| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0946 |
| --- | --- |

| Title: | CE1-related: Simplification of JVET-O0191 using 4 or 6 bit per initialization value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Two simplified variants of the context model initialization method of JVET-O0191 are proposed. The first one reduces the computational complexity by simplifying the computation of initialization values. The second variant reduces the number of bits required per initialization value from 6 to 4 bit. This allows to store the three initialization values and the 4 bit window size parameter in 16 bit per context model instead of 22 bit as necessary for the 6 bit variant. For the 6 bit variant, the luma BD rates over VTM-5.0 are 0.00% for AI, -0.01% for RA, 0.xx% for LB, 0.xx% for LP, 0.19% for RA-HighQP. For the 4 bit variant, the luma BD rates over VTM-5.0 are 0.07% for AI, 0.10 % for RA, 0.xx% for LB, 0.xx% for LP, and 0.48% for RA-HighQP.

# Proposed method

JVET-O0191 proposes the following context model initialization method:

q = Clip3(8, 24, qp >> 1) - 8

a = (initValue >> 3) * 9

b = (initValue & 7) * 9

s = 16 * a + (b - a) * q + 8

## Proposed variant 1 using 6 bits per initValue

In order to reduce the computational complexity, we propose the change the method of JVET-O0191 to:

q = Clip3(9, 23, qp >> 1) - 8

a = (initValue >> 3) * 9

b = (initValue & 7) * 9

s = 16 * a + (b - a) * q

The summation of 8 is removed from the computation of s. To compensate the effects, the clipping was narrowed from the interval [8,24] to [9,23]. Note that q only needs to be computed once for all context models whils s needs to be computed for each context model separately.

## Proposed variant 2 using 4 bits per initValue

In order to reduce number of bits required to store initValue, we propose the change the method of JVET-O0191 to:

q = Clip3(9, 23, qp >> 1) - 8

a = (initValue / 5 + 1)  << 4

b = (initValue % 5) << 4

s = 16 * a + (b - a) * q

Compared to variant 1, the computation of a and b is modified so that initValue is in the interval [0, 14]. Consequently, a can be one of the values {16, 32, 48} and b can be one of the values {0, 16, 32, 48, 64}.

Note that the integer division and modulo operation by 5 is only used to improve readability of the document. In hardware implementations, this can efficiently be implemented with a simple logic while in software, a small lookup table could be used. For the calculation of a, the LUT fits in a single 32 bit integer, while for b, it requires a 64 bit integer.

With variant 2, it is possible to store the three initialization values of a context model along with the window size parameter within a single 16 bit variable, while the 6 bit variant requires 22 bit.

# Experimental results

Initialization values for context variables have been derived using the same method as used for all CE1 tests (JVET-N10021). Therefore they can directly be compared with the CE1 test results.

Not all simulations are finished so far. Missing results will be updated as soon as possible.

## BD rates for the 6 bit variant using QP range 22 - 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,02% | -0,05% | 96% | 95% |
| Class A2 | 0,00% | -0,03% | -0,05% | 96% | 97% |
| Class B | 0,00% | -0,04% | -0,02% | 97% | 97% |
| Class C | -0,01% | -0,07% | 0,06% | 97% | 99% |
| Class E | -0,02% | -0,13% | -0,22% | 96% | 99% |
| Overall | 0,00% | -0,05% | -0,04% | 96% | 98% |
| Class D | 0,00% | 0,10% | -0,17% | 99% | 109% |
| Class F | -0,03% | 0,11% | 0,01% | 99% | 100% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,19% | 0,02% | 100% | 131% |
| Class A2 | 0,01% | 0,27% | -0,09% | 102% | 126% |
| Class B | 0,03% | 0,25% | -0,19% | 94% | 105% |
| Class C | -0,06% | 0,19% | -0,20% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | 0,23% | -0,13% | 97% | 111% |
| Class D | 0,02% | 0,55% | -0,18% | 95% | 102% |
| Class F | 0,02% | -0,02% | 0,14% | 95% | 99% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0,00% | 0,48% | -0,34% | 94% | 99% |
| Class E | 0,14% | -0,43% | -0,92% | 88% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,00% | 1,02% | -0,68% | 95% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low Delay P Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,08% | 0,16% | -0,20% | 94% | 97% |
| Class E | 0,07% | 0,08% | -1,15% | 85% | 92% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,05% | 0,81% | -0,75% | 94% | 99% |
| Class F | 0,00% | 0,99% | 0,37% | 93% | 95% |

## BD rates for the 6 bit variant using QP range 42 – 57 (high QP)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 highQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,19% | 1,60% | 0,79% | 101% | 105% |
| Class A2 | 0,29% | 1,36% | 0,34% | 100% | 102% |
| Class B | 0,18% | 1,32% | 0,34% | 99% | 103% |
| Class C | 0,11% | 1,06% | 0,89% | 100% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0,19% | 1,32% | 0,58% | 100% | 104% |
| Class D | 0,38% | 1,55% | 1,18% | 99% | 107% |
| Class F | 0,32% | 0,47% | 0,27% | 101% | 106% |

## BD rates for the 4 bit variant using QP range 22 – 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | -0,02% | -0,08% | 95% | 119% |
| Class A2 | 0,04% | -0,09% | -0,02% | 97% | 125% |
| Class B | 0,05% | -0,06% | 0,18% | 98% | 128% |
| Class C | 0,12% | 0,12% | 0,34% | 98% | 126% |
| Class E | 0,11% | 0,29% | 0,43% | 97% | 100% |
| Overall | 0,07% | 0,04% | 0,18% | 97% | 121% |
| Class D | 0,24% | 1,02% | 0,76% | 98% | 147% |
| Class F | 0,09% | 0,15% | 0,22% | 101% | 128% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,02% | 0,61% | 92% | 118% |
| Class A2 | 0,07% | 0,10% | 0,38% | 94% | 112% |
| Class B | 0,12% | 0,14% | 0,25% | 94% | 124% |
| Class C | 0,15% | 0,14% | 0,58% | 97% | 138% |
| Class E |  |  |  |  |  |
| Overall | 0,10% | 0,11% | 0,44% | 94% | 124% |
| Class D | 0,31% | 0,88% | 0,62% | 98% | 143% |
| Class F | 0,19% | 0,05% | 0,25% | 96% | 120% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0,33% | 0,80% | 0,98% | 98% | 102% |
| Class E | 0,59% | -1,22% | -0,20% | 87% | 92% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,57% | 0,38% | 0,76% | 98% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low Delay P Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0,26% | 0,24% | 1,07% | 98% | 107% |
| Class E | 0,54% | -1,10% | -0,36% | 87% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,56% | -0,21% | 0,03% | 95% | 98% |
| Class F | 0,37% | 0,13% | -0,07% | 92% | 89% |

## BD rates for the 4 bit variant using QP range 42 – 57 (high QP)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 highQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,23% | 1,48% | 1,10% | 101% | 122% |
| Class A2 | 0,24% | 0,81% | 0,72% | 98% | 123% |
| Class B | 0,54% | 1,35% | 1,84% | 98% | 111% |
| Class C | 0,76% | 1,57% | 2,03% | 104% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0,48% | 1,33% | 1,52% | 100% | 114% |
| Class D | 1,25% | 2,79% | 2,42% | 104% | 107% |
| Class F | 0,51% | 0,53% | 1,01% | 101% | 104% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).