| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0955 |
| --- | --- |

| Title: | Crosscheck of JVET-O0410 (CE6-related: Signalling of Low Frequency Non-Separable Transform) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Moonmo Koo 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This is cross-check report for JVET-O0410 (CE6-related: Signalling of Low Frequency Non-Separable Transform). The cross-check result match with what is provided by proponent in JVET-O0410.

Introduction

JVET-O0410 proposes the signalling of Low Frequency Non-Separable Transform (LFNST). In the current design, LFNST was determined according to the two types of the number of non-zero transform coefficients in a coding block. The proposed method removed two counters by using block size restriction and existing syntax elements, i.e., cbf, last_sig_coeff and coded_sub_block_flag. In addition, it is also proposed that the block size restriction is added to LFNST to meet VPDU constraints. If all of the following conditions are true, lfnst_idx is decoded.

- Both height and width of CU are equal or smaller than 64.

- Both last_sig_coeff and coded_sub_block_flag indicate no existence of non-zero coefficients in the zeroing area.

- cbf and last_sig_coeff satisfy one or more of the following conditions:

  - The block tree type is dual tree, cbf in luma block is not equal to 0 and last_sig_coeff of luma block indicates the non-DC position.

  - The block tree type is dual tree, one of the cbf in chroma block is not equal to 0 and last_sig_coeff of chroma blocks indicates the non-DC position.

  - The block tree type is single tree, one of the cbf in luma/chroma blocks and one of the last_sig_coeff in luma/chroma blocks indicates the non-DC position.

The proposed method is implemented on top of the VTM-5.0 and CE6-2.1a [2]. CE6-2.1a is the simplification proposal for LFNST by zeroing out all non-zero coefficients which are at the high-frequency area.

Code Analysis

The source code that proponent provided matches with what JVET-O0410 describes.

Experimental Results

The software package was obtained from the proponent. Code is compiled and tested following JVET common test condition [1], and compared with VTM 5.0 anchor. The available results match with what is provided by proponent.

On top of the VTM-5.0

On top of the CE6-2.1a

Reference

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19–27 March 2019.

- M. Siekmann et al., “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0094, 15th meeting, Gothenburg, SE, 3–12 July 2019.