| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0956 |
| --- | --- |

| Title: | Crosscheck of JVET-O0544 (Non-CE6: Modified LFNST Transform Matrices for Fast Implementation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Moonmo Koo 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This is cross-check report for JVET-O0544 (Non-CE6: Modified LFNST Transform Matrices for Fast Implementation). The cross-check result match with what is provided by proponent in JVET-O0544.

Introduction

JVET-O0544 proposes a slight modification to the LFNST matrices so that faster implementation can be achieved. Similar to the ordinary transform, the proposed modification allows dual implementation support, in which an LFNST transform can be either implemented by matrix multiplication or a fast algorithm to yield exactly the same results.

As the example introduced in JVET-O0544, let us consider computing the first coefficient (C) of the inverse LFNST transform of size 16x16 of the first set and first basis function,

C = LFNST_V * In

By slightly modifying LFNST_V to:

LFNST_V = [108   -40    26   -32     8   -25     8     2   -16    -9   -13     8    -2     4     2     0];

The coefficient can be computed as:

C=-13*(-In[0]*(28)+(In[1]<<3)-(In[2]<<1)+In[5]+In[9]+In[10])-(In[0]<<8)+(In[1]<<6)-(In[3]<<5)+(In[4]<<3)-In[5]*(12)+(In[6]<<3)+(In[7]<<1)-(In[8]<<4)+(In[9]<<2)+(In[11]<<3)-(In[12]<<1)+(In[13]<<2)+(In[14]<<1);

This, in turn, requires only 3 multiplications rather than 16 multiplications needed by a matrix multiply. Similarly, by modifying the vector to:

LFNST_V = [108   -40    26   -32     8   -27     8     2   -16    -9   -13     8    -2     4     1     0];

The coefficient can be computed as:

Out[0] =  (-13 * (-(In[0] << 3) + (In[1] << 3) - (In[2] << 1) + (In[5] << 1) + In[9] + In[10]) + (In[0] << 2) + (In[1] << 6) - (In[3] << 5) + (In[4] << 3) - In[5] + (In[6] << 3) + (In[7] << 1) - (In[8] << 4) + (In[9] << 2) + (In[11] << 3) - (In[12] << 1) + (In[13] << 2) + In[14]);

This requires single multiplication instead of 16.

The same procedure is done for the all the transform matrices to yield new matrices whose fast implementation requires much fewer operations.

Specifically, JVET-O0544 studies two variants with different degrees of changes with respect to the original matrices, of which cross-check results are presented in this contribution. In the first variant (Test 1), the maximum absolute difference (MAD) between the new and original LFNST matrices is less or equal to 2, while the second (Test 2) is less or equal to 4.

Code Analysis

The source code that proponent provided matches with what JVET-O0544 describes.

Experimental Results

The software package was obtained from the proponent. Code is compiled and tested following JVET common test condition [1], and compared with VTM 5.0 anchor. The available BD-rate results and most of run-time ones match with what is provided by proponent. However, RA encoding/decoding time results of Class A1/A2 for Test 1 in JVET-O0544 appear to be a little more deviated from 100%, which is not thought to be logically acceptable. Regarding this, the proponent said that the unexpected time numbers were caused by the fact that the relevant jobs were run on different clusters. Most of all, the inconsistent run times are not observed in the following cross-check results, which were generated from homogeneous cluster server. Therefore, it is confirmed that SW for JVET-O0544 does not have any run-time issues.

Test 1

Results on CTC

Results on low QP condition

Test 2

Results on CTC

Results on low QP condition

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010, March 2019.