| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0969-v1 |
| --- | --- |

| Title: | Crosscheck of method 2 in JVET-O0205 (CE8-related: Improvements on BDPCM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 man-shu.chiang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking method 2 in JVET-O0205 proposed by LGE. Method 2 in JVET-O0205 proposes line-shaped coefficient group (CG) for the block-wise differential pulse code modulation (BDPCM) based on setting the intra prediction mode for the BDPCM with the one used for the actual prediction. The simulation conducted by the crosschecker was partially done, and the partial results show that the the BD-rate results match exactly with those in JVET-O0205.

# Verification

LGE provided the source code of their proposed method described in JVET-O0205 [1]. One verification test was carried out. The verification test is for the proposed method 2 in JVET-O0205.

The test was conducted under the common test conditions (CTC) defined in [2] using the VTM5.0 software [3]. Table 1 shows the results of using VTM5.0 as the anchor and enabling the proposed method 2 in JVET-O0205. The BD-rates from our experimental results match exactly those in JVET-O0205. Please note that the runtime comparison may be not accurate.

Table 1. The results using VTM5.0 as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 146% | 124% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 149% | 125% |
| Class C | 0.00% | 0.00% | 0.00% | 148% | 128% |
| Class E | 0.00% | 0.00% | 0.00% | 148% | 125% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 147% | 129% |
| Class F | -0.18% | -0.11% | -0.08% | 148% | 126% |
| Class SCC | -0.28% | -0.14% | -0.15% | 148% | 123% |

# References

[1] S. Yoo, J. Choi, J. Nam, J. Heo, J. Lim, S. Kim, “CE8-related: Improvements on BDPCM,” Document of Joint Video Experts Team, JVET-O0205.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).