| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0975 |
| --- | --- |

| Title: | Crosscheck of JVET-O0634 (Non-CE9: BDOF simplifications) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomohiro Ikai | Tel: Email: | zhou.tianyang@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-O0634 (Non-CE9: BDOF simplifications). Proposed method in JVET-O0634 uses conditions on BDOF as follow:

1. Not applied in 8x8 CU case.

2. Not applied in 8x8/4xN CUs case.

3. Turn BDOF off when DMVR is activated for small blocks.

The crosscheck was carried out under the JVET common test condition. Experimental results (partial) match with those provided by the proponent.

Experimental Results

The software on top of VTM-5.0 was provided by the proponents [1]. Crosscheck experiments were performed under JVET common test conditions [2].

Following tables show the results of the crosscheck experiments under VTM-5.0 configuration.

- Performance results of Test1 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | -0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | 0.02% | 100% | 100% |
| Class B | 0.01% | 0.03% | -0.04% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Class D | 0.08% | 0.05% | 0.16% | 100% | 100% |
| Class F (optional) | 0.01% | 0.05% | 0.02% | 100% | 100% |

- Performance results of Test2 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.05% | 100% | 102% |
| Class A2 | 0.04% | -0.02% | -0.07% | 100% | 102% |
| Class B | 0.03% | 0.00% | 0.02% | 100% | 100% |
| Class C | 0.08% | 0.04% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.04% | 100% | 101% |
| Class D | 0.19% | -0.01% | 0.12% | 100% | 100% |
| Class F (optional) | 0.02% | 0.02% | 0.00% | 100% | 100% |

- Performance results of Test3 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | -0.01% | 100% | 102% |
| Class A2 | 0.01% | -0.01% | 0.00% | 101% | 102% |
| Class B | 0.02% | 0.02% | 0.01% | 100% | 100% |
| Class C | 0.02% | 0.01% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.01% | 100% | 101% |
| Class D | 0.09% | 0.07% | 0.10% | 100% | 99% |
| Class F (optional) | 0.02% | 0.04% | 0.02% | 100% | 100% |

Conclusion

It was confirmed that the simulation results (partial) of bitrate and PSNR match with those provided by the proponent.

Reference

- X. Chen, H. Yang, “Non-CE9: BDOF simplifications,” JVET-O0634, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.