| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0976 |
| --- | --- |

| Title: | Wavefront-based GRA and Related Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang and Miska M. Hannuksela | Tel: Email: | +1 858-335-9196 limin.2.wang@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution contains the following proposed items:

- It is proposed to indicate virtual boundary type in PPS. Type 0 is the same as currently in VVC Draft 5, and types 1 to 3 are respectively specified for vertical, horizontal, and wavefront-based virtual boundaries specified with syntax elements in the slice header. Types 1 to 3 are asserted to avoid the need for updating PPSs for turning off loop filtering.

- The contribution enables wavefront-based GRA (Gradual Random Access) and proposes related syntax to turn off loop filtering across a wavefront boundary. A GRA picture, in general, consists of three areas; clean, intra coded and dirty. With the wavefront-based GRA, all the reference pixels for the blocks in intra coded area are in clean and/or intra coded area. Hence, no restriction needs to be imposed on intra prediction for the blocks in the intra coded area.

# Introduction

## Vertical and horizontal GRA and related intra prediction constraints

Gradual Random Access (GRA) was introduced for low delay applications and discussed in several previous JVET meetings [1-7]. A Core Experiment (CE11) was formed for specific GRA test conditions at the 14th JVET meeting, Geneva, March 2019.

A GRA picture in general consists of three areas. They are clean area, intra coded area and dirty area. Two commonly-referred GRA types are vertical GRA and horizontal GRA. Fig. 1 shows an example of vertical GRA, where intra coded area (green) are spread over several pictures and a clean (yellow) area is gradually expanded vertically from the left to the right. Similarly, Fig. 2 illustrates the basic concept of horizontal GRA, where intra coded areas (green) are spread over several pictures and a clean (yellow) area is gradually expanded horizontally from the top to the bottom.

Fig. 1. Vertical GRA.

Fig. 2. Horizontal GRA

In both vertical and horizontal GRA, the reference pixels for CUs in the intra coded area may be in the dirty area, and hence, some restrictions on intra prediction need to be imposed, which likely affect the coding performance.

## Using virtual boundaries to indicate boundaries to turn off loop filtering

In HEVC, when exact_match_flag is equal to 1, it is required that the decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the recovery point SEI message shall be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous IRAP access unit, if any, in the bitstream.

If the same “exact match” is required, some restrictions have to be imposed for GRA. In general, only the coding information (such as reconstructed pixels, coding modes, etc.) associated with the clean areas and intra coded areas can be used for decoding the pictures between the random access point and the recovery point. For example, if in inter mode, the blocks in the clean area can only temporally refer to the clean areas in the previous pictures. If in intra mode, only the intra prediction modes that do not use the reference pixels in the dirty area can be employed to the blocks in intra coded area. The in-loop filtering (deblocking, SAO, ALF) can only be performed for the blocks in the clean area, but not for the boundary pixels next to the dirty areas.

In order to get “exact match”, both encoder and decoder need to switch off the loop filtering across the boundary between the dirty area and the intra/clean area.

The PPS syntax allows signalling of virtual boundaries across which the loop filtering is turned off. The virtual boundaries were motivated by 360° use cases, where filtering across projection faces is not desirable. It is possible to send a PPS with each picture between the random access point and the recovery point to update the virtual boundaries of PPS to turn off looping at a correct boundary for GRA. However, in our opinion, it is wasteful to send an entire PPS when only the virtual boundary positions are updated.

# Proposal

## Encoding for wavefront-based GRA

This contribution proposes a wavefront-based GRA. Wavefront GRA spreads intra coded areas (green) over pictures and gradually expands a clean (yellow) area diagonally from the top-left corner to the bottom-right corner, as shown in Fig. 3. Assume POC(n) is a random access point and POC(n+N-1) is a recovery point. At POC(n), the top-left corner area (green) is coded in intra mode. At POC(n+1), the intra coded area at POC(n) becomes the clean area (yellow), and the wavefront area (green) next to the clean area is coded in intra mode. At POC(n+2), the clean area and the intra coded areas at POC(n+1) becomes the clean area (yellow), and the wavefront area (green) next to the clean area is coded in intra mode. The process continues, and the clean area is gradually expanded. At POC(n+N-1), the clean area and the intra coded area at POC(n+N-2) becomes the clean area (yellow), and the bottom-right corner area (green) next to the clean area is coded in intra mode. Ideally, the numbers of intra coded blocks in intra coded areas in the N pictures should be similar, if not the same.

Fig. 3. Wavefront GRA, where the clean is in yellow, intra coded area in green and dirty area in blank.

One asserted advantage with wavefront GRA is all the information required for decoding the intra coded blocks in the wavefront area are in clean area and/or intra coded area. Fig. 4 shows an example of wavefront GRA slice/picture, where the blocks in yellow are in clean area, the blocks in green are in intra coded area, and the blocks in blank are in dirty area. As seen, for blocks in intra coded area, their left, above, above-left and above-right neighboring blocks are in either clean area or intra coded area. For example, for block 19 in intra coded area, its neighboring blocks 11 (left), 10 (above), and 2 (above-left ) are in clean area, and 18 (above-right) is in intra coded area. Another asserted advantage with wavefront GRA is that it can be used together with wavefront parallel processing to achieve parallel decoding.

Fig. 4. Wavefront GRA, where clean area is in yellow, intra coded area in green and dirty area in blank.

## Virtual boundaries

The related syntax is proposed in PPS and slice header, as shown in the following PPS syntax table and slice header syntax table.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_virtual_boundary_type | ue(v) |
| if( pps_virtual_boundary_type = = 0 ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } else if( pps_virtual_boundary_type >= 1 && pps_virtual_boundary_type <= 3 ) |  |
| pps_virtual_boundary_granularity | ue(v) |
| } |  |
| … | u(1) |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_virtual_boundary_type >= 1 && pps_virtual_boundary_type <= 3 ) { |  |
| num_virtual_boundaries | u(2) |
| for( i = 0; i < num_virtual_boundaries; i++ ) { |  |
| virtual_boundary_pos[ i ] | u(v) |
| if( pps_virtual_boundary_type = = 3 ) |  |
| virtual_boundary_len[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

pps_virtual_boundary_type equal to 0 specifies that the virtual boundaries across which loop filtering is turned off are specified with ( pps_num_ver_virtual_boundaries + 1 + pps_num_hor_virtual_boundaries + 1 ) syntax elements in the PPS. pps_virtual_boundary_type in the range of 1 to 3, inclusive, specify that the virtual boundaries are specified by pps_virtual_boundary_granularity, virtual_boundary_pos[ i ], and virtual_boundary_len[ i ].

NOTE: pps_virtual_boundary_type equal to 1, 2, 3 are suitable for vertical, horizontal, and diagonal gradual random access, respectively.

pps_virtual_boundary_granularity specifies the unit in which virtual_boundary_address and virtual_boundary_length are expressed. pps_virtual_boundary_granularity shall be in the range of 0 to 4, inclusive.

num_virtual_boundaries specifies the number of virtual boundaries across which loop filtering is turned off. The value of num_virtual_boundaries, when present, shall be the same in all slice headers of the same coded picture.

virtual_boundary_pos[ i ] specifies the start address of the virtual boundary across which loop filtering is turned off. When pps_virtual_boundary_type is equal to 1, PpsVirtualBoundariesPosX[ i ] is set equal to virtual_boundary_pos[ i ] * 2( pps_virtual_boundary_granularity+ 3 ), pps_num_ver_virtual_boundaries is inferred to be equal to num_virtual_boundaries, and pps_num_hor_virtual_boundaries is inferred to be equal to 0. When pps_virtual_boundary_type is equal to 2, PpsVirtualBoundariesPosX[ i ] is set equal to virtual_boundary_pos[ i ] * 2( pps_virtual_boundary_granularity+ 3 ), pps_num_hor_virtual_boundaries is inferred to be equal to num_virtual_boundaries, and pps_num_ver_virtual_boundaries is inferred to be equal to 0. The value of virtual_boundary_pos[ i ], when present, shall be the same with the same value of i in all slice headers of the same coded picture.

When pps_virtual_boundary_type is equal to 1, the number of bits to represent virtual_boundary_pos[ i ] is Ceil( Log2( pic_width_in_luma_samples ) – ( 3 + pps_virtual_boundary_granularity ) ). When pps_virtual_boundary_type is equal to 2, the number of bits to represent virtual_boundary_pos[ i ] is Ceil( Log2( pic_height_in_luma_samples ) – ( 3 + pps_virtual_boundary_granularity ) ). When pps_virtual_boundary_type is equal to 3, the number of bits to represent virtual_boundary_pos[ i ] is Ceil( Log2( pic_width_in_luma_samples * pic_height_in_luma_samples ÷ 2( pps_virtual_boundary_granularity +3 ) * 2 ) ).

virtual_boundary_len[ i ] specifies the length of the virtual boundary across which loop filtering is turned off. The value of virtual_boundary_len[ i ], when present, shall be the same with the same value of i in all slice headers of the same coded picture.

For example, in Fig. 4, the coordinator of the first intra GRA block of the intra coded area is (9, 0). Hence, virtual_boundary_pos[ i ] could be (9, 0) or (9 + GRABlockWidth * 0). There are 8 intra GRA blocks in intra coded area. Therefore, virtual_boundary_len[ i ] is equal to 8.

# Conclusions

This contribution proposes a wavefront-based GRA method. As compared with vertical and horizontal GRA, wavefront-based GRA does not impose any restrictions on intra predictions for the blocks in intra coded area. The related syntax in PPS and slice header helps to define the virtual boundaries of the three areas of a current GRA slice/picture.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2] K.Kazui, “AHG14: Study of methods for progressive intra refresh”, JVET-L0079, October 2018.

[3] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

[4] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Normative Intra Refresh Proposal”, JVET-L0161, October 2018.

[5] K.Kazui, “Revised software for ultra low-latency encoding”, JVET-N0080, March 2019.

[6] J.-M. Thiesse, D.Gommelet, D.Nicholson “AHG14: Updates on Intra Refresh Proposal”, JVET-N0391, March 2019.

[7] K. Kazui, J.M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 11 (CE11): Gradual random access”, JVET-N1031, March 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).