| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0977 |
| --- | --- |

| Title: | Crosscheck of JVET-O0209 (Non-CE6: LFNST simplification based on the methods proposed in CE6-2.1a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check reports of Test 1 and Test 2 in JVET-O0209. It is confirmed that the cross-check results are identical with the proponent’s results.

Summary of Test 1 and Test 2 of Proposal JVET-O0209

JVET-O0209 [1] tests the several simplifications for Low Frequency Non-Separable Transform (LFSNT) based on CE6-2.1 [2] and in this contribution Test 1 and Test 2 are crosschecked as follows:

Test 1: One simplification, i.e., (a) Max. 16 LFNST coefficients, is included. In this simplification, LFNST be applied to only one top-left sub-block, from which the number of LFNST coefficients cannot exceed 16, whereas 4x4 LFNST can be applied to each of two top-left adjacent 4x4 blocks in 4xN or Nx4 TU (N≥16), which ends up generating 32 LFNST coefficients in forward direction in VTM 5.0 design.

Test 2: Two simplifications, i.e., (c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied and (d) Change of the region where non-zero coefficients, are counted are included. Former simplification proposes additional zero-out of primary transform coefficients when 8x8 LFNST is applied. The latter simplification proposes modification of signaling an LFSNT index by checking the existence of non-zero coefficients by checking that last scanning position of last non-zero coefficient exceeds maximum allowable position of LFNST coefficients.

Source Code Analysis

The source code provided by the proponents was carefully analysed. It is confirmed that the implementation matches the description in JVET-O0209.

Simulation results

Simulations were conducted for all configurations and test sequences specified in the CTC [3]. The cross-check results are provided in the accompanying Excel documents. The cross checker could exactly reproduce the results provided by the proponent and reported in JVET-O0209. The measured encoder and decoder run time ratios are similar to those reported in JVET-L0209.

References

- M. Koo, J. Nam, J. Lim, M. Salehifar, S. Kim, “Non-CE6: LFNST simplification based on the methods proposed in CE6-2.1a”, JVET-O0209, Gothenburg, SE, July 2019

- M. Siekmann et al., “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0094, 15th meeting, Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.