| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0993 |
| --- | --- |

| Title: | Cross-check of JVET-O0568 (Non-CE8: An Alternative VPDU Memory Reuse for IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jingya Li Haiwei Sun Che-Wei Kuo Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com haiwei.sun@sg.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |  |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-O0568 proposed by Bytedance.

JVET-O0568 presents an alternative way to reuse on-chip VPDU memory for IBC when CTU size is 128x128. When CTU size is 128x128, two most recently coded VPDUs in the even-number VPDU row and one most recently coded VPDU in the odd-number VPDU row are kept as reference for IBC.

The verification task was done, and the BD-rate results match exactly with those in JVET-O0568 except one kbps value. Software implementation matches the proposal of the contribution.

# Verification

Bytedance provided the source code of their proposed method described in JVET-O0568 [1]. The provided software implementation matches that in the contribution.

Tests are conducted using the CE8 test conditions defined in [2] and are carried out by using the VTM5.0 software [3]. The VTM5.0 is used as the anchor.

The kbps from coding DaylightRoad2 at RA settings using QP27 is slightly different between the contribution and ours. Besides this test point, the BD-rates of the test from our experimental results match exactly those in JVET-O0568.

Table 1: Summary of the average BD bitrate results using the proposed method

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.02% | 0.07% | 140% | 100% |
| Class A2 | -0.78% | -1.00% | -0.97% | 147% | 100% |
| Class B | -0.78% | -0.77% | -0.78% | 156% | 100% |
| Class C | -0.30% | -0.34% | -0.21% | 156% | 99% |
| Class E | -1.29% | -1.22% | -1.43% | 160% | 101% |
| Overall | -0.64% | -0.66% | -0.65% | 152% | 100% |
| Class D | -0.55% | -0.56% | -0.61% | 149% | 98% |
| Class F | -0.78% | -0.71% | -0.75% | 101% | 99% |
| TGM | -1.82% | -1.85% | -1.87% | 100% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.23% | 0.14% | 106% | 99% |
| Class A2 | -0.06% | -0.41% | -0.05% | 106% | 99% |
| Class B | -0.13% | -0.48% | -0.63% | 109% | 100% |
| Class C | 0.15% | -0.33% | -0.12% | 114% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.38% | -0.22% | 109% | 100% |
| Class D | 0.10% | -0.23% | -0.06% | 114% | 98% |
| Class F | -0.55% | -0.68% | -0.58% | 101% | 100% |
| TGM | -1.09% | -1.14% | -1.15% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.35% | -0.11% | 119% | 97% |
| Class C | 0.08% | 0.25% | -0.33% | 125% | 100% |
| Class E | 0.03% | -0.27% | 0.62% | 113% | 99% |
| Overall | 0.05% | 0.16% | 0.00% | 119% | 99% |
| Class D | 0.08% | 0.23% | 1.17% | 126% | 99% |
| Class F | -0.32% | -0.25% | -0.68% | 100% | 98% |
| TGM | -0.71% | -0.86% | -0.85% | 101% | 99% |

# References

- Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, and Yue Wang “Non-CE8: An Alternative VPDU Memory Reuse for IBC”, Document of Joint Video Experts Team, JVET-O0568.

- X. Xu, Y.-H. Chao, Y.-C. Sun, and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, 14th Meeting: Geneva, CH, 19–27 March 2019.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM5.0.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).