| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1007 |
| --- | --- |

| Title: | Crosscheck of JVET-O0321 (Non-CE3: Removal of MIP mapping table) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jangwon Choi 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 jangwon84.choi@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This is cross-check report for JVET-O0321 (Non-CE3: Removal of MIP mapping table). The cross-check result match with what is provided by proponent in JVET-O0321.

Introduction

JVET-O0321 proposes to remove mapping table and replace them with a fixed mode index. Two tests were conducted:

1) Test 1: Replace the MIP-to-conventional mapping table with a fixed mode index PLANAR_IDX.

2) Test 2: Replace the conventional-to-MIP mapping table with a fixed mode index 0.

Experimental Results

Code is compiled and tested following JVET common test condition [1], and compared with VTM5 anchor. Results match with what is provided by proponent. Note that the results of encoding and decoding time have some fluctuation, because of the unstable cluster server.

Table 1 Result of Test 1 of JVET-O0321

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 102% | 106% |
| Class A2 | 0.03% | 0.02% | -0.02% | 102% | 103% |
| Class B | 0.01% | -0.04% | 0.09% | 100% | 99% |
| Class C | 0.00% | -0.12% | 0.05% | 97% | 100% |
| Class E | 0.03% | 0.01% | 0.03% | 103% | 103% |
| Overall | 0.01% | -0.02% | 0.04% | 100% | 102% |
| Class D | -0.03% | 0.19% | 0.00% | 100% | 102% |
| Class F | -0.02% | -0.02% | 0.08% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% | 106% | 106% |
| Class A2 | 0.07% | 0.22% | 0.06% | 104% | 101% |
| Class B | 0.01% | 0.04% | 0.01% | 106% | 100% |
| Class C | -0.02% | 0.15% | 0.01% | 107% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 106% | 101% |
| Class D | 0.00% | 0.07% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.13% | 0.17% | 101% | 100% |

Table 2 Result of Test 2 of JVET-O0321

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 104% | 106% |
| Class A2 | 0.00% | -0.01% | -0.02% | 99% | 103% |
| Class B | 0.01% | -0.04% | -0.01% | 102% | 101% |
| Class C | 0.01% | -0.09% | -0.11% | 98% | 99% |
| Class E | 0.00% | -0.04% | 0.10% | 102% | 101% |
| Overall | 0.00% | -0.04% | -0.01% | 101% | 102% |
| Class D | 0.00% | 0.14% | -0.03% | 100% | 101% |
| Class F | -0.01% | 0.00% | 0.05% | 101% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.13% | 102% | 102% |
| Class A2 | 0.02% | -0.07% | -0.01% | 108% | 104% |
| Class B | 0.02% | -0.02% | -0.14% | 105% | 99% |
| Class C | -0.03% | -0.19% | 0.00% | 107% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.02% | 106% | 101% |
| Class D | 0.01% | -0.20% | 0.15% | 100% | 99% |
| Class F | 0.03% | -0.06% | 0.02% | 100% | 100% |

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.