| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1016 |
| --- | --- |

| Title: | Non-CE4 : Simplified Triangle signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. Le Léannec, F. Galpin | Email: | antoine.robert@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

To simplify the TPM signaling, this contribution proposes to replace one of the indices by a flag specifying which single triangle merge index is signaled. Then the second index is derived from the signaled one.

The proposed simplification decreases the BD-rate performances by 0.09% in RA and by 0.15% in LDB. But the encoding complexity is also decreased by 4% in RA and 5% in LDB while decoding complexity is identical to VTM-5.0. Compared with AHG13 results, the proposed simplification can keep nearly 75% of the TPM gains in RA and more than 82% in LDB while reducing by half the encoding complexity.

# Introduction

The Triangle Partition Mode is the last merge mode that needs to signal one flag for the splitting direction as well as the two triangle merge indices, one for each partition. This signaling is costly compared to a regular merge signaling.

# Description

## Triangle partition in the VTM-5.0

In VTM5, a Triangle Partition Mode (TPM) is supported for inter prediction. It is only applied to CUs that are 8x8 or larger and is derived at CU-level as the remaining merge mode after the other merge modes including the regular merge mode, the MMVD mode, the subblock merge mode and the CIIP mode.

When this mode is used, a CU is split evenly into two triangle-shaped partitions, using either the diagonal split or the anti-diagonal split (Figure 1). Each triangle partition in the CU is inter-predicted using its own motion; only uni-prediction is allowed for each partition, that is, each partition has one motion vector and one reference index. The uni-prediction motion constraint is applied to ensure that same as the conventional bi-prediction, only two motion compensated predictions are needed for each CU.

Figure 1: Triangle partition based inter prediction

There are up to 5 uni-prediction candidates and an encoder tests all the combinations of candidates (one for each partition) in the 2 splitting directions. Therefore, the maximum number of tested combinations is 40 (5*4*2) with the actual CTC [1] (where MaxNumTriangleMergeCand = 5 and nb_combinations = MaxNumTriangleMergeCand * (MaxNumTriangleMergeCand - 1) * 2).

If TPM is used for a current CU, then a flag indicating the splitting direction (diagonal or anti-diagonal), and two merge indices (one for each partition) are signaled.

## Proposed simplified triangle signaling

In order to reduce the signaling of the TPM, it is proposed to replace one of the signaled indices by a flag indicating which index idxx is signaled. Then a derivation process of the other index idx1-x from idxx is also proposed. In that case, only one index and a flag have to be signaled.

The signaled flag, denoted merge_triangle_idx_num, indicates the number (x) of the following signaled index idxx. If idx0 is signaled, then idx1 is derived from idx0, otherwise this is idx0 that is derived from idx1. This flag is context based CABAC coded.

The proposed derivation process of the indices is as follows and as described in Table 1:

- If merge_triangle_idx_num is 0 (i.e. idx0 is signaled)

  - If idx0 is 0, then idx1 is set to 1,

  - and if idx0 is greater than 0, then idx1 is set to 0.

- If merge_triangle_idx_num is 1 (i.e. idx1 is signaled)

  - idx0 is set to 0 and idx1 is set to idx1 + 2.

Table 1: Mapping between idx0 and idx1
(red X: idx1 from signaled idx0, blue O: idx0 from signaled idx1)

|  |  | idx0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 |
| idx1 | 0 | / | XO | X | X | X |
|  | 1 | XO | / |  |  |  |
|  | 2 | O |  | / |  |  |
|  | 3 | O |  |  | / |  |
|  | 4 | O |  |  |  | / |

The same derivation process is applied whether it is idx0 or idx1 that is signaled. In that case, two indices pairs are in common ((0,1) and (1,0)), when idx1 is signaled, it is necessarily greater than 1. It is thus possible to signal idx1 - 2 instead of idx1.

Furthermore, by deriving one index idxx from the other, this reduces the maximum number of tested combinations at the encoder. It goes from 40 combinations (5*4*2, see 2.1) to only 16 ((5+3)*2 = (MaxNumTriangleMergeCand+(MaxNumTriangleMergeCand-2))*2). It reduces the number of tested combinations to less than half, thus the encoder complexity.

The VVC specification draft 5.0 [2] where the triangle partition signaling is involved becomes (highlighted in blue for the removals and in green for the additions):

#### 7.3.7.7  Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| … | … |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx_num[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.8.7  Merge data semantics

…

merge_triangle_split_dir[ x0 ][ y0 ] specifies the splitting direction of merge triangle mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_split_dir[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_triangle_idx_num[ x0 ][ y0 ] specifies which merge_triangle_idx is to be decoded. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx_num[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_triangle_idx[ x0 ][ y0 ] specifies the (merge_triangle_idx_num+1)-th merging candidate index of the triangular shape based motion compensation candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_triangle_idx0[ x0 ][ y0 ] specifies the first merging candidate index of the triangular shape based motion compensation candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx0[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_triangle_idx1[ x0 ][ y0 ] specifies the second merging candidate index of the triangular shape based motion compensation candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx1[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

…

#### 8.5.4.2  Derivation process for luma motion vectors for merge triangle mode

…

The motion vectors mvA and mvB, the reference indices refIdxA and refIdxB and the prediction list flags predListFlagA and predListFlagB are derived by the following ordered steps:

- The derivation process for luma motion vectors for merge mode as specified in clause 8.5.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the bi-prediction weight index bcwIdx and the merging candidate list mergeCandList.

- The variables m and n, being the merge index for triangle partition 0 and 1 resepctively, are derived using merge_triangle_idx_num[ xCb ][ yCb ] and merge_triangle_idx[ xCb ][ yCb ] merge_triangle_idx0[ xCb ][ yCb ] and merge_triangle_idx1[ xCb ][ yCb ] as follows:

- if ( merge_triangle_idx_num[ xCb ][ yCb ] == 1 )

m = 0			(8-447)

n = merge_triangle_idx[ xCb ][ yCb ] + 2	(8-448)

- otherwise

m = merge_triangle_idx[ xCb ][ yCb ]	(8-449)

n = ( m > 0 ) ? 0 : 1	(8-450)

m = merge_triangle_idx0[ xCb ][ yCb ]	(8-447)

n = merge_triangle_idx1[ xCb ][ yCb ] + ( merge_triangle_idx1[ xCb ][ yCb ] >= m ) ? 1 : 0	(8-448)

- …

| Table 9-11 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| merge_data( ) | … | … | … |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_split_dir[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx_num[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | TR | cMax = (merge_triangle_idx_num[ ][ ]) == 1 ? MaxNumTriangleMergeCand – 3 : MaxNumTriangleMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_idx0[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 2, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_split_dir[ ][ ] | bypass | na | na | na | na | na |
| merge_triangle_idx_num[ ][ ] | 0 | na | na | na | na | na |
| merge_triangle_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx0[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx1[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| … | … | … | … | … | … | … |

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-N1010 [1]. VTM-5.0 reference software is used with VTM configuration files.

Table 4 and Table 5 show the simulation results of the proposed simplified triangle signaling in random access and low delay B configurations. The proposed simplified triangle signaling decreases the BD-rate performances in random access by 0.09% and in low delay B by 0.15%. But encoding time is reduced by 4% in random access and by 5% in low delay B with the same decoding time as VTM-5.0.

Table 4: Simulation results of the simplified triangle signaling in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.09% | 0.22% | 96% | 100% |
| Class A2 | 0.08% | 0.13% | 0.20% | 96% | 100% |
| Class B | 0.07% | 0.08% | 0.07% | 96% | 100% |
| Class C | 0.12% | 0.11% | 0.08% | 96% | 101% |
| Overall | 0.09% | 0.06% | 0.13% | 96% | 100% |
| Class D | 0.11% | 0.05% | 0.12% | 96% | 100% |
| Class F (optional) | 0.06% | 0.13% | 0.11% | 97% | 100% |

Table 5: Simulation results of the simplified triangle signaling in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.11% | 0.38% | -0.01% | 96% | 99% |
| Class C | 0.16% | 0.49% | 0.20% | 96% | 101% |
| Class E | 0.19% | 0.12% | -0.57% | 93% | 100% |
| Overall | 0.15% | 0.35% | -0.08% | 95% | 100% |
| Class D | 0.22% | 0.37% | 0.76% | 95% | 100% |
| Class F (optional) | 0.00% | 0.19% | 0.41% | 96% | 100% |

In the report of the AdHoc Group 13 [3], the triangle partition mode performances are reported to be:

Table 6: AHG13 results of the TPM tool off test

|  | TPM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| RA | 0.35% | 0.63% | 0.67% | 92% | 100% | 92% | 99% |
| LDB | 0.88% | 1.34% | 1.02% | 87% | 101% | 88% | 103% |

The proposed method loses 0.09% in random access while saving 4% encoding time. It means that nearly 75% of the TPM gains are kept while the encoding time is reduced by 50%. In low delay B, with the proposed signaling, loses are of 0.15% with 5% encoding time saving. Again, more than 82% of the gains are preserved while encoding time is reduced by nearly 40%.

# Conclusion

To simplify the TPM signaling, this contribution proposes to replace one of the indices by a flag specifying which single triangle merge index is signaled. Then the second index is derived from the signaled one.

The proposed simplification decreases the BD-rate performances by 0.09% in RA and by 0.15% in LDB. But the encoding complexity is decreased by 4% in RA and 5% in LDB while decoding complexity is identical to VTM-5.0. Compared with AHG13 results, the proposed simplification can keep nearly 75% of the TPM gains in RA and more than 82% in LDB while reducing by half the encoding complexity.

# References

[1]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1010, 14th Meeting, Geneva, CH, March 2019.

[2]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1001, 14th Meeting, Geneva, CH, March 2019.

[3]	W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, S. Liu, D. Luo, “JVET AHG report: Tool reporting procedure (AHG13)”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O0013, 15th Meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).