| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1030-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0230 (Non-CE3: Interpolation filtering for intra prediction in non-diagonal directions) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Sung-Chang Lim Hahyun Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of interpolation filtering for intra prediction in non-diagonal directions proposed in JVET-O0230. In the first method replaces 4-tap cubic and Gaussian filter for luma, and bilinear filter for chroma by 4-tap piece-wise linear approximation of a second order polynomial. Additionally, in the second method, the piece-wise linear approximation interpolation or bilinear interpolation is selected based on comparison of two reference sample values difference and a threshold value. It is reported that the average luma BD-rates of the first and second method in AI are -0.01% and -0.09% compared to VTM 5.0, respectively.

- Introduction

In VTM 5.0, 4-tap cubic and Gaussian intra interpolation filters are used for luma angular prediction. One of those filters is selected by MDIS condition which is based on block size and intra prediction mode. For chroma angular prediction, bilinear interpolation filter is use.

The first proposed method in JVET-O0230 removes the cubic and Gaussian interpolation filters, and uses piece-wise linear approximation interpolation of a second order polynomial for both luma and chroma. In addition, [1 2 1] low pass filter is applied to two reference samples before performing interpolation when the MDIS condition is satisfied.

In the second proposed method, the piece-wise linear interpolation or bilinear interpolation is selected by comparing an absolute difference value of two reference samples and a predefined threshold value. When the difference value is less than or equal to the threshold value bilinear filter is applied. The threshold value is 16 and 8 for luma and chroma, respectively.

Experimental results

Experiments were performed based on VTM 5.0 software under JVET common test condition.

Table 1 and Table 2 show experimental results of the first and second proposed methods compared to VTM 5.0, respectively. All the results are matched with the ones in JVET-O0230.

Table 1. Results of the first proposed method over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.00% | 0.02% | 101% | 100% |
| Class A2 | 0.04% | -0.21% | -0.18% | 99% | 100% |
| Class B | 0.03% | -0.31% | -0.24% | 101% | 100% |
| Class C | -0.21% | -0.51% | -0.50% | 101% | 100% |
| Class E | 0.02% | -0.07% | -0.12% | 100% | 101% |
| Overall | -0.01% | -0.25% | -0.23% | 100% | 100% |
| Class D | -0.13% | -0.05% | -0.41% | 100% | 100% |
| Class F | -0.06% | -0.37% | -0.29% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.34% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | -0.17% | -0.30% | 100% | 100% |
| Class B | 0.03% | -0.62% | -0.69% | 100% | 100% |
| Class C | -0.08% | -0.58% | -0.76% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.46% | -0.49% | 100% | 100% |
| Class D | -0.06% | -0.24% | -0.19% | 101% | 100% |
| Class F | -0.01% | -0.40% | -0.20% | 101% | 100% |

Table 2. Results of the second proposed method over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.09% | 101% | 100% |
| Class A2 | -0.10% | -0.24% | -0.26% | 100% | 101% |
| Class B | -0.06% | -0.29% | -0.18% | 101% | 101% |
| Class C | -0.17% | -0.54% | -0.49% | 102% | 101% |
| Class E | -0.10% | -0.07% | -0.14% | 101% | 101% |
| Overall | -0.09% | -0.25% | -0.24% | 101% | 101% |
| Class D | -0.14% | 0.00% | -0.40% | 101% | 101% |
| Class F | -0.20% | -0.38% | -0.42% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | -0.04% | 101% | 100% |
| Class A2 | -0.05% | -0.12% | -0.18% | 101% | 100% |
| Class B | 0.00% | -0.31% | -0.45% | 101% | 100% |
| Class C | -0.07% | -0.58% | -0.43% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.29% | -0.31% | 101% | 100% |
| Class D | -0.01% | -0.23% | -0.28% | 101% | 100% |
| Class F | -0.10% | -0.42% | -0.31% | 101% | 100% |