| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1031-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0398 (Chroma block size restriction in dual tree intra coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Sung-Chang Lim Hahyun Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of chroma block size restriction in dual tree intra coding proposed in JVET-O0398. The proposed method restricts on 2xN chroma block size in dual tree case. It is reported that the average luma BD-rates of the proposed method in AI and RA are 0.02% and 0.02% compared to VTM 5.0, respectively.

- Introduction

In VTM 5.0, small chroma block sizes of 4x4, 4x2, and 2x4 are restricted in dual tree case. The proposed method in JVET-O0398 restricts on 2xN chroma block size in dual tree. When chroma block width is less than or equal to 4 vertical BT split is disabled. Likewise, vertical TT split is disabled when chroma block width is less than or equal to 8.

Experimental results

Experiments were performed based on VTM 5.0 software under JVET common test condition.

Table 1 shows experimental results of the proposed method compared to VTM 5.0. All the results are matched with the ones in JVET-O0398.

Table 1. Results of the proposed method over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.23% | 0.09% | 100% | 100% |
| Class A2 | 0.07% | 0.43% | 0.44% | 98% | 99% |
| Class B | 0.00% | 0.31% | 0.52% | 100% | 100% |
| Class C | 0.03% | 0.72% | 0.87% | 100% | 99% |
| Class E | -0.01% | -0.11% | -0.28% | 100% | 100% |
| Overall | 0.02% | 0.34% | 0.38% | 100% | 100% |
| Class D | 0.07% | 1.12% | 0.99% | 99% | 100% |
| Class F | 0.07% | 0.44% | 0.55% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.04% | 100% | 100% |
| Class A2 | 0.06% | 0.55% | 0.44% | 100% | 100% |
| Class B | 0.02% | 0.33% | 0.31% | 100% | 100% |
| Class C | 0.01% | 0.18% | 0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.27% | 0.27% | 100% | 100% |
| Class D | -0.01% | 0.58% | 0.69% | 100% | 100% |
| Class F | -0.01% | 0.47% | 0.39% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.18% | 0.19% | 100% | 99% |
| Class C | 0.00% | 0.36% | 0.28% | 101% | 100% |
| Class E | 0.12% | -0.16% | 0.15% | 100% | 100% |
| Overall | 0.04% | 0.15% | 0.21% | 100% | 100% |
| Class D | 0.04% | 0.00% | 1.22% | 101% | 100% |
| Class F | 0.02% | 0.12% | -0.18% | 100% | 100% |