| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1032-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0542 (Non-CE8: Modified intra mode associated with BDPCM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Sung-Chang Lim Hahyun Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of modified intra mode associated with BDPCM proposed in JVET-O0542. The intra prediction mode associated with a coding block coded by BDPCM mode is defined by prediction direction in BDPCM signalled using a syntax intra_bdpcm_dir_flag. It is reported that the average luma BD-rates of the proposed method for class F and TGM are -0.12% and -0.18% in RA, and -0.09% and -0.18% in LDB compared to VTM 5.0, respectively.

- Introduction

In VTM 5.0, intra prediction mode of BDPCM block is always set to planar mode. However, direction of BDPCM is signalled by a syntax intra_bdpcm_dir_flag. The proposed method JVET-O0542 sets intra prediction mode of BDPCM block using the intra_bdpcm_dir_flag. Namely, horizontal or vertical intra prediction mode is set to the BDPCM block according to the flag whether it is 0 or 1, respectively.

Experimental results

Experiments were performed based on VTM 5.0 software under JVET common test condition.

Table 1 shows experimental results of the proposed method compared to VTM 5.0. All the results are matched with the ones in JVET-O0542.

Table 1. Results of the proposed method over VTM 5.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | 0.04% | -0.08% | 101% | 100% |
| TGM | -0.18% | -0.06% | -0.07% | 101% | 99% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | 0.08% | 0.10% | 100% | 100% |
| TGM | -0.18% | -0.02% | -0.13% | 100% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.21% | -0.44% | 100% | 100% |
| TGM | -0.04% | 0.01% | 0.04% | 100% | 100% |