| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1037-v1 |
| --- | --- |

| Title: | AHG17: On AL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

At the Wednesday July 3rd, 2019 high-level syntax BoG meeting, the topic of VVC NAL unit header design was not concluded but would be revisited. During the discussion, there was a suggestion to use the HEVC NAL unit header syntax but use of the 6 bits for NAL unit type for future extension purpose. This contribution provides such a NAL unit header syntax for that revisit and and proposes this NAL unit header syntax for VVC.

# Introduction

The HEVC NAL unit header syntax is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

The NAL unit header syntax in JVET-M1001-v7 (the Marrakech output) is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

The NAL unit header syntax in JVET-N1001-v10 (the Geneva output) is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| nuh_layer_id | u(7) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

# Proposal

The following NAL unit header syntax is proposed for VVC (changed parts relative to the HEVC NAL unit header syntax are highlighted just for convenience):

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

Accordingly, the NAL unit header semantics are as follows:

forbidden_zero_bit shall be equal to 0.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that have nal_unit_type in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 1 – NAL unit types in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these nal_unit_type values, and in the design of decoders that interpret the content of NAL units with these nal_unit_type values. This Specification does not define any management for these values. These nal_unit_type values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same nal_unit_type value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 2 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes [Ed. Note: The table here is the same as in JVET-O0179, where changes relative to Table 7-1 in JVET-M1001-v7) are marked.]

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_4.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 2425..27 | RSV_NVCL2425.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

NOTE 3 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 4 – An instantaneous decoding refresh (IDR) picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having nal_unit_type equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture is the value of the nuh_layer_id of the VCL NAL units of the coded picture.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_VCL13, inclusive, TemporalId shall be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of a layer access unit. The value of TemporalId of a coded picture or a layer access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the layer access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to APS_NUT, TemporalId shall be equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when nal_unit_type is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

NOTE 5 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all layer access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes layer access units for which the TemporalId values are greater than the TemporalId of the layer access unit containing the SEI NAL unit.

nuh_reserved_zero_bit shall be equal to '0'. The value 1 of nuh_reserved_zero_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with nuh_reserved_zero_bit equal to '1'.

# Patent rights declaration(s)

Futurewei Technologies, Inc. does not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the contributor).