| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1038 |
| --- | --- |

| Title: | Crosscheck of JVET-O0748 (Non-CE3: On signalling for intra coding mode in the MPM list) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liang Zhao | Email: | leolzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of test 4 and test 5 in JVET-O0748. The software implementation as provided by the proponent of JVET-O0748 has been studied and confirmed to be aligned with the proposal description. Simulation results as provided in this contribution are reportedly matching with the results provided by proponent of JVET-O0748.

Crosscheck report

In JVET-O0748, in the 4th test, on top of JVET-O0925, the Planar flag is encoded using three contexts with Planar check of its neighbour which unified with the Planar check in the MPM list derivation. In the 5th test, on top of JVET-O0925, it is built on top of Test 4 where the first bin of MPM index is encoded using 3 contexts.

The SW implementation was provided by the proponent of JVET-O0748 and simulations are performed under common test condition (CTC) [1]. Two tests in JVET- O0748 have been crosschecked, which are listed in Table 1 and Table 2.

Table 1: Results of Test 4 over VTM-5.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.18% | -0.20% | 90% | 100% |  |  |  |  |  |
| Class A2 | -0.12% | -0.14% | -0.18% | 93% | 100% |  |  |  |  |  |
| Class B | -0.15% | -0.19% | -0.14% | 92% | 100% | 0.03% | 0.08% | -0.02% | 96% | 99% |
| Class C | -0.15% | -0.07% | -0.07% | 94% | 99% | -0.03% | -0.06% | -0.13% | 96% | 101% |
| Class E | -0.27% | -0.27% | -0.25% | 94% | 99% |  |  |  |  |  |
| Overall | -0.17% | -0.17% | -0.16% | 93% | 100% |  |  |  |  |  |
| Class D | -0.09% | 0.10% | -0.25% | 93% | 102% | 0.09% | 0.26% | 0.23% | 96% | 101% |
| Class F | -0.10% | 0.05% | 0.01% | 97% | 98% | -0.02% | 0.04% | 0.08% | 97% | 102% |

Table 2: Results of Test 5 over VTM-5

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.21% | -0.25% | 91% | 100% |  |  |  |  |  |
| Class A2 | -0.17% | -0.24% | -0.23% | 93% | 100% |  |  |  |  |  |
| Class B | -0.20% | -0.23% | -0.25% | 92% | 100% | -0.01% | -0.05% | -0.14% | 96% | 98% |
| Class C | -0.17% | -0.18% | -0.12% | 93% | 98% | -0.01% | -0.06% | -0.06% | 96% | 98% |
| Class E | -0.29% | -0.27% | -0.30% | 93% | 98% |  |  |  |  |  |
| Overall | -0.21% | -0.22% | -0.23% | 92% | 99% |  |  |  |  |  |
| Class D | -0.10% | 0.09% | -0.10% | 94% | 98% | 0.08% | 0.13% | 0.07% | 97% | 99% |
| Class F | -0.22% | 0.03% | -0.16% | 96% | 96% | -0.07% | -0.07% | 0.04% | 97% | 99% |

References

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.