| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–17 July 2019 | Document: JVET-O1039-v5 |
| --- | --- |

| Title: | BoG report on CE4 inter prediction with merge modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com |
| Source: | BoG |  |  |

_____________________________

# Summary

The CE4 BoG sessions on merge modifications were held 2045~2200 on July 3, 0900~1330 and 1500~1740 on July 4, 2000~2300 on July 5, 0900~1100 and 1945~2300 on July 6, and 0900~1100 and 1830~1900 on July 7. There are 63 contribution documents reviewed in this BoG under 6 categories as follows:

- Triangle prediction merge mode (25)

- Merge mode with MVD (3)

- Combined intra and inter prediction (6)

- Merge list construction (3)

- Merge mode syntax (18)

- Subblock temporal motion vector prediction (8)

The recommendations that were made during the BoG sessions cover the followings:

- General recommendation (3)

- Normative change (4 → 5)

- Non-normative change (1 → 4)

- Recommended CE4 test (2)

- Revisit in Track (1 → 6)

- Contribution not presented (17 → 0)

## Recommendations

### Recommended adoptions to VTM

General recommendation (3)

- It is commented that at this stage it is suggested to not to make changes to the number of CABAC context models for minor coding performance-wise impact.

- It is also mentioned what would be the right timing to start working on making changes to CABAC context models.

- It is a general agreement to “align spec to sw” first before proceeding to propose changes to the standard.

Normative changes (5)

JVET-O0222 Non-CE4: Modification of triangle merge mode enabling criterion [D. Liu, R. Yu (Ericsson)]

Since TPM supports 128xN/Nx128 CUs which exceed the size of 64x64 VPDU, the proposal disables these CU sizes in the application of TPM. (RA: 0.02% gain; LDB: 0.03% loss.)

It is stated that removing 128xN/Nx128 CUs helps align the processing unit to that of VPDU and also stated that this proposed method can reduce the storage size of weighting values (if they are not computed on the fly).

Same proposals: JVET-O0362 and JVET-O0249 Test 3.

The proponents of JVET-O0249 comments that disabling SBT in TPM is similar in spirit to remove 128xN/Nx128 CUs.

It is stated from a participant that this same topic was discussed in Marrakesh meeting in JVET-M0207/JVET-M0438. The related note is quoted from the CE10 BoG report (JVET-M0873), as below:

- It is remarked that 128x128 TPM might also violate VPDU condition. It is answered that this not the case, as it can be done on a 64x64 block basis.

As commented from several participants, it is noted that performing TPM on large block has complexity implications but no performance-wise benefit.

Recommendation: Adopt. (JVET-O0222/ JVET-O0249 Test 3/ JVET-O0362)

JVET-O0629 Non-CE4: Simplification of motion field for triangle partition [R.-L. Liao, Y. Ye, J. Chen (Alibaba)]

Method 1: Enlarge the unit of motion storage from 4x4 to 8x8 (RA:0.03%/99%/99%;  LB:0.04%/100%/99%)

A non-proponent commented that for larger CUs, this design creates motion field that fits blending process more closely.

With 8x8 unit storage, the number of comparisons to check whether each motion storage unit belongs to partition 1, partition 2 or blending area is reduced from 496 to 120.

It is also commented if a CE is formed to study TPM motion storage related techniques, complexity characteristics (i.e., number of comparisons to determine motion storage) should be reported.

A non-proponent supports adopting method 1 and also commented that the loss of 0.03% is relatively minor against VTM anchor.

Recommendation: Adopt method 1.

JVET-O0681 CE9-related: Disabled DMVR, BDOF and BCW for CIIP [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

It is proposed to disable BCW by reset the weighting to equal, and 1) reset BCW to equal weight, 2) inherit BCW index.

Performance: 1) -0.01% RA, 0.03% LDB (luma), 2) -0.01% RA, -0.02% LDB, no runtime impact observed.

It is claimed the benefit is software decoding complexity.

It is commented that conceptually not necessary to further weight inter as inter and intra will be combined with weighting.

It is asked whether we should do weighted prediction for CIIP. It is commented that weighted prediction is somewhat different as it is for fading and there is an offset in weighted prediction.

Two participants suggest adoption of method 2.

Recommendation: Adopt method 2 (inherit BCW index).

JVET-O0167 Non-CE4: On SbTMVP motion data derivation [J. Zhao, S. Kim (LGE)]

Method 1: Use MV from collocated subblock at another position as base motion vector, where the proposed position is at the immediately bottom-right motion storage unit relative to the center point of collocated block pointed to by temporal MV.

(RA: 0.00%; LDB: 0.00%)

The proponent claimed the benefits are:

- Simplify SbTMVP base motion data derivation by re-using one of the sub-CU’s motion data as base motion data.

- Avoid a separate base motion data derivation process, and therefore remove the associated temporal motion vector scaling.

The benefit is to simplify the operation for certain type of decoder implementation because it can reuse the TMVP derivation of subblocks.

One non-proponent expresses interest to have this feature.

Recommendation: Adopt method 1.

JVET-O0220 Non-CE4: On maximum number of subblock merge candidates [R. Yu, D. Liu (Ericsson)]

Method 2: The proposed fix solves the ambiguity in both spec and sw.

“when sps_affine_enabled_flag=0 and sps_sbtmvp_enabled_flag=1, there is one subblock merge mode, that is, sbTMVP. However, the problem is sbTMVP may not be always available, and thus a zero-MV affine candidate is used as a subblock merging candidate, which causes ambiguity.” -- quoted from JVET-O0328 Test 1.

In addition to sps_sbtmvp_enabled_flag, SbTMVP can be turned off by slice_temporal_mvp_enabled_flag.

Assuming five_minus_max_num_subblock_merge_cand is not present (i.e., sps_affine_enabled_flag=0):

MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand

= 5 – (5 − sps_sbtmvp_enabled_flag)

= sps_sbtmvp_enabled_flag   ……….. which is 1 in this case.

The method replaces the inference scheme of “5 – sps_sbtmvp_enabled_flag” by using “5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag)”.

(JVET-O0263 is identical to method 2)

Recommendation (Bugfix): Adopt the bugfix in method 2.

Non-normative changes (4)

JVET-O0379 Non-CE4: Modified mode decision process for triangle mode [F. Chen, L. Wang (Hikvision)]

Encoder-only fast algorithm to speed up TPM mode.

In VTM-5.0, there are 40 RDO checks at least for TPM mode when best mode is skip, this proposal reduces the number of RDO check from 40 to 3. These 3 are selected depending on SATD and bit count without CABAC estimation for the 40 TPM candidates. The same encoder-only algorithm also applies when the condition that the best mode is not skip is satisfied.

As reported, this encoder-only fast algorithm saves encoding time by 5% in RA and 7% in LDB with around 0.02% of BD-rate reduction in LBD case.

(RA:-0.01%/95%/101%;  LB:0.02%/93%/100%)

The cross-checker confirms that there is only one line of change in the current VTM software.

It is commended from a participant that JVET-O0280 proposes a SIMD-based implementation to speed up the blending process as for TPM. (~2% speed up to encoder)

Recommendation (SW): Adopt.

JVET-O0280 CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode [Y.-L. Hsiao, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

Encoder-only modification to TPM for SIMD implementation on TPM blending operation.

The encoding time is reduced by 2% for RA and 3% for LB without BD-rate changes.

Revisited in BoG (Revisited 10:40, July 7), there is no further issue spotted after code review by Xiang Li and (cc. to Frank Bossen).

The software implementation is confirmed.

Recommendation (SW): Adopt.

JVET-O0592 CE4-related: Motion estimation improvements	[X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

Method 1:

The proposed method picks up one of the previously decoded MVs as (a) the starting MV for integer motion search and (b) the initial MV candidate for SMVD.

The source of MVs are from 20 previously decoded CUs and these MVs are all rounded to integer precision before being used.

Method 2:

Perform a two-pass local refinement of CPMVs in affine ME process at ¼ sample precision.

The first pass checks 4 neighboring positions (above/bottom/left/right).

Then, the 4 remaining search points at corners are searched in the second pass only if the first-pass search updates CPMVs.

The results are reported with 2 methods: RA: -0.31%/101%/99%; LDB:-0.16%/101%/100%.

Several non-proponents express interest in this method. In addition, there is consistent coding performance improvement over all sequences.

Recommendation (SW): Adopt both methods and use both methods in CTC.

JVET-O0220 Non-CE4: On maximum number of subblock merge candidates [R. Yu, D. Liu (Ericsson)]

Method 1: Propose sw bugfix. Parse merge_subblock_flag conditioned on MaxNumSubblockMergeCand.

In the sepc text, merge_subblock_flag is parsed conditioned on MaxNumSubblockMergeCand>0; however, the sw refers to sps_sbtmvp_enabled_flag and sps_affine_enabled_flag to determined whether to parse merge_subblock_flag or not.

Recommendation (BugfixSW): Adopt the bugfix in method 1 to align sw to spec text.

### Recommended CE4 tests from following contributions (2)

JVET-O0489 Non-CE4: Geometrical partitioning for inter blocks [S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

This proposal introduces new geometric partitions for merge mode. This mode supports 32 angles & 5 distance values to indicate a variety of geometric partitioning patterns. In total there are only 140 new modes (but not 160=32*5) because some of the redundant partitioning patterns as for TPM can be removed.

A lookup-table based weighting function is introduces for blending samples on partitioning boundary inside a CU. The lookup table stores pre-calculated values of sine and cosine functions.

For motion storage, it sums up the four weight values at the corner of each 4x4 unit, and depending on the sum to differentiate whether uni-pred mv of triangle partitions or bi-pred mv should be stored.

For transform coding, it is required to remap a parallelogram shape of prediction residuals along the partitioning edge to square block for transform. For other samples, they are assumed to be zero.

(RA:-0.24%/105%/100%;)

If proposed transform is not applied, RA:-0.22%/106%/101%; LB:-0.44%/108%/102%.

The proponent claims that GEO is a straightforward extension to current TPM in VTM.

Visual quality comparisons are provided in the proposal, showing comparable viewing experience to that of the VTM anchor.

Non-proponents support further studying this tool due to the improved visual quality as compared to VTM-5.0 anchor.

Crosscheck number: JVET-O1010.

Recommendation: Further study in a CE.

JVET-O0522 Non-CE4: CIIP using triangular partitions [S. Blasi, G. Kulupana, A. Seixas Dias (BBC)]

Method 1:

Process-wise, the proposed mode is more close-related to CIIP but not TPM. The complexity overhead as compared with CIIP is the proposed mode requires to not blending the whole prediction samples of inter and intra, but instead, allocate each of them to respect triangle partition.

In addition, the same TPM blending is required for this mode. This aspect is later commented that this may not incur extra decoding burden as CIIP mode already require similar blending operation for the whole CU.

For the triangle that is intra-coded, only planar pred is used.

There are only 3 modes (instead of 4) for each TPM partitioning pattern, because this proposal does not allow intra pred to be used at bottom-right triangle.

The mapping from syntax to the 3 modes is adaptive depending on whether left and above neighbor blocks are intra or inter coded. In total, 4 different combinations.

(RA: -0.08%; Class C LDB: -0.18%)

It is suggested to add checking of sps_triangle_enable_flag for this mode.

It is commented that it is questionable whether the increase of decoder complexity (i.e., adding one new mode) could justify the improvement of coding improvement.

It is commented that whether the filter used in the TPM blending process is strong enough to filter the boundary between the intra predicted triangle and inter predicted triangle. As this would potentially introduce visual impact on this mode, subjective viewing is encouraged.

Three non-proponents express interest in this proposal and suggest further study in a CE.

Recommendation: Further study method 1 in CE.

## Open issues

### Revisit in track (6)

#### Simplified motion field storage

(Solution A) If both MVs share same ref pic list, then use right triangle's MV.

RA:0.00%; LB:0.06%

JVET-O0279 method 1 /   JVET-O0378 method 1 /  JVET-O0329 method 1 /  JVET-O0411 Test C /   JVET-O0660 method 3

It is commented that the mapping function from one reference pic list to the other may be just of implementation issue and can be done in slice level, that is, to use mapping tables for TPM CU. How this mapping table is formed is only an implementation-wise issue.

A participant commented that if a certain solution is selected from these solutions, the text in the spec as for TPM can be greatly reduced.

A participant commented that such simplification design (i.e., removing reference list mapping) can save 2 comparisons (i.e., one for mapping from L0 to L1 and the other for mapping from L1 to L0).

A non-proponent suggests to take no action for simplification.

| VTM | Solution A |
| --- | --- |

(Solution B) If both MVs share same ref pic list, then use bottom triangle partition's MV.

RA:0.00%; LB:0.07%

JVET-O0265 method 1 / JVET-O0629 method 2 / JVET-O0418 method 1 / JVET-O0329 method 1A /   JVET-O0378 method 2 / JVET-O0411 test B / JVET-O0279 method 1B

The consensus is based on the assumption that this simplification is desirable to select Solution B from Solutions A-C.

It is commented this Solution can simplify the text of the spec significantly.

| VTM | Solution B |
| --- | --- |

(Solution C) If both MVs share same ref pic list, then use upper triangle's MV.

RA:0.00%; LB:0.08%

JVET-O0279 method 1A / JVET-O0418 method 2 / JVET-O0329 method 1B / JVET-O0314 / JVET-O0411 Test A

One competing proponent commented that this configuration looks less promising as the upper one is located further away from succeeding CUs and would impact on the performance of spatial mv prediction, which explains why this configuration incurs slightly higher coding loss than others’.

| VTM | Solution C |
| --- | --- |

Although Solution B is generally agreed, there is still one proponent who proposed opposing solution and raised opinion to take no action.

Revisit in Track B.

#### JVET-O0285 CE4-related: On merging candidate list construction

Propose to enforce MaxNumMergeCand to be always 6 only in decoding process and no change to syntax signalling. (No experiment is required as this does not affect CTC cases)

The proponent claims this a bugfix since the decoder does not need to check whether the number of candidates has reached the maximal number.

It is requested to provide additional results on the configuration of MaxNumMergeCand=3,4,5 in addition to 2.

It is questioned on the cost of this additional check to this check. The proponent responds that the circuit design requires additional logic to accommodate different configurations of max merge cand numbers.

Revisited in BoG (1050, July 7) to review the additional results with MaxNumMergeCand 3, 4 or 5.

Revisit in Track B. Review results with MaxNumMergeCand 3, 4 or 5.

#### JVET-O0163 Non-CE4: Simplification on SbTMVP

Test 1:

The proposed method removes the condition check to switch adaptively between L0 and L1 that is used to determine which of the L0 and L1 motion (when both available) is used as the temporal MV for sbTMVP. The order become constant: L0 first, then if MvL0 does not point to collocated picture, then L1.

(RA:0.00%; LDB:0.01%)

(JVET-O0588 method 1is identical to method 1)

A non-proponent expresses there is no problem in the current design.

Another non-proponent commented the loss is at noise level and the benefit is the saving of the number for checking (2).

It is agreeable that the benefit from this proposal is to reduce the number of checks that is applied to the adaptation between L0 and L1 selection.

A non-proponent supports the proposed method.

Since there is no consensus to have this proposed feature, it may not be adequate to adopt this.

Revisit method 1 in Track B.

#### JVET-O0666 Fix for missing MV in 4xN and Nx4 triangle partition mode

It is claimed that this proposal can fix a problem in existing TPM where an update of the motion buffer of current coding unit is required. In the current design, bi-directional motion is stored for all 4x4 subblocks inside a 4xN or Nx4 CU using TPM. Without this fix, 1 bit needs to be stored for each 4x4 block to distinguish which motion vector from the bi-directional motion is used for which partition, in one possible implementation. And 128 bytes buffer is required for the storage of this indication flag of the whole CTU. It is claimed this additional 128 bytes could be saved.

It is commented that in another implementation where the motion compensation is performed for each 16x16 block considering pipeline design, the buffer required is 2 bytes.

It is commented that the update of the motion buffer of the current coding unit already exists in the current VTM-5.0. And there is no practical problem if the update is performed for TPM as well.

Results combined with JVET-O0411 method B.

Recommendation: revisit in Track B.

#### JVET-O0795 CE4-related: On the context variables and syntax elements of merge index

Method 2.1: Move merge_triangle_split_dir after merge_triangle_idx1. Group bypass bins of triangle_idx 1 and merge_triangle_split_dir.

Method 2.2: Group bypass bins from TPM cand idx.

It is commented that the benefit from grouping bypass bins is unclear.

Revisit: Track B. (Solicit comments from hardware experts) This contribution is to be revisited in Track B with JVET-O0456 in the category of CE4 BoG on general inter prediction.

#### Merge mode syntax

(Solution C) Change syntax tree of merge mode flags and reduce checks.

Proposal: JVET-O0249 Test 1.

RA: -0.08%; LDB: -0.06%

|  | → |  |
| --- | --- | --- |

(Solution D) Impose more condition checks to avoid parsing redundancy completely.

Proposal: JVET-O0431 / JVET-O0586 / JVET-O0309 / JVET-O0585 method 1 / JVET-O0534/ JVET-O0361 / JVET-O0278.

RA:-0.20%; LDB: -0.22%. (Note: both anchor and test disable TPM.)

Revisit: Track B (Solution C and D). The proponents are suggested to form offline discussion and provide statistical data.

### Revisit in BoG (0)

### Contribution not reviewed yet (0)

# Triangle prediction merge mode (25)

JVET-O0128 Non-CE4: simplification of triangle mode MV context storage [J. Li, C.S. Lim (Panasonic)]

This contribution is a response to AHG16 comment on complexity of MV context storage of triangle mode. Two methods are proposed. The first method is code optimization that moves CU level MV index mapping to slice level by using mapping list table. The second method is further simplification of MV index mapping process. The first method has no impact on coding efficiency. The second method has coding impact of 0.01% luma BD-rate in RA and 0.00% luma BD-rate in LDB over VTM-5.0 anchor.

Method 1 [implementation]: Move CU-level MV mapping function to slice-level. It is reported no coding performance-wise change.

Method 2: In blending area, if both MVs are from the same reference picture list, the first MV is stored in the motion field of TPM CU. (RA: 0.01% loss; LDB: no impact.)

No action.

JVET-O0724 Crosscheck of JVET-O0128 (Non-CE4: Simplification of triangle mode MV context storage) [R.-L. Liao (Alibaba)]

JVET-O0217 Non-CE4: Simplification of motion vector storing process for triangle merge mode [Y. Ahn, D. Sim (Digital Insights)]

In current VVC WD, the additional motion vector storing process is used to store motion information of triangle merge mode. Especially, a reference picture mapping process is performed to store the motion information of the blending area. This contribution proposes a simplification of motion vector storing process for triangle merge mode to remove the reference picture mapping process. The proposed method applies the adaptive selection to determine which motion information to be stored in the blending area according to the reference index. The proposed method shows 0.00% and 0.16% luma BD-rate in RA and LB configuration, respectively.

When the two uni-prediction MVs of triangle partitions refer to the same ref list, then the MV with smaller refidx is selected to replace the bi-prediction MVs at the blending area. In case that first triangle partition MV and the second one happens to have identical refidx, the MV of the upper triangle partition’s MV is selected.

RA: 0.00%; LB: 0.16% loss (Class E 0.27% loss, 0.42% loss in Johnny).

Missing crosscheck report.

(Refer to the discussion at the end of this section)

JVET-O0222 Non-CE4: Modification of triangle merge mode enabling criterion [D. Liu, R. Yu (Ericsson)]

In VTM-5.0, the triangular merge mode divides a coding unit into two triangular partitions. Uni-directional motion compensation is applied for each triangular partition and the diagonal edge between the two triangular partitions is blended by weighting matrices. The triangular merge mode is allowed for CU with either of its dimension greater than 64. To accommodate VPDU pipeline design but still realize the correct blending, different weighting matrices need to be applied for each VPDU since the blending area for each VPDU is different. The contribution proposes to disable the triangular merge mode when width equals 128 or height equals 128, such that the handling for each VPDU is unified and storage of weighting matrices for different VPDU is saved. The proposed method was tested under the VTM-5.0 random access (RA) and low delay (LD) under common test configuration. The BD-rate impact is -0.02% for RA and 0.03% for LD.

Since TPM supports 128xN/Nx128 CUs which exceed the size of 64x64 VPDU, the proposal disables these CU sizes in the application of TPM. (RA: 0.02% gain; LDB: 0.03% loss.)

It is stated that removing 128xN/Nx128 CUs helps align the processing unit to that of VPDU and also stated that this proposed method can reduce the storage size of weighting values (if they are not computed on the fly).

Same proposals: JVET-O0362 and JVET-O0249 Test 3.

The proponents of JVET-O0249 comments that disabling SBT in TPM is similar in spirit to remove 128xN/Nx128 CUs.

It is stated from a participant that this same topic was discussed in Marrakesh meeting in JVET-M0207/JVET-M0438. The related note is quoted from the CE10 BoG report (JVET-M0873), as below:

- It is remarked that 128x128 TPM might also violate VPDU condition. It is answered that this not the case, as it can be done on a 64x64 block basis.

As commented from several participants, it is noted that performing TPM on large block has complexity implications but no performance-wise benefit.

Recommendation: Adopt. (JVET-O0222/ JVET-O0249 Test 3/ JVET-O0362)

JVET-O1041 Crosscheck of JVET-O0222 (Non-CE4: Modification of triangle merge mode enabling criterion) [F. Chen (Hikvision)]

JVET-O0265 Non-CE4: Simplified motion field storage for TPM [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

In this contribution, three simplified motion field storage methods for TPM are proposed. In method #1, the mapping process is removed for the storage of motion information in the weighted area. In method #2, bi-prediction motion is always stored. In method #3, the changes of method #1 and method #2 are combined under the low delay check. Simulation results reportedly show that there is no overall coding performance change for method #3 compared to VTM5.0.

Method 1: If both MVs are from same ref list, select bottom triangle’s MV. (RA:0.00%; LB:0.07%)

Identical proposals: JVET-O0629 method 2 / JVET-O0418 method 1 / JVET-O0329 method 1A / JVET-O0378 method 2 / JVET-O0279 method 1B / JVET-O0411 test B

Method 2: If both MVs are from same ref list, change bottom triangle partitoin’s ref picture list idx to the other (RA:0.06%; LB:0.00%). However, refidx availability check in the other reference picture list is required but is not considered in this proposal, and because CTC happens to have two identical reference picture list in Low-Delay case, thus Method 2 happens to avoid this bug.

Method 3: For non low-delay case, Method 1 is taken; For low-delay case, Method 2 is taken (RA:0.00%; LB:0.00%)

The proponent mentioned that the number of lines in sepc regarding TPM can be greatly reduced.

(Refer to the discussion at the end of this section)

JVET-O0676 Crosscheck of JVET-O0265: Non-CE4: Simplified motion field storage for TPM [J. Zhao (LGE)]

JVET-O0279 CE4-related: Simplification of motion vector storing process for triangle prediction mode [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes two methods to simplify the motion vector (MV) storing process at partition boundary for triangle prediction mode. Compared with VTM5.0, in method 1, the sequential reference picture search procedure is removed, and motion vector selection either depending on partition direction or not is applied when the two MVs of the two partitions point to the same reference picture list. In method 2, the motion vector selection by partition direction is always applied. Results are summarized as follows.

Proposed method 1
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM5.0-LB: {Y-BD-rate=0.06%, EncT=100%, DecT=100%}

Proposed method 1 variant a
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=98%}
VTM5.0-LB: {Y-BD-rate=0.08%, EncT=100%, DecT=100%}

Proposed method 1 variant b
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=98%}
VTM5.0-LB: {Y-BD-rate=0.07%, EncT=100%, DecT=100%}

Proposed method 2
VTM5.0-RA: {Y-BD-rate=0.04%, EncT=100%, DecT=102%}
VTM5.0-LB: {Y-BD-rate=0.17%, EncT=100%, DecT=101%}

Method 1: select MVA or MVB depending on partitioning direction, when both ref picture list are identical.

Identical proposal: JVET-O0378 method 1 / JVET-O0329 method 1 / JVET-O0411 Test C / JVET-O0660 method 3

Method 1a: select MVA, when both ref picture list are identical.

Identical proposals: JVET-O0418 method 2 / JVET-O0329 method 1-B / JVET-O0314 / JVET-O0411 Test A

Method 1b (identical to JVET-O0265 method 1): select MVB, when both ref picture list are identical.

Method 2: select MVA or MVB depending on partitioning direction and it does not depend on whether ref picture lists are the same or not to each motion vector. The larger coding loss in LDB case is that because the motion field of the entire TPM

Identical proposal: JVET-O0660 method 1 / JVET-O0329 method 2.

(Refer to the discussion at the end of this section)

JVET-O0835 Crosscheck of JVET-O0279 (CE4-related: Simplification of motion vector storing process for triangle prediction mode) [H. Lee, S.-C. Lim, J. Lee, J. Kang (ETRI)]

JVET-O0314 Non-CE4: Modification of MV storage in triangle merge mode [P.-H. Lin (Foxconn)]

(Identical to JVET-O0279 method 1a)

JVET-O0759 Crosscheck of JVET-O0314 (Non-CE4: Modification of MV storage in triangle merge mode) [H.-J. Jhu (Kwai Inc.)]

JVET-O0329 Non-CE4: Simplification of motion vector storage process for triangle merge mode [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

This contribution presents simplified methods on motion vector storage process for triangle merge mode. In method 1, when a 4x4 subblock is type-2 and two triangular partitions have motion information in the same reference list, the reference picture mapping process is removed. Instead, one of two triangular partition motion information is assigned based on triangular split direction information. In method 1-variant A, MVB and RefIdxB is always assigned to 4x4 subblock. In method 1-variant B, MVA and RefIdxA is always assigned to 4x4 subblock. In method 2, type-2 is removed so that a 4x4 subblock can be either type-0 or type-1. Compared with VTM-5.0, the method 1 incurs an average 0.00% and 0.06% luma BD-rate changes for RA and LDB, respectively. The method 1-variant A incurs an average 0.XX% and 0.XX% luma BD-rate changes for RA and LDB, respectively. The method 1-variant B incurs an average 0.00% and 0.08% luma BD-rate changes for RA and LDB, respectively. The method 2 incurs an average 0.04% and 0.17% luma BD-rate changes for RA and LDB, respectively.

Method 1 is identical to JVET-0279 method 1 (partition-direction based selection, when both MVs share the same ref pic list).

Method 1-A is identical to JVET-O0265 method 1.

Method 1-B is identical to JVET-O0279 method 1a.

Method 2 is identical to JVET-O0279 method 2.

JVET-O0881 Crosscheck of JVET-O0329 (Non-CE4: Simplification of motion vector storage process for triangle merge mode) [R. Yu, D. Liu (Ericsson)]

JVET-O0362 Non-CE4: Modified enabling condition for triangle mode [F. Chen, L. Wang (Hikvision)]

(Identical to JVET-O0222).

JVET-O0725 Crosscheck of JVET-O0362 (Non-CE4: Modified Enabling Condition for Triangle Mode) [R.-L. Liao (Alibaba)]

JVET-O0378 Non-CE4: Simplification of MV context storage for triangle mode [F. Chen, L. Wang (Hikvision)]

Currently, the MV context storage of triangle merge mode seems to be unnecessarily complicated. In this contribution, three methods are proposed to simplify the MV context storage. The first method selects one of the uni-prediction motion vector based on the split mode for the weighted areas. The second one just selects the bottom MV. The third one selects one of the uni-prediction motion vector based on the split mode or the average MVs for the weighted areas. The forth one keeps current method unchanged if the two reference lists are totally the same. It is reported that the first method brings 0.00% and 0.06% loss for RA and LB configurations respectively compared to VTM-5.0. The second one only brings -0.01% and 0.07% loss for RA and LB respectively compared to VTM-5.0. The third one only brings 0.02% and 0.04% loss for RA and LB respectively compared to VTM-5.0. And the forth one only brings -0.01% and 0.00% loss for RA and LB respectively compared to VTM-5.0.

Method 1 is identical to JVET-0279 method 1 (direction-based selection)

Method 2 is identical to JVET-O0265 method 1

Method 3: Average MVA and MVB if they both come from the same reference picture list and same reference picture; otherwise, method 1 is taken. (RA:0.02%; LB:0.04%)

Method 4: Slice-level to check whether two ref lists are identical to each other. If yes, swap ref picture list from one to the other (RA:-0.01%; LB:0.00%); otherwise, method 1 is taken.

(Refer to the discussion at the end of this section)

JVET-O0880 Crosscheck of JVET-O0378 (Non-CE4: Simplification of MV context storage for triangle mode) [D. Liu, R. Yu (Ericsson)]

JVET-O0379 Non-CE4: Modified mode decision process for triangle mode [F. Chen, L. Wang (Hikvision)]

In VTM-5.0, the triangle prediction mode brings additional 8% and 12% encoding time for RA and LB configurations, respectively. Especially for LB configuration, the complexity is a heavy burden. In this contribution, a fast mode decision method of the triangle prediction mode is reported for encoder. This method forces the triangle prediction mode in the encoder to conduct rough RDO process to save the candidate number of true RDO modes. It is reported that the method saves 5% and 7% encoding time for RA and LB configurations respectively. And it brings 0.01% gains and 0.02% loss for RA and LB configurations respectively compared to VTM-5.0.

Encoder-only fast algorithm to speed up TPM mode.

In VTM-5.0, there are 40 RDO checks at least for TPM mode when best mode is skip, this proposal reduces the number of RDO check from 40 to 3. These 3 are selected depending on SATD and bit count without CABAC estimation for the 40 TPM candidates. The same encoder-only algorithm also applies when the condition that the best mode is not skip is satisfied.

As reported, this encoder-only fast algorithm saves encoding time by 5% in RA and 7% in LDB with around 0.02% of BD-rate reduction in LBD case.

(RA:-0.01%/95%/101%;  LB:0.02%/93%/100%)

The cross-checker confirms that there is only one line of change in the current VTM software.

It is commended from a participant that JVET-O0280 proposes a SIMD-based implementation to speed up the blending process as for TPM. (~2% speed up to encoder)

Recommendation (SW): Adopt.

JVET-O0726 Crosscheck of JVET-O0379 (Non-CE4: Modified Mode Decision Process for Triangle Mode) [R.-L. Liao (Alibaba)]

JVET-O0388 Non-CE4: On the context of triangle merge index [F. Chen, L. Wang (Hikvision)]

Currently, both of the two indices of the triangle prediction mode reuse the context of the regular merge index. Although the candidates of the triangle prediction mode is the same as that of regular merge mode, the two indices of the triangle prediction mode have different distribution feature since they must be different from each other. For this purpose, two more contexts are added for the triangle index coding. It is reported that the modification brings 0.00% gains and 0.06% gains for RA and LB configurations respectively compared to VTM-5.0.

In VTM-5, TPM mode reuses the context of merge index of regular merge mode. This proposal introduces two new contexts respectively for the 2 merge idx of TPM mode.

The results show coding performance-wise benefit only for LDB case, with 0.06% gain on average, where the peak gain is from Class E with 0.19% gain and peak loss from SlideEditing with 0.21% loss.

A participant mentioned that this proposal does not increase the number of context-coded bins of TPM mode. The only thing required is to switch the context that is to be used for regular merge and TPM mode.

Further study.

General recommendation to Track B:

It is commented that at this stage it is suggested to not to make changes to the number of CABAC context models for minor coding performance-wise impact.

It is also mentioned what would be the right timing to start working on making changes to CABAC context models.

JVET-O0727 Crosscheck of JVET-O0388 (Non-CE4: On the Context of Triangle Merge Index) [R.-L. Liao (Alibaba)]

JVET-O0411 Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage [K. Reuzé, C.-C. Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

Triangle Partition Mode (TPM) requires the generation of a bi-prediction motion vector (biMv) for motion field storage in the blending area. It was mentioned in AHG16 that the process for generating this biMv is unnecessarily complex. It is thus proposed to simplify the generation of this biMv. Three methods are proposed to simplify this process. A first method provides X.XX% in RA / X.XX% in LDB. A second method provides X.XX% in RA / X.XX% in LDB. A third method provides no BD-rate loss in RA / X.XX% in LDB. A fourth method provides X.XX% in RA / X.XX% in LDB.

Test A is identical to JVET-O0279 method 1a

Test B is identical to JVET-O0265 method 1

Test C is identical to JVET-O0279 method 1

Test D: non-square handling

JVET-O0418 CE4-related: Simplification of motion vector storage operation for triangle merge mode [X. W. Meng (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

(Method 1 is identical to JVET-O0265 method 1)

(Method 2 is identical to JVET-O0279 method 1a)

JVET-O0995 Crosscheck of JVET-O0418 (CE4-related: Simplification of motion vector storage operation for triangle merge mode) [J. Li (Panasonic)]

JVET-O0489 Non-CE4: Geometrical partitioning for inter blocks [S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

Flexible non-rectangular partitioning for motion compensation blocks is proposed in this contribution. Triangle inter block is subset of proposed solution. On top of VTM-5.0 proposed method solution provides 0.24% and 0.55% luma BD-rate gain for RA and LD-B with ~5% encoding run-time increment and no decoding run-time change. Beside BD-rate gain proposed solution provides clear visual quality improvement.

This proposal introduces new geometric partitions for merge mode. This mode supports 32 angles & 5 distance values to indicate a variety of geometric partitioning patterns. In total there are only 140 new modes (but not 160=32*5) because some of the redundant partitioning patterns as for TPM can be removed.

A lookup-table based weighting function is introduces for blending samples on partitioning boundary inside a CU. The lookup table stores pre-calculated values of sine and cosine functions.

For motion storage, it sums up the four weight values at the corner of each 4x4 unit, and depending on the sum to differentiate whether uni-pred mv of triangle partitions or bi-pred mv should be stored.

For transform coding, it is required to remap a parallelogram shape of prediction residuals along the partitioning edge to square block for transform. For other samples, they are assumed to be zero.

(RA:-0.24%/105%/100%;)

If proposed transform is not applied, RA:-0.22%/106%/101%; LB:-0.44%/108%/102%.

The proponent claims that GEO is a straightforward extension to current TPM in VTM.

Visual quality comparisons are provided in the proposal, showing comparable viewing experience to that of the VTM anchor.

Non-proponents support further studying this tool due to the improved visual quality as compared to VTM-5.0 anchor.

Recommendation: Further study in a CE.

JVET-O1010 Crosscheck of JVET-O0489 (Non-CE4: Geometrical partitioning for inter blocks) [A. Wieckowski (HHI)]

JVET-O0513 Non-CE4: Adaptive blending filtering for TPM [S. Esenlik, H. Gao, B. Wang, A. Kotra, Z. Zhao, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

(To discuss in CE8 BoG)

JVET-O0921 Crosscheck of JVET-O0513: Non-CE4: Adaptive blending filtering for TPM [X. Xiu (Kwai Inc.)]

JVET-O0552 Non-CE4: Extension of triangle partition mode to P-slices [K. Reuzé, C.-C Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

It is proposed to extend the use of TPM to P-slices, by removing the blending operation performed on the diagonal, and by only storing uni motion vectors, using the different proposed simplifications from JVET-O0411. Using the first method provides X.XX% coding gains in LDP configuration. The second method provides X.XX% coding gains in LDP configuration. The third method provides 0.30% coding gains in LDP configuration. The fourth method provides 0.39% coding gains in LDP configuration.

It is commented that the worst-case memory access is increased by the proposed method for LDP configuration. 4x4 uni-prediction vs. 4x8 uni-prediction.

An increase of encoder run-time over 20% is observed.

It is commented that there is a redundancy in syntax design of merge mode in current VTM-5.0 for LDP. Simply removing this redundancy could give about 0.22% coding gain for LDP.

Further study is encouraged.

JVET-O1028 Crosscheck of JVET-O0552 (Non-CE4: Extension of triangle partition mode to P-slices) [I. Zupancic (HHI)]

JVET-O0601 CE4-related: On motion vector population under triangle prediction mode [X. Wang, Y.-W. Chen, X. Xiu, T.-C. Ma (Kwai Inc.)]

In the current VVC under triangle prediction mode, 4x4 blocks in each triangle partition are populated with the respective uni-prediction motion vector (MV) for that partition. For those blocks sitting on the triangle partition border, a potentially bi-prediction MV is derived from the two uni-prediction MVs and assigned to them. At decoder side, certain logic is needed to classify each individual 4x4 block into one of the three cases and assign a MV to the block accordingly. In this document, a method is proposed to simplify such motion vector population process, with each quarter area of a triangle mode CU filled with a same motion vector. Simulation results show that the proposed simplification method leads to {Y, U, V} BD-rate variations of {0.00%, -0.07%, 0.02%} and {0.02%, 0.23%, 0.05%} for RA and LD configurations, respectively.

The proposed change is to store bi-pred MVs not only on the diagonal 4x4 units but also to the whole quarter area of the CU sitting along the partitioning boundary. Therefore, all the 4x4 units inside a quarter shares the same motion information.

RA: 0.00%; LB: 0.02% (higher chroma loss in Class E with 0.56% loss, and peak chroma loss of 1.11% in FourPeople).

A participant commented that the proposed quarter-by-quarter motion storage would introduce mismatch between the motion field that is used for MC and that for deblocking.

It is also commented that the mismatch could potentially impact on the visual quality at CU boundaries.

No action.

JVET-O1034 Crosscheck of JVET-O0601: CE4-related: On motion vector population under triangle prediction mode [H. Wang (Qualcomm)]

JVET-O0629 Non-CE4: Simplification of motion field for triangle partition [R.-L. Liao, Y. Ye, J. Chen (Alibaba)]

In this contribution, the motion field storage for triangle partition mode is simplified. Firstly, 8x8 subblock is used to store motion instead of 4x4 subblock in VVC draft 5. Secondly, the bi-prediction motion generation process for subblocks along the diagonal/anti-diagonal is modified: when both of the uni-prediction motion vectors of the two triangle partitions are from the same reference picture list, the process of converting one of them to another reference picture list is removed. Instead, the motion vector of the bottom triangle partition is stored. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.03%, 0.04%, 0.00%} for RA and {0.04%, 0.17%, -0.05%} for LDB when 8x8 subblock is used. {0.00%, -0.05%, -0.02%} for RA and {0.09%, 0.02%, -0.38%} for LDB when the motion vector of bottom triangle partition is used. {0.02%, -0.02%, 0.01%} for RA and {0.09%, 0.08%, 0.06%} for LDB when both simplifications are applied.

Method 1: Enlarge the unit of motion storage from 4x4 to 8x8 (RA:0.03%/99%/99%;  LB:0.04%/100%/99%)

A non-proponent commented that for larger CUs, this design creates motion field that fits blending process more closely.

With 8x8 unit storage, the number of comparisons to check whether each motion storage unit belongs to partition 1, partition 2 or blending area is reduced from 496 to 120.

It is also commented if a CE is formed to study TPM motion storage related techniques, complexity characteristics (i.e., number of comparisons to determine motion storage) should be reported.

A non-proponent supports adopting method 1 and also commented that the loss of 0.03% is relatively minor against VTM anchor.

Method 2: Identical to JVET-O0265 method 1 (i.e., if both MVs are from the same ref picture list, bottom triangle’s MV is taken) (RA:0.00%/99%/100%; LB:0.07%/99%/99%)

Method 3: Test 1+2 (RA:0.02%/99%/99%; LB:0.09%/99%/99%)

Recommendation: Adopt method 1.

JVET-O0792 Crosscheck of JVET-O0629 (Non-CE4: Simplification of motion field for triangle partition) [Y.-L. Hsiao (MediaTek)]

JVET-O0645 Non-CE4/8: Blending-off Switch for TPM Mode [C.-C. Chen, Y. Zhang, K. Reuze, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)]

(To discuss in CE8 BoG)

JVET-O0912 Crosscheck of JVET-O0645: Non-CE4/8: Blending-off Switch for TPM Mode [W. Zhu (Bytedance)]

JVET-O0646 Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode [C.-C. Chen, K. Reuze, Y. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm)]

(To discuss in CE8 BoG)

JVET-O0763 Crosscheck of JVET-O0646 (Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode) [H. Yang (InterDigital)]

JVET-O0653 Non-CE4: Motion field storage simplification for triangular prediction mode [T. Solovyev, S. Ikonin, E. Alshina (Huawei)]

This contribution describes modification of motion field storage for triangular prediction mode. Alternative motion buffer filling pattern is proposed. The only motion information is used for the whole block except the area adjacent to the block borders. For the area adjacent to the block borders motion buffer filling method is the same as it is in VTM 5.0. The simulation results reportedly show no performance change for RA and 0.07% luma BD-rate loss for LDB configurations.

Non-boundary 4x4 units and the 2 corner 4x4 units along the partitioning boundary are all with bi-pred MVs. The rest are assigned with either one of the triangle partitions’ uni-pred MV.

(RA: 0.00%; LDB: 0.07%)

A non-proponent commented the proposed method still needs to figure out whether uni-pred MV is to be store at the bottom or upper line or to bi-pred MVs are to be store at the corners along the partition boundary.

The proponent claimed that in one implementation is to fill the whole CU with bi-pred MVs and then fill the remaining units close to CU boundary with either the partition 1’s MV or partition 2’s MV, and the benefit is that the number of comparisons as compared to that of TPM can be reduced.

A non-proponent commented that the proposed method may have to access the memory buffer that is used to store motion field twice, firstly for bi-pred MVs to fill all the 4x4 units per CU and secondly to update some of the boundary 4x4 units with either one of the uni-pred motion from partition 1 and 2.

No action.

JVET-O0660 Simplification of MV storage for blending block in triangle mode [K. Panusopone, L. Wang (Nokia)] [late]

This contribution proposes two approaches to simplify MV storage for blending blocks in triangle mode. The first approach eliminates blending blocks and only one MV is stored for all blocks in triangle mode. The second approach always stores one MV when MVs of first and second triangle partition are from the same reference list. The results show that methods 1 and 2 from the first approach have luma BD-rate of approximately 0.04% and 0.03% for RA configuration, respectively. The results also show that methods 3 and 4 from the second approach have luma BD-rate of approximately 0.00% and -0.01% for RA configuration, respectively.

Method 1 is identical to JVET-O0279 method 2. (RA: 0.04% ; LB: 0.17%)

Method 2 refers to bottom triangle’s MV if CU width > CU height and diagonal split. Otherise, it refers to right triangle’s MV.  (RA: 0.03% ; LB: 0.12%)

Method 3 is identical to JVET-O0279 method 1. (RA: 0.00% ; LB: 0.06%)

Method 4: if both MVs share identical ref picture list, apply Method 2. (RA: -0.01% ; LB: 0.07%)

Missing crosscheck.

Further study encouraged.

JVET-O0666 Non-CE4: Fix for missing MV in 4xN and Nx4 triangle partition mode [K. Reuzé, C.-C Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

In triangle partition mode, the current triangle MC doesn’t use the motion information from the motion buffer, which is different from all other modes. It is asserted that this is not good for hardware implementation. It needs to be modified it so that triangle MC can be performed by only using the motion information in motion buffer. However, using the current motion storage design for triangle, there is a corner case (in 4xN/Nx4 block) that doesn’t have both motion information for partition 0 and partition 1.

Results show no coding difference in RA configuration and 0.01% BD-rate loss in LDB configuration.

(Chaired by Haitao Yang)

It is claimed that this proposal can fix a problem in existing TPM where an update of the motion buffer of current coding unit is required. In the current design, bi-directional motion is stored for all 4x4 subblocks inside a 4xN or Nx4 CU using TPM. Without this fix, 1 bit needs to be stored for each 4x4 block to distinguish which motion vector from the bi-directional motion is used for which partition, in one possible implementation. And 128 bytes buffer is required for the storage of this indication flag of the whole CTU. It is claimed this additional 128 bytes could be saved.

It is commented that in another implementation where the motion compensation is performed for each 16x16 block considering pipeline design, the buffer required is 2 bytes.

It is commented that the update of the motion buffer of the current coding unit already exists in the current VTM-5.0. And there is no practical problem if the update is performed for TPM as well.

(Revisited in the morning of July 7, 10:50, chaired by Haitao.)

Results combined with JVET-O0411 method B.

Recommendation: revisit in Track B.

JVET-O1059 Crosscheck of JVET-O0666 (Non-CE4: Fix for missing MV in 4xN and Nx4 triangle partition mode) [F. Henry (Orange), G. Clare (bcom)]

JVET-O0280 CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode [Y.-L. Hsiao, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes single instruction multiple data (SIMD) implementation for the weighted sample prediction process of triangle prediction mode. Compared with VTM5.0, the encoding time is reduced by 2% for RA and 3% for LB without BD-rate changes.

Encoder-only modification to TPM for SIMD implementation on TPM blending operation.

After revisiting in BoG (Revisited 10:40, July 7), there is no further issue spotted after code review by Xiang Li and (cc. to Frank Bossen).

The software implementation is confirmed.

Recommendation (SW): Adopt.

JVET-O0859 Crosscheck of JVET-O0280 (CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode) [T. Hashimoto (Sharp)]

JVET-O1016 Non-CE4: simplified triangle signaling [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (InterDigital)] [late] [miss]

Withdrawn.

(Discussion for TPM storage at 1600, July 4)

(A) If both MVs share same ref pic list, then use right triangle's MV.

RA:0.00%; LB:0.06%

JVET-O0279 method 1 /   JVET-O0378 method 1 /  JVET-O0329 method 1 /  JVET-O0411 Test C /   JVET-O0660 method 3

It is commented that the mapping function from one reference pic list to the other may be just of implementation issue and can be done in slice level, that is, to use mapping tables for TPM CU. How this mapping table is formed is only an implementation-wise issue.

A participant commented that if a certain solution is selected from these solutions, the text in the spec as for TPM can be greatly reduced.

A participant commented that such simplification design (i.e., removing reference list mapping) can save 2 comparisons (i.e., one for mapping from L0 to L1 and the other for mapping from L1 to L0).

A non-proponent suggests to take no action for simplication.

(B) If both MVs share same ref pic list, then use bottom triangle partition's MV.

RA:0.00%; LB:0.07%

JVET-O0265 method 1 / JVET-O0629 method 2 / JVET-O0418 method 1 / JVET-O0329 method 1A /   JVET-O0378 method 2 / JVET-O0411 test B / JVET-O0279 method 1B

The consensus is based on the assumption that this simplification is desirable to select Solution B from Solutions A-C.

It is commented this Solution can simplify the text of the spec significantly.

(C) If both MVs share same ref pic list, then use upper triangle's MV.

RA:0.00%; LB:0.08%

JVET-O0279 method 1A / JVET-O0418 method 2 / JVET-O0329 method 1B / JVET-O0314 / JVET-O0411 Test A

One competing proponent commented that this configuration looks less promising as the upper one is located further away from succeeding CUs and would impact on the performance of spatial mv prediction, which explains why this configuration incurs slightly higher coding loss than others’.

(D) Always use right triangle's MV.

RA:0.04%; LB:0.17%

JVET-O0279 method 2 / JVET-O0660 method 1 / JVET-O0329 method 2

This solution saves 1 additional comparison as compared to Solution A-C; however, according to the loss in LDB, this configuration does not seem to reach an acceptable performance-complexity trade-offs. It is also commented that saving this comparison cannot justify 0.1% loss in LDB case.

(E) If both MVs share same ref pic list, then use the MV with smaller refIdx and if two MVs happen to have same refIdx, then use upper triangle's MV.

JVET-O0217 | RA: 0.00%; LB: 0.16% loss (Class E 0.27% loss, 0.42% loss in Johnny).

This solution actually imposes extra condition check compared with Solution B and the coding loss is the highest among all the solutions.

No action.

(F) If both MVs share same ref pic list, then map bottom triangle’s ref pic list idx to the other (incomplete design)

JVET-O0265 method 2 | RA:0.06%; LB:0.00%

(G) If both MVs share same ref pic list, then:

If non low-delay case, then use bottom triangle partition's MV

Otherwise, map bottom triangle’s ref pic list idx to the other (incomplete design)

JVET-O0265 method 3 | RA:0.06%; LB:0.00%

(H) If same ref pic list and same ref pic POC, then average MVA and MVB.

Otherwise, If same ref list, then use right triangle's MV

JVET-O0378 method 3 | RA:0.02%; LB:0.04%

Compared with Solution B, this solution introduces one more condition check and it also requires to one additional 2 ADD and 2 SHIFT operations to average x and y components.

The increase of complexity does not provide extra coding performance improvement and thus is not justifiable to be consider for adoption.

No action.

(I) If both MVs share same ref pic list and two ref pic lists are identical, then map ref pic list idx to the other

Otherwise, if same ref list, then use right triangle's MV

It is commented from a participant that the reference picture list condition is somewhat overfitting to the Low-Delay configuration in CTC, and thus there is no observable performance-wise difference in LDB case.

JVET-O0378 method 4 | RA:-0.01%; LB:0.00%

Since there is no non-proponent supports this solution, no further action is required.

Although Solution B is generally agreed, there is still one proponent who proposed opposing solution and raised opinion to take no action.

Revisit in Track B.

# Merge mode with MVD (3)

JVET-O0415 Non-CE4: Fix of POC distance condition for MMVD [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In this contribution, two simplified MVD derivation methods for MMVD are introduced by restricting the usage of the POC difference. In method 1, using the POC difference is restricted when a current block refers any long-term reference picture. In method 2, using the POC difference is always restricted. For method 1, the experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC. For method 2, the experimental results reportedly show 0.01%(0.06%) BD-rate changes with 100%(100%) and 100%(101%) encoding and decoding run-time compared to VTM5.0 as anchor in RA(LB) configurations.

Method 1 performs LTRP check first before POC distance check.

It is commented that as compared with VTM, there is one missing state (i.e., distance offset assigning to mvdL1). In some encoding scenario where LTRP is used, the impact on coding performance is a concern.

Method 2 always assigns distance offset to L0, and then performs distance offset scaling (if not long-term refPic).

The benefit of this method is that it removes 1 condition check that is used to determine which reference pic is with lower POC distance relative to the current picture.

RA: 0.01% / LB: 0.06% / no obvious encoding and decoding time impact from the saving of 1 condition check.

Since there is no non-proponent support further studying the proposed methods, no action.

JVET-O0868 Crosscheck of JVET-O0415 (Non-CE4: Fix of POC distance condition for MMVD) [N. Zhang (Bytedance)]

JVET-O0420 CE4-related: Reduce scaling operation number for MMVD [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

This contribution proposes to remove the scaling process in MMVD when the starting MV is a temporal motion vector predictor (TMVP), which can reduce worst case number of scaling operation at decoder side when MMVD is enable and keep same scaling operation number for TMVP at both MMVD on and off.

Experimental results show that the proposed method has no performance impact on VTM-5.0 anchor.

According to the proposed method, if the base vector of MMVD is inferred from TMVP merge candidate, then:

-- Distance offset is copied if both reference pics are forward reference pics (or backward ref pics);

-- Distance offset is flipped if only one reference pic is forward ref picture

Result is bit-exact as the VTM anchor.

It is commented that as the the decoder requires 1 additional check to differentiate TMVP from others.

However, the benefit is that the proposed methods remove the distance offset scaling process in the case that TMVP is selected to be the base motion vector.

It is questionable which of the options is more complicated: (1) the proposed method, (2) disallow TMVP from MMVD.

It is agreed that in average case, the proposed method could benefit to decoder complexity.

It is commented that design inconsistency between TMVP and other merge candidates for MMVD is introduced, which may not be desirable. In addition, such inconsistency is not translated to coding-performance benefit.

No action.

JVET-O0761 Crosscheck of JVET-O0420 (CE4-related: Reduce scaling operation number for MMVD) [H.-J. Jhu (Kwai Inc.)]

JVET-O0633 Non-CE4: On MMVD candidate flag coding [X. Chen, H. Yang (Huawei)]

This contribution studies the performance impact on mmvd_cand_flag using bypass coding. Experiments were conducted on top of the VTM-5.0 with Random-Access testing condition. The performance results are summarized as follows.

RA: (Y) 0.00%, (U) 0.00%, (V) -0.02%, (Enc.) 100%, (Dec.) 102%.

LB: (Y) -0.02%, (U) 0.12%, (V) -0.10%, (Enc.) 100%, (Dec.) 101%.

The proposed method is to replace the context-coded bin by using bypass coding for mmvd_cand_flag.

(RA: 0.00%; LDB: -0.02%).

The benefit is saving one context-coded bin.

The major gain is from Class E in LDB (0.08% gain, but also incurring a peak loss of 0.59% in chroma)

It is commented that this may not be the right timing to start working on context-wise fine-tuning.

No action.

JVET-O0927 Crosscheck of JVET-O0633 (Non-CE4: On MMVD candidate flag coding) [T. Hashimoto, T. Ikai (Sharp)]

# Combined intra and inter prediction (6)

JVET-O0384 CE4-related: CIIP Simplification [J. Y. Lee (Sejong Univ.), W. Lim, G. Bang, H. Y. Kim (ETRI)] [miss] [late]

At the last meeting, the number of intra prediction modes in CIIP is reduced to one. This contribution further simplifies the CIIP mode by simply reducing the number of merge candidates from six to two. Similar to MMVD, the proposed method only checks the first and second Merge candidates, and then signals a flag to indicate what Merge candidate is combined with the intra mode in CIIP. The experimental result shows that the proposed simplification has negligible loss.

The proposed method is to apply CIIP only to the first 2 merge candidates.

(RA: 0.03%; LDB: 0.05%; from cross-checker’s partial result, the encoding time reduction is around 1%)

The proposed method removes the parsing of merge cand idx when CIIP flag is on, and in addition, a ciip_cand_flag is introduced to indicate which of the 2 merge candidates is selected.

It is questionable whether the proposed method can be simulated by an encoder-only change which enforces encoder to test only the first 2 merge candidates for CIIP.

There is no non-proponent expressing interest to this proposed method.

Crosscheck report: JVET-O1022.

It is encouraged further study outside a CE.

JVET-O1022 Crosscheck of JVET-O0384 (CE4-related: CIIP Simplification) [K. Choi (Samsung)]

JVET-O0421 Non-CE4: On CIIP Merge mode with no residual [G. Laroche, C. Gisquet, J. Taquet, P. Onno (Canon)]

This contribution presents a modification of the CIIP Merge mode. With the current VVC specification, it is not possible to signal a CIIP mode without residual. This contribution proposes to signal a CIIP Merge mode without residual thanks to a cu_cbf flag.

It is reported that the proposed modification obtains the average BDR results of 0.04% 0.00%, for respectively RA and LD configurations compared to VTM5.0 without modification of the CABAC initialisation context values.

The propose method allows CIIP to signal root_cbf_flag in the case when no residual exists for the current CU.

It is noted that in VTM, CIIP is not allowed for merge skip mode.

(RA: 0.04%; LDB: 0.00%)

It is commented that the introduced root_cbf_flag for CIIP merge mode is not translated to coding performance-wise benefit.From past analysis in CE10 (JVET-L0100), it is reported that CIIP skip mode incurs performance drop as compared with CIIP without skip.

No action.

JVET-O1083 Crosscheck report of JVET-O0421 (Non-CE4: On CIIP Merge mode with no residual) [A. Filippov, V. Rufitskiy (Huawei)]

JVET-O0522 Non-CE4: CIIP using triangular partitions [S. Blasi, G. Kulupana, A. Seixas Dias (BBC)]

In this contribution, it is proposed to allow usage of triangular partitions in Combined Inter-Intra Prediction (CIIP). As in conventional CIIP, an intra- and inter-predicted predictions are combined to form a prediction for the current block. The combination between the two predictors is performed following the partitioning scheme used in the Triangle mode. The intra predictor is used in one of the two triangular partitions, and the merge inter-predictor is used in the other triangular partition. Three possible Triangle CIIP modes are considered with different triangle split and/or usage of inter or intra prediction in the two partitions. Usage of the specific Triangle CIIP mode is explicitly signalled in the bitstream as an alternative to conventional CIIP. Results show the approach achieves XX% gains in RA configuration for 101% encoder complexity and no additional decoder complexity. Particularly significant gains are obtained for Class C, where average -0.21% gains are obtained. Gains of XX are obtained in the LD-B configuration.

An additional variant of the method is also proposed where a mode different than Planar may be used in the intra-prediction process. Such mode is inferred from the neighbouring blocks and does not require any additional signalling. Results show the approach achieves -0.10% gains in RA configuration for 102% encoder complexity and no additional decoder complexity.  Gains of XX are obtained in the LD-B configuration.

Method 1:

Process-wise, the proposed mode is more close-related to CIIP but not TPM. The complexity overhead as compared with CIIP is the proposed mode requires to not blending the whole prediction samples of inter and intra, but instead, allocate each of them to respect triangle partition.

In addition, the same TPM blending is required for this mode. This aspect is later commented that this may not incur extra decoding burden as CIIP mode already require similar blending operation for the whole CU.

For the triangle that is intra-coded, only planar pred is used.

There are only 3 modes (instead of 4) for each TPM partitioning pattern, because this proposal does not allow intra pred to be used at bottom-right triangle.

The mapping from syntax to the 3 modes is adaptive depending on whether left and above neighbor blocks are intra or inter coded. In total, 4 different combinations.

(RA: -0.08%; Class C LDB: -0.18%)

It is suggested to add checking of sps_triangle_enable_flag for this mode.

It is commented that it is questionable whether the increase of decoder complexity (i.e., adding one new mode) could justify the improvement of coding improvement.

It is commented that whether the filter used in the TPM blending process is strong enough to filter the boundary between the intra predicted triangle and inter predicted triangle. As this would potentially introduce visual impact on this mode, subjective viewing is encouraged.

Three non-proponents express interest in this proposal and suggest further study in a CE.

Method 2:

Method 1 + the triangle coded by intra mode is no longer planar but is inferred from neighboring CU’s intra mode (if exists).

(RA: -0.10%; LDB: ???)

The extra gain of 0.02% that is provided from the adaptive intra mode selection seems consistent over all class of sequences.

Additional method: It was with 3 mode for the proposed TPM+CIIP mode and is reduced to only 2, showing minor impact on encoder runtime and no impact on performance as compared with method 2.

Recommendation: Further study method 1 in CE.

JVET-O1029 Crosscheck of JVET-O0522 (Non-CE4: CIIP using triangular partitions) [I. Zupancic (HHI)]

JVET-O0537 Non-CE: Weighted intra and inter prediction mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

(Discussed in Track)

JVET-O0778 Crosscheck of JVET-O0537 (Non-CE: Weighted intra and inter prediction mode) [Y.-W. Chen (Kwai Inc.)]

JVET-O0661 Non-CE4/8: Simplification of CIIP on IBC mode [Y. Zhang, C.-C. Chen, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)]

Withdrawn

JVET-O0681 CE9-related: Disabled DMVR, BDOF and BCW for CIIP [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

(Chaired by Xiang Li)

In the Core Experiment (CE) 9-2.2 on complexity reduction on combined inter and intra prediction (CIIP), it’s tested to disable decoder-side motion vector derivation (DMVR) and bi-directional optical flow (BDOF) when generating the inter prediction samples in the CIIP mode. In this contribution, it’s proposed to further disable Bi-prediction with CU-level weight (BCW) when generating the inter prediction samples in the CIIP mode. The average BD-rate change of the proposed method is {-0.01%, -0.08%, -0.08%} for RA and {0.03%, 0.09%, -0.32%} for LDB, respectively. The average BD-rate change of the proposed method 2 is {-0.01%, -0.07%, -0.04%} for RA and {-0.02%, 0.11%, 0.04%} in LDB, respectively.

It is proposed to disable BCW by reset the weighting to equal, and 1) reset BCW to equal weight, 2) inherit BCW index.

Performance: 1) -0.01% RA, 0.03% LDB (luma), 2) -0.01% RA, -0.02% LDB, no runtime impact observed.

It is claimed the benefit is software decoding complexity.

It is commented that conceptually not necessary to further weight inter as inter and intra will be combined with weighting.

It is asked whether we should do weighted prediction for CIIP. It is commented that weighted prediction is somewhat different as it is for fading and there is an offset in weighted prediction.

Two participants suggest adoption of method 2.

Recommendation: Adopt method 2 (inherit BCW index).

JVET-O0952 Crosscheck of JVET-O0681 (CE9-related: Disabled DMVR, BDOF and BCW for CIIP) [Y.-W. Chen (Kwai Inc.)]

# Merge list construction (3)

JVET-O0285 CE4-related: On merging candidate list construction [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to replace the MaxNumMergeCand variable with a fixed value equal to six only during merging candidate list construction. In addition, it is proposed to change the history-based motion vector predictor (HMVP) table size from the variable (MaxNumMergeCand − 1) to five. It is claimed that the hardware decoder can be simplified by removing the support of the variable derivation process. No BD-rate change is reported for the common test conditions (CTC).

Propose to enforce MaxNumMergeCand to be always 6 only in decoding process and no change to syntax signalling. (No experiment is required as this does not affect CTC cases)

The proponent claims this a bugfix since the decoder does not need to check whether the number of candidates has reached the maximal number.

It is requested to provide additional results on the configuration of MaxNumMergeCand=3,4,5 in addition to 2.

It is questioned on the cost of this additional check to this check. The proponent responds that the circuit design requires additional logic to accommodate different configurations of max merge cand numbers.

Revisited in BoG (1050, July 7) to review the additional results with MaxNumMergeCand 3, 4 or 5.

Revisit in Track B. Review results with MaxNumMergeCand 3, 4 or 5.

JVET-O0879 Crosscheck of JVET-O0285 (CE4-related: On merging candidate list construction) [H. Gao (Huawei)]

JVET-O0501 Non-CE4: Temporal merge modes simplification [F. Galpin, F. Le Léannec, A. Robert, T. Poirier (InterDigital)]

This contribution reports the result of simplifying the temporal merge modes construction by isolating the temporal predictors in a specific list. A second test also removes some temporal candidates to simplify further the decoding process. The results show a loss of 0.03% in RA and 0.22% in LDB for test 1, and 0.07% for RA and 0.33% for LDB for the second test.

Test 1 pull out sbTPMV from sub-block merge list in another merge candidate list which contains only sbTMVP.

(RA: 0.03% loss; LDB: 0.22% loss)

One new flag is introduced to differentiate the selection of subblock cand list and the new can list.

- The proposed method is to moves a certain bit that indicates the candidate index of sbTMVP to an upper signaling level as that for subblock_merge_flag.

- The proposed method also reduces the max number of subblock candidate by 1.

Considering that the syntax and semantics cannot be translated to encoding performance-wise benefit, no action.

Test 2: Test 1 + Move regular TMVP to this new candidate list. First is sbTMVP and the second is TMVP in the new cand list.

Additionally, disable CIIP/MMVD for the TMVP candidate.

JVET-O0904 Crosscheck of JVET-O0501 (non-CE4: temporal merge modes simplification) [H. Huang (Qualcomm)]

JVET-O0589 Non-CE4: Simplification of HMVP in merge list construction [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

In current VVC, the MV redundancy checking is applied to the first two HMVP candidates against the left and above spatial candidates (up to 4 MV redundancy checking) when inserting the HMVP candidate into the regular merge list. It is proposed to apply the MV redundancy check to only the first HMVP candidate. Moreover, the insertion order of HMVPs from the HMVP table is also changed to a reverse order (i.e. insertion from the first to latest entry HMVP) because the HMVPs which is located closer to the first entry has less possibility to be identical to the spatial neighboring MVs.

Besides, when inserting HMVP candidates into the regular merge list, there is one check to see whether the total number of available merge candidates reaches the maximally allowed merge candidates minus one. If the size reaches the maximum size minus one, the merge candidate list construction process from HMVP is terminated. The motivation of this is to keep at least one entry for the pair-wise candidates for better coding efficiency. This contribution presents a method to remove this check on HMVP candidates with minor impact to the coding efficiency.

The results reportedly show that the first method introduces 0.06% (RA), 0.02%(LDB) BD-rate changes while the second method introduces -0.03% (RA), 0.04%(LDB) BD-rate changes against VTM-5.0. The combination of the two methods introduces 0.01% (RA), 0.03%(LDB) BD-rate changes against VTM-5.0. With the proposed methods, the number of MV redundancy checking is reduce from 4 to 2 and the check on the number of available merge candidates when inserting HMVPs can be removed.

In VTM, when inserting HMVP (at first 2 entries) into merge candidate list, pruning check at left and above spatial candidates are required (i.e., up to 4 comparisons).

Method 1:

Replace the mentioned “first 2 entries” by “first entry only” (i.e., up to 2 comparisons)

The accessing order of HMVP is changed to “from the oldest to the latest”.

(RA: 0.06%; LDB: 0.02%)

It is commented what is the performance change of “from the oldest to the latest”. The propoenet says only less than 0.1% loss form their internal results.

One non-proponent expresses no interest in this method.

Method 2: Move the inserting position of HMVP to the one after pairwise candidate.

It is noted that in the spec, HMVP candidate cannot be inserted to the last entry of merge candidate list.

With this change, when inserting HMVP into merge list, the check of “last entry” can be avoided.

(RA: -0.03%; LDB: 0.04%)

It is commented that in the merge candidate list generation, the decoder still has to generate the whole merge candidate list. Therefore, the worst-case complexity remains the same.

It is also commented that pairwise merge candidate is less performing than HMVP, so moving HMVP to a position after pairwise may bring negative impact on coding performance of merge modes in some cases (e.g. LDB).

It is also commented that in the current design, the decoder only has to check whether the number of derived merge candidates reaches maximum size minus 1 before pairwise cand is inserted.

As the benefit form the change seems unclear, it is suggested to take no action on this.

Method 3: Method 1 + 2

(RA: 0.01%; LDB: 0.03%)

JVET-O0698 Crosscheck of JVET-O0589 (Non-CE4: Simplification of HMVP in merge list construction) [Y.-H. Chao (Qualcomm)]

# Merge mode syntax (18)

JVET-O0220 Non-CE4: On maximum number of subblock merge candidates [R. Yu, D. Liu (Ericsson)]

In VTM-5.0, subblock merge candidate list contains subblock temporal motion vector prediction (SbTMVP) candidate and affine candidates. For each inter (P/B) slice, a slice level variable MaxNumSubblockMergeCand is derived to control the length of the subblock merge candidate list. It is asserted that there are following two issues with regards to the MaxNumSubblockMergeCand. Suggested modifications are also presented.

- In the VTM-5.0 reference software, the parsing of merge_subblock_flag is conditioned on SPS enabling flags for SbTMVP and affine, i.e. sps_sbtmvp_enabled_flag and sps_affine_enabled_flag. Given the VTM-5.0 syntax, it can have MaxNumSubblockMergeCand = 0 for some slices. For those slices, there would be no presence of subblock merge candidates, which would make the merge_subblock_flag always 0.

It is suggested to change the VTM-5.0 software to make the parsing of merge_subblock_flag conditioned on MaxNumSubblockMergeCand instead.

- The MaxNumSubblockMergeCand is inferred to 1 when the SPS enabling flags sps_affine_enabled_flag = 0 and sps_sbtmvp_enabled_flag = 1. In this case, the intention is let the subblock merge candidate list contain one SbTMVP candidate. However, the SbTMVP candidate generation process also follows the slice_temporal_mvp_enabled_flag. When the slice_temporal_mvp_enabled_flag is 0, the slice disallows any temporal motion vector prediction. In other words, for the combination of sps_affine_enabled_flag = 0, sps_sbtmvp_enabled_flag = 1 and slice_temporal_mvp_enbaled_flag = 0, the subblock merge candidate list would only contain one less meaningfull zero motion candidate.

It is proposed to set the five_minus_max_num_subblock_merge_cand to be 5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag) when five_minus_max_num_subblock_merge_cand is not present.

Method 1: Propose sw bugfix to parse merge_subblock_flag conditioned on MaxNumSubblockMergeCand

In the sepc text, merge_subblock_flag is parsed conditioned on MaxNumSubblockMergeCand>0; however, the sw refers to sps_sbtmvp_enabled_flag and sps_affine_enabled_flag to determined whether to parse merge_subblock_flag or not.

Method 2: Same problem as JVET-O0328 Test 1 raised. The proposed fix solves the ambiguity in both spec and sw.

“when sps_affine_enabled_flag=0 and sps_sbtmvp_enabled_flag=1, there is one subblock merge mode, that is, sbTMVP. However, the problem is sbTMVP may not be always available, and thus a zero-MV affine candidate is used as a subblock merging candidate, which causes ambiguity.” -- quoted from JVET-O0328 Test 1.

In addition to sps_sbtmvp_enabled_flag, SbTMVP can be turned off by slice_temporal_mvp_enabled_flag.

Assuming five_minus_max_num_subblock_merge_cand is not present (i.e., sps_affine_enabled_flag=0):

MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand

= 5 – (5 − sps_sbtmvp_enabled_flag)

= sps_sbtmvp_enabled_flag

The method replaces the inference scheme by using “5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag)”.

(JVET-O0263 is identical to method 2)

Recommendation (BugfixSW): Adopt the bugfix in method 1 to align sw to spec text.

Recommendation (Bugfix): Adopt the bugfix in method 2.

JVET-O0221 Non-CE4: On merge data syntax [R. Yu, D. Liu (Ericsson)]

In the merge data syntax of VTM-5.0, the signalling order of different merge alternatives is regular merge, MMVD, subblock merge, CIIP and triangle merge. It is asserted that there are two drawbacks with such design: 1) The merge alternatives such as MMVD, subblock merge, CIIP and triangle can all be turned off either on a sequence level or on a slice level. With the existing VTM-5.0 syntax, redundant bits would be spent for the merge alternatives that precede the alternatives that get disabled. 2) The behavior of decoding is not properly defined when all the merge alternatives are signalled to be off for a CU. The proposal discusses around these drawbacks and presents several options for consideration.

Method 1: Impose additional condition checks to avoid redundant syntax.

(LDP: -0.22%/-0.20%/-0.26%/102%/116%)

This method is a subset of JVET-O0586.

It is commented that this proposed method partially solves the whole parsing redundancy.

Method 2: Impose bitstream conformance constraint on the CU-level flag (i.e., general_merge_flag), so that one of the merge type flag should be 1 when general_merge_flag=1.

Method 3: Move regular merge mode to the end of the merge data syntax.

Same as JVET-O0305 method 3

(RA:  0.03%/-0.02%/-0.08%/100%/107%)

(LDB: 0.14%/0.30%/0.32%/100%/118%)

JVET-O0249 Non-CE4: Merge Modes Signaling [H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

In the current VTM, there are five merge modes including: the regular merge mode, the merge mode with motion vector difference, the subblock-based merge mode, combined inter and intra prediction mode, and triangle partition mode.  The merge mode signaling follows the order described. In this contribution, two changes are proposed. First, it’s proposed to disable triangle merge prediction mode for a CU that has width or height larger than 64. Second, it’s proposed to group the merge modes in to 3 categories: 1) subblock-based merge mode, 2) regular merge mode and merge mode with motion vector difference, 3) combined inter and intra prediction mode and triangle merge prediction mode. Then a first bin is used to indicate whether the first group is used, and if the first bin is 0, a second bin is used to decide between second group and third group. For the second and third groups, one bin is used to specify the selected mode. The Y, U, V BD-rate performance for RA and LDB under common test conditions respectively are {-0.08%, -0.12%, -0.19%} and {-0.06%, 0.09%, 0.02%}. In the high QP test, the BD-rate performance for RA and LDB are {-0.24%, -0.35%, -0.39%} and {-0.38%, -0.55%, -0.65}, respectively. For the first change only, the BD-rate performance for RA and LDB are {-0.02%, 0.00%, 0.00%} and {0.03%, 0.07%, -0.10%}, respectively.

(Chaired by Haitao Yang)

It is noted that the proposal is composed of two aspects, the first one is about TPM restriction and the second is about merge mode signaling.

Separate results for the aspect 2 on merge syntax cleanup (no aspect 1 on TPM restriction) is requested.

It is asked whether the proposed merge syntax structure would increase number of bins since the most frequently used merge mode is moved to the end of the parsing procedure.

It is commented there is observable coding gain in high QP configuration, which may indicate that the most frequently used merge mode may not be regular merge mode. No statistics is shown.

It was asked the impact on the throughput of entropy coding. One participant commented that the worst case throughput is not changed.

Merge mode distribution data is requested and be compared with VTM-5.0. It is commented that this distribution statistics should be provided for all the proposals for comparative study. It is clarified that the statistics include the CU size and the merge mode.

It is commented that the syntax table of the proposed method is cleaner than the current design.

Low QP results are requested by one participant.

It is commented that the clean up of the current merge syntax is important. Coding gain along with the cleanup is welcomed.

Revisit when the separate compression performance results and the merge mode distribution statistics for the proposed merge syntax cleanup results are available.

Statistical result to be presented in Track B. (No revisit required in BoG)

JVET-O0728 Crosscheck of JVET-O0249 (Non-CE4: Merge Modes Signaling) [R.-L. Liao (Alibaba)]

JVET-O0263 Non-CE4: Cleanups on syntax design for sub-block coding tools [K. Zhang, L. Zhang, H. Liu, N. Zhang, Z. Deng, Y. Wang (Bytedance)]

(Identical to JVET-O0220)

JVET-O0278 CE4-related: On signalling of merge flags [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The current merge syntax design may suffer decoder crash and redundant merge flag signalling outside common test conditions (CTCs). This contribution proposes to check the availability of all the following types of merge modes before signalling current type of merge mode flag. With this method, one default type of merge mode is guaranteed and redundant flag signalling can be removed when all the following types of merge modes are not available. The proposed fix has no impact to CTCs, and results of a non-CTC setting are summarized as follows.

Proposed method over VTM5.0 when Triangle Prediction Mode (TPM) is disabled:
VTM5.0-RA: {Y-BD-rate = -0.20%, EncT = 99%, DecT = 100%}
VTM5.0-LB: {Y-BD-rate = -0.22%, EncT = 99%, DecT = 100%}

Propose to check the availability of remaining merge modes (after the current one) before parsing the mode flag of the current merge mode.

If none is available, the current merge type flag need not be parse and is inferred to be 1.

The benefit is two-fold: (a) it avoids the problem that no merge mode is valid, (b) it also avoids redundant merge mode flag signaling.

(RA:-0.20%; LDB: -0.22%) Note: both CTC and test disable TPM.

JVET-O0431 / JVET-O0586 / JVET-O0309 / JVET-O0585 method 1 / JVET-O0534/ JVET-O0361.

The proponent of JVET-O0534 clarifies that it introduces additional encoder-only change and thus results in 0.01% BD-rate difference as compared to  JVET-O0278. Normative part is identical to JVET-O0278.

JVET-O0828 Crosscheck of JVET-O0278 (CE4-related: On signalling of merge flags) [H. Chen (Huawei)]

JVET-O0305 Non-CE4: Undefined condition of merge mode syntax [K. Abe, T. Toma, J. Li, V. Drugeon (Panasonic)]

This contribution proposes to modify the syntax structure of merge data. regular_merge_flag was introduced into VTM5.0 at the previous meeting, however, this modification brings an undefined condition when none of the merge modes are selected. This contribution presents three solutions for this issue: (1) the block is coded as regular merge mode when none of the merge modes are selected; (2) the block is coded as fallback merge mode when none of the merge modes are selected; (3) revert to VTM4.0 syntax structure. Simulation results reportedly show that (1) and (2) have no bitstream difference with VTM5.0, (3) has 0.03% loss for RA and 0.14% loss for LDB. And (1) has the least amount of change for the VVC specification text.

Method 1: When none of merge modes is available, the proposed method introduces a fallback mode that is regular merge.

In addition, merge_idx is signalled after all merge mode decisions.

(no performance change)

It is commented that the regular merge mode can be represented in 2 different way: one is with regular_merge_flag=1 and the other is the proposed method, which is considered desirable behavior.

No action.

Method 2: Method 1 + change fallback mode, that is, the fallback merge mode is with uni-pred motion RefIdxL0=0 and MVL0=(0,0)

(no performance change)

As when the fall back mode condition is satisfied, the decoder behavior is the same as create a new merge cand list which contains only one zero mv cand, which does not seem to be a deriable way to fix this issue.

No action.

Method 3: Revert to VTM4.0 syntax structure. Specifically, remove regular_merge_flag and bring back merge_triangle_flag. The regular merge mode is determined when merge_triangle_flag=0.

(RA: 0.03%; LDB: 0.14%)

(Same as JVET-O0221 method 3)

JVET-O0309 Non-CE4: Merge mode signalling overhead reduction [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

(Identical to JVET-O0278)

JVET-O0829 Crosscheck of JVET-O0309 (Non-CE4: Merge mode signalling overhead reduction) [H. Chen (Huawei)]

JVET-O0330 Non-CE4: Syntax element ‘regular_merge_flag’ signalling [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

(The proponent claimed presentation is not necessary.)

JVET-O0770 Crosscheck of JVET-O0330 (Non-CE4: Syntax element ‘regular_merge_flag’ signaling) [Y.-W. Chen (Kwai Inc.)]

JVET-O0361 Non-CE4: Remove the redundancy among the flag coding for merge modes [F. Chen, L. Wang (Hikvision)]

(Identical to JVET-O0278)

JVET-O1008 Crosscheck of JVET-O0361 (Non-CE4: Remove the redundancy among the flag coding for merge modes) [J. Choi (LGE)]

JVET-O0422 Non-CE4: On mode flags signalling for Merge [G. Laroche, C. Gisquet, J. Taquet, P. Onno (Canon)]

This contribution presents a modification of Merge flags coding. The target of this modification is that each code word of the Merge modes flags corresponds to one available Merge type. In the current VVC specification, some Merge flags are transmitted when one or more Merge type is disabled at SPS, slice or block level.

In this contribution, the Skip mode is available only for the Regular and IBC Merge mode. The non-residual SubBlock, MMVD and Triangle Merge modes are signalled as a Merge mode with a cu_cbf equal to 0. For the Merge mode, the Merge list is signalled as the previous VTM-4.0.

With the proposed modification, each Merge flag corresponds to its own availability (SPS, Slice or block size).

It is reported that the proposed modification obtains the average BDR results of 0.03% 0.11%, -0.23% for respectively RA, LD and LDP configurations compared to VTM5.0 without modification of the CABAC initialisation context values.

When the Triangle Prediction Mode is disabled, -0.16% and -0.21% BDR gains are obtained for respectively RA and LD configurations. And when all Merge modes are disabled, the proposed modifications obtain -0.11%, -0.25% and -0.24 BDR gains for respectively RA, LD and LDP configurations.

This proposal imposes a constraint on merge modes that it only allows skip mode for IBC and regular merge mode.

For those merge modes which does not have skip flag support, root_cbf_flag is introduced.

This proposal also moves the signaling of regular merge mode flag to the end of merge mode signaling for non-skip merge.

Test 1: as above (RA: 0.03%; LDB: 0.11%; LDP: -0.23%)

The gain from LDP is actually from a VTM’s bug and thus the gain from the proposal method is unclear.

The proponent stated that the coding loss in RA/LDB is caused by not well-trained CABAC context models.

It is commented to fix issue and further study is encouraged.

Test 2: (Anchor+TPMoff vs Test+TPMoff)

Test 3: CTCs + TPM/Affine/SubBlock/MMVD/CIIP disabled.

It is commented that the gain of Test 2 & 3 is from removing redundant signalling that is from disabling some merge modes.

JVET-O1084 Crosscheck report of JVET-O0422 (JVET-O0422 Non-CE4: On mode flags signalling for Merge) [A. Filippov, V. Rufitskiy (Huawei)]

JVET-O0431 Non-CE4: On Merge data signaling [H. Chen, X. Chen, H. Yang (Huawei)]

(Identical to JVET-O0278)

JVET-O1025 Crosscheck of JVET-O0431 (Non-CE4: On Merge data signalling) [Z.-Y. Lin (MediaTek)]

JVET-O0500 Non-CE4: Affine and sub-block modes coding clean-up [F. Galpin, F. Le Léannec, A. Robert, T. Poirier (InterDigital)]

This contribution reports the result of cleaning up the coding of the affine and sub-block mode. The goal is to make the coding of the sub-block and affine flag mode consistent between the syntax and the CABAC coding.

The average luma BD-rate are -0.05% and 0.01% for the RA and LDB configurations, respectively for test1. The average luma BD-rate are -0.03% and a 0.03% for the RA and LDB configurations, respectively for test2.

Method 1: Add additional context for merge_subblock_flag. (It is noted that in VVC, merge_subblock_flag and inter_affine_flag share the same context.)

(RA: -0.05%; LDB: 0.01%)

The cross-checker commented that the proposed methods requires introducing more context than 1.

It looks there is certain misalignment between sw and spec; however, this is caused by CtxIdxOffset table is undefined, and thus it becomes unclear whether having a shared not non-shared context between merge_subblock_flag and inter_affine_flag is intended. (Refer to a undefined table as in Table 9-5 in JVET-N1001_v10)

This proposal proposes a method to eliminate the ambiguity between spec text and sw by introducing separate context model for merge_subblock_flag and inter_affine_flag.

A proponent of subblock merge list commented that sharing the context model for merge_subblock_flag and inter_affine_flag is the intended design as in spec text.

The cross-checker supports having this separate context models for merge_subblock_flag and inter_affine_flag in spec text.

It is also commented to not have this change in the current standard developing stage due to that the impact is relatively minor to justify this change.

The cross-checker asked to revisit this method in Track B.

Tie break.

There is no consensus to revisit this method in Track; however, it depends on the proponent’s or non-proponents’ decision to raise this issue in Track.

Method 2: Method 1 + Remove affine context derivation.

(RA: -0.03%; LDB: 0.03%)

The cross-checker commented that this method reduces context by 2 and reduces dependency between the current CU and neighboring blocks’ affine flags to derive context modeller.

The cross-checker commented that the compression benefit seems not sufficient to take this action.

No action on this method.

JVET-O0903 Crosscheck of JVET-O0500 (non-CE4: affine and sub-block modes coding clean-up) [H. Huang (Qualcomm)]

JVET-O0534 Non-CE4: Redundancy removal for merge mode signaling [J. Ye, X. Li, S. Liu (Tencent)]

(Identical to JVET-O0278)

JVET-O0560 Merge Mode Syntax [L. Wang, K. Panusopone (Nokia)]

This contribution proposes possible changes for merge mode syntax. Option 1 removes the last condition check in merge_date(), option 2 moves regular merge mode to the end of merge_data(), and option 3 adds regular merge mode as the last merge mode.

Propose to fix the issue that no merge mode is available when general_merge_flag=1.

Method 1: Remove the last condition check of MergeTriangleFlag. Thus, if other merge mode is not available, the CU has to be coded in triangular mode.

If only regular merge mode is sps-level indicated, the method still requires send a regular_merge_flag, which redundant.

Method 2: Same as VTM-4.0.

(identical to JVET-O0305)

Method 3: Add regular merge mode to the end of merge mode checks as a default merge mode.

JVET-O0585 Non-CE4: Merge syntax for removing redundant syntax signaling or fall back merge mode [H. Jang, N. Park, J. Nam, S. Kim, J. Lim (LGE)]

Merge data syntax in current VVC is being signalled redundantly and has potential problem with decoder crash for non-common test conditions. In this contribution, two solutions are proposed to remove the above mentioned issues.

Method 1: propose updated conditions for each merge syntax signaling to avoid redundant signaling and invalid (undefined) mode

Method 2: fall back merge mode is defined to avoid undefined mode as follows.

a. Merge candidate index 0 without syntax signaling

b. Zero motion without syntax signaling

The method 1 shows -0.20% and -0.22% BD-rate changes for RA and LDB respectively when triangle prediction is disabled. The experimental results of fall back mode by the method 2-a and method 2-b show 0.00%, 0.00% and 0.00% BD-rate change over VTM-5.0.

Method 1: identical to JVET-O0278.

Method 2a: fallback merge mode to set regular merge mode with candidate index 0 without any syntax signaling.

Method 2b: fall back merge mode with zero motion and ref idx equal to 0.

JVET-O0973 Crosscheck of JVET-O0585 (Non-CE4: Merge syntax for removing redundant syntax signaling or fall back merge mode) [M.-S. Chiang (MediaTek)]

JVET-O0586 Non-CE4: Redundancy removal in merge modes signaling [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, S. Ye, X. Wang (Kwai Inc.)]

(Identical to JVET-O0278)

JVET-O0697 Crosscheck of JVET-O0586 (Non-CE4: Redundancy removal in merge modes signaling) [Y.-H. Chao (Qualcomm)]

JVET-O0639 Non-CE4: Context reduction for merge mode [J. Ye, X. Li, C. Auyeung, S. Liu (Tencent)]

(The proponent claimed this need not be presented)

JVET-O0911 Crosscheck of JVET-O0639: Non-CE4: Context reduction for merge mode [W. Zhu (Bytedance)]

JVET-O0795 CE4-related: On the context variables and syntax elements of merge index [?? (Mediatek)] [miss] [late]

This contribution proposes to simplify the context variables and syntax elements of merge index. Compared with VTM5.0, in method 1, only one context variable is used for merge index syntax elements. In method 2.1, by moving merge_triangle_split_dir after merge_triangle_idx1, the number of switching between contexts coded bins and bypass coded bins for triangle prediction mode (TPM) merge data syntax is reduced from four to three. In method 2.2, the first bins of merge_triangle_idx0 and merge_triangle_idx1 are first signalled and followed by the remaining bins of merge_triangle_idx0 and merge_triangle_idx1 and merge_triangle_split_dir. The simulation results are reporeted as follows.

Method 1:
VTM5.0-RA: {Y-BD-rate = 0.05%, EncT = 99%, DecT = 98%}
VTM5.0-LB: {Y-BD-rate = 0.05%, EncT = 100%, DecT = 99%}

Method 2.1: Move merge_triangle_split_dir after merge_triangle_idx1. Group bypass bins of triangle_idx 1 and merge_triangle_split_dir.

Method 2.2: Group bypass bins from TPM cand idx.

It is commented that the benefit from grouping bypass bins is unclear.

Revisit: Track B. (Solicit comments from hardware experts) This contribution is to be revisited in Track B with JVET-O0456 in the category of CE4 BoG on general inter prediction.

JVET-O0849 Crosscheck of JVET-O0795 (CE4-related: On the context variables and syntax elements of merge index) [R.-L. Liao (Alibaba)]

(Discussion on merge syntax redundancy solutions at 10:15, July 7, chaired by Chun-Chi)

(A) Impose bitstream conformance constraint on the CU-level flag (i.e., general_merge_flag), so that one of the merge type flag should be 1 when general_merge_flag=1.

(JVET-O0221 method 2 / JVET-O0560 method 1)

Form developer point of view, a clear definition of process in spec text is more preferred.

No action.

(B) Revert back VTM-4.0 solution. (JVET-O0221 method 3/ JVET-O0560 method 2 / JVET-O0305 method 3)

It is commented that reverting back to VTM-4.0 could incur coding performance drop.

No non-proponent expresses support on having this in Track discussion.

(C) Change syntax tree of merge mode flags and reduce checks (JVET-O0249 Test 1).

(D) Imposing more condition checks to avoid parsing redundancy completely (JVET-O0431 / JVET-O0586 / JVET-O0309 / JVET-O0585 method 1 / JVET-O0534/ JVET-O0361 / JVET-O0278).

(E)  Impose a constraint on merge modes that it only allows skip mode for IBC and regular merge mode.

For those merge modes which does not have skip flag support, root_cbf_flag is introduced.

This proposal also moves the signaling of regular merge mode flag to the end of merge mode signaling for non-skip merge.

Revisit: Track B (Solution C and D). The proponents are suggested to form offline discussion and provide statistical data.

# Subblock Temporal motion vector prediction (8)

JVET-O0163 Non-CE4: Simplification on SbTMVP [H. Chen, H. Yang (Huawei)]

This contribution proposes to simplify the derivation of SbTMVP candidate in sub-block merge mode in two aspects. Firstly, the adaptive L0/L1 order for checking spatial motion vectors when deriving the temporal motion vector is removed. It is proposed to always check L0 motion vector firstly. Secondly, the availability check of SbTMVP candidate and the derivation of base motion data is removed. If the collocated sub-block of a sub-block is not inter coded, the motion of that sub-block is set to zero.

For aspect 1, simulation results reportedly show on average 0.00%/0.01% luma BD-rate in RA/LB configurations, compared to VTM-5.0. For aspect 2, simulation results reportedly show on average 0.02%/0.01% luma BD-rate in RA/LB configurations. Simulation results for the combination of two aspects reportedly show on average 0.02%/0.05% luma BD-rate in RA/LB cases.

Test 1:

The proposed method removes the condition check to switch adaptively between L0 and L1 that is used to determine which of the L0 and L1 motion (when both available) is used as the temporal MV for sbTMVP. The order become constant: L0 first, then if MvL0 does not point to collocated picture, then L1.

(RA:0.00%; LDB:0.01%)

(JVET-O0588 method 1is identical to method 1)

A non-proponent expresses there is no problem in the current design.

Another non-proponent commented the loss is at noise level and the benefit is the saving of the number for checking (2).

It is agreeable that the benefit from this proposal is to reduce the number of checks that is applied to the adaptation between L0 and L1 selection.

A non-proponent supports the proposed method.

Since there is no consensus to have this proposed feature, it may not be adequate to adopt this.

Test 2:

Removes the derivation of base motion and always infers it to be zero vector, and thus the availability check of SbTMVP can be avoided. Once a collocated subblock is intra, this subblock is filled with zero motion vector.

(RA:0.02%; LDB:0.01%)

It is commented that even replacing an intra subblock by using zero motion would incur some problems: (1) potential artifact between blocks with actual motion and block with zero motion and (2) there is no coherence between the collocate block and collocated blocks’ collocated reference (that is why such block is originally coded by intra).

(The proponent of JVET-O0167 claims to fix the problem, too.)

Test 3: Test 1 + 2

(RA:0.02%; LDB:0.05%)

Revisit method 1 in Track B.

General recommendation to Track B:

It is a general agreement to “align spec to sw” first before proceeding to propose changes to the standard.

JVET-O0926 Crosscheck of JVET-O0163 (Non-CE4: Simplification on SbTMVP) [Y.-C. Lin (MediaTek)]

JVET-O0167 Non-CE4: On SbTMVP motion data derivation [J. Zhao, S. Kim (LGE)]

In Subblock-based TMVP, base motion data is firstly derived. When sub-CU MV is not available, base motion data is used as sub-CU’s motion data. Both base motion data and sub-CU motion data derivation are similar to TMVP and require temporal motion vector scaling. It is asserted that a separate base motion data derivation process is unnecessary. This contribution proposes to simplify SbTMVP base motion data derivation by aligning it with sub-CU motion derivation position, and reuse motion data of a sub-CU as base motion data instead. This reduces the number of MV scaling and reduces the SbTMVP complexity. Results show that there is no loss. Two tests are done:

(1) Use MV of sub-CU near center of the CU as base motion data. Results are: In RA configuration, 0.00%, -0.03%, -0.03% for Y,U,V respectively; in LD configuration,  0.00%, 0.08%, -0.08% for Y,U,V respectively

(2) Use MV of the top-left sub-CU as base motion data. Results are: In RA configuration, 0.00%, -0.01%, 0.03% for Y,U,V respectively; In LD configuration, 0.04%, 0.09%, 0.00% for Y,U,V respectively.

Method 1: Use MV from collocated subblock at another position as base motion vector, where the proposed position is at the immediately bottom-right motion storage unit relative to the center point of collocated block pointed to by temporal MV.

(RA: 0.00%; LDB: 0.00%)

The proponent claimed the benefits are:

- Simplify SbTMVP base motion data derivation by re-using one of the sub-CU’s motion data as base motion data.

- Avoid a separate base motion data derivation process, and therefore remove the associated temporal motion vector scaling.

The benefit is to simplify the operation for certain type of decoder implementation because it can reuse the TMVP derivation of subblocks.

One non-proponent expresses interest to have this feature.

Method 2: Use MV from first motion storage unit on the collocated subblock motion field at the top-left corner, not center point’s, as base motion vector.

(RA: 0.00%; LDB: 0.04%)

No interest is shown from participants on method 2.

Recommendation: Adopt method 1.

JVET-O0867 Crosscheck of JVET-O0167 (Non-CE4: On SbTMVP base motion data derivation) [N. Zhang (Bytedance)]

JVET-O0283 CE4-related: Simplification for SbTMVP [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, it is proposed to remove all position clipping operations for subblock-based temporal motion vector prediction (SbTMVP) and use a method of two-subblock check to conditionally adjust the temporal motion vector (MV) to zero. It is asserted that the SbTMVP derivation is significantly simplified. It is reported that proposed method results in 0.00% and 0.01% luma BD-rates with 100% and 100% encoding time and 98% and 99% decoding time over VTM5.0-RA and VTM5.0-LB, respectively.

In VTM, motion source block may be located outside the position clipping area and thus requires clipping.

Propose to remove clipping by enforcing out-of-range unit in the motion source block to be zero mv.

(RA: 0.00%; LDB: 0.01%)

The proponent claims this proposed method can completely remove the clipping process (320 for 64 16x16 blocks) and each clipping comprises 4 comparisons and 2 assignments. The added subblock check operations is 64 for 64 16x16 blocks. Each check operation comprises 4 comparison and 1 assignment.

(JVET-O0604 is identical to this proposed method.)

No action.

JVET-O1071 Crosscheck report of JVET-O0283: CE4-related: Simplification for SbTMVP [X. Xu (Tencent)]

JVET-O0328 Non-CE4: On Sub-block Merge Candidates List [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

In current VVC design, there is an ambiguity in the derivation process for subblock merging candidate list. In detail, when ‘sps_affine_enabled_flag’ is set to 0 and ‘sps_sbtmvp_enabled_flag’ is set to 1, only subblock-based temporal motion vector prediction (SbTMVP) could be inserted into a subblock merge list. However, because SbTMVP is not always available, even if affine model based motion compensation is not used for inter prediction, a zero affine candidate is used as a subblock merging candidate. In order to prevent this ambiguity, this contribution proposes that SbTMVP candidate is always set available by setting to a default motion with zero reference index when collocated center subblock is not inter-coded. Simulation results reportedly show that Test 1 achieves 0.00% and 0.04% Y-BD rate changes for RA and LB configurations and Test 2 achieves 0.00% and 0.00% Y-BD rate changes for RA and LB configurations.

Test 1:

When collocated central CU is intra coded, zero MV with refIdx=0 is set to be the base motion data.

Difference from JVET-O0163 is that it still requires to check whether based motion is available or not,

while JVET-O0163 method 2 always regards it as zero MV.

Base motion vector is can be either BI zero MV (B-slice) or UNI zero MV (P-slice).

(RA: 0.00%; LDB: 0.04%)

The proposal raises a potential ambiguity in VTM that when sps_affine_enabled_flag=0 and sps_sbtmvp_enabled_flag=1, there is one subblock merge mode, that is, sbTMVP. However, the problem is sbTMVP may not be always available, and thus a zero-MV affine candidate is used as a subblock merging candidate, which causes ambiguity.

Test 2:

Same as Test 1 but base motion vector is UNI zero MV.

(RA: 0.00%; LDB: 0.00%)

There is no consensus to fix this ambiguity. Take no action on this.

JVET-O0790 Crosscheck of JVET-O0328 (Non-CE4: On sub-block merge candidates list) [Y.-L. Hsiao (MediaTek)]

JVET-O0423 Non-CE4: On SbTMVP derivation using HMVP [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

This contribution proposes to replace the SbTMVP dependency on the offset determined by accessing a spatial neighbor, by the use of an appropriate motion vector from the stored motion candidate when updating the HMVP candidate list. In addition, as this motion vector can be considered to be almost available, compared to said spatial neighbor, the availability check for the SbTMVP is removed. This is asserted to offer a simplification, for e.g. a software decoder, or an encoder privileging one type of candidate or another in the Subblock Merge list. . It is reported that the proposed modifications obtain the average Y-BDRate results of -0.01%/ and 0.01% for respectively RA and LDB configurations compared to VTM5.0.

The proposed method replaces both temporal motion and base motion vector by using the latest HMVP candidate, so that SbTMVP is always available.

In addition, zero MV is used at the beginning of each CTU row since HMVP is empty.

(RA:-0.01%; LDB:0.01%)

No action.

JVET-O0887 Crosscheck of JVET-O0423 (Non-CE4: On SbTMVP derivation using HMVP) [F. Henry (Orange), G. Clare (bcom)]

JVET-O0588 CE4-related: On SbTMVP motion shift derivation [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

(Method 1 is identical to JVET-O0168 method 1)

JVET-O0780 Crosscheck of JVET-O0588: CE4-related: On SbTMVP m\otion shift derivation [Y.-C. Sun (Alibaba)]

JVET-O0592 CE4-related: Motion estimation improvements	[X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

In this contribution, two encoder improvements are proposed to enhance the motion estimation (ME) efficiency of the VTM-5.0 encoder, including 1) adding the motion vector (MV) candidates from previously inter coding units (CUs) for the starting point selection of integer-pel ME; 2) local refinement of the control-point motion vectors (CPMVs) for affine ME. Compared to the VTM-5.0 anchors, the proposed encoder improvements reportedly provides average {Y, U, V} BD-rate savings of {0.31%, 0.52%, 0.53%} and {0.16%, 0.16%, 0.10%} for RA and LD configurations, respectively, with 1% encoding time increase.

Method 1:

The proposed method picks up one of the previously decoded MVs as (a) the starting MV for integer motion search and (b) the initial MV candidate for SMVD.

The source of MVs are from 20 previously decoded CUs and these MVs are all rounded to integer precision before being used.

Method 2:

Perform a two-pass local refinement of CPMVs in affine ME process at ¼ sample precision.

The first pass checks 4 neighboring positions (above/bottom/left/right).

Then, the 4 remaining search points at corners are searched in the second pass only if the first-pass search updates CPMVs.

The results are reported with 2 methods: RA: -0.31%/101%/99%; LDB:-0.16%/101%/100%.

Several non-proponents express interest in this method. In addition, there is consistent coding performance improvement over all sequences.

Recommendation (SW): Adopt both methods and use both methods in CTC.

JVET-O0932 Crosscheck of JVET-O0592 (CE4-related: Motion estimation improvements) [G. Li (Tencent)]

JVET-O0604 CE4-related: Simplification on block location validation in SbTMVP [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

(Identical to JVET-O0283)

JVET-O0785 Crosscheck of JVET-O0604: CE4-related: Simplification on block location validation in SbTMVP [Y.-C. Sun (Alibaba)]