| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1040v2 |
| --- | --- |

| Title: | AHG8: Summary of Resolution Adaptivity related proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Stephan Wenger Ye-Kui Wang Miska M. Hannuksela | Tel: Email: | stewe@stewe.org ye-kui.wang@futurewei.com miska.hannuksela@nokia.com |
| Source: | Tencent, Futurewei |  |  |

_____________________________

# Abstract

This contribution provides a summary of proposals related to resolution adaptivity. A list of design questions is provided for each categories of proposals to help structure the review of the proposals.

# Design Questions

## Signaling of picture sizes (and conformance windows)

List of sizes in SPS, referred to by index in PPS (O0133, O0332)

Maximum size in SPS, actual size in PPS (O0204, O0303 O0319, O0395)

## DPB management

Per picture, irrespective of size

Based on picture size (O0133)

## Resampling Filter(s)

Which filter to use?

- Existing interpolation filter (O0134)

- For downsampling, two sets of integer versions of 8-tap Lanczos filters optimized for 2:1 and 4:1 (O0240)

- For downsampling, 9 tap (7 tab complexity).  For upsampling, 4 tap, 32 phases (O0319)

- SHVC resampling process (O0395)

Picture-based or block-based specification of the filtering process?

If picture-based specification of the filtering process, specify (in SPS) the maximum number of reference pictures that need to be resampled to be used as reference for any coded picture (O0395)

Possibility to turn on "wraparound" selection sample location of input samples for filtering (for equirectangular projection) (O0395)

## Constraints on coding tools

TMVP generally allowed but the collocated picture shall have the same resolution as the current picture (O0134, O0303, O0395)

DMVR generally allowed but disabled for a coding block when referring to different resolution (O0134, O0242, O0303)

BDOF generally allowed but disabled for a coding block when referring to different resolution (O0134, O0242, O0303)

Single flag allows disabling all TMVP, DMVR, BDOF per slice, presence gated by SPS-level flag (O0184)

The existing affine fallback mode shall be triggered for a coding block a coding block when referring to different resolution (O0303)

# List of Contributions

JVET-O0133 AHG8: Support for reference picture resampling - handling of picture size signalling, conformance windows, and DPB management [Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

V2

For support of the reference picture resampling (RPR) feature in VVC, the following aspects are proposed in this contribution:

- When RPR is enabled, additional picture sizes and conformance windows are signalled in the SPS. In the PPS, an index to the list of picture resolutions and conformance windows is signalled to specify the size of pictures referring to the PPS.

- For DPB management, reference pictures of different sizes are stored in the same DPB. For the determination of DPB fullness, each decoded picture is associated with the PictureSizeUnit variable that specifies the relative size of each picture. When a picture is added into the DPB, DPB fullness is incremented by PictureSizeUnit of the added picture; when a picture is removed from the DPB, DPB fullness is decremented by PictureSizeUnit of the removed picture.

The proposed text changes are included in an attachment to this contribution, with change marks relative to JVET-N1001-v8.

Note that JVET-O0134 includes a discussion and proposal on resampling, TMVP, DMVR, and BDOF for RPR support in VVC.

In v2 of this contribution, the attached specification text has been modified for the rounding process in the resampling process, to match what's implemented in the software, which is included as an attachment in JVET-O0134.

JVET-O0134 AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF [Y.-C Sun, T.-S Chang, J. Lou (Alibaba), Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei)]

V2

For support of the reference picture resampling (RPR) feature in VVC, the following aspects for handling of coding tools are proposed in this contribution:

- Reference pictures are stored in the DPB in its original size. Block-based resampling is applied when the resolutions of the current picture and the reference picture are different. The resampling reuses the existing interpolation filter.

- TMVP may be enabled when RPR is enabled; however, it is constrained that a reference picture with a resolution different from the resolution of the current picture shall not be used as the collocated picture for slices in the current picture.

- DMVR may be enabled when RPR is enabled; however, the use of DMVR for the current picture using a reference picture is disallowed when the resolutions of the current picture and the reference picture are different.

- BDOF may be enabled when RPR is enabled; however, the use of BDOF for the current picture using a reference picture is disallowed when the resolutions of the current picture and the reference picture are different.

The proposed text changes are included in an attachment to JVET-O0133, with change marks relative to JVET-N1001-v8. The simulation results and the source code of the software are included as attachments to this contribution.

Note that JVET-O0133 includes a discussion and proposal on picture size signalling, conformance windows, and DPB management for RPR support in VVC.

In v2 of this contribution, the attached source code of the software has been updated, for some minor bug fixes and clean-ups. These updates do not affect the simulations, thus the simulation results remain the same as in v1 of this contribution.

JVET-O0184 AHG8: On ARC indicator [Y. He, Y. He, A. Hamza (InterDigital)]

V1

This contribution proposes to signal constraint flag in SPS and slice header to constrain certain coding tools for ARC/RPR.

JVET-O0204 AHG8: On Adaptive Resolution Change (ARC) High-Level Syntax (HLS) [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

V1

This contribution relates to Adaptive Resolution Change (ARC). The contribution presents suggested High-Level Syntax for supporting ARC. It is asserted that the proposed syntax offers a solution with low signalling overhead, with a high degree of flexibility in terms of signalled resolutions and with support for introducing new resolutions at any point in a Coded Video Sequence.

JVET-O0240 AHG8: Adaptive Resolution Change (ARC) with downsampling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

V1

This contribution relates to Adaptive Resolution Change (ARC). The contribution asserts that several of the use-cases for ARC involves downsampling and it is suggested to use filters with low-pass characteristics for the cases where a reference picture is of higher resolution than the current picture. For the one-step approach of ARC this would mean to use a different interpolation filter in the Motion Compensation process. For the two-step approach of ARC (where a new reference picture is created from a reference picture with different resolution) it would mean to generate a downsampled reference picture using filters adjusted for downsampling. The contribution includes two suggested sets of interpolation filters: one for 2:1 scaling and one for 4:1 scaling. Both are integer versions of 8-tap Lanczos filters.

JVET-O0242 AHG8: Enabling BDOF and DMVR for reference picture resampling [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

V1

This contribution proposes enabling BDOF and DMVR for the cases when reference pictures and the current picture sizes are the same in the reference picture resampling. The additional check for the picture sizes is performed for BDOF and DMVR enable conditions.

JVET-O0303 AHG8: Adaptive Resolution Change [P. Chen, T. Hellman, B. Heng, W. Wan, M. Zhou (Broadcom)]

V2

This contribution proposes a method to support adaptive resolution change within a coded video sequence. It includes the signalling of adaptive resolution change in parameter sets and modifications to the current motion compens3edgb ated prediction process in the VTM when there is a resolution change between a current picture and its reference pictures. These modifications are limited to motion vector scaling and subpel location derivations. There are no changes in the existing motion compensation interpolators with the asserted benefits of the proposed solution that these interpolators can be reused and does not introduce new processing blocks. The contribution also provides recommendations on constraining the interaction of certain coding tools with adaptive resolution changes as they are asserted to add additional complexity and memory bandwidth.

JVET-O0319 AHG8: On Adaptive Resolution Change [P. Topiwala, M- Krishnan, W. Dai (FastVDO)]

V2

ARC has received increased attention in the past three meeting cycles, and numerous documents consider ARC for VVC. This contribution focuses on an ARC solution that supports any resampling ratio. The solution requires an additional flag in the PPS to notify a change in resolution and two additional parameters for the resampled height and width. The resampling filters used in the proposed solution are a 9-tap downsampling filter (complexity of 7-tap), and 4-tap upsampling filters with 32 phases, which is also part of the existing design of VVC. Gains of about 0.4dB in PSNR (using phase 0 of the upsampling filter) are reported without encoding, when compared to filters described in JVET-M0135, on a subset of test data. An average BD-rate gain of about 2.35% is reported for the luma(Y) channel with compression.

JVET-O0332 AHG8: Signaling and filtering for Reference Picture Resampling (RPR) [B. Choi, S. Wenger, S. Liu (Tencent)]

Proposed are the updates of JVET-N0052, which focus on signaling aspects for reference picture resampling (a.k.a. adaptive resolution change) and filter designs.

The key features of the proposed RPR scheme includes the followings:

- Support of adaptive (decoded) picture resolution change within a coded video sequence.

- Support of constant reference picture resolution for simplification of motion compensation process.

- Support of constant output picture resolution for guided display resolution

- Support of adaptive resampling mode (option1: with additional filtering, option2: without additional filtering)

In order to support the above features, the following high-level syntax modifications are proposed:

- Minimum/Maximum picture resolution (in DPS)

- A flag indicating that RPR is enabled in a coded video sequence (in SPS)

- Decoded picture resolutions in a table (in SPS)

- A flag indicating that any reference picture has the same spatial resolution, and the constant reference picture size. (in SPS)

- A flag indicating that any output picture has the same spatial resolution, and the constant output picture size. (in SPS)

- An index indicating the decoded picture size among the candidates signaled in SPS (in PPS)

- A flag indicating that motion vector scaling for temporal motion vector prediction is disabled (in PPS)

- Filter mode selection (in PPS)

JVET-O0395 AHG8: On adaptive resolution changing and scalable coding [M. M. Hannuksela, A. Aminlou (Nokia)]

V1

This contribution proposes an approach that uses a single layer (i.e. all NAL units have the same NuhLayerId value) and can be used for adaptive resolution changing as well as for spatial, quality, and view scalability.

JVET-O0641 AHG8: AHG 8: JVET Common test conditions for adaptive resolution change (ARC) [M. G. Sarwer, R. -L Liao, J. Chen, J. Luo, Y. Ye (Alibaba)] [late]

V2

This contribution proposes the common test conditions for adding adaptive resolution change support into VVC. It is asserted that establishing the adaptive resolution change CTC enables the JVET to select coding tools with better performance vs. complexity trade-offs. This contribution further suggests to test both modes of resampling, i.e. picture-based and block-based, in a core experiment before selecting the solution that offers better performance vs. complexity trade-off.