| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1042 |
| --- | --- |

| Title: | AHG17: Varying-length NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes a NAL unit header design, which

- uses a 1-byte NAL unit header for all NAL unit types when the bitstream contains a single layer only

- uses a 2-byte NAL unit header when up to 16 layers are present in the bitstream

- enables extending the NAL unit header to 3 bytes in a future VVC version, which could be used e.g. for further extending the layer identifier value range to 11 bits (up to 2048 layers)

# Motivation

In the proposed design, a 1-byte NAL unit header is used for all NAL unit types when the bitstream contains a single layer only. The bitrate savings are asserted to be equal to or slightly better what were reported by Ericsson in JVET-O0237, which are reproduced in Table 1 and Table 2 for convenience.

Table 1.	Estimated results for encoding NAL unit headers with 1 byte instead of 2 bytes for the CTC (source: JVET-O0237)

|  | 4k video streaming 10 Mbps @ 60 fps | 1080p video 2 Mbps @ 60 fps | 4k 360 video 8 Mbps @ 60 fps |
| --- | --- | --- | --- |
| Slices per picture | 20 | 5 | 96 |
| SEI messages per picture | 2 | 1 | 2 |
| APSs per picture | 0.33 | 0.33 | 0.33 |
| bit savings (bits per second): | 10 718 | 3038 | 47 198 |
| bit savings (%): | 0.11 | 0.15 | 0.59 |

Table 2 Estimated bitrate savings for some additional non-CTC use cases (source: JVET-O0237)

In the proposed design, a 2-byte NAL unit header is used when the bitstream contains multiple layers. 16 layers can be indicated with the 2-byte header, which is asserted to be sufficient for envisioned use cases for independent layers (e.g. 3DoF+), spatial scablability, and typical multiview scalability. The NAL unit header is extensible further to the third byte.

# Proposed specification text changes

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| ext_or_tid_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| if( ext_or_tid_plus1 = = 0 ) { |  |
| nuh_temporal_id_plus1 | u(3) |
| nuh_layer_id | u(4) |
| nuh_reserved_zero_bit | u(1) |
| } |  |
| } |  |

zero_tid_required_flag equal to 0 specifies that zero_tid_required_flag does not impose any additional constraints on the value of nuh_temporal_id1_plus1.

ext_or_tid_plus1 equal to 0 specifies that nuh_temporal_id_plus1, nuh_layer_id, and nuh_reserved_zero_bit are present. ext_or_tid_plus1 greater than 0 is used to specify a temporal identifier for the NAL unit. When NalUnitType is equal to DPS_NUT, EOB_NUT, or VPS_NUT, ext_or_tid_plus1 shall not be equal to 0.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. When not present, nuh_temporal_id_plus1 is inferred to be equal to ext_or_tid_plus1.

The value of nuh_temporal_id_plus1 shall not be equal to 0. When zero_tid_required_flag is equal to 1, the value of nuh_temporal_id_plus1 shall be equal to 1.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

…

nuh_layer_id_plus1 minus 1 specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. When not present, nuh_layer_id is inferred to be equal to 0. The value of nuh_layer_id_plus1 shall be in the range of 1 to 1260 to 15, inclusive. The value of 127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 for nuh_layer_id_plus1 in a NAL unit.

NOTE 7 – The value of 127 for nuh_layer_id_plus1 may be used to indicate an extended layer identifier in a future extension of this Specification.

The variable NuhLayerId is derived as follows:

NuhLayerId = nuh_layer_id_plus1 − 1		(7-3)

The value of NuhLayerId shall be the same for all VCL NAL units of a coded picture. The value of NuhLayerId of a coded picture is the value of the NuhLayerId of the VCL NAL units of the coded picture.

# Extending NuhLayerId value range in a future VVC version

The syntax below enables to extend the NuhLayerId value space to 11 bits, i.e. 0 to 2047, inclusive. The proponents expect this to be sufficient for envisioned super-multiview and lenslet usage.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| ext_or_tid_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| if( ext_or_tid_plus1 = = 0 ) { |  |
| nuh_temporal_id_plus1 | u(3) |
| nuh_layer_id | u(4) |
| nuh_extension_flag | u(1) |
| if( nuh_extension_flag ) { |  |
| nuh_lid_msb | u(7) |
| nuh_reserved_zero_bit | u(1) |
| } |  |
| } |  |

nuh_lid_msb specifies the most significant bits of NuhLayerId. When present, nuh_lid_msb shall be greater than 0, When not present, nuh_lid_msb is inferred to be 0.

The variable NuhLayerId is derived as follows:

NuhLayerId = ( nuh_lid_msb << 4 ) + nuh_layer_id

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).