| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1046-v2 |
| --- | --- |

| Title: | BoG report on CE4 related contributions for general inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Chun-Chi Chen | Email: | haitao.yang@huawei.com chunchic@qti.qualcomm.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

Summary

Three sessions were organized for BoG meeting, 35 contributions have been reviewed.

- 1800 to 2200 on July 4

- 0900 to 1400 on July 5

- 1540 to 1930 on July 5

- 1715 to 1815 on July 7

JVET-O0500 was identified to be more closely related CE4 BoG on merge modifications, and was recommended to be reviewed in that BoG.

Seven contributions on the alignment of BDOF and PROF operations are planned to be jointly reviewed with CE9 BoG.

  - Recommendations

    - Recommended adoptions

Normative change

O0164 / O0587 (method 2): remove scaling of spatial motion vector candidates in AMVP list construction.

O0567: add a bitstream conformance that MvdL1 of SMVD mode shall be within the range of [-215, 215-1].

O0284 (method 2) / O0579 (method 3): sym_mvd_flag is not signalled and is inferred to be false when mvd_l1_zero_flag is true.

O0438: affine AMVR control flag is conditioned on affine control flag in SPS.

O0366 (method 2): simplify BCW index derivation by setting it to the BCW index of the first candidate spatial neighbor block in the process of constructing an affine merge candidate.

O0414 (method 2): put a constraint that SMVD can only be applied to STRPs. No impact on CTC results.

SW change

O0567: align SW with text in MVD clipping at the encoder side.

O0126: align SW with text in the binarization of BCW index by using the truncated Rice code in the text.

Bugfix

O0572 (method 1): align text with SW in the derivation of SMVD reference pictures, by adding the initialization at the beginning of the second stage derivation.

    - Revisits in track

O0434: TMVP derivation modification in two aspects, 1) apply clipping to bottom-right collocated location if the location is outside the picture or belong to different CTU line, 2) remove the derivation of central collocated motion vector.

O0254: disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than 16.

O0456: grouping the syntax elements of motion vector differences.

O0795 grouping bypass coded bins of TPM syntax. This contribution is from CE4 BoG on merge modifications.

    - Discussion

Constraint on lMvd[ compIdx ] range: extend to [-217, 217-1], or keep [-215, 215-1] as in the current design.

Context reduction: remove context of sym_mvd_flag (O0468).

    - Revisits in BoG

O0456: grouping the syntax elements of motion vector differences.

O0424: derive the target reference picture for TMVP scaling from the HMVP candidate list.

O0439: derive the target reference picture for TMVP scaling by searching list 0 for the target reference picture of the collocation motion vector.

O0414 (bugfix): skip long-term reference picture in the derivation of reference picture for SMVD mode.

O0553 / O0664: revisit together with proposals on alignment of BDOF and PROF operations.

O0595: to be reviewed.

O0253: to be reviewed.

O0252: motion offset calculation to be reviewed.

All revisits listed was done in the session of CE4 BoG on July 7.

CE4 related contributions

Motion vector coding (17)

List of contributions in this category:

| JVET-VC number | Title |
| --- | --- |
| JVET-O0164 | Non-CE4: Simplification on AMVP list construction |
| JVET-O0253 | Non-CE4: Simplification of HMVP in AMVP mode |
| JVET-O0284 | CE4-related: Interaction between symmetric MVD and mvd_l1_zero_flag |
| JVET-O0414 | Non-CE4 : Symmetric-MVD control considering the reference picture type |
| JVET-O0424 | Non-CE4: On TMVP reference frame selection using HMVP |
| JVET-O0434 | Non-CE4: Simplification of TMVP |
| JVET-O0439 | Non-CE4: Modification of the temporal merging candidate for the regular merge mode |
| JVET-O0440 | Non-CE4: A modified method to derive the temporal merge candidate for long-term reference picture |
| JVET-O0441 | Non-CE4: Short-tap interpolation for bi-prediction |
| JVET-O0456 | Non-CE4: Grouping the syntax elements of motion vector differences |
| JVET-O0468 | Non-CE4 : simplified SMVD mode coding |
| JVET-O0567 | Non-CE4: Cleanups on MVD ranges |
| JVET-O0572 | Non-CE4: On availability of default reference pictures used in SMVD |
| JVET-O0579 | Non-CE4 : SMVD processing when mvd_l1_zero_flag is set equal to 1 |
| JVET-O0587 | Non-CE4: Simplification of MVP list construction |
| JVET-O0607 | CE4-related: On handling the interaction of the mvd_l1_zero_flag and sym_mvd_flag |
| JVET-O0652 | CE4-related: Line buffer size reduction method for switchable interpolation filter |

MVP list construction

JVET-O0164 Non-CE4: Simplification on AMVP list construction [H. Chen, H. Yang (Huawei)]

In current VVC, a two pass approach is conducted for searching the spatial motion vectors in AMVP list construction. This contribution proposes to remove the second pass search of spatial motion vectors. It is claimed that with this modification, spatial motion vector scaling is totally removed from VTM-5.0. Simulation results reportedly show 0.01%/-0.03% luma BR-rate for RA/LB configurations, compared to VTM-5.0.

The discussion was chaired by C.-C. Chen.

It is proposed to remove the MV scaling process when above candidates’ refidx do not match the refidx of current block motion. (RA: 0.01%; LDB: -0.03%)

In worst case, this proposal reduces 1 MV scaling operations for spatial AMVP.

It commented that 24% of the samples in RA and 43% of coded samples in LDB contain scaled spatial AMVP. (JVET-O0587)

The proponent mentioned that when removing the MV scaling for spatial AMVP candidates, it actually increase the chance of HMVP candidate to get into the AMVP list.

A participant supports this method because even in the current AMVP design, there is a constraint imposed to AMVP list construction that the number of MV scaling for spatial candidates is only one which reduces the complexity of the list construction. It is also commented that removing the MV scaling for spatial candidates completely follow the same design philosophy to achieve a lower-complexity design of AMVP list construction.

Recommendation: Adopt.

JVET-O0253 Non-CE4: Simplification of HMVP in AMVP mode [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

The same modification was proposed and discussed in the previous meeting. There was a concern on the coding performance on low QP.

This proposal presented results using both CTC QP and low-QP setting, and show no performance difference.

No action.

JVET-O0587 Non-CE4: Simplification of MVP list construction [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

Method 2 is the same as JVET-O0164.

Recommendation: adopt method 2.

MVD coding

JVET-O0456 Non-CE4: Grouping the syntax elements of motion vector differences [Y.-J. Chang, Y. Zhang, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

In this contribution, it is proposed to reorganize the MVD syntax elements into two groups depending on whether an MVD syntax element is context coded or bypass coded aiming at improving the CABAC engine’s throughput. The grouped design is implemented on top of VTM 5.0. The simulation results under Common Test Conditions compared to VTM-5.0 shows no coding efficiency change while improving CABAC throughput.

It is commented that there is a cost to do the switching between context coded bins and by-pass coded bins. This proposal reduce the number of switching from 5 to 1.

It commented that grouping together all by-pass coded bins for abs_mvd_minus2 of at most three motion vectors for a CU is better than parsing the abs_mvd_minus2 of each motion vector component separately.

It is commented that if all abs_mvd_greater0_flag are grouped together and followed by all abs_mvd_greater1_flag, it is even better than the current proposal, since the switching between the context for greater0 and greater1 is further removed. It is agreed that this is a better solution than what is proposed.

One non-proponent support this.

Revisit after the text for the new proposal is provided.

During the revisit in the morning of July 7, it is suggested to be reviewed in track to have opinions from hardware experts.

Recommendation: revisit in track.

JVET-O0567 Non-CE4: Cleanups on MVD ranges [Z. Deng, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

This contribution proposes to fix the following problems in VVC draft 5,

- The MVD derived by affine AMVR with 1/16-pel resolution may exceed the current MVD range (e.g., 16-bit in VVC draft).

- The MvdL1 of SMVD-coded blocks may exceed the current valid MVD range at corner cases.

- In the VTM-5.0 reference software, a reduced range of MVD constraint is applied, which does not align with the current VVC working draft.

In this contribution, two methods are proposed addressing the problems. In method #1, it is proposed to constrain the signaled MVD according to the MVD resolution. In method #2, it is proposed to constrain the internal MVD instead of the signaled MVD. With method #1 or #2, the MVD out-of-range problem in SMVD is fixed and the reference software is aligned with the working draft. It is reported that the proposed method provides almost the same performance as the anchor in terms of both BD-rate and running time.

The third aspect propose align the SW with text, by removing MVD clipping. No results for this single change. It is commented this should not have an impact on compression performance because large MVD rarely happens with current CTC.

In the current VTM-5.0, the parsed MVD is always 16-bit regardless of the precision of the MVD. The magnitude of MVD differs when using different precision, e.g. 1/16, 1/4, 1, in affine AMVR. If 1/16 precision is chosen, the magnitude of MVD is at most 2K pixels, which is claimed to be insufficient when coding picture with large resolution.

It is commented that there is no such constraint since an encoder has an option of using 1/4-pel or 1-pel precision instead of 1/16-pel precision if an MVD larger than 2K is desired.

The first aspect of the proposal is to make MVD cover 4K distance regardless of the MVD precision. This seems like a proposal for coding gain.

The second aspect of the proposal is clipping the MVD within the range of [-215, 215-1] for SMVD.

Method 2 of the second aspect is adding a bitstream conformance for MVD of SMVD mode, it is proposed to be constrained to [-215, 215-1]. There are participant agreed that this constraint is useful in implementation.

Recommendation:

1) adopt method 2 of the second aspect, adding a bitstream conformance constraint that MvdL1 of SMVD mode is within the range of [-215, 215-1]. The original constraint, lMvd[ compIdx ] shall be in the range of −215 to 215 – 1, is still kept.

2) adopt the third aspect of the proposal, which aligns the SW with text by removing MVD clipping. SW fix of encoder operations.

Discuss whether the constraint on lMvd[ compIdx ] range should be changed to [-217, 217-1].

TMVP modifications

JVET-O0424 Non-CE4: On TMVP reference frame selection using HMVP [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

The derivation of the TMVP candidate, and similarly of the bottom-right control point for constructed merge affine candidates, is currently always targeting the first reference frame. However, when the temporal distances between reference pictures is small, it is asserted that this is not as likely to offer the best prediction. It is therefore proposed to derive the reference index to use from the HMVP candidate list. It is reported that the proposed modifications obtain the average Y-BDRate results of 0.00%/-0.09%/-0.13%, for respectively Test1 in RA, LDB and LDP configurations, and 0.01%/-0.12% for Test2 in RA/LDB, compared to VTM5.0.

It is commented that this proposal could potentially increase memory bandwidth of motion compensation.

The proposal is for coding gain.

Two methods are proposed. In method 1, the proposed change is applied to all TMVP derivation. In method 2, the change is not applied to TPM.

It is reported a similar method is proposed in JVET-N0203. Similar coding gain is observed in LDB. It was suggested to identify the source of coding gain.

It is observed the coding gain comes along with an increase of encoder run-time. The crosschecker reported a negligible encoder run-time increase in all cases.

Further study is encouraged. See notes in JVET-O0439.

Revisit when results with constant QP is available.

Partial results are available by the time this contribution is revisited. For LDB, the performance decreases from -0.05% to -0.02% for Class C, and from -0.23% to 0.08% for Class E, from -0.24% to -0.16%.

Further study is encouraged.

JVET-O0434 Non-CE4: Simplification of TMVP [H. Chen, H. Yang (Huawei)]

This contribution proposes to align the process of illegal collocated position for TMVP with SbTMVP. It is proposed to apply clipping to bottom-right collocated location if the location is outside the picture or belong to different CTU line. In addition, the derivation of central collocated motion vector is removed. Simulation results for the simplified method reportedly show that the luma BD-rate 0.01% and 0.00% in RA and LB cases, respectively, when compared to VTM-5.0.

The discussion was chaired by C.-C. Chen.

If bottom-right candidate is not available, its position is clipped, and thus extra logic may be required and is not considered as simplification.

When the proposed method is applied, the central TMVP candidate is removed from regular inter AMVP / regular merge / affine AMVP / affine construction candidate.

The proponent commented that if BR position moves toward the bottom-right corner insider the current CU, the performance impact is ~0.06% for RA in VTM-4.0 from proponent’s internal results.

The proponent also stated that if the configuration is to just remove center position without clipping for bottom-right position, the coding loss is around 0.06% in RA in VTM-4.0 from proponent’s internal results.

It is commented that whether this 0.06% gain justify to take the approach with BR candidate only + clipping.

A participant commented that according to past experience, bottom-right position could be more performing than the center one.

It was asked that why not just keep center candidate only.

It is also commented that it is unclear whether removing which of center position and bottom-right position would achieve better performance if keeping only one of center and bottom-right positions is intended.

The cross-checker confirms that the proposed method actually simplifies the derivation process of TMVP.

A non-proponent expresses interest in this method.

Another non-proponent supports adoption of the propose method.

One participant suggested no action because there is no clear evidence showing what kind of complexity is reduced and how significant it is.

It is clarified that part of the complexity-wise saving is one on-chip memory access to center candidate position.

Revisit in Track B.

JVET-O0439 Non-CE4: Modification of the temporal merging candidate for the regular merge mode [G. Lee, G. Kim, S. Cha, D. Nam, J. Han (Sejong University)]

In this contribution, when the regular merge mode is used, if the reference picture of the collocated block is in the reference picture list of the current picture, then it is considered as target picture. If the reference picture list of the current picture does not include the reference picture of the collocated block, then the picture related to the index 0 in the reference picture list is set to the target picture as in VTM 5.0. The target picture is used to make the temporal merge candidate, where the collocated MV is scaled based on the difference between POCs of the target picture and the reference picture of the collocated block.

When the proposed method is applied, the simulation results reportedly show the average 0.04%, -0.18% and -0.21% BD-rate changes are achieved on top of VTM-5.0 for RA, LDB and LDP configurations, respectively, under the common test conditions. There is no change in the encoding and decoding running times.

The reported coding gain is larger for LDB compared with JVET-O0424. Meanwhile, the performance loss is bigger than JVET-O0424 in RA configuration.

It is commented that the coding gain may come from referencing a coded picture with higher quality. The guess is the coding gain depends on the GOP structure. If this is the case, encoder may choose put high quality picture in the position of ref_idx 0 to obtain the coding gain.

Further study is encouraged, for identifying the source of coding gain and come up with a solution accordingly.

Revisit when results with constant QP is available.

During the revisit, the performance change for RA and for LDB is explained in detail in a revision of this contribution.

Further study is encouraged.

JVET-O0440 Non-CE4: A modified method to derive the temporal merge candidate for long-term reference picture [G. Lee, G. Kim, S. Cha, D. Nam, J. Han (Sejong University)]

This contribution proposes a modified method to set the target picture when the long-term reference picture is considered. In the merge mode of VTM 5.0, because the target picture is set to the picture whose index is 0 in the reference picture list, if the type of the picture of index 0 is different from that of the reference picture of the co-located picture, the temporal merge candidate is not used. In this contribution, when the co-located block refers to the long-term reference picture, the target picture is set to a picture in the reference picture list whose type is long-term, even if the index of the picture is not 0. The proposed method can make a temporal merge candidate when the type of the reference picture having index 0 is short-term while that of the reference picture of the co-located picture is long-term.

It is commented that the long term reference picture of the collocated block may be different from the long term reference picture in the list, and in this case it is not reasonable to reuse the collocated motion vector to get prediction from a different long term reference picture.

No results are shown. Because there is no long term reference picture in current CTC.

It is encouraged to provide evidence of the benefit of the proposed method.

Symmetric motion vector coding modifications

Start at 9:10 am Friday morning.

JVET-O0284 CE4-related: Interaction between symmetric MVD and mvd_l1_zero_flag [M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM5.0, to align with the VVC draft specification, the checking of mvd_l1_zero_flag was removed when signalling sym_mvd_flag. Therefore, a mechanism is required to handle the behavior of symmetric motion vector difference (SMVD) when sym_mvd_flag and mvd_l1_zero_flag are both true for tests outside the common test conditions (CTC). In this contribution, three methods are proposed to solve the potential issue.

Method 1: when mvd_l1_zero_flag is true, MVD for list 1 (L1) is inferred to be zero even if sym_mvd_flag is true.

Method 2: sym_mvd_flag is not signalled and is inferred to be false when mvd_l1_zero_flag is true.

Method 3: mvd_l1_zero_flag is signalled but shall not be applied when the enabling condition of SMVD is satisfied.

It is reported that in CTC, no BD-rates are changed by methods 1, 2, and 3.

It is commented that mvd_l1_zero_flag is a slice level syntax, and use it to control the behavior of a block level coding tool is reasonable. Method 2 is preferred out of the reason.

Another participant supports method 2.

Recommendation: adopt method 2 (same as method 3 of JVET-O0579).

JVET-O0414 Non-CE4: Symmetric-MVD control considering the reference picture type [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In VTM 5.0, the symmetric MVD (SMVD) scheme allows to derive MVD of the reference list 1 (L1_MVD) from MVD of the reference list 0 (L0_MVD) without signaling of the MVD when the sym_mvd_flag is enabled. The symmetric MVD scheme is motivated that L1_MVD is precisely derived from L0_MVD when both reference pictures are similar and the movement of the object is constant in certain direction. Hence, it is important to define more accurate conditions for applying SMVD. Therefore, it is proposed to consider the reference picture type to determine whether sym_mvd_flag syntax is signaled or not. The experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC.

It fixes the bug in the derivation of reference index for SMVD mode when there is a long-term reference picture in the list.

The crosschecker support adoption of method 2.

Revisit after the text is reviewed by the proponent of SMVD.

During the revisit, one participant also support adopt method 2.

Recommendation: adopt method 2.

JVET-O0468 Non-CE4: simplified SMVD mode coding [F. Le Léannec, Y. Chen, T. Poirier (InterDigital)]

This contribution proposes to simplify the coding of the SMVD mode.

In VTM-5.0, the sym_mvd_flag syntax element indicates the use of symmetrical motion vector difference coding on the CU level. It is coded in context-based mode.

This contribution proposes to code this flag in bypass mode. It reportedly brings +0.01% Luma BD-rate difference over VTM-5.0 in RA configuration.

It is reported by the proponent that this proposal not only save one context, but also change one context coded bin to a by-pass coded bin.

Recommendation: report back to track to discuss all contributions received in CE4 on context reduction.

JVET-O0572 Non-CE4: On availability of default reference pictures used in SMVD [H. Liu, L. Zhang, K. Zhang, Z. Deng, N. Zhang, Y. Wang (Bytedance)]

In Symmetric Motion Vector Difference (SMVD) mode, the reference index is not signaled, and a default reference index is derived for each prediction direction implicitly. However, the derivation process of the default reference indices is different between the reference software and the working draft. With the working draft, it is allowed that the two default reference pictures are both before or both after the current picture in the displaying order. However, it is disallowed in the reference software. This contribution proposes to align the working draft to the reference software. In addition, a second solution is proposed to replace two steps in the reference derivation process by one step. For both solutions, no performance changes are observed under common test conditions.

According to the current draft spec of SMVD, it is possible to derived two reference picture and both of them are in the same direction away from the coding picture which is not the intention of the original design of SMVD.

Method 1 proposes to add an initialization at the beginning of the second stage of SMVD reference picture derivation. It is reported that the same initialization is performed at the beginning of the first stage derivation already.

Method 2 changes the existing derivation procedure and also claimed to solve the abovementioned problem.

It is noted that method 2 may produce different results compared with method 1. However, the results are the same under the current CTC.

The original proponent of SMVD support adoption of method 1.

Further study on method 2 is encouraged, to identify whether this is a pure editorial change compared with method 1.

Recommendation: adopt method 1.

JVET-O0579 Non-CE4: SMVD processing when mvd_l1_zero_flag is set equal to 1 [H. Jang, N. Park, J. Nam, S. Kim, J. Lim (LGE)]

This proposal points out a mismatch issue between draft text and software implementation. Specifically, in VTM 5.0, sym_mvd_flag is signalled regardless of mvd_l1_zero_flag. However, in draft text, slice level constraint (i.e., mvd_l1_zero_flag) has higher priority than CU level constraint (sym_mvd_flag) so that mvd for reference list 1 is set to 0 when mvd_l1_zero_flag is equal to 1. Therefore, in this proposal, three candidate methods are suggested to be considered as follows:

- Change draft spec to align with SW

- Change SW to align with draft spec

- Back to VTM-4.0

Method 3 of this proposal is the same as method 2 in JVET-O0284.

Method 2 of this proposal is the same as method 1 in JVET-O0284.

It is noted that in the current VTM-5.0, the CU level control flag of SMVD is signalled even if mvd_l1_zero_flag is 1. If the SMVD flag is 1, mvd_l1_zero_flag is override on a particular CU.

JVET-O0607 CE4-related: On handling the interaction of the mvd_l1_zero_flag and sym_mvd_flag [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

The first solution is align VTM-5.0 with draft spec, which is the same as method 1 in JVET-O0284

In the second solution, when both mvd_l1_zero_flag and sym_mvd_flag is equal to 1, both the L0 MVD and the L1 MVD are set to be zero. This is another way of signalling zero MVD and is redundant with the current MVD coding mechanism.

Others

JVET-O0441 Non-CE4: Short-tap interpolation for bi-prediction [X. W. Meng (PKU), X. Zheng (DHI), S. S. Wang, S. W. Ma (PKU)]

This contribution presents a short-tap interpolation based bi-prediction to reduce the memory bandwidth consumption of conventional LDB configuration compared to LDP configuration. This proposal proposes to use 4-tap interpolation filter for small blocks of normal bi-prediction and affine bi-prediction. Compare to conventional LDP configuration, the proposed method can achieve 5.83% coding gain with almost the same memory bandwidth consumption.

The proposed method is reported to have the coding loss of 0.22%/0.52% for RA/LDB. The coding loss on class D is 1.02%/2.51% for RA/LDB.

BD-rate is reported to be -5.83% for LDP. It is noted that bi-prediction is enabled in the test when compared with LDP anchor which use uni-prediction only.

It is reported the encoder run-time increase of the proposed short-tap LDB compared with regular LDB is approximately the same.

It is asked whether there is an impact on subjective quality by using a 4-tap interpolation filter. The proponent reported this is not checked yet.

It is asked how much memory bandwidth could be saved by the proposed method. Analysis of bandwidth is provided. The worst case bandwidth is reported to be reduced. It is suggested to analyze the memory bandwidth using the memory bandwidth measure already integrated in VTM-5.0.

Further study is encouraged.

JVET-O0652 CE4-related: Line buffer size reduction method for switchable interpolation filter [T. Solovyev, S. Ikonin, E. Alshina (Huawei)] [late]

The discussion was chaired by L. Zhang.

This contribution describes modification of switchable interpolation filter index propagation process for method proposed in CE4-1. In order to reduce the line buffer size, borrowing of interpolation filter index is restricted to be within the current CTU line. The simulation results on top of CE4-1.2 reportedly show 0.02% and 0.03% luma BD-rate loss for RA and LDB configurations respectively. The simulations results on top of CE4-1.3 reportedly show 0.02% luma BD-rate loss for RA configuration and 0.02% luma BD-rate gain for LDB configuration.

It is clarified that the coding time is not reliable.

Why the loss becomes bigger on top of CE4-1.2 with bug fix (the adopted version)?

It is claimed that merge may become better due to the bug fix so that the loss is slightly higher.

The benefit is claimed to save 1 bit per 4x4 block in the line buffer, that is, 120 bytes for 4K video which is roughly 1%.

Revisit after full results are ready.

(Revisited 1800 chaired by Chun-Chi)

Full results is still not available yet.

Further study is encouraged.

Affine motion compensation (17)

List of contributions in this category:

| JVET-VC number | Title |
| --- | --- |
| JVET-O0123 | Non-CE9: Alignment of BDOF refinement process with PROF |
| JVET-O0252 | CE4-related: Alignment and simplification of PROF and BDOF |
| JVET-O0254 | Non-CE4: Size constrain for inherited affine motion prediction |
| JVET-O0281 | CE4-related: Alignment of displacement bitdepths for BDOF and PROF |
| JVET-O0286 | CE4-related: Extension of phase-variant affine subblock motion compensation in CE4-2.5 |
| JVET-O0313 | CE4-related: On Conditions for enabling PROF |
| JVET-O0438 | Non-CE4: Cleanup for affine AMVR enabling flag in SPS level |
| JVET-O0500 | non-CE4: affine and sub-block modes coding clean-up |
| JVET-O0530 | Non-CE4: Adaptive subblock size for affine motion compensation |
| JVET-O0553 | Non-CE4: On prediction refinement with optical flow |
| JVET-O0580 | CE4-related: Additional tests for prediction refinement with optical flow for affine mode (CE4-2.1) with 4x8 sub-block size |
| JVET-O0583 | CE4-related: Prediction refinement with optical flow for chroma components for affine mode |
| JVET-O0593 | CE4-related: Harmonization of BDOF and PROF |
| JVET-O0594 | CE4-related: Prediction sample padding unification for BDOF and PROF |
| JVET-O0595 | CE4-related: Modified adaptive motion vector resolution (AMVR) for affine mode |
| JVET-O0624 | CE4-related: Unified padding for BDOF and PROF |
| JVET-O0664 | CE4-related: Simplifications on PROF |

PROF simplification and alignment with BDOF

Reviewed in the joint session of CE4 BoG and CE9 BoG, chaired by Semih Esenlik. See notes in JVET- O1133.

JVET-O0123 Non-CE4: Alignment of BDOF refinement process with PROF [J. Li, C. Lim (Panasonic)]

JVET-O0252 CE4-related: Alignment and simplification of PROF and BDOF [H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

Aspect 3 can save about 900 bytes in some implementation. Follow the criteria when discussing contributions of the same kind in Track B, this is not attractive.

No action.

JVET-O0281 CE4-related: Alignment of displacement bitdepths for BDOF and PROF [C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0583 CE4-related: Prediction refinement with optical flow for chroma components for affine mode [Y. He, W. Chen (InterDigital)]

JVET-O0593 CE4-related: Harmonization of BDOF and PROF [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-O0594 CE4-related: Prediction sample padding unification for BDOF and PROF [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-O0624 CE4-related: Unified padding for BDOF and PROF [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)]

PROF modifications

JVET-O0313 CE4-related: On Conditions for enabling PROF [Y.-C. Yang, P.-H. Lin (Foxconn)]

In the SW of CE4-2.1a there is an implementation optimization for PROF to skip the refinement on affine coded CUs if the control point motion vectors of a CU are the same. It is reported here the percentage of affine coded CUs that would skip PROF operation is about 24%.

It is proposed in this contribution to calculate max delta MVx and delta MVy of a 4x4 subblock and compare the two values with a threshold and then decide whether skip the PROF refinement. The percentage of skipping PROF for affine coded blocks is around 35%.

It is noted the proposed method is an implementation optimization and does not change the simulation results.

It is noted the claimed simplification does not bring encoder and decoder run-time decrease.

Further study on encoder fast algorithms for PROF is encouraged.

JVET-O0553 Non-CE4: On prediction refinement with optical flow [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

In CE4-2.1 prediction refinement with optical flow (PROF) is tested. In this contribution two small changes are proposed. First, small-block level (4x4 or 8x8) PROF border extension is modified such that border extension is at bigger-sized sub-block such as 16x16 level or full block level when affine motion is not big, which is reportedly more friendly for SIMD implementation. Second, MV derivation is changed for better precision. The simulation results reportedly show that average BD rate savings for RA, LDB, LDP respectively are as follows.

Full block border extension + accurate delta MV on top of CE4-2.1.a: -0.50% (RA), -0.33% (LDB), -0.43% (LDP) over VTM-5 (-0.02%, -0.01%, -0.10% over CE4-2.1.a).

16x16 sub-block border extension + accurate delta MV on top of CE4-2.1.a: -0.50% (RA), -0.37% (LDB), -0.46% (LDP) over VTM-5 (-0.02% and -0.05%, -0.13% over CE4-2.1.a).

Full block border extension + accurate delta MV on top of CE4-2.1.c: -0.48% (RA), -0.40% (LDB), -0.43% (LDP) over VTM-5 (-0.02%, -0.05%, -0.10% over CE4-2.1.c).

16x16 sub-block border extension + accurate delta MV on top of CE4-2.1.c: -0.48% (RA), -0.40% (LDB), -0.46% (LDP) over VTM-5 (-0.02%, -0.05%, -0.13% over CE4-2.1.c).

As the first aspect of this contribution, adaptive block size for padding is proposed. On top of CE4-2.1a, the padding is perform for 4x4 or 16x16 subblock, based on checking the condition maxc,d,e,f<T. The subblock size for MC is still 4x4.

For the second aspect of this contribution, subblock MV instead of CPMV is used for the calculation of delta MV. With the proposed method, delta MV of each sample inside an affine coded CU is calculated. In contrast, the delta MV of samples inside a single 4x4 subblock is calculated in CE4-2.1a. The number of additions for delta MV calculation is increased (M/4)*(N/4) times, where MxN is the CU size. It is claimed by the proponent that the additional complexity is not proportional to the number of 4x4 subblocks inside a CU if SIMD optimization is performed.

For the third aspect, the center position of a 4x4 subblock is changed to (1.5,1.5) from (2,2), which change the MV derivation of subblocks. per-pixel delta MV calculation of a 4x4 subblock is modified.

Each aspect has slight coding gain. The total coding gain for all three aspects are -0.02% and -0.05%, -0.13% over CE4-2.1.a.

There is a related contribution for aspect 1, JVET-O0624.

Revisit aspect 1 together with JVET-O0624. See notes in JVET-O1133.

Revisit aspect 2 together with JVET-O0252. See notes in JVET-O1133.

Revisit aspect 3 after revisit of aspect 1 and 2. See notes in JVET-O1133.

JVET-O0580 CE4-related: Additional tests for prediction refinement with optical flow for affine mode (CE4-2.1) with 4x8 sub-block size [Y. He, W. Chen (InterDigital), W. Zeng, E. Cai (Ubilinx)]

Based on the CE4 test 2.1, this contribution proposes to use 4x8 sub-block for affine bi-prediction for luma component. Two tests are included to test using 6-tap and 8-tap interpolation filter for bi-prediction, respectively. The simulation results reportedly show that comparing to VTM-5.0, average BD rate savings for RA and LD respectively are {-0.43%, -0.08%, -0.04%} and {-0.29%, -0.03%, -0.09%} for 6-tap filter; {-0.49%, -0.10%, -0.09%} and {-0.35%, -0.02%, -0.22%} for 8-tap filter.

Compared with CE4-2.1a, test 1 here has a performance drop of 0.05% for RA and 0.03% for LDB. It is claimed by the proponent the proposal will benefit the computation complexity, by reducing the number of multiplications and additions as shown below in case of bi-prediction.

| Tests | Settings for affine bi-prediction | BD rate relative to VTM-5.0 RA-Y; LB-Y | Multiplications per 4x4 block (uni/bi) | Additions per 4x4 block (uni/bi) |
| --- | --- | --- | --- | --- |
| VTM-5.0 | 4x4, 6 tap | 0%; 0% | 400/800 | 350/700 |
| CE4-2.1a | 4x4, 6 tap | -0.48%; -0.32% | 432/864 | 430/860 |
| CE4-2.1b | 8x8, 6 tap | -0.40%; -0.27% | 432/744 | 430/760 |
| CE4-2.1c | 8x8, 8 tap | -0.46%; -0.35% | 432/976 | 430/984 |
| Test 1 | 4x8, 6 tap | -0.43%; -0.29% | 432/744 | 430/760 |
| Test 2 | 4x8, 8 tap | -0.49%; -0.35% | 432/976 | 430/984 |

Compared with CE4-2.1b, test 1 can get additional coding gain.

No action.

JVET-O0664 CE4-related: Simplifications on PROF [H. Chen, H. Yang (Huawei)]

In CE4-2.1, Prediction refinement with optical flow (PROF) for affine mode was tested. It’s reported the luma BD-rate for RA and LD respectively are -0.48%, and -0.32% in test a. In this contribution, two simplified aspects for PROF are proposed. The first aspect is to apply PROF to affine coded block with uni-prediction for reducing the complexity of PROF. In the second aspect, the refinement offset is clipped within [-4092, 4091] to ensure the refined value is within 16-bit signed range.

Simulation results for aspect 1 reportedly show that the luma BD-rate -0.40% and -0.17% in RA and LB cases, respectively, when compared to VTM-5.0.

Simulation results for aspect 2 reportedly show that the luma BD-rate -0.48% and -0.30% in RA and LB cases, respectively, when compared to VTM-5.0.

It is proposed to clip the offset to 13 bits instead of 14 bits to ensure the dynamic range within 16 bits, which was confirmed by crosschecker.

It was commented that if an implementation needs to store the intermediate data, the proposed method is desirable. But if an implementation does not need to store the intermediate data, it is not necessary to do this.

It was commented there is a related proposal to be reviewed jointly with CE9 BOG.

Review this one together with JVET-O0594. See notes in JVET-O1133.

Others

JVET-O0254 Non-CE4: Size constrain for inherited affine motion prediction [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

The inherited affine motion prediction requires extra storage for the control-point motion vectors (CPMVs) of the neighboring coded blocks. This contribution proposes size constrain for inherited affine motion prediction that reduces the memory size for CPMV storage. The Y, U, V BD-rates changes compares to VTM-5.0 are 0.02%, 0.01%, -0.06% in RA and 0.02%, 0.04%, 0.17% in LDB, respectively.

It is claimed by proponent that the proposed method reduces the memory size for CPMV storage by half, in a memory optimized implementation. If the memory optimized implementation is not applied, the proposed method reduces the memory size for CPMV storage to one quarter.

It is noted that there are several contributions had been proposed in the previous meetings along this direction, some of the proposal can save up to 90% local buffer.

It is noted that the same change was put into CE for study and was reviewed in the previous meeting, but no action was taken on this.

It is commented that the amount of storage saving is not worth doing this modification.

One participant express the opinion that the saving of local buffer is desired.

One participant consider this saving here is not critical.

Recommendation: revisit in Track.

JVET-O0286 CE4-related: Extension of phase-variant affine subblock motion compensation in CE4-2.5 [T.-D. Chuang, C.-Y. Lai, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

The phase-variant affine subblock MC is extended to allow using filters with different phases within a horizontal row or a vertical column for better approximating the sample-based affine MC in an affine subblock.

For the proposed method-A, the average luma BD-rates are -0.61% and -0.28% in RA and LB, respectively.

The phase variant affine MC has been disused in CE. No action here.

JVET-O0438 Non-CE4: Cleanup for affine AMVR enabling flag in SPS level [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

This contribution presents a syntax change to remove unnecessary signalling of sps_affine_amvr_enabled_flag when sps_affine_enabled_flag is equal to 0. When sps_affine_enabled_flag is equal to 0, the coding unit does not use affine motion compensation and the affine AMVR enabling flag does not have impact on the decoding process. Therefore, if affine motion compensation is not enabled by SPS level flag, it is proposed not to signal the affine AMVR enabling flag. It reportedly shows 0.00%/0.00%/0.00% (Y/U/V) average BD-rate change with RA and LDB configurations.

It is commented by the crosschecker it is a straightforward fix of the current layout of affine related control flags in SPS.

It is commented that the using of affine AMVR does not have to depend on the using of regular AMVR. The proposed change here share the same spirit.

It is noted that in the current draft spec, affine AMVR and regular AMVR is already decoupled and can be enabled separately.

Recommendation: adopt the suggested position change of affine AMVR flag in SPS.

JVET-O0500 Non-CE4: Affine and sub-block modes coding clean-up [F. Galpin, F. Le Léannec, A. Robert, T. Poirier (InterDigital)]

This is re-allocated to be reviewed by BoG on inter prediction with merge modifications.

JVET-O0530 Non-CE4: Adaptive subblock size for affine motion compensation [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

In this document, it is proposed to adaptively switch affine subblock size and interpolation filter based on affine model parameters. When the maximum absolute value of MV difference between neighboring 4x4 blocks is below a threshold, affine subblock size is set to 8x8 and 8-tap luma interpolation filter is used for affine motion compensation; otherwise, 4x4 subblock size and 6-tap luma interpolation filter is used as in current VVC working draft. The proposed method is implemented on top of VTM-5.0, and simulations were conducted based on common test conditions. Two threshold values were tested. Comparing with VTM-5.0 anchor, minor coding gain is observed with runtime saving, especially for LDB:

Threshold-1: RA -0.02%, 99%, 100%; LB -0.01%, 96%, 99%

Threshold-2: RA -0.03%, 100%, 100%; LB -0.04%, 96%, 99%.

The suggested adaptive selection of 4x4 or 8x8 subblock size applies to both uni-prediction and bi-prediction.

When 8x8 subblock size is chosen, 8-tap interpolation filter is used.

This proposal is implemented on top of CE4-2.1c PROF. When 8x8 subblock size is chosen, the padding of subblock boundary and the calculation of delta MV is performed on 8x8 basis.

It is possible the subblock size are different for each of the prediction in case of bi-prediction. The proponent mentioned the subblocks size for the two prediction can be enforced to be the same.

It is reported that the subblock size for MC and for MV storage are different in the implementation of CE4-2.1c. It is asked whether it is the same in the implementation of this proposal. The proponent can confirm this.

It is claimed by the proponent this proposal can save average complexity and reduce encoder run-time.

Similar adaptive subblock size for affine MC was proposed several meetings ago. However, this feature was removed when adopting affine MC, because this feature does not help the worst case complexity even though the average complexity can be reduced. See JVET-K0184.

It is commented by one participant that JVET-K0184 shown 0.1% coding performance drop, which is the key reason to remove the feature.

It is commented by the proponent of JVET-K0184 that the reason to remove the feature is because the worst case complexity is not reduced.

It is commented when turning off PROF, with the proposed method, 8-tap interpolation filter is used when the subblock size is 8x8, which is the reason why CE4-2.1a is preferred over CE4-2.1c during the discussion of CE4 tests.

It is commented by the proponent that there is a benefit of reducing average complexity with slight coding gain the proposed method so may worth re-considering enabling both 4x4 and 8x8 subblock size for affine MC and for PROF.

It is commented that the current implementation of the motion compensation in case of the fall back mode is activated can be optimized to achieve encoder run-time saving.

It is reported by the crosschecker that 1.27% coding gain is observed for BQSquare in LDB with 5% encoder run-time saving.

One participant suggests no action on this.

The crosschecker suggest adopt this.

One participant commented that a decoder need to spend efforts on determining the subblock size and switch back and forth between 4x4 and 8x8 processing.

One participant commented that two different logics are required to both affine MC and PROF if we are going to have this adaptive subblock size.

One participant suggests further study this in CE together with JVET-K0184.

It is commented there is no consensus to revisit this in track.

There is a consensus that the proponent is suggested to raise the issue and discuss this in track.

JVET-O0595 CE4-related: Modified adaptive motion vector resolution (AMVR) for affine mode [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

In the VVC specification draft 5, adaptive motion vector resolution (AMVR) is applied to affine mode to reduce the overhead of signaling the motion vector differences (MVDs). In the current affine AMVR design, the MVD resolution of one affine block can be adaptively selected among 1/4-pel, 1/16-pel and 1-pel. In this contribution, one modified MVD resolution set {1/4-pel, 1/8-pel and 1-pel} is proposed for the affine AMVR mode. Simulation results show that compared to the VTM-5.0 anchors, the proposed modification reportedly provides {Y, U, V} BD-rate savings of {0.07%, 0.10%, 0.15%} for RA configuration without increasing the encoding and decoding time. Additionally, when the affine AMVR is enabled for LD configuration, the corresponding BD-rate savings of the proposed modification are {0.05%, 0.05%, 0.40%}.

No action.

BCW modifications (4)

List of contributions in this category:

| JVET-VC number | Title |
| --- | --- |
| JVET-O0126 | Non-CE4: Align coding of BPWA index with normal truncated rice binarization process |
| JVET-O0366 | Non-CE4 : Simplifications on BCW index derivation process |
| JVET-O0367 | Non-CE4 : The proposed BCW index derivation for pairwise candidate |
| JVET-O0516 | non-CE4: BCW for pairwise candidate |

JVET-O0126 Non-CE4: Align coding of BPWA index with normal truncated rice binarization process [J. Li, C.S. Lim (Panasonic)]

This contribution proposes to align coding of BPWA index with normal truncated rice binarization process in VTM5.0. The proposed method has coding impact of 0.01% luma BD-rate in RA and 0.03% luma BD-rate in LDB over VTM-5.0 anchor.

It is reported by the proponent the specification of this different truncated rice binarization is not correctly describe in the draft spec.

One participant commented this is not described in the text.

It seems like a mismatch between the SW and the draft spec on this issue.

Recommendation: adoption (SW), align SW with text on the binarization method of BCW index.

JVET-O0366 Non-CE4: Simplifications on BCW index derivation process [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In affine merge mode, for inherited candidates, BCW index is inherited from spatial neighbor block which is coded with affine mode while for constructed candidates, BCW index is derived from indices of the each CPs. In this proposal, the number of comparisons on BCW index derivation process for affine constructed merge mode, which was adopted in the previous meeting is reduced as follows.

- CASE 1: The number of comparison is reduced to 1.

- CASE 2: The number of comparison is reduced to 0.

The experimental results for CASE1 reportedly show 0.00% (0.00%) BD-rate changes with 100% (100%) and 100% (98%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations. And the experimental results for CASE2 reportedly show 0.01% (-0.03%) BD-rate changes with 100% (100%) and 100% (101%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations.

It is reported by proponent that the worst case comparison in BCW index derivation for affine mode is reduced to 0 from 25 in method 2.

The crosschecker support method 2.

It is asked what is the coding gain of BCW index inheritance for affine constructed candidate. One participant reported it is -0.02% for RA and -0.05% for LDB.

Recommendation: adopt method 2.

JVET-O0367 Non-CE4: The proposed BCW index derivation for pairwise candidate [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In regular merge mode, the BCW index of spatial neighbor block is inherited when current CU is bi-predicted block. And, BCW index of the pairwise candidate is set to default mode (i.e. equal weight). The pairwise candidate has similar characteristics with affine constructed mode because this candidate is constructed by combining between merge candidates. Hence, derivation methods of BCW index for pairwise candidate are proposed for consistency in derivation design as follows.

This proposal is not for simplification. On average there is a slight performance drop. For class A2, the coding loss is 0.06% for RA.

No action.

JVET-O0516 Non-CE4: BCW for pairwise candidate [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (InterDigital)]

Method 2 in this proposal is the same as the method 2 in JVET-O0367.

Method 3 in this proposal is the same as the method 1 in JVET-O0367.

Method 1 is based on method 2 with additional logic but the same compression performance.

No action.

LIC (6)

List of contributions in this category:

| JVET-VC number | Title |
| --- | --- |
| JVET-O0190 | Non-CE4: LIC model parameters using forward mapped luma samples in LMCS |
| JVET-O0257 | CE4-related: Simplified local illumination compensation |
| JVET-O0457 | CE4-related: LIC flag inherited per component |
| JVET-O0470 | CE4-related: encoder simplification of local illumination compensation |
| JVET-O0531 | Non-CE4: On LIC for Affine |
| JVET-O0605 | CE4-related: On exclusively applying LIC and PROF |

JVET-O0190 Non-CE4: LIC model parameters using forward mapped luma samples in LMCS [J. Shingala, A. Natesan (Ittiam)]

This proposal relates to simplifying the normative method of deriving the linear model parameters for Local illumination compensation (LIC) in the mapped (reshaped) domain by using the neighbour reconstruction samples and forward mapped neighbour inter predicted samples when Luma mapping with chroma scaling (LMCS) is enabled. The proposed method computes the LIC scaling parameter ‘a’ in the forward mapped (reshaped) domain and the offset parameter ‘b’ in the original (unmapped) domain so that the decode pipeline order of Motion Compensation (which includes LIC) is followed by forward mapping of inter prediction as per the existing scheme of LMCS. The method of deriving LIC model parameters for chroma is unchanged as chroma samples are not reshaped.

The proposed method to derive the LIC parameters in the mapped (reshaped) domain was implemented on top of CE 4-3.1a core LIC experiment in VTM-5.0. Test results reportedly show BD-rate gain of 0.47% (Y), 0.35% (U), 0.46% (V) with encoding time of 118% and decoding time of 100% for RA and BD-rate  gain of 0.34% (Y), 0.22% (U), 0.50% (V) with encoding time of 116% and decoding time of 100% for LDB configuration under CTC test conditions.

It is proposed to derive the scaling factor in the mapped domain. And forward mapping of reference template samples are performed for this purpose.

The claimed benefit is complexity reduction because the forward mapping is less complex than the inverse mapping.

The scaling factor in the mapped domain is applied to the prediction which is in the original domain. It is claimed by the proponent that this does not have a negative impact on the performance, which is confirmed by the simulation results.

The offset is derived in the original domain, which requires the inverse mapping of the average of the neighboring template sample values.

It is noted that there is a mismatch in the design because the scaling factor in the mapped domain is applied to the prediction which is in the original domain.

It is commented that there may be a negative impact on compression performance if the mapping function is irregular.

No action.

JVET-O0257 CE4-related: Simplified local illumination compensation [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

No need for presentation as suggested by the proponent.

JVET-O0457 CE4-related: LIC flag inherited per component [P. Bordes, F. Urban, F. Le Léannec, F. Galpin, T. Poirier (InterDigital)]

No need for presentation as suggested by the proponent.

JVET-O0470 CE4-related: encoder simplification of local illumination compensation [F. Urban, P. Bordes, F. Le Léannec, F. Galpin, T. Poirier (InterDigital)]

No need for presentation as suggested by the proponent.

JVET-O0531 Non-CE4: On LIC for Affine [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

No need for presentation as suggested by the proponent.

JVET-O0605 CE4-related: On exclusively applying LIC and PROF [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

No need for presentation as suggested by the proponent.