| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3-12 July 2019 | Document: JVET-O1048-v2 |
| --- | --- |

| Title: | Non-CE11: Non-CIP normative Gradual Random Access |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Gommelet, Jean-Marc Thiesse, Didier Nicholson | Tel: Email: | +33 1 46 73 06 06 david.gommelet@vitec.com jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC 99 rue Pierre Sémard 92320 Chatillon, France |  |  |

_____________________________

# Abstract

This contribution introduces an improvement to CE 11 Test 2.2 to support Gradual Random Access. Instead of using a full CIP-like constraint on all intra CUs of the refreshed region, this contribution only take care of not using reference sample from the non-refresh area. There is no impact on bitrates fluctuations which are keeping smooth as in CE 11 Test 2.2. The resulting improvement over CE11 test 2.2 is of -3.41% in class E, -1.05% in class C and -1.22% in class D.

The contribution also proposes a less-intrusive option leveraging the same principle: virtual boundaries brought at slice level are proposed to be used for disabling intra prediction across virtual boundary. This option is intended to be less complex to support than CIP since the resulting impact on intra reference pixels filling is kept similar as for picture boundaries.

# Proposed Modifications

## Option 1: Virtual boundary

The proposed modification in this sub-section is build upon CE11 Test 2.1.a with virtual boundary. Instead of using the CIP tool at picture level, it is proposed to use virtual boundary for disabling intra prediction across boundary. Indeed, it was raised that using CIP (either at frame level of CU level) could be somewhat complex due to the need to fill multiple hole of various sizes. By using virtual boundary to disable intra prediction, this proposal allows limiting this filling process of reference pixel to only vertical or horizontal boundary as already present at picture boundaries. The effect, compared to CIP, is represented in figure 1.

Figure 1. Intra prediction reference disabling in case of CIP (right) or proposed virtual boundary modification.

This proposed modification leads to three cases as represented in figure 2:

- The virtual boundary crosses the CB: all reference pixels are marked as non-available for intra prediction.

- The virtual boundary is at the left boundary of the CB: all the pixels of the left column are marked as non-available for intra prediction.

- The virtual boundary crosses the top-right reference pixels of the CB: all the pixels on the right of the boundary are marked as non-available.

As a result, the intra prediction is avoided in both direction of the virtual boundary. However, for intra refresh scenario, for case B, it is asserted that CU on the right of the boundary are more often coded with inter mode (in the example with column moving from left to right and it is the same in the other direction) and for case A, it is likely that the encoder will avoid selecting such CU partitioning leading to optimize coding efficiency.

Figure 2. Additional reference disabling with proposed modification.

The proposed modification achieved GRA thanks to these two modifications to current draft text:

- Adding the virtual boundary signalization at slice level.

- Adding flag pps_intra_across_virtual_boundaries_disabled_flag

## Option 2: Proposed modification with GRA signalization

To solve the drawback represented in figure 2, the proposed modification can be applied together with the GRA signalization proposed in CE 11 test 2.2. When apply to CE 11 test 2.2 with specific GRA signalization, the proposed modification will result in the constraint represented in following figure. Only reference pixels from the dirty region to the GRA region will be marked as non-available for intra prediction.

Figure 3. Intra prediction reference disabling on GRA boundary.

For a column pattern, the proposed text modifications in the section 8.4.5.2.7 Reference sample availability marking process are as follow with support of the two scanning directions:

- If one or more of the following conditions are true, the sample p[x][y] is marked as “not avilable for intra prediction”: - If availableN is equal to FALSE, the sample refUnfilt[ x ][ y ] is marked as "not available for intra prediction". - sps_intra_refresh_enabled_flag is equal to 1 and slice_intra_refresh_direction_flag is equal to 0 and x + cu_width < (slice_intra_refresh_position + intra_refresh_width) and xNBY >= (slice_intra_refresh_position + intra_refresh_width) - sps_intra_refresh_enabled_flag is equal to 1 and slice_intra_refresh_direction_flag is equal to 1 and x >= slice_intra_refresh_position and xNBY < slice_intra_refresh_position

# Experimental Results

The simulations are run in the same way as CE 11 test. The results below are obtained on top of CE11 test 2.2, so including the specific GRA signalization (section 1.2). It is intended that similar improvements can be obtained when apply without specific GRA signalization (section 1.1).

The reported improvement over CE11 test 2.2 is of -3.41% in class E, -1.05% in class C and -1.22% in class D.

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0_CE11_22.exe |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class C | -1,05% | -2,13% | -2,31% | 97% | #NOMBRE! |
| Class E | -3,41% | -4,63% | -5,05% | 100% | #NOMBRE! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | -1,22% | -1,78% | -1,68% | 100% | #NOMBRE! |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |

Table 1 Performance of proposed modification (Option 2) over CE11 test 2.2.

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 with N0438 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class C | 4,60% | 0,59% | 1,23% | 94% | #NOMBRE! |
| Class E | 10,17% | 3,95% | 6,57% | 97% | #NOMBRE! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | 7,50% | 10,03% | 10,69% | 101% | #NOMBRE! |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |

Table 2. Performance of proposed modification (Option 2) over CE11 anchor.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] D.Gommelet, J.-M. Thiesse, D.Nicholson, “CE11: Gradual Random Access (GRA) using encoder and normative restrictions (Tests CE11-2.1.a, CE11-2.1.b and CE11-2.2)”, JVET-O0100, July 2019.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).