| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1049 |
| --- | --- |

| Title: | Crosscheck of JVET-O0084 (8-bit implementation and simplification of MIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

This document reports the verification of results presented in JVET-O0084, where a variant of MIP using 8-bit coefficient tables is described. In addition to the table bit depth reduction, a number of other adjustments are also made: use of raw samples in the upsampling process and alignment of reference sample padding with “conventional” prediction modes. The code was inspected and found to be consistent with the description in the document. BD results also match. The tool off test was not conducted.

# Results

## Adjustments only

The following results were obtained when doing only the adjustments re upsampling and padding.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | -0.12% | -0.04% | -0.03% | 99% | 100% |
| Class A2 | -0.01% | -0.05% | -0.07% | -0.02% | 99% | 98% |
| Class B | -0.01% | -0.10% | -0.07% | -0.03% | 98% | 97% |
| Class C | 0.02% | -0.06% | 0.03% | 0.01% | 98% | 94% |
| Class E | -0.01% | -0.02% | -0.02% | -0.01% | 98% | 101% |
| Overall | -0.01% | -0.07% | -0.04% | -0.01% | 98% | 98% |
| Class D | 0.03% | 0.17% | -0.14% | 0.04% | 95% | 90% |
| Class F | -0.01% | 0.00% | 0.04% | 0.00% | 99% | 100% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | -0.28% | 0.02% | -0.01% | 99% | 101% |
| Class A2 | 0.02% | 0.03% | -0.12% | 0.01% | 99% | 100% |
| Class B | 0.01% | -0.11% | -0.22% | -0.01% | 97% | 96% |
| Class C | -0.02% | -0.24% | -0.04% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.15% | -0.10% | -0.01% | 99% | 99% |
| Class D | 0.03% | -0.01% | 0.03% | 0.04% | 103% | 100% |
| Class F | 0.02% | -0.05% | 0.20% | 0.03% | 100% | 98% |

## All changes

The following results were obtained when enabling all changes.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.09% | -0.14% | -0.25% | -0.10% | 101% | 98% |
| Class A2 | -0.05% | -0.10% | -0.12% | -0.07% | 99% | 100% |
| Class B | -0.06% | -0.12% | -0.08% | -0.06% | 98% | 97% |
| Class C | -0.02% | -0.07% | 0.04% | -0.02% | 98% | 95% |
| Class E | -0.06% | -0.08% | -0.07% | -0.06% | 96% | 93% |
| Overall | -0.05% | -0.10% | -0.08% | -0.06% | 99% | 97% |
| Class D | -0.03% | 0.05% | -0.02% | -0.01% | 96% | 91% |
| Class F | -0.01% | 0.05% | -0.06% | 0.00% | 95% | 95% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.06% | -0.35% | -0.06% | -0.07% | 97% | 100% |
| Class A2 | -0.02% | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class B | -0.03% | -0.01% | -0.22% | -0.04% | 97% | 96% |
| Class C | -0.02% | -0.08% | -0.01% | -0.03% | 100% | 98% |
| Class E |  |  |  |  |  |  |
| Overall | -0.03% | -0.09% | -0.09% | -0.04% | 98% | 98% |
| Class D | 0.01% | -0.07% | 0.09% | 0.00% | 101% | 97% |
| Class F | -0.06% | -0.14% | -0.01% | -0.05% | 100% | 99% |