| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1058 |
| --- | --- |

| Title: | Cross-check of JVET-O0380 (Non-CE6: Simplification on MTS index coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Y. Chen F. Le Léannec 975, av. des Champs Blancs CS 17616 35576 Cesson-Sévigné | Tel: Email: | +33(0)256856325 ya.chen@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

The cross-checker reports the source code provided by Hikvision was carefully analysed and found to be consistent with the description in JVET-O0380. The cross checker could exactly reproduce the results reported in JVET- O0380 as the “proposed technique”. The measured encoder and decoder run time ratios are similar to those reported in JVET- O0380, considering the timing measurements inaccuracy.

# Cross-check of JVET-O0380

The document reports the result of a cross check for JVET- O0380[1]:

- The source code provided by Hikvision was carefully analysed. It is confirmed that the implementation matches the description in JVET- O0380.

- Simulations were conducted according to the common test conditions[2] for the VTM-5.0 in the AI, RA and LDB configuration settings.

- The simulations are completed. The test results match the results reported in JVET- O0380 as the “proposed technique”. The run times measured are consistent with the run times provided by the proponent, assuming some timing measurements errors.

One should notice in the VTM-5.0, there is a mismatch with spec in tu_mts_idx syntax.

In current VTM-5.0 implementation:

enum MTSIdx

{

MTS_DCT2_DCT2 = 0,

MTS_SKIP = 1,

MTS_DST7_DST7 = 2,

MTS_DCT8_DST7 = 3,

MTS_DST7_DCT8 = 4,

MTS_DCT8_DCT8 = 5

};

According to specification (JVET-N1001-v9):

tu_mts_idx[ x0 ][ y0 ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated luma transform block.

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | 1 | 1 | 2 | 2 |

The specification should be replaced according to the VTM-5.0 implementation as:

tu_mts_idx[ x0 ][ y0 ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated luma transform block. When tu_mts_idx[ x0 ][ y0 ] equals to 1, it specifies no transform is applied to the luma transform block.

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | x | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | x | 1 | 1 | 2 | 2 |

Partial detailed results can be found in the attached spreadsheets. The partial results of the cross check are summarized below (others are still running):

# Test 1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | -0.06% | 101% | 100% |
| Class B | 0.01% | -0.04% | 0.07% | 101% | 103% |
| Class C | 0.00% | -0.08% | 0.05% | 100% | 101% |
| Class E | -0.01% | -0.02% | -0.04% | 100% | 99% |
| Overall | 0.00% | -0.04% | 0.01% | 100% | 101% |
| Class D | 0.02% | 0.01% | -0.09% | 101% | 100% |
| Class F | -0.01% | -0.04% | -0.09% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.09% | 101% | 101% |
| Class A2 | 0.01% | 0.13% | -0.01% | 101% | 102% |
| Class B | 0.00% | -0.13% | -0.14% | 100% | 102% |
| Class C | -0.08% | 0.01% | -0.15% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.07% | 100% | 102% |
| Class D | -0.02% | 0.23% | 0.14% | 100% | 101% |
| Class F | 0.00% | -0.05% | 0.03% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.07% | 0.05% | 0.21% | 101% | 109% |
| Class E | 0.01% | 0.62% | 0.62% | 101% | 108% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.36% | -0.01% | 100% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Test 2:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.07% | -0.07% | 100% | 101% |
| Class B | 0.01% | -0.03% | 0.06% | 100% | 103% |
| Class C | 0.00% | -0.08% | 0.05% | 100% | 102% |
| Class E | -0.01% | -0.01% | -0.04% | 99% | 100% |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 101% |
| Class D | 0.01% | 0.02% | -0.09% | 100% | 100% |
| Class F | -0.01% | -0.04% | -0.09% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.20% | 0.10% | 100% | 101% |
| Class A2 | 0.00% | 0.06% | -0.06% | 101% | 102% |
| Class B | 0.01% | -0.16% | -0.19% | 100% | 103% |
| Class C | -0.07% | 0.02% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.08% | -0.09% | 100% | 102% |
| Class D | -0.02% | 0.22% | 0.13% | 100% | 101% |
| Class F | 0.00% | -0.02% | 0.03% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.05% | 0.10% | -0.29% | 102% | 109% |
| Class E | 0.11% | 0.28% | 0.59% | 102% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.10% | 0.55% | 100% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- X. Gao, F. Chen and L. Wang, “Non-CE6: Simplification on MTS index coding”, JVET- O0380, Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.