| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1061 |
| --- | --- |

| Title: | Lossless coding for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Xianglin Wang Kwai Inc. 8910 University Center Lane, Suite 400, San Diego, CA, 92122 Tangi Poirier, Fabrice Le Léannec, Karam Naser, Edouard Francois InterDigital 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE Hyeongmun Jang, Junghak Nam, Naeri Park, Jungah Choi, Seunghwan Kim, Jaehyun Lim LG Electronics Inc. | Email: | tsung-chuanma@kwai.com yiwenchen@kwai.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com hm.jang@lge.com |
| Source: | InterDigital KWAI LG Electronics Inc. |  |  |

_____________________________

# Abstract

This joint proposal combines the technology describes in the three contributions [3], [4] and [5] from 15th JVET meeting about lossless coding. Both Software and specification text are proposed.

The lossless coding mode is achieved by simply bypassing transform, quantization, and in-loop filters (de-blocking filter, sample adaptive offset, and adaptive loop filter). In VVC, several new tools are developed for transform, quantization and in-loop filter but some of them does not support lossless coding.

Meanwhile, lossless mode is included in the VTM software but the encoder fails with --TransquantBypassEnable=1 as reported by ticket #205 and is not included in the Draft Text as reported by ticket #333. In order to support lossless coding in VVC, this contribution suggests cu_transquant_bypass_flag and operation of low-level coding tools when cu_transquant_bypass_flag is equal to 1 as follows:

- Transform/quantization bypass enable flag at PPS

- Transform/quantization bypass flag for coding unit level

- Syntaxes related transform which is MTS, LFNST, SBT and transform skip.

- Sign data hiding disable for transform/quantization bypass block.

- Disable LMCS for lossless mode

- Disable JointCbCr for transform/quantization bypass block.

- ALF processing for transform/quantization bypass block.

# Proposed method

## Transform/quantization bypass enable flag at PPS

Transform/quantization bypass enable flag is proposed in PPS.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

transquant_bypass_enable_flag equal to 1 specifies that cu_transquant_bypass_flag is present. transquant_bypass_enable_flag equal to 0 specifies that cu_transquant_bypass_flag is not present.

## Transform/quantization bypass flag for coding unit level

In VTM-5.0, cu_transquant_bypass_flag is allowed in coding unit level as below. However this syntax is not described in draft specification yet. In this proposal, it is recommended that draft specification match to SW.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if(transquant_bypass_enable_flag) |  |
| cu_transquant_bypass_flag | ae(v) |
| ….. |  |
| } |  |

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process and the in-loop filter process are bypassed when cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

## Syntax signaling related transform which is SBT, MTS, LFNST and transform skip

If cu_transquant_bypass_flag is set equal to 1, scaling and transform process are bypassed. Hence, syntaxes which is related transform processing are not required. The proposed method suggest that MTS, LFNSST index, sbt flag and transform skip flag is not signalled when cu_transquant_bypass_flag is equal to 1.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if(!cu_transquant_bypass_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if(!cu_transquant_bypass_flag && Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) ) |  |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / 2 |  |
| hC = CbHeight[ x0 ][ y0 ] / 2 |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| if( ( tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( !cu_transquant_bypass_flag && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else if(!cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| else |  |  |
| log2ZoTbWidth = log2TbWidth |  |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else if(!cu_transquant_bypass_flag[ x0 ][ y0 ]) |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| else |  |  |
| log2ZoTbHeight = log2TbHeight |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| …. |  |
| if( dep_quant_enabled_flag ¦ ¦ !sign_data_hiding_enabled_flag) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| ….. |  |
| } |  |  |

## Sign data hiding disable for transform/quantization bypass block

In VTM-5.0 SW, sign data hiding is disabled when cu_transquant_bypass_flag is equal to 1. However this part also is missed in draft spec. in order to match draft specification to SW. in this proposal, sign data hiding is disabled when cu_transquant_bypass_flag is equal to 1.

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( dep_quant_enabled_flag ¦ ¦ !sign_data_hiding_enabled_flag ¦¦ cu_transquant_bypass_flag) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| } |  |  |

## Disable LMCS for lossless mode

In the proposed method, it is suggested that LMCS is disabled when lossless mode is enabled.

slice_lmcs_enabled_flag equal to 1 specifies that luma mappin with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| ….. |  |
| if( sps_lmcs_enabled_flag && ! transquant_bypass_enabled_flag) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(5) |
| if( !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| ….. |  |
| byte_alignment( ) |  |
| } |  |

### Picture reconstruction process #### General Inputs to this process are: - a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component, - the variables nCurrSw and nCurrSh specifying the width and height, respectively, of the current block, - a variable cIdx specifying the colour component of the current block, - an (nCurrSw) x (nCurrSh) array predSamples specifying the predicted samples of the current block, - an (nCurrSw) x (nCurrSh) array resSamples specifying the residual samples of the current block. Output of this process is a reconstructed picture sample array recSamples. Depending on the value of the colour component cIdx, the following assignments are made: - If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y. - Otherwise, if cIdx is equal to 1, tuCbfChroma is set equal to tu_cbf_cb[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C. - Otherwise (cIdx is equal to 2), tuCbfChroma is set equal to tu_cbf_cr[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C. Depending on the value of slice_lmcs_enabled_flag, the following applies: - If transquant_bypass_enable_flag is equal to 1 or slice_lmcs_enabled_flag is equal to 0, the (nCurrSw)x(nCurrSh) block of the reconstructed samples recSamples at location ( xCurr, yCurr ) is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1: recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) (8-993) - Otherwise (transquant_bypass_enable_flag is equal to 0 and slice_lmcs_enabled_flag is equal to 1), the following applies: - If cIdx is equal to 0, the following applies: - The picture reconstruction with mapping process for luma samples as specified in clause 8.7.5.2 is invoked with the luma location ( xCurr, yCurr ), the block width nCurrSw and height nCurrSh, the predicted luma sample array predSamples, and the residual luma sample array resSamples as inputs, and the output is the reconstructed luma sample array recSamples. - Otherwise (cIdx is greater than 0), the picture reconstruction with luma dependent chroma residual scaling process for chroma samples as specified in clause 8.7.5.3 is invoked with the chroma location ( xCurr, yCurr ), the transform block width nCurrSw and height nCurrSh, the coded block flag of the current chroma transform block tuCbfChroma, the predicted chroma sample array predSamples, and the residual chroma sample array resSamples as inputs, and the output is the reconstructed chroma sample array recSamples.

## Disable JointCbCr for transform/quantization bypass block

Joint coding of Cb-Cr is not inversible so are also be disabled at CU-level if the cu_transquant_bypass_flag is true

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) ) |  |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / 2 |  |
| hC = CbHeight[ x0 ][ y0 ] / 2 |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| if( ( tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded && !cu_transquant_bypass_flag) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( !cu_transquant_bypass_flag && tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

## ALF processing for transform/quantization bypass block

The proposed method is that ALF is bypassed when cu_transquant_bypass_flag of current sample is equal to 1. Current ALF process allows filtering even though cu_transquant_bypass_flag is equal to 1. Current DBF and SAO processing already considers transform/quantization bypassed block.

Subcluase 8.9.5.2 Coding tree block filtering process for luma samples - The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows: - If pcm_loop_filter_disabled_flag and pcm_flag[ xCtb+ x ][ yCtb + y ] are both equal to 1, the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = recPictureL[ hx, vy ] (8-1207) - Othersiwe if cu_transquant_bypass_flag[ xCtb+ x ][ yCtb + y ] is equal to 1, the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = recPictureL[ hx, vy ] (8-1207) - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum ) (8-1208) Subclause 8.8.5.4 Coding tree block filtering process for chroma samples - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows: - If pcm_loop_filter_disabled_flag and pcm_flag[ ( xCtbC + x ) * SubWidthC ][ ( yCtbC + y ) * SubHeightC ] are both equal to 1, the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = recPicturec[ hx, vy ] (8-1259) - Otherwise, if cu_transquant_bypass_flag[ ( xCtbC + x ) * SubWidthC ][ ( yCtbC + y ) * SubHeightC ] is equal to 1, the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = recPicturec[ hx, vy ] (8-1259) - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum ) (8-1260)

# Experimental result

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementation are shown in Table 1 below against the HM-16.19 under CTC [2] with the additional configurations “--TransquantBypassEnable=1 --CUTransquantBypassFlagForce=1 --CostMode=lossless”. Results of TGM classes are also included.

Table 1. Performance comparison between HM-16.19 and proposed modifications on VTM-5.0 (to be updated, *: only finished sequence)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | HM | VTM |  | HM | VTM |  | HM | VTM |  |
| Class A1 | 2.1 | 2.2 | -6.11% | 2.2 | 2.3 | -3.95% |  |  |  |
| Class A2 | 1.7 | 1.7 | -3.57% | 1.8 | 1.9 | -2.59% |  |  |  |
| Class B | 2.0 | 2.1 | -4.26% | 2.2 | 2.3 | -2.67% | x.xx | x.xx | x.xx% |
| Class C | 2.0 | 2.1 | -5.34% | 2.5 | 2.6 | -3.24% | 2.5 | 2.6 | -2.79% |
| Class D | 1.8 | 1.9 | -5.37% | 2.6 | 2.8 | -3.75% | 2.7 | 2.7 | -3.15% |
| Class E | 2.7 | 2.9 | -6.94% |  |  |  | 3.2 | 3.3 | -2.57% |
| Class F | 4.5 | 5.5 | -16.77% | 30.3 | 40.3 | -14.18% | 61.9* | 79.9* | -14.31% |
| Overall | 2.1 | 2.2 | -5.25% | 2.2 | 2.2 | -3.11% | 1.9* | 2.0* | -1.78% |
| TGM | 6.2 | 13.1 | -48.71% | 81.3 | 114.0 | -30.10% | 108.0 | 142.8 | -26.87% |
| Enc Time[%] | 3337% |  |  | 1078% |  |  | 838% |  |  |
| Dec Time[%] | 192% |  |  | 162% |  |  | 131% |  |  |

# Additional proposed method

It is also suggested that further restriction is applied on top of the proposed method to improve complexity reduction and to prevent potential lossy compression.

## Disallowing BDOF and DMVR

It is observed that BDOF and DMVR do not introduce coding performance improvement for lossless coding. In this proposal, it is proposed to disabled BDOF and DMVR for a lossless coded CU.

- - - - - - - -

# Experimental result including additional restriction for lossless mode

For Table 2, the result snows the comparison between HM-16.19 and proposed implementation with disabling BDOF and DMVR. The Table 3. compares the difference between TEST 1 and TEST 2. As the result shown, disabling DMVR and BDOF for lossless VVC reduces 5% runtime in both encoding and decoding without coding performance loss.

Table 2. Performance comparison between HM-16.19 and proposed modifications on VTM-5.0 (Disable BDOF and DMVR) (to be updated)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | HM | VTM |  | HM | VTM |  | HM | VTM |  |
| Class A1 | 2.1 | 2.2 | -6.11% | 2.2 | 2.3 | -3.98 % |  |  | 0.00% |
| Class A2 | 1.7 | 1.7 | -3.57% | 1.8 | 1.9 | -2.61% |  |  | 0.00% |
| Class B | 2.0 | 2.1 | -4.26% | 2.2 | 2.3 | -2.72% | x.x | x.x | x.xx % |
| Class C | 2.0 | 2.1 | -5.34% | 2.5 | 2.6 | -3.19% | 2.5 | 2.6 | -2.79% |
| Class D | 1.8 | 1.9 | -5.37% | 2.6 | 2.8 | -3.60% | 2.7 | 2.7 | -3.15% |
| Class E | 2.7 | 2.9 | -6.94% |  |  |  | x.x | x.x | x.xx % |
| Class F | 4.5 | 5.5 | -16.77% | 30.3 | 40.3 | -14.20% | x.x | x.x | x.xx % |
| Overall | 2.1 | 2.2 | -5.25% | 2.2 | 2.2 | -3.12% | x.x | x.x | x.xx % |
| TGM | 6.2 | 13.1 | -48.71% | 81.3 | 113.8 | -30.08% | 108.0 | 142.8 | -26.87% |
| Enc Time[%] | 3201% |  |  | 1025% |  |  | XXX% |  |  |
| Dec Time[%] | 183% |  |  | 154% |  |  | XXX% |  |  |

Table 3. Performance comparison between proposed VVC with BDOF/DMVR (TEST1) and w/o BDOF/DMVR (TEST2). TEST 1 is anchor while TEST 2 is test.

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | TEST1 | TEST2 |  |
| Class A1 | 2.3 | 2.3 | -0.02% |
| Class A2 | 1.9 | 1.9 | -0.02% |
| Class B | 2.3 | 2.3 | -0.05% |
| Class C | 2.6 | 2.6 | 0.06% |
| Class D | 2.8 | 2.8 | 0.16% |
| Class E | 0.0 | 0.0 | 0.00% |
| Class F | 40.3 | 40.3 | -0.01% |
| Overall | 2.2 | 2.2 | -0.01% |
| TGM | 114.0 | 113.8 | 0.00% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 95% |  |  |

Moreover, the VTM with the proposed modifications also supports the mixed of lossy and lossless CUs coding (i.e. the middle area of each picture is forced to be coded in lossless mode while the other area are lossy coded) with the configurations “--TransquantBypassEnable=1 --CUTransquantBypassFlagForce=0 --CostMode=mixed_lossless_lossy”. Some examples of the reconstructed pictures are shown in Figure1. In these examples, a rectangle located in the middle is the region of interest and is coded as lossless while other area are lossy coded using the regular configurations. Figure 1 is an example for mixed_lossless_lossy coding with proposed modification in VTM while Figure 2 is the original source frame for comparison.

Sequence: BlowingBubble, Qp=32, CostMode=mixed_lossless_lossy, Frame # 389, Low delay B Picture

Figure 1. reconstructed pictures with the mixed coding of lossy and lossless CUs.

Sequence: BlowingBubble, Frame # 389

Figure 2. original pictures of Figure 1.

# Conclusion

In this proposal, several modifications are proposed to VVC to support lossless coding. Two tests are provided in this proposal.

Test1 is lossless VVC with DMVR and BDOF. The result of Test1 is -5.25% (AI), -3.11% (RA) and -xx.xx%(LB) for CTC sequences, respectively. For class F, the result is -16.77% (AI), -14.18% (RA) and -xx.xx%(LB). For class TGM, the result is -48.71% (AI), -30.10% (RA) and -26.87%(LB).

Test2 is lossless VVC without DMVR and BDOF. The result of Test2 is -5.25% (AI), -3.12% (RA) and -xx.xx%(LB) for CTC sequences, respectively. For class F, the result is -16.77% (AI), -14.20% (RA) and -xx.xx%(LB). For class TGM, the result is -48.71% (AI), -30.08% (RA) and -26.87 %(LB).

According to the above results, it is recommended to adopt the proposed modifications into VVC.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, Geneva, CH, March 2019.

- “On lossless coding for VVC”, T. Poirier, F. Le Léannec, K. Naser, E. Francois, Document JVET-O0460, Gothenburg, SE, July 2019

- “Comments on transform quantization bypassed mode”, H. Jang, J. Nam, N.Park, J. Choi, S. Kim, J. Lim, Document JVET-O0584, Gothenburg, SE, July 2019

- “Modifications to support the lossless coding”, T.-C. Ma, Y.-W. Chen, X. Xiu, X. Wang, Document JVET-O0591, Gothenburg, SE,

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).