| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1062-v1 |
| --- | --- |

| Title: | Cross-check of JVET-O0372 (Non-CE6: A Simplification for MTS Index Signaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Anish Tamse Minsoo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com mss.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross check results of JVET-O0372 titled “Non-CE6: A Simplification for MTS Index Signaling”. The simulation was carried out under CTC. The results provided by the proponent were verified.

# Cross Check Results

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Proponent provided the source code of their proposal based on the VTM-5.0 software and results. The results match with those provided by the proponent.

The proposal proposes to simplify the coding of MTS index by using fewer context models. More specifically, the context model for the 0th bin is fixed to 0 (instead of computing based on cqtDepth whose output ranged from 0-5)

Table 1: Assignment of ctxInc to syntax elements tu_mts_idx with context coded bins (VTM 5.0)

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | cqtDepth | 6 | 7 | 8 | na | na |

Table 2: Assignment of ctxInc to syntax elements tu_mts_idx with context coded bins (proposed)

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | 0 | 6 | 7 | 8 | na | na |

Table 1 shows the results of test with the proposed method.

Table 1: Summary of the BD-rates, EncT, and DecT for Test 1

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | 0.00% |  | -0.04% |  | 0.03% |  | 99% |  | 99% |  |
| Class A2 |  | 0.00% |  | -0.05% |  | -0.04% |  | 97% |  | 99% |  |
| Class B |  | 0.00% |  | -0.05% |  | 0.01% |  | 98% |  | 99% |  |
| Class C |  | 0.00% |  | -0.11% |  | -0.03% |  | 99% |  | 98% |  |
| Class E |  | -0.01% |  | -0.04% |  | -0.05% |  | 98% |  | 99% |  |
| Overall |  | 0.00% |  | -0.06% |  | -0.01% |  | 98% |  | 99% |  |
| Class D |  | 0.00% |  | 0.05% |  | -0.12% |  | 99% |  | 96% |  |
| Class F |  | -0.01% |  | 0.02% |  | 0.06% |  | 99% |  | 99% |  |
|  |  |  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -0.02% |  | -0.15% |  | 0.11% |  | 99% |  | 99% |  |
| Class A2 |  | 0.01% |  | -0.05% |  | -0.06% |  | 99% |  | 99% |  |
| Class B |  | 0.00% |  | 0.06% |  | -0.11% |  | 98% |  | 99% |  |
| Class C |  | -0.04% |  | -0.36% |  | -0.08% |  | 99% |  | 98% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.01% |  | -0.12% |  | -0.05% |  | 99% |  | 99% |  |
| Class D |  | -0.05% |  | 0.19% |  | -0.07% |  | 99% |  | 98% |  |
| Class F |  | -0.05% |  | 0.13% |  | -0.06% |  | 98% |  | 97% |  |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class C |  | 0.04% |  | -0.01% |  | 0.05% |  | 99% |  | 100% |  |
| Class E |  | 0.01% |  | -0.20% |  | 0.56% |  | 98% |  | 99% |  |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class D |  | 0.04% |  | -0.07% |  | 0.09% |  | 98% |  | 96% |  |
| Class F |  | -0.06% |  | -0.11% |  | 0.40% |  | 98% |  | 97% |  |

# Reference

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, March 2019.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).