| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1063-v1 |
| --- | --- |

| Title: | Cross-check of JVET-O0373 (Non-CE6: An Improved Context Modeling for LFNST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Anish Tamse Minsoo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com mss.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosscheck results of JVET-O0373 titled “Non-CE6: An Improved Context Modeling for LFNST”. The simulation was carried out under CTC. The results provided by the proponent were verified.

# Cross Check Results

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Proponent provided the source code of their proposal based on the VTM-5.0 software and results. The results match with those provided by the proponent.

The proposal proposes to improve the coding performance by adding another context to the first bin of LFNST index. More specifically, the changes to the coding scheme in the code are as follows:

#if QC_LFNST_Separate_CTX

unsigned cctx = 0;

if( CS::isDualITree( *cu.cs ) && isLuma( cu.chType ) )

{

if( cu.firstTU->mtsIdx < MTS_DST7_DST7 )

{

cctx = 1; // Luma DCT-2

}

}

if( isChroma( cu.chType ) )

{

if( cu.firstTU->mtsIdx < MTS_DST7_DST7 )

{

cctx = 2; // Chroma DCT-2

}

}

#else

unsigned cctx = 0;

if( cu.firstTU->mtsIdx < MTS_DST7_DST7 && CS::isDualITree( *cu.cs ) ) cctx++;

#endif

Table 1 shows the results of test with the proposed method.

Table 1: Summary of the BD-rates, EncT, and DecT for Test 1

# Reference

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, March 2019.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).