| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1065-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0470 on encoder simplification of local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents cross-check results for JVET-O0470 on encoder simplification of local illumination compensation. The cross-checker results match the result of the proponent presented in JVET-O0470.

# Introduction

In JVET-O0470 encoder modification is applied to CE4-3.2b test. There are two encoder modification tested:

Test 1: Uni-directional motion is stored in the motion search process to enable LIC-on test even if LIC-off motion is bi-directional

Test 2: Test 1 with additional motion search with MRSAD is performed when LIC usage is available.

# Test results

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.36% | -0.21% | 107% | 102% |
| Class A2 | -0.17% | -0.32% | -0.27% | 109% | 103% |
| Class B | -0.23% | -0.11% | -0.25% | 103% | 98% |
| Class C | -0.16% | -0.26% | -0.29% | 105% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.24% | -0.26% | 106% | 100% |
| Class D | 0.03% | 0.03% | 0.39% | 107% | 103% |
| Class F | -0.37% | -0.23% | -0.24% | 101% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.05% | -0.24% | 107% | 102% |
| Class C | -0.12% | -0.46% | -0.30% | 108% | 104% |
| Class E | 0.07% | 0.27% | -0.51% | 99% | 93% |
| Overall | -0.12% | -0.11% | -0.33% | 105% | 100% |
| Class D | 0.04% | -0.81% | -0.35% | 103% | 87% |
| Class F | -1.35% | -1.17% | -1.90% | 101% | 94% |

Test 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.56% | -0.38% | 120% | 102% |
| Class A2 | -0.34% | -0.29% | -0.42% | 121% | 102% |
| Class B | -0.38% | -0.18% | -0.33% | 114% | 98% |
| Class C | -0.24% | -0.37% | -0.28% | 115% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.33% | -0.34% | 117% | 99% |
| Class D | -0.01% | 0.06% | 0.15% | 115% | 102% |
| Class F | -0.52% | -0.35% | -0.33% | 110% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.24% | -0.11% | 121% | 101% |
| Class C | -0.26% | -0.43% | -0.83% | 120% | 104% |
| Class E | 0.04% | 0.20% | -0.26% | 116% | 95% |
| Overall | -0.23% | -0.20% | -0.39% | 119% | 100% |
| Class D | -0.02% | -0.24% | -0.37% | 114% | 91% |
| Class F | -1.69% | -1.91% | -1.81% | 111% | 98% |

Test 2