| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1076-v1 |
| --- | --- |

| Title: | Report of BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Jill Boyce (Intel) | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com jill.boyce@intel.com |
| Source: | BoG coordinators |  |  |

_____________________________

# Abstract

The BoG met on the following days and time slots:

- 3 July 2019 Wednesday from 16:00 to 21:12

- 5 July 2019 Thursday from 09:00 to 20:35

The BoG recommends the following:

- Adopt nonref_flag, in the slice header, unconditioned. [JVET-O0181]

- Adopt the normative changes to the low-level decoding processes for treating rectangular regional boundaries as picture boundaries proposed in JVET-O0138, JVET-O0141, and JVET-O0394. Apply to the boundaries of sub-pictures, where each sub-picture consists of one or more rectangular-mode slices that collectively form a rectangular region. [JVET-O0138, JVET-O0141, JVET-O0394]

- Adopt signalling sub-picture location and size in the SPS (unless a sub-picture based GRA solution is agreed in the CE11 discussion), using the SPS syntax in O0555 for this purpose as a starting point, with the sub-picture ID replaced with grid index (not considering scalability, ARC/RPR, GRA). [JVET-O0555]

- Adopt the following, pending on review of the text (J. Boyce to coordinate the work on the text):

  - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether the sub-picture boundaries are treated as picture boundaries. [JVET-O0141]

  - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether in-loop filtering (deblocking, SAO, and ALF) across sub-picture boundaries are enabled. [JVET-O0141]

# Review of contributions

## AHG17: General high-level syntax (64)

### NAL unit header and reference picture indication (7

General discussion on NUH

Chaired by J. Boyce, 3 July.

The existing HEVC design was mentioned as being also worth considering. The number of bits for NUT values using 5 bits instead of 6 was considered to be sufficient.

Having an extension mechanism is desirable. Can either reserve a particular layer value, or can reserve a bit. If a layer value is reserved, the existing 7 bits can’t be used for the extension, and would require additional bits in the extension, but is more efficient when the extension isn’t used.

It was suggested that a variable length design allowing a single byte to be used in the typical case be considered. This was discussed at the previous meeting.

Revisit the contributions in this area.

JVET-O0146 AHG17: On unspecified NAL unit types [Y.-K. Wang (Futurewei)]

Chaired by J. Boyce, 3 July.

Currently, the TemporalId of all the unspecified NAL unit types have to be equal to 0. This contribution proposes to reallocate NAL unit types such that some unspecified NAL unit types can have TemporalId greater than 0.

JVET-O0179 On NAL Unit Header Design [S. Deshpande (Sharp)]

Chaired by J. Boyce, 3 July.

Modifications are proposed to NAL unit header design.

It is proposed to redesign the NAL unit header as follows:

- The first byte consists of syntax element nuh_layer_id. For this syntax element, values 188 onwards are forbidden. Value 0 to 126, inclusive are defined to have meaning same as currently in VVC WD for independent layers. Additionally values 127 to 187, inclusive are reserved – to be possibly used for spatial/ view/ SNR scalability later.

- The second byte consists of 5-bit syntax element nal_unit_type, followed by 3-bit syntax element nuh_temporal_id_plus1.

In this proposal, 8 bits are allocated to nuh_layer_id, with values 127 to 187 reserved.

Some support was expressed for this contribution.

A question was raised about how this approach would be used for larger number of layers in an extension. It is noted that the existing draft spec also doesn’t explicitly describe that case.

Additional proposals if the reordering of syntax elements proposal were not adopted.

Proposal 1: temporal_id_plus1 is replaced by temporal_id, because of the position of temporal_id in the current design doesn’t require avoiding 0. If we keep the current syntax, there is support for doing this, unless any problems are identified.

Proposal 2: remove the reserved bit in the NUH and give layer id another bit.

Proposal 3: changes the restriction of allowable temporal id values with particular NUT values.

JVET-O0237 AHG17: Compact NAL unit header [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

Chaired by J. Boyce, 3 July.

This contribution proposes two changes related to the NAL unit header in VVC.

Firstly, it is proposed to signal an extension flag in the NAL unit header that specifies if the NAL unit header is signaled with one byte or if the NAL unit header is extended with one additional byte. The extension byte is needed to express a NuhLayerId larger than 0 or for certain less commonly used NAL unit types (e.g. reserved or unspecified NAL unit types).

More specifically, the following changes to the NAL unit header are proposed in this contribution:

- Signal a vcl_flag indicating whether the NAL unit is a VCL or non-VCL NAL unit

- Signal nal_unit_type_lsb with 2 bits instead of 4

- Signal a nuh_extension_flag specifying whether a 1-byte NAL unit header extension is present

- If the extension is present signal nuh_layer_id_plus1, nal_unit_type_ext_bit and nuh_reserved_zero_bit in a second NAL unit header byte, else infer nuh_layer_id_plus1 to be equal to 1 and nal_unit_type_ext_bit and nuh_reserved_zero_bit to be equal to 0

- Signal nuh_layer_id_plus1 with 6 bits as in HEVC instead of 7 bits

- Set NALUnitType equal to ( zero_tid_required_flag  <<  4 ) + (vcl_flag << 3 ) + ( nal_unit_type_ext_bit << 2 ) + nal_unit_type_lsb

The estimated luma BD rate numbers by the change are reported to be 0.0%/0.0%/-0.1%/-0.1% for AI/RA/LDB/LDP respectively and the proponents claim higher savings when multiple slices per pictures are used.

Secondly, it is claimed by the proponents that in the current VVC draft there are no reserved IRAP NAL unit type values and that the unspecified NAL unit types are required to have temporal ID equal to 0. This contribution proposes to reserve two IRAP NAL unit type values and to allow two of the four unspecified NAL unit types to have a non-zero temporal ID, at the cost of removing two of the reserved non-IRAP NAL unit type values.

Second aspect is similar to JVET-O0146, but splits the existing reserved VCL NUTs into IRAP and non IRAP types. Some support was expressed that if the NUT syntax is kept and the allocation of values changed, this may be better than the JVET-O0146 proposal.

First aspect proposes variable length NAL unit, where second byte is only present sometimes. Reduces the number of layers than can be signaled and allocates  a bit to indicate that more that 2 bytes are present in the NUH. Redefines the first of the 4 NUT bits for vcl type.

It was remarked that at the previous meeting it had been discussed to use the temporal Id to condition the presence of the second bit.

JVET-O1037 AHG17: On NAL unit header [Y.-K. Wang (Futurewei)]

JVET-O1042 AHG17: Varying-length NAL unit header [M. M. Hannuksela (Nokia)]

JVET-O0359 AHG17: On identification of reference and non-reference picture [T. Suzuki (Sony)]

Chaired by Y.-K. Wang, July 3rd.

Proposes to use additional NAL unit types for trailing and STSA pictures (not for other types of pictures, e.g., RASL, RADL) for reference/non-reference picture indication.

If to have this indication for trailing and STSA pictures, why not for RASL and RADL pictures as well?

It seems that then the indication should also be specified for RASL and RADL. And if we do that for RASP and RADL too, then we probably don't have sufficient NAL unit type value space remaining.

It was agreed that if to have this indication for trailing and STSA pictures, we should also have it for RASL and RADL pictures as well.

See notes under JVET-O0181.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 2 in the contribution belongs to this agenda item.

Chaired by Y.-K. Wang, July 3rd.

Proposes slnr_flag and nonref_flag for non-IRAP slices at the beginning of the slice header syntax, to indicate whether this slice belongs to is a sub-layer non-reference picture or a sub-layer reference picture, and whether the picture that this slice belongs to may be not used for inter prediction, respectively.

The claimed benefit: Knowing this information allows pictures to be discarded under certain situations (e.g. playback is lagging real-time).

It was agreed to have only the nonref_flag, in the slice header, unconditioned.

### Parameter set mechanism and parsing dependency (6)

JVET-O0113 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Futurewei), J. Boyce (Intel)]

JVET-O0155 AHG17: Resolving PPS parsing dependency on SPS [Hendry, Y.-K. Wang (Futurewei)]

JVET-O0236 AHG17: Making PPS parsing independent of SPS [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-O0490 AHG17: Independent parsing of parameter sets [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 1 (on parameter set activation) and proposal 4 (on DPS referencing by SPS) in the contribution belong to this agenda item.

JVET-O0268 AHG17: Signalling SPS base and patch [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

### IOP point definition and signalling (2)

JVET-O0044 AHG17: Signalling zero or more Sub-Profiles [S. Wenger, B. D. Choi, S. Liu (Tencent)]

JVET-O0417 AHG17: On general constraint information [P. Bordes, M. Kerdranvat (InterDigital)]

### Picture types, random access, and sub-layer switching (5)

JVET-O0147 AHG17: Constraints on IRAP pictures and leading pictures [Hendry, Y.-K. Wang (Futurewei)]

JVET-O0226 AHG17: On constraint about position of leading pictures and trailing pictures associated to an IRAP [V. Drugeon, K. Abe (Panasonic)]

JVET-O0149 AHG17: On external decoding refresh (EDR) [Y.-K. Wang, J. Chen (Futurewei), H. Yu, L. Yu (ZJU)]

JVET-O0393 AHG17: On cross-RAP referencing [M. M. Hannuksela (Nokia)]

JVET-O0235 AHG17: Ensuring temporal switching with STSA pictures [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

### Reference picture list signalling (2)

JVET-O0148 AHG17: On reference picture list signalling [Y.-K. Wang, Hendry (Futurewei)]

JVET-O0244 AHG17: On zero delta POC in reference picture structure [V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

### Miscellaneous HLS topics (14)

JVET-O0151 AHG17: On no_output_of_prior_pics_flag [Y.-K. Wang (Futurewei)]

JVET-O0154 AHG17: Miscellaneous AHG17 topics [Y.-K. Wang (Futurewei)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposal 5 (also covered in JVET-O0152) in the contribution belongs to this agenda item.

JVET-O0178 On DPB Parameters [S. Deshpande (Sharp)]

JVET-O0201 AHG17: On Gradual Random Access (GRA) [M. Coban, A. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0234 AHG17: Slice header extension [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-O0238 AHG17: Parameters in SPS or slice headers [M. Pettersson, P. Wennersten, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-O0241 AHG17: On Decoding Process Steps [S. Deshpande (Sharp)]

JVET-O0270 AHG17: Signalling refreshed regions associated with a GRA picture [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0391 AHG17: Avoiding dependencies on externally controlled variables in the decoding process [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)]

JVET-O0392 AHG17: Output of SEI messages [M. M. Hannuksela (Nokia)]

JVET-O0497 AHG17: On layer grouping for independent regions and ARC [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

JVET-O0783 AHG17: Control of transform skip residual coding [L Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0891 AHG6/AHG17: Cubemap-based projection syntax for 360-degree videos [Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

### APS and ALF parameters signalling (8)

JVET-O0245 AHG17: On TemporalId and NuhLayerId in APS [V. Seregin, M. Coban, A. K. Ramasubramonian, N. Hu, M. Karczewicz (Qualcomm)]

JVET-O0246 AHG17: Separate luma and chroma ALF APS types [V. Seregin, N. Hu, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0299 AHG17: APS support for default and user defined scaling matrices [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-O0300 AHG17: Clean-up of ALF Syntax Parameters [S. Paluri, J. Lim, S. Kim (LGE)]

JVET-O0301 AHG17: Simplification of ALF Clipping Coefficients in the APS [S. Paluri, J. Lim, S. Kim (LGE)]

JVET-O0302 AHG17: Simplification of ALF Coefficients in the APS [S. Paluri, J. Lim, S. Kim (LGE)]

JVET-O0893 Crosscheck of JVET-O0302 (Non-CE5: Simplification of ALF Coefficients in the APS) [N. Hu (Qualcomm)]

JVET-O0491 AHG17: High-level syntax cleanup [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-O0561 AHG17: APS ID Control Mechanism [S. Paluri, J. Lim, S. Kim (LGE)]

### Virtual boundary signaling (3)

JVET-O0339 AHG17: On virtual bounary signaling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0340 AHG17: On modifiaction of virtual boundaries signaling [G. Bang, W. Lim, H.Y. Kim (ETRI)]

JVET-O0445 [AHG17] Signaling of uniform virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. P. Choi (Samsung)]

### HRD (11)

JVET-O0043 AHG17: Clarification on role of pic_struct to HRD and the output picture process [C. Fogg (MovieLabs)]

JVET-O0152 AHG17: HRD support for GRA and changing the term GRA to GDR [Y.-K. Wang (Futurewei)]

JVET-O0153 AHG17: HRD clean-ups [Y.-K. Wang (Futurewei)]

JVET-O0177 On HRD Signalling [S. Deshpande (Sharp)]

JVET-O0180 On Buffering Period [S. Deshpande (Sharp)]

JVET-O0189 AHG17: Support of decoding unit in HRD [K. Kazui (Fujitsu)]

JVET-O0227 AHG17: Parsing dependency of picture timing SEI message on SPS [V. Drugeon (Panasonic)]

JVET-O0228 AHG17: Harmonized HRD parameters signaling [V. Drugeon (Panasonic)]

JVET-O0495 AHG17: On HRD for Open GOP and DRAP [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

JVET-O0496 AHG17: On HRD for splicing [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)

JVET-O0667 DPB size analysis of VTM5.0 using RA configuration in current CTC [X. Ma, H. Yang (Huawei)]

### VUI and SEI (6)

JVET-O0041 AHG17: On signalling of frame field information [Y.-K. Wang (Futurewei), K. Sühring (HHI), C. Fogg (MovieLabs)]

JVET-O0042 AHG17: Replicating pic_struct into separate elements for future extensibility [C. Fogg (MovieLabs)]

JVET-O0435 AHG17: on pic_struct [S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen (Dolby)]

JVET-O0360 AHG17: On sRGB and sYCC [T. Suzuki (Sony)]

JVET-O0622 AHG2/AHG17: Draft text to update BT.2100 references and ICTCP HLG coding equations [S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen]

JVET-O0492 AHG17: On merging of bitstreams and conformance [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

## AHG12: high-level parallelism and coded picture regions (42)

### General (1)

JVET-O0800 A summary of HLS proposals on coded picture regions [Y.-K. Wang (Futurewei)] [late]

The design questions provided in this contribution were used by the coordinators of the BoG on high-level syntax to structure the discussions of other contributions in this category.

### Sequence-level independent regions (12)

Contributions in this category were discussed on 4 July, 2019, 0900 to 2035 in the BoG on high-level syntax (chaired by Y.-K. Wang, with the discussion and decision on sub-picture ID signalling chaired by S. Deshpande).

General discussion on coding of sequence-level independent regions

- To support coding of sequence-level independent regions based on a normative approach or based on non-normative encoder motion constraints like in MCTS as in HEVC?

  - Based on a normative approach (all the 11 proposals in 6.18.1)

  - Based on encoder motion constraints (none)

- If to support coding of sequence-level independent regions based on a normative approach,

  - to have the normative approach based on treating of rectangular regional boundaries as picture boundaries in decoding motion compensation? (O0138, O0141, O0394)

Comments: It was mentioned that the low-level changes for treating of rectangular regional boundaries as picture boundaries in decoding motion compensation seem clean and acceptable.

A suggestion: Replace "rectangular slices" above with "raster-scan slices" and remove the rectangular slices from VVC. This suggestion was objected by multiple participants.

Agreed with the normative changes to the low-level decoding processes for treating rectangular regional boundaries as picture boundaries proposed in O0138, O0141, and O0394. Apply to the boundaries of sub-pictures, where each sub-picture consists of one or more rectangular-mode slices that collectively form a rectangular region.

Agreed with the following, pending on review of the text (J. Boyce to coordinate the work on the text):

- In the SPS syntax agreed below for signalling of sub-picture size and position, add a flag for each sub-picture to control whether the sub-picture boundaries are treated as picture boundaries.

- In the SPS syntax agreed below for signalling of sub-picture size and position, add a flag for each sub-picture to control whether in-loop filtering (deblocking, SAO, and ALF) across sub-picture boundaries are enabled.

Note that the existing raster scan slice mode would still exist in the VVC design.

    - Should treating of rectangular regional boundaries as picture boundaries based on sub-pictures or slices?

      - Based on slices (O0138, O0394)

      - Based on sub-pictures (O0141, O0394)

What is the use case of allowing for more than one slice within a sub-picture? One possible usage mentioned was that allowing splitting one sub-picture into multiple slices could be useful for delay optimization.

In a layered approach for sub-pictures, treating sub-picture boundaries as picture boundaries comes naturally with the current VVC draft text.

It was commented that, in a layered approach, the client decoder needs to run multiple decoder instances, one for each layer. However, it seems that the approach in O0334 does not necessarily need multiple decoder instances.

On the layered approach in O0334, the following comments/questions were expressed:

- Different sub-pictures within an AU have different POC values. Why? Different sub-pictures with different POC values are decoded by one decoder but have different decoding times. In that case, what is the picture output process? Then it works without using layer ID.

- RPL signalling can be significant less efficient.

- Potentially more POC LSBs might be needed for signalling of POC values, which may affect coding efficiency.

- How would these affect delta POC based scaling of MVs and sample values?

- When merging different sub-pictures layers into one layer, rewriting of slice header would be needed due to the use of different POC values for sub-pictures originated from the same picture.

- Can different sub-pictures originated from the same picture have different values of picture_output_flag? It seems they can be just required to be the same.

- How about HDR behavior? From the CPB behavior point of view, the single-layer process would conceptually work, just the picture rate is n times as great as if no sub-pictures and no layering were used. From the DPB point of view, it could be similarly as the CPB behavior, and in addition to that, the output process for the entire picture after merging the decoded sub-pictures could be specified normatively; this is currently not addressed.

- Some encoder constraints are needed to enable sub-picture based sub-bitstream extraction.

- It was commented that the proposal could be applied to any type of layers, not just sub-picture layers.

- The reference picture marking needs to be layer based, i.e., layer ID is not just metadata.

- It was commented that if we decide to have layered based approach for sub-pictures, we need to consider whether the layered approach here would also be used for scalability.

- This approach does not need a change to any low-level decoding process.

- With a layered approach, there is no way to enable in-loop filtering across the boundaries of the sub-pictures. This could be an issue for some use cases and not for some other use cases.

- If a layered approach is specified, would we still envision the support of decoding based on the original spatial resolution and picture rate?

- Document O0497, O0393 are related to the approach in O0334.

- Can sub-pictures originated from the same picture have different tile/brick configurations? No, the intent is that they have the same tile/brick configuration, but currently there is no constraint for this.

  - Why?

- How is the cropping window defined? Do they refer to the same SPS or different SPSs?

The proponent and other interested participants were requested to work on completing the layered approach based on O0334, taking into account the comments noted above, possibly taking into account related aspects in O0497 and O0393. Revisit after the output of the offline work is available.

On single-layer based approaches:

- The sub-picture sizes would be less flexible than a layered approach, because sub-pictures not at the picture boundaries must contain complete CTUs.

  - Is that flexibility useful? For evenly dividing a CMP picture into sub-pictures for the 6 faces.

    - Note that the CMP face seam issue related to evenly dividing into 6 faces have been addressed by the virtual boundaries.

    - It seems that there is no other technical benefit for evenly dividing a CMP picture into sub-pictures for the 6 faces.

  - On disadvantage of this flexibly for the layered approach is that this disallows merging the coded sub-pictures of different layers into one layer.

Is there a coding efficiency impact for the layered approach if there are many sub-pictures (e.g., 96) and they do not share parameter sets? Same question for POC and RPL signalling.

It was commented that we need a single layer approach anyway, because allowing in-loop filtering across sub-picture boundaries is needed, and in addition to that, we should investigate decoding of multiple layers as if they were in the same layer.

  - to have the normative approach based on other approaches (none)

- To code sequence-level independent regions of one picture in one layer or in multiple layers?

  - One layer, all sub-pictures of one picture are in one layer (O0138, O0141, O0176, O0182, O0183, O0394, O0493, O0555, O0700)

  - Multiple layers, each sub-picture region is in its own layer (O0334, O0473)

General discussion on signalling of sequence-level independent regions

- Where to signal location, ID, and size of sequence-level independent region?

  - In the SPS (O0141, O0555)

  - In the PPS (O0176)

  - In the VPS (O0182, O0334)

  - In a new decoding control NAL unit (O0394)

  - In a new parameter set (O0473)

  - In a new SEI message (O0700)

It was firstly agreed to exclude b, c, and f.

On sub-picture location and size (not considering scalability, ARC/RPR, GRA):

Agreed to signal sub-picture location and size in the SPS unless a sub-picture based GRA solution is agreed in the CE11 discussion.

The information on sub-pictures and the sub-picture location and size for each sub-picture in the SPS describes the exactly complete pictures referring to the SPS.

On sub-picture ID:

The discussion and decision of this piece was chaired by S. Deshpande.

In O0141 and O0555, the sub-picture ID is mainly used for sub-bitstream extraction purpose.

There was no consensus to include the signalling of sub-picture IDs in the sub-picture design in the output VVC draft text from this meeting.

On the syntax for signalling of sub-picture location and size in the SPS:

It was agreed to use the SPS syntax in O0555 for this purpose as a starting point, with the sub-picture ID replaced with grid index (not considering scalability, ARC/RPR, GRA).

Regrading items 2) and 3) below:

The BoG agreed that it is generally useful to have some information signalling conformance points for (sets of) sub-pictures. Details are for further study.

- If to specify conformance for sequence-level independent regions, where to signal the conformance region (e.g., output sub-picture sets)?

  - In the SPS (O0141, O0555)

  - In the VPS (O0183)

- For conformance for sequence-level independent regions, to explicitly signal the PTL information or to indicate that the level for any sub-picture or sub-picture set can be derived based on scaling of the picture size relative to the sub-picture or sub-picture set sample size, respectively, when not possible, signal PTL in an SEI message?

  - To explicitly signal (O0141)

  - To indicate that the level can be derived, and if not, signal in an SEI message (O0555)

- To signal a sub-picture ID in the slice header? Not for now.

  - Yes (O0141, O0555)

General discussion on other aspects covered in these contributions

- Signal maximum decoded picture width and height in DPS and decoded picture width and height in PPS, and remove pic_width_in_luma_samples and pic_height_in_luma_samples from SPS? (O0182) Leave this for the ARC/RPR discussion.

- Update the sub-bitstream extraction process to support sub-picture extraction and reposition across multiple layers (O0183)? See notes elsewhere.

- Provide the possibility to specify partitioning to tiles, bricks, and rectangular slices in SPS, and when the partitioning is specified in SPS, it is not present in the PPS? (O0394) No action for now.

- Add a new SEI message for nesting of SEI messages for output sub-picture sets? (O0493) Not for now.

- Use a grid to indicate the sub-picture size and position? (O0555) Yes, see notes above.

- Use neighboring information to indicate neighbors of a layer and thus its position in the total layout? (O0473)

JVET-O0138 AHG12: Treating region boundaries as picture boundaries [Hendry, S. Hong, J. Chen, Y.-K. Wang (Futurewei)]

JVET-O0141 AHG12: Sub-picture based motion-constrained independent regions [Y.-K. Wang, Hendry, J. Chen (Futurewei), R. Skupin, Y. Sanchez, K. Suehring (HHI)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposal 6 in the contribution belongs to this agenda item.

JVET-O0182 AHG12: On picture and sub-picture signaling [Y. He, A. Hamza (InterDigital)]

JVET-O0183 AHG12: On layer-based sub-picture extraction and reposition [Y. He, A. Hamza (InterDigital)]

JVET-O0334 AHG8/AHG12: On sub-picture partitioning support with layers [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-O0394 AHG12: On independently coded picture regions [M. M. Hannuksela (Nokia)]

JVET-O0471 AHG8/AHG12: Layer concepts clarifications and improvements for immersive media use cases [E. Thomas, A. Gabriel (TNO)]

JVET-O0473 AHG8/AHG12: On generating final output picture from independent VVC layers [E. Thomas, A. Gabriel (TNO)] [late]

JVET-O0493 AHG12: On independent regions and output sub-picture sets [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-O0555 AHG12: Sub-pictures and sub-picture sets with level derivation [J. Boyce, L. Xu (Intel)]

JVET-O0700 AHG12: On sub-picture info SEI message [Y. He, A. Hamza (InterDigital)] [late]

### Mixed NAL unit types within a picture (2)

Chaired by J. Boyce 3 July.

From JVET-O0800:

List of design questions

- To support mixed NAL unit types within a picture or not?

  - Yes (O0140, O0498)

- If yes to 1),

  - Based on a PPS flag that specifies whether all VCL NAL units of a picture have the same NAL unit type (O0140)? Or

  - Base on allowing slices within a picture to have different sets of reference pictures drived from the RPLs and changing the reference marking process such that marking does not happen with the first VCL NAL unit when slices within a picture do have different sets of reference pictures derived from the RPLs (O0498)

- If yes to 1), add the following constraint?

  - For any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type (O0140)

What are the implications for decoder implementations in a mixed NAL unit case?

IRAP vs. non-IRAP type impacts parsing and decoder process.

POC process depends on IRAP vs non-IRAP. For a mixed pictures, use the non-IRAP style numbering.

Bitstream merging was mentioned as a possible use case that would benefit, to avoid the need to have to rewrite NUT. Other use cases also discussed Independent decoder regions like sub-pictures are an intended use case, with mix of IRAP and non-IRAP pictures. 360 video in particular is a use case, where some regions have more frequent IRAP than other regions.

Question about if prediction is restricted to enable the some random access behavior. How would such a constraint be expressed?

If this feature is used, how does a decoder determine that a picture is decodable? Would the restriction be imposed in an application spec or in the VVC spec?

The JVET-O0140 contribution requires that the mixed flag be set only when a picture uses both IRAP and non-IRAP VCL NAL units in the same picture.

Question about interaction of leading pictures with this feature. May want to disable using leading pictures when this feature is used.

It was suggested that this feature is most useful when sub-pictures or MCTS is used.

Revisit after sub-pictures contributions have been reviewed.

JVET-O0140 AHG12: On mixed NAL unit types within a picture [Y.-K. Wang, Hendry (Futurewei)]

See notes above.

JVET-O0498 AHG17: On marking process and unequal NALU types per AU [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

This contribution proposes changes to the current specification as follows:

- Allow VCL NAL unit within the same AU to have different NAL unit types.

- Allow VCL NAL units within the same AU to have different Reference Picture Lists

- Define a marking process that does not happen with the first VCL NAL unit, when different RPLs are present for different VCL NAL units.

This approach doesn’t require mixed VCL NAL units to be used when the flag is set. This could be a problem for decoders to know when they are able to start decoding.

A question raised about HRD behavior.

This contribution requires changes to the decoder for reference picture list handling.

### Slice, tile, and brick signalling in PPS and slice header (15)

JVET-O0143 AHG12: Signalling of addresses for rectangular slices [R. Sjöberg, M. Damghanian, M. Pettersson, J. Ström (Ericsson), Hendry, Y.-K. Wang (Futurewei)]

JVET-O0156 AHG12: On brick signaling [W. Lim, G. Bang, H. Y. Kim (ETRI)]

JVET-O0172 On rectangular slice address signaling [B.-K. Lee (Xris)]

JVET-O0173 On brick information signaling [B.-K. Lee (Xris)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposals 1 to 4 in the contribution belongs to this agenda item.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 3 in the contribution belongs to this agenda item.

JVET-O0199 AHG12: On rectangular slices [M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0225 AHG17: On signalling of bottom right brick for rectangular slices [V. Drugeon, T. Nishi (Panasonic)]

JVET-O0269 AHG17: Suggested syntax improvement on PPS [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0338 AHG12: On signaling for slice, tile and brick [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0403 AHG17: On brick index signalling [P. Bordes, M. Kerdranvat (InterDigital)]

JVET-O0452 AHG12: On tile and brick partition [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0453 AHG12: On rectangular slice signaling [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0454 AHG12: On sub-bitstream extraction [Y.-J. Chang, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0610 AHG17: Association of VCL NAL units to coded pictures [B. Heng, W. Wan (Broadcom)]

### Signalling of brick/WPP entries (4)

JVET-O0144 AHG12: On end_of_subset_one_bit, end_of_brick_one_bit, and byte alignment in the slice data syntax [Hendry, Y.-K. Wang (Futurewei)]

JVET-O0145 AHG12: Miscellaneous AHG12 topics [Y.-K. Wang, Hendry (Futurewei)]

JVET-O0215 AHG12: Clean up on wavefront parallel processing signaling [T. Ikai, E. Sasaki, T. Chujoh (Sharp)]

JVET-O0306 AHG17: Entry point offsets without start code emulation [K. Abe, T. Toma, V. Drugeon (Panasonic)]

### New tile/brick/WPP partitioning schemes (5)

JVET-O0198 AHG12: On brick partitioning [M. Coban, A. Ramasubramonian, Y.-J. Chan, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0358 AHG12: On uniform tile spacing [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

JVET-O0390 AHG17: On tile and brick signalling [M. M. Hannuksela (Nokia)]

JVET-O0511 AHG12: On tiles and bricks partitioning [N. Ouedraogo, E. Nassor (Canon)]

JVET-O0462 AHG12: Separated Luma and Chroma wavefront for dual tree [T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

JVET-O1075 AHG17: AHG12: On tile and brick signalling (combination of JVET-O0390 and JVET-O0551) [M. M. Hannuksela (Nokia), N. Ouedraogo, E. Nassor, F. Denoual (Canon)]

### Loop filtering across boundaries of independently coded regions (4)

JVET-O0142 AHG12: On turning off ALF filtering at brick and slice boundaries [Y.-K. Wang, J. Chen, Hendry (Futurewei)]

Also included in the agenda item 6.5.

JVET-O0494 AHG12: On filtering of independently coded region [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Also included in the agenda item 6.5.

JVET-O0654 AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Also included in the agenda item 6.5.

JVET-O0858 Crosscheck of JVET-O0654 (AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter) [H. Liu (Bytedance)]

JVET-O0662 Non-CE5: Modified ALF filtering for Slice, Brick and Virtual boundaries [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

Also included in the agenda item 6.5.

## AHG8: layered coding and resolution adaptivity (25)

Contributions in this category were discussed XXday X July XXXX–XXXX in Track X (chaired by XXX).

### Reference picture resampling (RPR) (12)

JVET-O1040 AHG8: AHG8: Summary of Resolution Adaptivity related proposals [S. Wenger (Tencent), Y.-K. Wang (Futurewei)]

JVET-O0133 AHG8: Support for reference picture resampling - handling of picture size signalling, conformance windows, and DPB management [Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

JVET-O0134 AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF [Y.-C Sun, T.-S Chang, J. Lou (Alibaba), Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei)]

JVET-O0757 Crosscheck of JVET-O0134 (AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF) [H.-J. Jhu (Kwai Inc.)]

JVET-O0184 AHG8: On ARC indicator [Y. He, Y. He, A. Hamza (InterDigital)]

JVET-O0204 AHG8: On Adaptive Resolution Change (ARC) High-Level Syntax (HLS) [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-O0240 AHG8: Adaptive Resolution Change (ARC) with downsampling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-O0242 AHG8: Enabling BDOF and DMVR for reference picture resampling [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0303 AHG8: Adaptive Resolution Change [P. Chen, T. Hellman, B. Heng, W. Wan, M. Zhou (Broadcom)]

JVET-O0701 Crosscheck of JVET-O0303 (AHG8: Adaptive Resolution Change) [Y. He (InterDigital)]

JVET-O0319 AHG8: On Adaptive Resolution Change [P. Topiwala, M- Krishnan, W. Dai (FastVDO)]

JVET-O0332 AHG8: Signaling and filtering for Reference Picture Resampling (RPR) [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0395 AHG8: On adaptive resolution changing and scalable coding [M. M. Hannuksela, A. Aminlou (Nokia)]

Only the RPR part in the contribution belongs to this agenda item.

JVET-O0641 AHG8: AHG 8: JVET Common test conditions for adaptive resolution change (ARC) [M. G. Sarwer, R. -L Liao, J. Chen, J. Luo, Y. Ye (Alibaba)] [late]

### Scalabitliy (13)

JVET-O0045 AHG8: Spatial Scalability using Reference Picture Resampling [S. Wenger, B. D. Choi, S. C. Han, X. Li, S. Liu (Tencent)]

JVET-O0135 AHG8: Scalability for VVC – general [Y.-K. Wang, Hendry, J. Chen (Futurewei)]

JVET-O0136 AHG8: Scalability for VVC - handling of POC, RPL, and RPM [Y.-K. Wang, Hendry, J. Chen (Futurewei)]

JVET-O0185 AHG8/AHG17: On layer dependency signaling [Y. He, A. Hamza (InterDigital)]

JVET-O0243 AHG8: On inter-layer reference for scalability support [V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-O0333 AHG8: On spatial scalability support with reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0335 AHG8: E-Sport Use cases of VVC region-wise scalability and its system support [B. Choi, S. Zhao, I. Sodagar, S. Wenger, S. Liu (Tencent)]

JVET-O0336 AHG8: Region-wise scalability support with reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0337 AHG8/AHG17: Layered Random Access point (LRA) [B. Choi, S. Wenger, S. Liu (Tencent)] [miss] [late]

JVET-O0395 AHG8: On adaptive resolution changing and scalable coding [M. M. Hannuksela, A. Aminlou (Nokia)]

Only the scalability part in the contribution belongs to this agenda item.

JVET-O0436 AHG8/AHG17: indication of shutter angle for variable frame rate application [S. McCarthy, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen (Dolby)]

JVET-O0471 AHG8/AHG12: Layer concepts clarifications and improvements for immersive media use cases [E. Thomas, A. Gabriel (TNO)]

JVET-O0473 AHG8: On HRD operations for layers [E. Thomas, A. Gabriel (TNO)] [miss] [late]