| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1076-v4 |
| --- | --- |

| Title: | Report of BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Jill Boyce (Intel) | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com jill.boyce@intel.com |
| Source: | BoG coordinators |  |  |

_____________________________

# Abstract

The BoG met on the following days and time slots:

- 3 July 2019 Wednesday from 16:00 to 21:12

- 4 July 2019 Thursday from 09:00 to 20:35

- 5 July 2019 Friday from 09:00 to 20:50

- 6 July 2019 Saturday from 09:00 to 13:20, and from 7:20 to 21:15

- 7 July 2019 Sunday from 09:00 to 11:15

The BoG recommends the following:

- Adopt the nonref_flag, in the slice header, unconditioned, to indicate whether the picture may be used by other pictures for inter prediction reference. [JVET-O0181]

- Adopt a sub-picture design based on the following pieces:

  - The normative changes to the low-level decoding processes for treating rectangular regional boundaries as picture boundaries proposed in JVET-O0138, JVET-O0141, and JVET-O0394. Apply to the boundaries of sub-pictures, where each sub-picture consists of one or more rectangular-mode slices that collectively form a rectangular region. [JVET-O0138, JVET-O0141, JVET-O0394]

  - Adopt signalling sub-picture location and size in the SPS (unless a sub-picture based GRA solution is agreed in the CE11 discussion), using the SPS syntax in O0555 for this purpose as a starting point, with the sub-picture ID replaced with grid index (not considering scalability, ARC/RPR, GRA). [JVET-O0555]

  - Adopt the following, pending on review of spec text (J. Boyce to coordinate the work on the spec text):

    - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether the sub-picture boundaries are treated as picture boundaries. [JVET-O0141]

    - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether in-loop filtering (deblocking, SAO, and ALF) across sub-picture boundaries are enabled. [JVET-O0141]

- The BoG agreed that it is generally useful to have some information signalling conformance points for (sets of) sub-pictures. The BoG requests to discuss at track on whether it is sufficient to have information in SEI message for applications, or should we rather normatively specify conformance points for (sets of) sub-pictures.

- Adopt a reference picture resampling (RPR) design based on the following pieces (draft spec text to be prepared. V. Seregin to coordinate):

  - On signalling of picture sizes and conformance window [JVET-O0182, JVET-O0204, JVET-O0303, JVET-O0319, JVET-O0395]:

    - Signal the maximum size in the SPS.

    - Signal the actual picture size in the PPS, using the same coding as currently for the SPS. Need to move some derivations equations that rely on active resolution currently in the SPS to the PPS.

    - Move the conformance window as currently in the SPS to the PPS.

  - Use the resolution of the cropped output pictures calculating the scaling factor between the current and reference pictures. Detailed study required by low-level coding tools experts. [JVET-O0204]

  - DPB is managed based on the maximum picture size (i.e., each picture store is of the size of a decoded picture of the maximum spatial resolution signalled in the SPS, regardless of the actual picture size).

  - A block-based resampling process based on the existing VVC interpolation filter, for both upsampling and downsampling. [JVET-O0134, JVET-O0303, JVET-O0242].

  - The following constraints on TMVP, DMVR, and BDOF:

    - TMVP generally allowed but the collocated picture shall have the same resolution as the current picture. [JVET-O0134, JVET-O0242, JVET-O0303, JVET-O0395]

    - DMVR generally allowed but disabled for a coding block when referring to different resolution. [JVET-O0134, JVET-O0242, JVET-O0303]

    - BDOF generally allowed but disabled for a coding block when referring to different resolution. [JVET-O0134, JVET-O0242, JVET-O0303]

- Discuss whether the existing affine fallback mode shall be triggered for a coding block when referring to a reference picture with a different spatial resolution than the current picture, which is proposed in JVET-O0303.

- Establish CTC for reference picture resampling (RPR), based on the following details, coordinated by J. Luo:

  - Resampling ratios: 2:1, 1.5:1, and optionally 4:1, use same ratio in both dimensions.

  - Test configurations: 1) low delay, fixed in time, like 0./5 sec low, 0.5 sec high, and 2) random access, I picture only.

- Establish a CE on reference picture resampling filters, coordinated by V. Seregin.

- Adopt changing the coding of pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] from u(v) to u(13).

- Adopt, pending on review of spec text (to be prepared by S. Deshpande and Y.-K. Wang), using phrases like "the referred xPS" in the VVC specification instead of phrases like "the active xPS" and not to specify a parameter sets activation process. [JVET-O0113]

- Adopt the following: signal num_tiles_in_pic_minus1 under the syntax condition "if( uniform_tile_spacing_flag  &&  brick_splitting_present_flag )", to avoid a PPS parsing dependency on SPS. [JVET-O0236]

- Adopt the frame field information SEI message, for separately signalling of the frame field information and the picture timing information in different SEI messages, and it is constrained that, when field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS. Accordingly, the condition "if( CpbDpbDelaysPresentFlag )" is removed from the picture timing SEI message. [JVET-O0041]

  - Discuss whether the frame field information SEI message should be included in the base VVC spec (as the picture timing SEI message) or in the separate SEI part (as other SEI messages that do not directly affect the specification of conformance).

- Adopt the following changes for signalling of sub profiles: allow signalling of zero or more sub profile fields, use 8 bits for the count, and use 32 bits for the sub profile field. [JVET-O0044]

- Adopt a constraint to require that, for P and B slices, the number of active entries in RPL 0 is greater than 0, and for B slices, the number of active entries in RPL 1 is greater than 0. [JVET-O0148]

- Adopt the following changes: [JVET-O0244]

Additionally signal weighted prediction enabled flags in SPS, which are signalled in the PPS currently, and to disallow zero delta POC values when weighted prediction is not enabled. Editorial: move the condition out of the RPL structure syntax.

- Adopt the following: [JVET-O0151]

  - The value of no_output_of_prior_pics_flag for CRA pictures starting a new CVS is used similarly as for IDR pictures.

  - The no_output_of_prior_pics_flag is also signalled for GRA pictures and applied for GRA pictures starting a new CVS similarly as for IDR pictures.

  - The no_output_of_prior_pics_flag is also signalled for the reserved IRAP VCL NAL unit types. [Ed. (YK): I realized after the BoG session that we should not do this, because the slice header syntax for the reserved IRAP VCL NAL unit types may be different.]

- Adopt text for the basically empty Annex B on byte stream format. The text is exactly the same as in HEVC, except for the addition of DPS_NUT and APS_NUT, along with VPS_NUT, SPS_NUT and PPS_NUT, in the semantics of zero_byte. [JVET-O0154]

- Adopt moving of slice_pic_order_cnt_lsb to be before recovery_poc_cnt in the slice header. [JVET-O0152, JVET-O0176]

- Adopt coding of sps_max_dec_pic_buffering_minus1[ i ] and sps_max_num_reorder_pics[ i ] using delta coding. [JVET-O0178]

  - Discuss whether sps_max_latency_increase_plus1[ i ] should also be delta-coded. Are the values of sps_max_latency_increase_plus1[ i ] are also monotonic non-decreasing?

- Adopt conditionally signalling of sps_sub_layer_ordering_info_present_flag based on the value of sps_max_sub_layers_minus1. [JVET-O0178]

- Adopt changing the coding of recovery_poc_cnt to ue(v) instead of se(v), thus disallowing negative values that allow output gaps. [JVET-O0201]

The BoG was aware that negative and zero values were allowed for recovery_poc_cnt in HEVC and the same behavior (allowing output gaps) was also allowed in AVC. Does the track know why an output gap was allowed in AVC and HEVC? Should we further disallow value zero?

- Adopt the slice header extension mechanism that is essentially identical to that in HEVC. [JVET-O0234]

- Adopt the following, but do it in the PPS instead of SPS. Further study whether some of those should be just in the PPS only: [JVET-O0238]

- Add a sps_or_slice_flag in the SPS that specifies whether the following syntax elements are always signalled in slice headers or conditionally signalled in the SPS or slice headers:

  - dep_quant_enabled_flag

  - ref_pic_list_sps_flag

  - slice_temporal_mvp_enabled_flag

  - mvd_l1_zero_flag

  - collocated_from_l0_flag

  - six_minus_max_num_merge_cand

  - five_minus_max_num_subblock_merge_cand

  - max_num_merge_cand_minus_max_num_triangle_cand

- If sps_or_slice_flag is equal to 1, the following applies:

  - The syntax elements are present in the SPS.

  - For each of these syntax elements, if the value in the SPS is equal to 0, the syntax element is also present in the slice header, and the value signalled in the slice header is used. Otherwise, the syntax element is not present in the slice header and the value signalled in the SPS is used.

- Otherwise, the syntax elements are not present in the SPS and are present in the slice header.

- Adopt the following bug fixes: [JVET-O0241]

  - Setting of PictureOutputFlag in the general decoding process for a coded picture and in the process for generating unavailable reference pictures.

  - Correctly setting the POC value for long-term reference pictures in the decoding process for generating unavailable reference pictures.

  - Add the inference of slice_pic_order_cnt_lsb for the generated LTRP in the decoding process for generating unavailable reference pictures.

- Adopt a set of constraints on IRAP, RASL, and RADL pictures that are technically identical as in HEVC. [JVET-O0147]

- Adopt the following constraint: [JVET-O0147, JVET-O0226]

If field_seq_flag is equal to 0 and the current picture is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture preceding picA in decoding order, and there shall be no non-leading picture between picA and picB in decoding order.

- Conditionally adopt the following items if no proposal of allowing mixed NAL unit types is adopted at this meeting: [JVET-O0235]

  - All VCL NAL units of a picture shall have the same NAL unit type.

  - A picture or a layer access unit is also referred to as having a NAL unit type equal to the NAL unit type of the coded slice NAL units of the picture or layer access unit.

- Adopt the following constraints: [JVET-O0235]

  - STSA pictures shall have TemporalId greater than 0.

  - Constraints on RPLs such to make sure that step-wise sub-layer switching can occur at STSA pictures.

- Adopt the following constraints: [JVET-O0245]

  - TemporalId of an APS NAL unit shall be greater than or equal to that of the layer access unit containing the NAL unit.

  - The value of TemporalId of VPS and DPS NAL units shall be equal to 0.

  - When multiple ALF APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple ALF APSs with the same value of adaptation_parameter_set_id shall have the same content.

  - A constraint on layer ID of an APS NAL unit:

    - If scalability with inter-layer prediction is adopted: A slice shall only refer to APS NAL units with layer ID(s) of layers the layer containing the slice directly or indirectly depends on.

    - Otherwise: A slice shall only refer to APS NAL units with the same layer ID.

- Adopt moving of scaling list matrices from SPS to APS, and constrain that all slices of a picture shall refer to the same matrix APS ID. [JVET-O0299]

- Adopt a clarification in the draft text that different types of APSs have independent APS ID value spaces.

- Discuss the following:

  - What are the maximum numbers of matrix and ALF APSs that may be needed?

  - What is the maximum number of APSs for each type that may need to be referred to by slices of one picture?

- Adopt the following constraints: [JVET-O0428]

  - When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

  - All slices within a picture referring to an LMCS APS shall refer to the same LMCS APS.

- Adopt to allow only u(n), u(v), ue(v), or se(v) coded syntax elements in syntaxes for slice header and all parameter sets, thus without using tu(v), tb(v), etc. [JVET-O0491]

- Adopt to allow association of a buffering period SEI message with a GRA picture, and some minor bug fixes and editorial improvements proposed in JVET-O0152, excluding the change of the trailing picture definition. [JVET-O0152]

- Adopt the editorial bug fixes for HRD related texts proposed in JVET-O0153. [JVET-O0153]

- Adopt the following: [JVET-O0177]

  - To allow signaling of HRD parameters for temporal sub-layers optionally controlled by a flag.

  - To remove timing_info_present_flag, and conditionally signal syntax elements num_units_in_tick and time_scale based on hrd_parameters_present_flag.

- Adopt the following: [JVET-O0180, JVET-O0228]

  - Addition of a loop of temporal sub-layers into the buffering period SEI message syntax, to signal the initial CPB removal delay values for either all sub-layers or just highest sub-layer.

  - Addition of a syntax element that is equal to sps_max_sub_layer_minus1 to avoid parsing dependency on SPS.

  - A constraint that bp_cpb_cent_minus1[ i ] shall be equal to sps_cpb_cent_minus1[ i ].

  - An editorial improvement to keep tracking of the BitRate and CpbSize values for each sub-layer.

- Adopt the decoding unit based HRD design that is the same as in HEVC. [JVET-O0189]

- Adopt to remove a parsing dependency of the picture timing SEI message on SPS by signalling the number of temporal sub-layers of the bitstream within the picture timing SEI message. [JVET-O0227]

- Adopt replicating the compound functionality of pic_struct into separate elements (pic_output_period, bottom_pic_flag, field_pairig_idc) in an SEI message. [JVET-O0042]

- Adopt the correction to the semantics of the transfer characteristics and matrix coefficients syntax elements to support IEC 61966-2-1, as proposed in JVET-O0360. [JVET-O0360]

- Adopt to mandate the presence of AU delimiter for each AU.

- Adopt a bug fix to the slice data syntax such that byte alignment is ensured at the end of each brick/WPP subset. [JVET-O0144]

- Confirm the following design principles on scalability support in VVC:

  - To specify a scalability support in VVC edition 1.

  - Since from technical perspective there is no undue burden to non-scalable applications when adding scalability, it can be assumed that scalability ought to be in baseline/main (non-scalable profiles).

  - Signal the layer dependency information in the VPS.

  - VPS needs to be included in non-scalable bitstreams.

  - Support of outputting only the highest layer (for typical scalability) and support of outputting all layers (for multiview) are both needed.

- Review unified scalability proposals, focusing POC, RPL, and RPM, prepared following the HLS BoG agreements on the scalability category of proposals.

- Revisit/discuss the following documents: JVET-O0335, JVET-O0336, JVET-O0471, JVET-O0473.

# Review of contributions

## AHG17: General high-level syntax (67)

### NAL unit header and reference picture indication (7)

General discussion on NUH

Chaired by J. Boyce, 3 July.

The existing HEVC design was mentioned as being also worth considering. The number of bits for NUT values using 5 bits instead of 6 was considered to be sufficient.

Having an extension mechanism is desirable. Can either reserve a particular layer value, or can reserve a bit. If a layer value is reserved, the existing 7 bits can’t be used for the extension, and would require additional bits in the extension, but is more efficient when the extension isn’t used.

It was suggested that a variable length design allowing a single byte to be used in the typical case be considered. This was discussed at the previous meeting.

Revisit the contributions in this area.

JVET-O0146 AHG17: On unspecified NAL unit types [Y.-K. Wang (Futurewei)]

Chaired by J. Boyce, 3 July.

Currently, the TemporalId of all the unspecified NAL unit types have to be equal to 0. This contribution proposes to reallocate NAL unit types such that some unspecified NAL unit types can have TemporalId greater than 0.

JVET-O0179 On NAL Unit Header Design [S. Deshpande (Sharp)]

Chaired by J. Boyce, 3 July.

Modifications are proposed to NAL unit header design.

It is proposed to redesign the NAL unit header as follows:

- The first byte consists of syntax element nuh_layer_id. For this syntax element, values 188 onwards are forbidden. Value 0 to 126, inclusive are defined to have meaning same as currently in VVC WD for independent layers. Additionally values 127 to 187, inclusive are reserved – to be possibly used for spatial/ view/ SNR scalability later.

- The second byte consists of 5-bit syntax element nal_unit_type, followed by 3-bit syntax element nuh_temporal_id_plus1.

In this proposal, 8 bits are allocated to nuh_layer_id, with values 127 to 187 reserved.

Some support was expressed for this contribution.

A question was raised about how this approach would be used for larger number of layers in an extension. It is noted that the existing draft spec also doesn’t explicitly describe that case.

Additional proposals if the reordering of syntax elements proposal were not adopted.

Proposal 1: temporal_id_plus1 is replaced by temporal_id, because of the position of temporal_id in the current design doesn’t require avoiding 0. If we keep the current syntax, there is support for doing this, unless any problems are identified.

Proposal 2: remove the reserved bit in the NUH and give layer id another bit.

Proposal 3: changes the restriction of allowable temporal id values with particular NUT values.

JVET-O0237 AHG17: Compact NAL unit header [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

Chaired by J. Boyce, 3 July.

This contribution proposes two changes related to the NAL unit header in VVC.

Firstly, it is proposed to signal an extension flag in the NAL unit header that specifies if the NAL unit header is signaled with one byte or if the NAL unit header is extended with one additional byte. The extension byte is needed to express a NuhLayerId larger than 0 or for certain less commonly used NAL unit types (e.g. reserved or unspecified NAL unit types).

More specifically, the following changes to the NAL unit header are proposed in this contribution:

- Signal a vcl_flag indicating whether the NAL unit is a VCL or non-VCL NAL unit

- Signal nal_unit_type_lsb with 2 bits instead of 4

- Signal a nuh_extension_flag specifying whether a 1-byte NAL unit header extension is present

- If the extension is present signal nuh_layer_id_plus1, nal_unit_type_ext_bit and nuh_reserved_zero_bit in a second NAL unit header byte, else infer nuh_layer_id_plus1 to be equal to 1 and nal_unit_type_ext_bit and nuh_reserved_zero_bit to be equal to 0

- Signal nuh_layer_id_plus1 with 6 bits as in HEVC instead of 7 bits

- Set NALUnitType equal to ( zero_tid_required_flag  <<  4 ) + (vcl_flag << 3 ) + ( nal_unit_type_ext_bit << 2 ) + nal_unit_type_lsb

The estimated luma BD rate numbers by the change are reported to be 0.0%/0.0%/-0.1%/-0.1% for AI/RA/LDB/LDP respectively and the proponents claim higher savings when multiple slices per pictures are used.

Secondly, it is claimed by the proponents that in the current VVC draft there are no reserved IRAP NAL unit type values and that the unspecified NAL unit types are required to have temporal ID equal to 0. This contribution proposes to reserve two IRAP NAL unit type values and to allow two of the four unspecified NAL unit types to have a non-zero temporal ID, at the cost of removing two of the reserved non-IRAP NAL unit type values.

Second aspect is similar to JVET-O0146, but splits the existing reserved VCL NUTs into IRAP and non IRAP types. Some support was expressed that if the NUT syntax is kept and the allocation of values changed, this may be better than the JVET-O0146 proposal.

First aspect proposes variable length NAL unit, where second byte is only present sometimes. Reduces the number of layers than can be signaled and allocates a bit to indicate that more that 2 bytes are present in the NUH. Redefines the first of the 4 NUT bits for VCL type.

It was remarked that at the previous meeting it had been discussed to use the temporal Id to condition the presence of the second bit.

JVET-O1037 AHG17: On NAL unit header [Y.-K. Wang (Futurewei)]

JVET-O1042 AHG17: Varying-length NAL unit header [M. M. Hannuksela (Nokia)]

JVET-O0359 AHG17: On identification of reference and non-reference picture [T. Suzuki (Sony)]

Chaired by Y.-K. Wang, July 3rd.

Proposes to use additional NAL unit types for trailing and STSA pictures (not for other types of pictures, e.g., RASL, RADL) for reference/non-reference picture indication.

If to have this indication for trailing and STSA pictures, why not for RASL and RADL pictures as well?

It seems that then the indication should also be specified for RASL and RADL. And if we do that for RASP and RADL too, then we probably don't have sufficient NAL unit type value space remaining.

It was agreed that if to have this indication for trailing and STSA pictures, we should also have it for RASL and RADL pictures as well.

See notes under JVET-O0181.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 2 in the contribution belongs to this agenda item.

Chaired by Y.-K. Wang, July 3rd.

Proposes slnr_flag and nonref_flag for non-IRAP slices at the beginning of the slice header syntax, to indicate whether this slice belongs to is a sub-layer non-reference picture or a sub-layer reference picture, and whether the picture that this slice belongs to may be not used for inter prediction, respectively.

The claimed benefit: Knowing this information allows pictures to be discarded under certain situations (e.g. playback is lagging real-time).

It was agreed to have only the nonref_flag, in the slice header, unconditioned.

### Parameter set mechanism and parsing dependency (6)

JVET-O0113 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Futurewei), J. Boyce (Intel)]

The discussion was chaired by M. M. Hannuksela on July 5th.

This contribution proposes the following two aspects:

- To allow parsing dependency between different types of parameter sets, mainly to avoid the burden in designing of parameters for making sure that there is no such parsing dependency, and at the same time to avoid some redundant signalling overhead of lower-level parameter sets; and

- To not specify a parameter sets activation process but rather use phrases like "the referred xPS" in the VVC specification instead of phrases like "the active xPS".

There are some hidden constraints in the parameter set activation text as mentioned in O0181:

- Various referred parameter sets having the same content

- Availability of parameter sets before they are used

- How many parameter sets can be referred/activated e.g. per layer

Such constraints would need to be included also with the proposed design. The proponent expressed that some constraints are present already in the present text but probably not all.

It was commented that SPS should be independently parsable (i.e., not depending on VPS or DPS).

Should APS be independently parsable? In VVC Draft 5 it is.

Agreed pending text review and without allowing parsing dependency.

Futurewei volunteered to provide the SW.

JVET-O0155 AHG17: Resolving PPS parsing dependency on SPS [Hendry, Y.-K. Wang (Futurewei)]

The discussion was chaired by M. M. Hannuksela on July 5th.

This contribution asserted that there is dependency for parsing of some PPS syntax elements on SPS syntax in the current VVC working draft. The parsing dependency occurs in the parsing of the following syntax elements:

- Tiles and bricks related syntax elements. For deriving the values of NumTilesInPic and NumBricksInPic, which are needed for parsing other syntax elements that are also present in the PPS, the parsing process needs some information from SPS.

- Positions of virtual boundaries. For parsing syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ], the number of bits that represents those syntax elements depend on the picture width and height that are only available from SPS.

If the JVET decides to keep parsing independency between parameter sets, to solve the above parsing dependency issues, the following is proposed:

- For tile and brick signalling, the following is proposed:

  - When a picture is partitioned into more than one tile, regardless whether uniform tile partitioning is used or not, num_tile_columns_minus1 and num_tile_rows_minus1 are always signalled. The derivation process of colWidth[ i ] and RowHeight[ i ] is modified to incorporate the values of num_tile_columns_minus1 and num_tile_rows_minus1.

  - When one or more tiles are partitioned into more than one brick, regardless whether uniform brick partitioning is used or not, num_brick_rows_minus1[ i ] is always signalled. The derivation process of rowHeight2[ j ] is modified to incorporate the value of num_brick_rows_minus1[ i ]

It was commented that JVET-O0358 might have some relation to aspect 1.

For aspect 1, see notes below on O0236.

O0143 is related to proposed aspect 2 (on avoiding the parsing dependency specifically). Revisit aspect 2 when O0143 is reviewed.

- For signalling of virtual boundaries, the number of bits that represent the syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] is signalled explicitly.

It was commented that the naming of pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] does not indicate that the units are not luma samples.

O0236 and O0490 have related proposals on avoiding parsing dependency for virtual boundary signalling.

The picture width and height will be signalled in the PPS, as per the decisions on ARC. Then, there would no longer be parsing dependency issue for virtual boundary signalling.

A revisit is needed only if the track/plenary does not agree on the picture width and height signalling in the PPS

JVET-O0236 AHG17: Making PPS parsing independent of SPS [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

The discussion was chaired by M. M. Hannuksela on July 5th.

The proponents of this contribution have the opinion that keeping the PPS parsing independent of the SPS is a desirable property in VVC assuming VVC will end up in an HEVC-like process for parameter set activation where parameter sets are activated by VCL NAL units. If the parsing of the PPS then depends on the SPS, PPSes may need to be stored in coded format by a decoder until a VCL NAL unit triggers a PPS activation.

If the VVC design would differ from the HEVC design and allow parsing dependencies between the PPS and SPS, the proponents do not propose to make any of the changes described in this contribution.

Two claimed PPS parsing dependency issues in the draft VVC specification, JVET-N1001-v8, were identified by the proponents.

The first issue is that the number of bits for the u(v) syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] are determined by the picture size which is specified in the SPS. The proposed solution is to replace

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x[ i ] * 8

by

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_coarse_x[ i ] * 512 + pps_virtual_boundaries_fine_x[ i ] * 8

where pps_virtual_boundaries_coarse_x is a ue(v) syntax element and pps_virtual_boundaries_fine_x is a u(6) syntax element. The same is proposed for PpsVirtualBoundariesPosY.

The second issue is that currently the for-loop count for parsing brick syntax elements is equal to NumTilesInPic. If uniform_tile_spacing_flag is equal to 1, then NumTilesInPic depends on the pic_width_in_luma_samples  and pic_heigh_in_luma_samples syntax elements in the SPS. This contribution proposes to add a PPS syntax element num_tiles_in_pic_minus1 in case uniform_tile_spacing_flag and brick_splitting_present_flag are both equal to 1 where the value of num_tiles_in_pic_minus1 shall be set equal to NumTilesInPic – 1, with syntax as follows:

| brick_splitting_present_flag | u(1) |
| --- | --- |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| ... |  |

It was commented that signalling the number of tile columns and rows might be more efficient than signalling the number of tiles in a picture.

Agreed on the syntax above to conditionally signal num_tiles_in_pic_minus1.

It was commented that another option would be to indicate the CTB size in PPS (with the constraint that it shall be the same as the signalled CTB size in SPS). This would also enable deriving the tile-and-brick partitioning (in units of samples) by parsing the PPS only (without a need to resolve the referencing to SPS).

JVET-O0490 AHG17: Independent parsing of parameter sets [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

The discussion was chaired by M. M. Hannuksela on July 5th.

This contribution proposes to keep the design principle of independent parsing of parameter sets and also proposes solutions for the parsing dependencies introduced the VVC draft 5.

The proponent said that the solutions are not detailed. Noted.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 1 (on parameter set activation) and proposal 4 (on DPS referencing by SPS) in the contribution belong to this agenda item.

The discussion was chaired by M. M. Hannuksela on July 5th.

Proposal 1: Parameter sets activation/ referencing specification text is proposed

Why is an activation process needed at all?

- It was commented that writing specification text might be easier, since phrase like "active SPS" can be used. A counter argument was expressed that it would be equally easy to say "referred SPS". Another comment was that since there are multiple active APSs, the phrase "the active APS" is ambiguous.

O0113 is related (as it avoids the activation process).

It was asked whether activation or referencing process cause implementation differences? It was asserted that no implementation differences are implied.

It was commented that keeping the term "active parameter set" is desirable (even if there is no activation process).

Some of the constraints proposed in O0181 will be incoporated into the revised version of O0113.

Proposal 4: A semantics restriction is proposed on the DPS referred to by SPS, for each CVS in the bitstream.

It was asked whether a different DPS ID can be referred (as long as they have the same content).

It was commented that e.g. in splicing the latter CVS might have had a DPS that has stricter requirements than the former CVS of the spliced bitstream. Would that be ok?

It was asked if there is a definition of the phrase "the same content". Referring to DPS ID could be clearer.

Further study on proposal 4.

JVET-O0268 AHG17: Signalling SPS base and patch [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes, for discussion, to specify the base part and patch part for the sequence parameter set (SPS) and signal them based on their base and patch types for the SPS referenced by the coding tools. The SPS base may be shared or reused in terms of several aspects for an application. It is alleged this may help improve the sharing (e.g. across layers) and flexibility in the syntax. The proponent said that "sharing" provides bit count efficiency, since the SPS base can be shared by multiple SPS patches.

Proposal 1: specify and signal a SPS as the SPS base and the SPS patch, and a new NAL unit type of SPS_BASE_NUT.

Proposal 2: specify the sequence parameter set patch structure with the patch types for update processing.

Proposal 3: specify the relative origin information in the SPS.

Can SPS base be in a different layer? The proponent responded yes.

Any estimation of how many bits could be saved? The proponent responded that no such analysis has been done.

It was commented that partial parameter set updating was studied in the development of HEVC and was found to be challenging for memory copying/overwriting.

The signalling of sub-picture locations will be dealt with in the off-line work related to O0334.

No action.

### IOP point definition and signalling (2)

JVET-O0044 AHG17: Signalling zero or more Sub-Profiles [S. Wenger, B. D. Choi, S. Liu (Tencent)]

On signalling of zero sub-profile:

An option is to specify that the value 0 of general_sub_profile_idc indicates that no additional constraint is imposed.

On signalling of multiple sub-profiles for the same bitstream:

It was commented that using 8 bits for the count is better, to not loose byte alignment.

It was commented that the length of the sub profile field should be 32 bits instead of 24 bits for IUT-T T.35 codes.

It was commented that a particular bitstream may conform to more than one profile and also more than one sub profile.

Agreed, use 8 bits for the count, and use 32 bits for the sub profile field.

JVET-O0417 AHG17: On general constraint information [P. Bordes, M. Kerdranvat (InterDigital)]

TBP.

### Picture types, random access, and sub-layer switching (5)

JVET-O0147 AHG17: Constraints on IRAP pictures and leading pictures [Hendry, Y.-K. Wang (Futurewei)]

Chaired by J. Boyce, July 7th.

Proposal 1: A set of constraints on IRAP, RASL, and RADL pictures that are technically identical as in HEVC.

Proposal 2: Proposes the following constraint:

If field_seq_flag is equal to 0 and the current picture is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture preceding picA in decoding order, and there shall be no non-leading picture between picA and picB in decoding order.

Proposal 1: agreed.

Proposal 2 agreed.

JVET-O0226 AHG17: On constraint about position of leading pictures and trailing pictures associated to an IRAP [V. Drugeon, K. Abe (Panasonic)]

Chaired by J. Boyce, July 7th.

Option 1: Add the following constraint: At most one trailing picture associated with an IRAP picture may precede, in decoding order, all leading pictures that are associated with the same IRAP picture.

Option 2: Add the following constraints:

When field_seq_flag is equal to 0 and a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture.

When field_seq_flag is equal to 1, at most one trailing picture may precede, in decoding order, all leading pictures that are associated with the same IRAP picture.

Option 2 is essentially the same as proposal 2 of JVET-O0147. Agreed, using the wording in proposal 2 of JVET-O0147.

JVET-O0149 AHG17: On external decoding refresh (EDR) [Y.-K. Wang, J. Chen (Futurewei), H. Yu, L. Yu (ZJU)]

JVET-O0393 AHG17: On cross-RAP referencing [M. M. Hannuksela (Nokia)]

JVET-O0235 AHG17: Ensuring temporal switching with STSA pictures [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

This contribution claims that there are four issues related to STSA pictures in the draft VVC specification (JVET-N1001-v8) and proposes changes to resolve them.

1) The first claimed issue is that STSA pictures seem to be allowed to have TemporalId equal to 0. This is claimed not to be the case for HEVC and it is proposed that such a restriction is added to the draft VVC specification.

2) The second claimed issue is that a restriction that all coded slice NAL units of an AU shall have the same value of NalUnitType, is missing. It is proposed that such a restriction is added to the draft.

3) The third claimed issue is that an STSA picture currently does not guarantee that temporal switching is possible since the draft allows to include pictures preceding the STSA in decoding order of the same temporal layer as active entries in the RPL of the STSA picture and/or following pictures. It is proposed that two constraints, similar to what can be found in HEVC, are added to the list of RPL decoding process bitstream conformance constraints in section 8.3.2 of JVET-N1001-v8.

4) The fourth claimed issue is that currently it is allowed for an STSA picture and pictures that follow it in decoding order to use an APS of the same temporal layer that preceded the STSA picture. It is proposed to add restrictions to prohibit usage of PPS and APS information that precedes the STSA access unit in decoding order.

Discussions:

1) Agreed.

2) Agreed that all VCL NAL units of a picture shall have the same NAL unit type if the proposal of allowing mixed NAL unit types is not adopted at this meeting. The second piece is also agreed but replacing AU with layer AU.

3) Agreed in principle. It was noted that such constraints would also be needed for other types of pictures, like RASL, RADL, etc.

4) Same as in HEVC, the proposed constraint cannot be applied for PPSs, because of scenarios with externally provided PPSs. Further study encouraged whether this can be applied to APSs without affecting scenarios with externally provided APSs.

### Reference picture list signalling (2)

JVET-O0148 AHG17: On reference picture list signalling [Y.-K. Wang, Hendry (Futurewei)]

Chaired by M. M. Hannuksela, July 6th.

- Add a constraint to require that, for P and B slices, the number of active entries in RPL 0 is greater than 0, and for B slices, the number of active entries in RPL 1 is greater than 0.

Agreed.

- Change the syntax and semantics of num_ref_idx_active_override_flag and num_ref_idx_active_minus1[ i ], asserted to make the syntax and semantics simpler and more intuitive for understanding.

It was commented that VVC Draft 5 is not broken.

The following that had been stroke out in the contribution should be kept:

- if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 )

- For i equal to 0 or 1, when the current slice is a B slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ i ] is not present, num_ref_idx_active_minus1[ i ] is inferred to be equal to 0.

- When the current slice is a P slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ 0 ] is not present, num_ref_idx_active_minus1[ 0 ] is inferred to be equal to 0.

For further study.

- Move ltrp_in_slice_header_flag from the ref_pic_list_struct( ) to the SPS syntax.

It was commented that this proposal removes the functionality to control on individual RPL basis signalled in SPS whether it contains long-term reference pictures or not for achieving bit count savings in SPS.

No action.

JVET-O0244 AHG17: On zero delta POC in reference picture structure [V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

In this contribution, it is proposed to additionally signal weighted prediction enabled flag in SPS, which is signaled in the PPS currently, and to disallow zero delta POC values when weighted prediction is not enabled.

Agreed. Editorially move the condition out of the RPL structure syntax.

### Miscellaneous HLS topics (13)

JVET-O0151 AHG17: On no_output_of_prior_pics_flag [Y.-K. Wang (Futurewei)]

Chaired by M. M. Hannuksela, July 6th.

This contribution proposes the following two aspects:

- The value of no_output_of_prior_pics_flag for CRA pictures starting a new CVS is used similarly as for IDR pictures.

- The no_output_of_prior_pics_flag is also signalled for GRA pictures and applied for GRA pictures starting a new CVS similarly as for IDR pictures.

It was suggested that no_output_of_prior_pics_flag should be present also in reserved IRAP VCL NAL units types. Agreed, with the suggestion included.

JVET-O0154 AHG17: Miscellaneous AHG17 topics [Y.-K. Wang (Futurewei)]

Chaired by M. M. Hannuksela, July 6th.

- Text for the empty clause on order of VCL NAL units and association to coded pictures, such that the decoding order of VCL NAL units within a coded picture is clearly specified.

It was commented that there are proposals (e.g. O0176 proposal 4) related to aspect 1. See notes regarding those proposals.

- Text for the basically empty Annex B on byte stream format. The text is exactly the same as in HEVC, except for the addition of DPS_NUT and APS_NUT, along with VPS_NUT, SPS_NUT and PPS_NUT, in the semantics of zero_byte. Agreed.

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposal 5 (also covered in JVET-O0152) in the contribution belongs to this agenda item.

Proposes to move slice_pic_order_cnt_lsb before recovery_poc_cnt.

Agreed.

JVET-O0178 On DPB Parameters [S. Deshpande (Sharp)]

Proposal 1) It is proposed to code sps_max_dec_pic_buffering_minus1[ i ] and sps_max_num_reorder_pics[ i ] using delta coding.

2) It is proposed to conditionally signal sps_sub_layer_ordering_info_present_flag based on the value of sps_max_sub_layers_minus1.

Comment:

1) Seems making sense in principle. Why not for sps_max_latency_increase_plus1[ i ] as well? It seems not easy to analyze whether it is monotonic non-decreasing.

Agreed.

2) Makes sense. Agreed.

JVET-O0201 AHG17: On Gradual Random Access (GRA) [M. Coban, A. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

1) Proposal part 1:

Proposes the following change (adding the highlighted bullet item):

For each GRA picture in the bitstream, the following applies:

– If the current picture is the first picture in the bitstream in decoding order or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

– If the current picture is before the RpPicOrderCntVal of previous GRA picture in decoding order with NoIncorrectPicOutputFlag equal to 1, the variable NoIncorrectPicOutputFlag is set equal to 1.

– Otherwise, NoIncorrectPicOutputFlag is set equal to 0.

The proposal is based on the following justification:

"An encoder can decide to encode an IRAP picture or start another GRA process after signalling a GRA picture with NoIncorrectPicOutputFlag equal to 1 before its associated recovery point picture in decoding order. In this situation, it is proposed to replace the RpPicOrderCntVal of the decoding process with PicOrderCntVal of the IRAP picture for IRAP case, and with PicOrderCntVal + recovery_poc_cnt, where PicOrderCntVal and recovery_poc_cnt are the PicOrderCntVal and recovery_poc_cnt of the new GRA picture and set NoIncorrectPicOutputFlag equal to 1. Basically, a GRA picture with NoIncorrectPicOutputFlag equal to 1’s RpPicOrderCntVal is valid until an IRAP or a new GRA_NUT or its associated recovery point picture (or a later poc value) is decoded."

2) Proposal part 2: Proposes to change the coding of recovery_poc_cnt from se(v) to ue(v), thus disabling gaps in picture output as least the GRA picture from which random accessing occurs would have later output order than the recovery point when the value of recovery_poc_cnt is negative.

Discussions:

Proposal part 1:

It was commented that this would disallow overlapping GRA periods, while the overlapping GRA periods can be used for the use case that the proposal is addressing. No action.

Proposal part 2:

Interesting. Seems to be a valid point. Why did we allow such picture output gaps in AVC and HEVC for GDR/GRA indicated by the recovery point SEI message.

Agreed to change coding of recovery_poc_cnt to ue(v) instead of se(v). The BoG was aware that negative and zero values were allowed for recovery_poc_cnt in HEVC and the same behavior was also allowed in AVC. Does the track know why negative values were allowed since AVC? Should we further disallow value zero?

JVET-O0234 AHG17: Slice header extension [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

Proposes to add the slice header extension mechanism that is essentially identical to that in HEVC. HEVC slice header extension design in HEVC edition 1 was later made use in HEVC Annex F and Annex I.

Agreed.

JVET-O0238 AHG17: Parameters in SPS or slice headers [M. Pettersson, P. Wennersten, R. Sjöberg, M. Damghanian (Ericsson)]

The proponents report that some of the slice parameters in VVC stay constant for all slices in the bitstream for some CTC configurations. This contribution proposes a mechanism to enable signaling some of the slice parameters in VVC either in the SPS or in the slice header.

More specifically, the following changes to VVC are proposed in this contribution (the spec text actually is editorially improved compared to the summary below, to avoid the same syntax element names in different syntax structures):

- Add a sps_or_slice_flag in the SPS that specifies whether the following syntax elements are always signalled in slice headers or conditionally signalled in the SPS or slice headers:

  - dep_quant_enabled_flag

  - ref_pic_list_sps_flag

  - slice_temporal_mvp_enabled_flag

  - mvd_l1_zero_flag

  - collocated_from_l0_flag

  - six_minus_max_num_merge_cand

  - five_minus_max_num_subblock_merge_cand

  - max_num_merge_cand_minus_max_num_triangle_cand

- If sps_or_slice_flag is equal to 1, the following applies:

  - The syntax elements are present in the SPS.

  - For each of these syntax elements, if the value in the SPS is equal to 0, the syntax element is also present in the slice header, and the value signalled in the slice header is used. Otherwise, the syntax element is not present in the slice header and the value signalled in the SPS is used.

- Otherwise, the syntax elements are not present in the SPS and are present in the slice header.

The luma BD-rate numbers for the CTC are reported to be 0.0%/0.0%/-0.1%/0.0% for AI/RA/LDB/LDP respectively and the proponents claim higher gains when multiple slices per picture are used.

Discussions:

The analysis was performed based on VTM 5.0, but the RPL implementation only got integrated only since VTM 5.1.

Agreed, but do this in the PPS instead of SPS. Further study whether some of those should be just in the PPS only.

JVET-O0241 AHG17: On Decoding Process Steps for PictureOutputFlag and Unavailable Reference Pictures [S. Deshpande (Sharp)]

1) Proposal 1: Modifications are proposed for setting of PictureOutputFlag during decoding process for a coded picture and for general decoding process for generating unavailable reference pictures.

Agreed (bug fixes).

2) Proposal 2: A bug-fix is proposed to the decoding process for generating unavailable reference pictures to correctly set the POC value for long-term reference pictures.

Agreed (bug fix). Additional, add the inference of slice_pic_order_cnt_lsb for the generated picture.

JVET-O0270 AHG17: Signalling refreshed regions associated with a GRA picture [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Chaired by S. Deshpande, July 6th.

1) Proposal 1: proposes to add refreshed_region_flag to the slice header when gra_enabled_flag is equal to 1.

The semantics of refreshed_region_flag in the proposal is not sufficient to enable correct decoding of the slices with refreshed_region_flag equal to 1 and to enable a correct recovery after the indicated recovery point.

JVET-O0124 (part of CE 11) proposes the same flag, but therein it affects the decoding process. Currently, that topic in CE 11 is to be revisited. A revisit on this could be asked if this is not addressed by CE 11 activities.

2) Proposal 2: proposes to add pps_gra_enabled_flag to PPS and condition refreshed_region_flag in the slice header based on pps_gra_enabled_flag equal to 1.

No action.

3) Proposal 3: proposes to move the GRA related syntax to be before slice_address, immediately after the first slice header syntax element, slice_pic_parameter_set_id.

The syntax elements slice_address and slice_type are more important than the GRA information, e.g., for AU detection, trick mode playback.

No action.

JVET-O0391 AHG17: Avoiding dependencies on externally controlled variables in the decoding process [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)]

1) A decoding control NAL unit is proposed with the syntax that includes the following syntax elements, gated by presence flags:

a) target_layer_id: the identifier of the layer that is to be decoded

b) highest_temporal_id: the identifier of the highest sub-layer that is to be decoded

c) handle_as_cvs_start_flag, indicating if the associated CRA or GRA picture is treated as a picture that starts a CLVS

d) no_output_of_prior_pics_flag, when equal to 1, specifying that the pictures whose output time precedes the decoding time of the associated IDR picture are not output

2) The variables TargetLayerId, HighestTid, and HandleAsCvsStartFlag are derived from the respective syntax elements of the decoding control NAL unit(s) rather than set by external means.

3) no_output_of_prior_pics_flag is removed from the slice header syntax. NoOutputOfPriorPicsFlag is derived from no_output_of_prior_pics_flag in the decoding control NAL unit.

Discussions:

It was commented that, adding a NAL unit ...

It was commented that this is rather a systems issue, and using a new NAL unit seem to try bypass the systems functionality.

[Add comments, including a comment related to encryption].

No action.

JVET-O0392 AHG17: Output of SEI messages [M. M. Hannuksela (Nokia)]

Proposes to include a flag (sei_output_flag) in the general SEI message syntax, i.e. sei_message( ). When the flag is equal to 1, the SEI message is output together with the associated picture.

Question: How to test it?

It was commented that the problem statement (quoted below) is convincing:

In some operating systems and/or device architectures, the player application might not be able to pass metadata to the rendering process in a picture-synchronized manner but rather only the video decoder might be capable of doing that. This might apply particularly for encrypted video.

For example, frame packing is widely used for stereoscopic video systems, e.g., in some 3D video systems and in almost all 3600 video systems, but there seemed to be no complaint about that the frame packing SEI message is needed for the systems to work.

It was commented that adding a flag into the syntax is a waste of a bit, as for a particular bitstream, whether an SEI message is needed is determined by the type of the SEI message, thus the SEI type contains the information already.

One scenario: use a v1 decoder to decode a bitstream containing an "essential" SEI message defined in v2 of the spec, this flag equal to 1 mandates that such SEI messages shall not be ignored but be passed to the post-processing function.

It was pointed out that this flag has some overlap with the SEI manifest SEI message.

Further study encouraged.

JVET-O0497 AHG17: On layer grouping for independent regions and ARC [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

The contribution proposes a layer group signalling in the VPS with an indication of associated layers and where all or only one layer needs to be decoded, to address the following two aspects:

- Layered Independent Regions jointly decodable as single layer

- Indication of layers that can be used for ARC or open GOP switching

The proposed grouping indicates how many and which streams are grouped together into a so called Layer Group to be processed jointly by a decoder as a single layer, i.e., a decoder set by external means to decode a specific Layer Group encounters a NAL unit of said Layer Group, it decodes the NAL unit as if they had the same LayerId value.

Similarly, the Layer Group concept can be used for ARC/RPR. In a multi-layer approach, it may be the case that not all layers can be used for ARC/RPR, but that switching from one layer to another without an IDR is constrained so that it can only be done between identified layers in a Layer Group. Therefore, a grouping mechanism is required that indicates which group of layers have been encoded to allow for switching by using ARC/RPR and that only a single layer per access unit (e.g. the highest layer_id present) is to be decoded. Since in this case, again, the layers within a layer group are considered as if they belonged to the same layer, the marking process for another layer of the same Layer Group might be affected by another layer within the same Layer Group.

Discussion:

On the subpicture functionality, it was commented that some aspects need to be further worked out, including sharing PPSs, placing subpictures correctly, etc.

It was commented that, if PTL and/or HRD parameters are provided for each group, this may make sense.

Further study encouraged.

JVET-O0783 AHG17: Control of transform skip residual coding [L Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Resolved somewhere else.

JVET-O0487 Geometry padding for cube based 360 degree video using uncoded areas [J. Sauer, M. Bläser (RWTH Aachen)]

Chaired by J. Boyce, 7 July.

This document describes how coding tools for cube based 360° video can be implemented without changes to the core design. VVC was extended with a mechanism that specifies uncoded areas in the decoded picture buffer, which are filled by specifically processed samples. For 360° video coding these areas are filled with samples for the purposes of geometry padding or filtering across face boundaries. The specification of these processes does not necessarily have to be part of VVC, but could also be part in an independent standard which would need to be implemented in combination with VVC to enable 360° video applications.

It is suggested that such a mechanism might also be useful for other applications, such as scalable, multi-view or any kind of multi-layer coding with need for specific inter-layer processing.

Requires changes to the decoding process, with multiple pages of spec text. Proposes using the APS to signal parameters. There was concerned expressed about adding a coding tool which is applicable to only a specific application, but would burden all implementations. No action.

JVET-O0488 Parameterizable cubemap for 360 degree video coding [J. Sauer, M. Bläser (RWTH Aachen)]

Chaired by J. Boyce, 7 July.

This document describes a parameterizable cubemap, which is able to mimic the behaviour of other cubemap formats, e.g. CMP, EAC based on its configuration. The parameters of the cubemap are signaled with an SEI message. Using this, the specific mapping function of a cubemap format does no longer have to be hard coded, i.e. it is sufficient to implement this format instead of implementing CMP, EAC, ….

The equation is a Taylor’s series. It was tested with 9 coefficients.

JVET-O0891 AHG6/AHG17: Cubemap-based projection syntax for 360-degree videos [Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

Chaired by J. Boyce, 7 July.

This document describes a parameterizable cubemap, which is able to mimic the behaviour of other cubemap formats, e.g. CMP, EAC based on its configuration. The parameters of the cubemap are signaled with an SEI message. Using this, the specific mapping function of a cubemap format does no longer have to be hard coded, i.e. it is sufficient to implement this format instead of implementing CMP, EAC, ….

Provides two flags to indicate for horizontal and vertical dimensions if conventional CMP or generalized cube mapping is used. Unclear that it is necessary to divide in to horizontal and vertical components. Also signals these flags per face, which seems unnecessary.

Face size is determined based on the coded picture size and the layout. Would be better to use the conformance cropped size.

This design is specifically directed at supporting the functionalities available in 360Lib. The region-wise packing SEI is a superset of the functionality. This proposed syntax would be fewer bits for the targeted functionalities.

Key questions:

- Should we define a generalized cube map SEI with equations to enable different projection mapping functions?

  - Which equations & syntax?

- Should the new cube map SEI include layout and padding functionality?

Support was expressed for having a generalized mapping function.

Side activity (coordinated by J.-L. Lin) to modify syntax based on discussion. Consider adding an EAC specific option. Revisit.

### APS and ALF parameters signalling (9)

JVET-O0245 AHG17: On TemporalId and NuhLayerId in APS [V. Seregin, M. Coban, A. K. Ramasubramonian, N. Hu, M. Karczewicz (Qualcomm)]

1) Proposal 1: Change the constraint on TemporalId of an APS NAL unit, which is currently required to be equal to that of the containing layer AU, to be either "greater than or equal to" or "less than or equal to".

2) Proposal 2: Constrain that the NuhLayerId of an APS NAL unit shall be the same as the NuhLayerId of the coded slice referring to the APS NAL unit.

Discussions:

1) Agreed to remove the bullet item on TemporalId of APS NAL units.

Also agreed that to add a constraint to require that the value of TemporalId of VPS and DPS NAL units to be 0, same as for SPS. This is obvious, but currently missing in the latest draft spec.

2) Agreed on same APS content constraint. For the piece on layer ID, agreed to do as proposed if scalability with inter-layer prediction is not supported, otherwise agreed to allow the layer ID of the APS to be the same as the current picture or the layer ID of a (directly or indirect) dependent layer.

JVET-O0246 AHG17: Separate luma and chroma ALF APS types [V. Seregin, N. Hu, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes to have independent value spaces for APS ID of different types of APSs and to split ALF APS type into luma and chroma ALF APS types.

On the proposal of having independent value spaces for APS ID of different types of APSs, that seemed the intent of the APS design already.

On splitting ALF APS type into luma and chroma ALF APS types:

It was commented that this would at least increase overhead of NAL unit header and either start codes or length fields.

Further study encouraged, to study how often one of luma and chroma ALF parameters stay the same and the other one changes, and how many bits can be saved by separating the ALF APS into luma and chroma ALF APS types.

JVET-O0299 AHG17: APS support for default and user defined scaling matrices [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes to add the support for signaling of scaling list matrices in the APS.

Comments:

Do scaling list matrices have the properties similarly as ALF and LMCS parameters that would make them better signalled in the APS rather than in SPS/PPS/SH? Such properties are like the following:

1) A set of parameters may be shared by different slices of a picture, and by multiple pictures, but they change often across pictures.

2) Within a bitstream, many sets of parameters may be applied.

It seems that the scaling list matrices also have such properties.

Agreed, but constrain that all slices of a picture shall refer to the same matrix APS ID.

Agreed to clarify in the spec that different types of APSs have independent APS ID value spaces.

Suggest to discuss in track on

- What are the maximum numbers of matrix and ALF APSs that may be needed?

- What is the maximum number of APSs for each type that may need to be referred to by slices of one picture?

JVET-O0300 AHG17: Clean-up of ALF Syntax Parameters [S. Paluri, J. Lim, S. Kim (LGE)]

Resolved elsewhere.

JVET-O0301 AHG17: Simplification of ALF Clipping Coefficients in the APS [S. Paluri, J. Lim, S. Kim (LGE)]

Resolved elsewhere.

JVET-O0302 AHG17: Simplification of ALF Coefficients in the APS [S. Paluri, J. Lim, S. Kim (LGE)]

Resolved elsewhere.

JVET-O0893 Crosscheck of JVET-O0302 (Non-CE5: Simplification of ALF Coefficients in the APS) [N. Hu (Qualcomm)]

Resolved elsewhere.

JVET-O0428 AHG16/AHG17: LMCS related clean-ups [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Zhao, S.H. Kim (LGE)]

Note that if the 3rd part of this proposal is agreed, the specification text for that part should be reviewed and confirmed by HLS experts.

Already discussed and adopted Saturday July 6 in Track B except the APS part which needs review and confirmation by HLS BoG. Notes from JVET-O_Notes_d3 below, notes on adoption should be in JVET-O_Notes_d4 when available:

The expression of the constraints were agreed in principle. Some editorial clean ups are needed; delegated to the editors.

JVET-O0491 AHG17: High-level syntax cleanup [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

Recent adoptions related to ALF introduced new syntax element types into the VVC specification: tb(v) and tu(v), for truncated binary and truncated unary codes. tu(v) is currently only used in one location in the slice header, tb(v) in multiple locations in the slice header and the APS. This contribution proposes to replace these codes with u(n), u(v), or ue(v), without significant change in coding efficiency.

Agreed to suggest to track/plenary that all syntax elements in all parameter sets and slice header to use u(n), u(v), ue(v), or se(v), nothing else like tu(v), tb(v), etc.

JVET-O0561 AHG17: APS ID Control Mechanism [S. Paluri, J. Lim, S. Kim (LGE)]

Proposes a mechanism for allocation of a portion of the APS IDs for each tool carried within the APS for a given coded video sequence (CVS) is proposed.

No action.

JVET-O0090 CE5-4: alternative luma filter sets and alternative chroma filters for ALF [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

The syntax looks OK.

It was suggested that proponents of all agreed proposals that touch the alf_data( ) syntax to provide a single text for the changes to the alf_data( ) syntax, in a new document.

Revisit when that text is available.

### Virtual boundary signaling (3)

JVET-O0339 AHG17: On virtual bounary signaling [B. Choi, S. Wenger, S. Liu (Tencent)]

Proposes to use delta coding for pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ], still coded as u(v), with the length changing as the index increases.

It was commented that it's generally an overkill to do this kind of length-changing u(v) coding for one array, as this just makes parsing of one array of syntax elements more difficult due the need of calculating the length each time with the logarithmic math. Unless it is shown that it clearly saves a LOT of bits, it's not worth to do.

Options:

1) Agree with the proposal

2) Use delta and ue(v)

3) Use u(13), not delta coded

4) No action (u(v), not delta coded)

Agreed on option 3). BD will do the software integration.

JVET-O0340 AHG17: On modification of virtual boundaries signaling [G. Bang, W. Lim, H.Y. Kim (ETRI)]

Proposes

1) A delta based virtual boundaries signalling, and

2) A uniform virtual boundaries signaling

On 1): See notes under JVET-O0339.

On 2): It was commented that since there can only be up to 3 virtual boundaries, it seems not worth do an additional mode for the signalling. It was commented that in the worst case this can save 49 bits. A similar proposal is included in JVET-O0445. In the cube map projection case, there are 3 virtual boundaries (2 vertical and 1 horizontal), this saves 37 bits.

No action.

JVET-O0445 [AHG17] Signaling of uniform virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. P. Choi (Samsung)]

See notes under JVET-O0340.

### HRD (11)

JVET-O0043 AHG17: Clarification on role of pic_struct to HRD and the output picture process [C. Fogg (MovieLabs)]

JVET-O0152 AHG17: HRD support for GRA and changing the term GRA to GDR [Y.-K. Wang (Futurewei)]

Chaired by S. Deshpande July 7th.

This contribution proposes some text changes asserted to enable the specification of HRD conformance for bitstream parts containing GRA pictures.

The following two aspects are proposed:

- It is proposed to enable the specification of HRD conformance for bitstream parts containing GRA pictures by allowing association of a buffering period SEI message with a GRA access unit.

Agreed with 2.2 except the definition for trailing pictures will not be changed.

2.3, 2.6 resolved by decision on JVET-O0176

2.4, 2.5, 2.8, 2.9: agreed

- Editorially, it is proposed to change the term gradual random access (GRA) to gradual decoding refresh (GDR).

Agreed.

JVET-O0153 AHG17: HRD clean-ups [Y.-K. Wang (Futurewei)]

Chaired by M. M. Hannuksela.

The following aspects are proposed:

- Move the hrd_parameters( ) syntax and semantics to clause 7.

- Remove "vui_" from all syntax elements in hrd_parameters( ) starting with "vui_".

- Rename hrd_parameters( ) to general_hrd_parameters( ) to avoid the name subset issue.

All aspects above agreed (editorial bug fixes).

In addition, it is suggested to discuss and resolve the following issues:

- Clarify the relationship between vui_cpb_cnt_minus1[ ] in hrd_parameters( ) and bp_cpb_cnt_minus1 in the buffering period (BP) SEI message.

There are related contributions. See notes elsewhere.

- Step 5 of clause C.1 selects the BP SEI message and picture timing (PT) SEI message that apply to a particular value of HighestTid. However, the HighestTid value to which a BP or PT SEI message applies is currently not clear.

It was commented that an approach similar to HEVC scalable nesting SEI message could be used.

It was commented that VVC Draft 5 is complete in this regard, but the text should be clarified.

No action.

JVET-O0177 On HRD Signalling [S. Deshpande (Sharp)]

Proposal 1: It is proposed to allow signaling of HRD parameters for temporal sub-layers optionally controlled by a flag.

Proposal 2: It is proposed to remove timing_info_present_flag, and conditionally signal syntax elements num_units_in_tick and time_scale based on hrd_parameters_present_flag.

Proposal 3: It is proposed to signal the syntax elements num_units_in_tick and time_scale with a minus1 coding.

Comments:

1) Agreed.

2) Agreed.

3) No action.

JVET-O0180 On Buffering Period [S. Deshpande (Sharp)]

Proposal 1:

•	It is proposed to signal initial CPB removal delay values for different highest temporal sub-layers supported by the bitstream.

•	Operation of coded picture buffer is modified to use the appropriate initial buffering delay parameters based on the HighestTid.

Proposal 2: It is proposed to keep track of the BitRate and CpbSize values for each temporal sub-layer representation separately and use the appropriate variable values during the operation.

Comments:

1) Is this kind of adding loops into the BP/PT SEI messages the right approach to address the scalability issue? What if we also add layered based scalability in addition to the sub-layer based scalability we have now? Wouldn't the scalable nesting scheme in HEVC be a much more scalable solution?

It was commented that adding loops for temporal scalability seems making sense.

Suggestion: This should be consistent with other agreements: add flag to enable signalling for either all sub-layers or just highest sub-layer.

Suggestion: Add a syntax element that is equal to sps_max_sub_layer_minus1 to avoid parsing dependency on SPS, and further require that bp_cpb_cent_minus1[ i ] is equal to sps_cpb_cent_minus1[ i ].

Agreed with the proposal and with the suggestions.

2) It's a good editorial improvement. Agreed.

JVET-O0189 AHG17: Support of decoding unit in HRD [K. Kazui (Fujitsu)]

This contribution proposes to introduce the decoding unit (DU) based HRD operation. CPB and DPB parameters and operations in HEVC are suggested as the starting point.

Agreed.

JVET-O0227 AHG17: Parsing dependency of picture timing SEI message on SPS [V. Drugeon (Panasonic)]

It is suggested to remove a parsing dependency, believed to be not intended by the author, of the picture timing SEI message on the active SPS by signalling the number of temporal sub-layers of the bitstream within the picture timing SEI message.

Agreed.

JVET-O0228 AHG17: Harmonized HRD parameters signaling [V. Drugeon (Panasonic)]

It is suggested to modify the syntax and semantics of the buffering period SEI message by:

1)	Adding a loop on temporal sub-layers;

2)	Adding requirements that syntax elements repeated in the buffering period SEI message shall be equal to the corresponding syntax elements from the active SPS.

Agreed. See more notes under JVET-O0180.

JVET-O0495 AHG17: On HRD for Open GOP and DRAP [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

Proposes to add one mechanism that enables signalling of an alternative set of HRD parameters for removal of RASL pictures and for removal of pictures between an IRAP picture and a DRAP picture associated with the IRAP picture. The proposal is twofold:

1) Indicating only the initial CPB removal delay for the original/complete bitstream in the buffering period SEI message but adding indication of the presence of alternative removal times to following picture timing SEI messages (similar to irap_cpb_params_present_flag in HEVC) and indication of whether the removal of pictures has happened (same as use_alt_cpb_params_flag in HEVC).

2) Different from the HEVC alternative timing design, an indication of offsets to the original CPB removal times in picture timing SEI messages that applies when AUs are removed from the IRAP up to the AU associated to the picture timing SEI message.

It was commented that the alternative HRD parameters apply starting from the AU containing the BP SEI message, while the parameters are now signalled in the picture later in decoding order.

Further study encourage.

JVET-O0496 AHG17: On HRD for splicing [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)

1) This contribution provides details of an asserted bug of the HRD model of VVC and HEVC, specifically when splicing. The first aspect of this contribution proposes a bug fix for the reported issue.

2) The second aspect has two options. In option 1 it is proposed to add a flag to the PT SEI message that allows a splicer to easily identify pictures within the bitstream, at which seamless splicing can be achieved. In option 2, in addition to the option 1 flag, a value that may be different than the InitCpbRemovalDelay is signalled in the BP SEI message. With this signalling, a bistream that has InitCpbRemovalDelay greater than that of the current bitstream but less than the signalled value can be seamlessly spliced together.

1) Further study encouraged.

2) It was commented that this a good direction. If to be done, option 2 seems more preferable option 1. Revisit in track.

JVET-O0667 DPB size analysis of VTM5.0 using RA configuration in current CTC [X. Ma, H. Yang (Huawei)]

### VUI and SEI (9)

JVET-O0041 AHG17: On signalling of frame field information [Y.-K. Wang (Futurewei), K. Sühring (HHI), C. Fogg (MovieLabs)]

Chaired by M. M. Hannuksela, July 5th.

This contribution proposes to signal the frame field information using a separate SEI message instead of in the picture timing information SEI message, and it is further proposed that, when field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS.

The condition "if( CpbDpbDelaysPresentFlag )" is redundant and should be removed.

Agreed with the removal of the if-condition above. To be discussed in the Track/Plenary review whether the frame-field information SEI message will be included in the base VVC standard or in the separate SEI part.

JVET-O0042 AHG17: Replicating pic_struct into separate elements for future extensibility [C. Fogg (MovieLabs)]

Chaired by J. Boyce, 7 July.

This proposal breaks the compound functionality of pic_struct into separate elements (pic_output_period, bottom_pic_flag, field_paired_with_next_opposite_field_flag, output_fields_from_frame_flag, top_field_first_flag) to meet the extensibility needs of future video applications. It is not proposed to replace the existing pic_struct in SEI messages inherited from AVC and HEVC, which would be supported for legacy applications and transcoding, though equivalent functionality is proposed via the new elements.  Some of the pic_struct modes in HEVC may now be depreciated in industrial use, and therefore functionality may be undesirable to carry forward in a more compact set of new syntax constructs for VVC. Powerpoint slides are also included in version 2 (revision 1) of this proposal. The element pic_output_period is essentially a downloadable DeltaToDivisor that has been derived from pic_struct table in HEVC (and copied into the VVC working draft). The bottom_pic_flag originates from AVC slice header to indicate the current coded field parity (conveyed in pic_struct 1, 2, 9, 10, 11, 12 scenarios). field_paired_with_next_opposite_field_flag identifies whether the previous or next field of opposite parity is a paired with the current field (intended for systems that encourage fields to occur in pairs, i.e. no dangling fields). top_field_first originates from MPEG-2, which indicates the field (top or bottom) of a frame is that is first output (in pic_struct 3, 4, 5, 6 scenarios).  Compared to the 4-bit pic_struct bitstream element, any possible field sequence combination of bottom_pic_flag and field_paired_with_next_opposite_field_flag, or frame sequence combination of output_fields_from_frame_ flag and top_field_first_flag consumes 2 bits.  The effective length of pic_output_period starts at 3-bits if pic_output_period is coded with an unsigned exp-Golumb code, i.e. u(e). Should pic_output_period be specified with a variable length code, or the interpreted picture period of 0 not be desired to be expressed via pic_output_period (such as by changing the bitstream element to pic_output_period_minus1), then the nominal length of pic_output_period (or pic_output_period_minus1) is 1-bit. These new elements can be further conditionally signalled by presence or enabled flags at a higher layer, such as picture parameter set (PPS) or sequence parameter set (SPS).

pic_output_period value of 0 could serve as a no output pic flag, or could have a minus1 capability. Could gate presence in the slice by an SPS or PPS flag.

It was remarked that we have avoided having syntax conditioned on VUI parameters, and this syntax relies on field_seq_flag, although it could be moved to the main SPS.

There is an existing pic_output_flag in the slice header, which has its presence conditioned by a presence flag. Could possibly replace the pic_output_flag with the proposed pic_output_period.

Should these parameters be in the slice header or an SEI? Some support expressed for keeping it in an SEI message.

JVET-O0041 has already been adopted this meeting, and would require the presence of the frame_field_info SEI when field_seq_flag is equal to 1.

Recommend to adopt the SEI message option, and that it be placed in the same section of the specification as the picture timing SEI and HRD.

JVET-O0435 AHG17: on pic_struct [S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen (Dolby)]

Withdrawn.

JVET-O0360 AHG17: On sRGB and sYCC [T. Suzuki (Sony)]

This was also presented in JCT-VC for AVC and HEVC, and the proposal was agreed therein.

This contribution proposes to correct transfer_characteristics and matrix coefficients to support IEC 61966-2-1. The semantics of transfer characteristics (Table E.4), and the semantics of matrix coefficients (Table E.5) should be corrected.

Agreed.

JVET-O0622 AHG2/AHG17: Draft text to update BT.2100 references and ICTCP HLG coding equations [S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen]

Resolved elsewhere.

JVET-O0492 AHG17: On merging of bitstreams and conformance [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Chaired by Y.-K. Wang, July 5th.

This contribution proposes to add signalling for merging of several independently coded regions (layer, tiles or bricks) into a joint single-layer bitstream. It is argued in the contribution that such system design is a relevant implementation choice for addressing devices with a single decoder instance only and for the inherent synchronization of decoded output in such a design.

1) The first aspect of the contribution adds an indication towards applicable merging operations and identification of mergeable bitstreams. An SEI message is proposed that carries a mergeability_id to represent a set of constraints defined by external means such as an application specification or a service operator. The mergeability_id is associated with a mergeability_type describing which NAL unit types and syntax structures are required to be rewritten in the respective merging operation.

2) The second aspect of the contribution adds an indication towards HRD conformance of merged bitstreams to the PTL syntax. A target_level_idc is given in combination with a target_level_percentage that a merged bitstream would occupy in a merged bitstream with a level equal to target_level_idc.

On proposal 1):

Interesting idea.

It was commented that using 6 bits for mergeability_id might not be enough to avoid potential conflict of IDs, particularly if the semantics of the ID is NOT defined and controlled by a single organization.

It was commented that such information could also be left for systems, e.g., file format or DASH to carry.

It was commented that it is possible to just use the ID itself without a type field, if the semantics of the type field is incorporated into the semantics of the ID.

Further study encouraged.

On proposal 2):

Isn't the actual level of the current bitstream as well as other parameters like spatial resolution can be used for the same purpose as the additional information herein?

It was commented that if this is to be included into the specification, it should be SEI message instead.

Is it possible to have multiple target level?

Further study encouraged.

## AHG12: high-level parallelism and coded picture regions (45)

### General (1)

JVET-O0800 A summary of HLS proposals on coded picture regions [Y.-K. Wang (Futurewei)] [late]

The design questions provided in this contribution were used by the coordinators of the BoG on high-level syntax to structure the discussions of other contributions in this category.

### Sequence-level independent regions (12)

Contributions in this category were discussed on 4 July, 2019, 0900 to 2035 in the HLS BoG, chaired by Y.-K. Wang, with the discussion and decision on sub-picture ID signalling chaired by S. Deshpande.

General discussion on coding of sequence-level independent regions

- To support coding of sequence-level independent regions based on a normative approach or based on non-normative encoder motion constraints like in MCTS as in HEVC?

  - Based on a normative approach (all the 11 proposals in 6.18.1)

  - Based on encoder motion constraints (none)

- If to support coding of sequence-level independent regions based on a normative approach,

  - to have the normative approach based on treating of rectangular regional boundaries as picture boundaries in decoding motion compensation? (O0138, O0141, O0394)

Comments: It was mentioned that the low-level changes for treating of rectangular regional boundaries as picture boundaries in decoding motion compensation seem clean and acceptable.

A suggestion: Replace "rectangular slices" above with "raster-scan slices" and remove the rectangular slices from VVC. This suggestion was objected by multiple participants.

Agreed with the normative changes to the low-level decoding processes for treating rectangular regional boundaries as picture boundaries proposed in O0138, O0141, and O0394. Apply to the boundaries of sub-pictures, where each sub-picture consists of one or more rectangular-mode slices that collectively form a rectangular region.

Agreed with the following, pending on review of the text (J. Boyce to coordinate the work on the text):

- In the SPS syntax agreed below for signalling of sub-picture size and position, add a flag for each sub-picture to control whether the sub-picture boundaries are treated as picture boundaries.

- In the SPS syntax agreed below for signalling of sub-picture size and position, add a flag for each sub-picture to control whether in-loop filtering (deblocking, SAO, and ALF) across sub-picture boundaries are enabled.

Note that the existing raster scan slice mode would still exist in the VVC design.

    - Should treating of rectangular regional boundaries as picture boundaries based on sub-pictures or slices?

      - Based on slices (O0138, O0394)

      - Based on sub-pictures (O0141, O0394)

What is the use case of allowing for more than one slice within a sub-picture? One possible usage mentioned was that allowing splitting one sub-picture into multiple slices could be useful for delay optimization.

In a layered approach for sub-pictures, treating sub-picture boundaries as picture boundaries comes naturally with the current VVC draft text.

It was commented that, in a layered approach, the client decoder needs to run multiple decoder instances, one for each layer. However, it seems that the approach in O0334 does not necessarily need multiple decoder instances.

On the layered approach in O0334, the following comments/questions were expressed:

- Different sub-pictures within an AU have different POC values. Why? Different sub-pictures with different POC values are decoded by one decoder but have different decoding times. In that case, what is the picture output process? Then it works without using layer ID.

- RPL signalling can be significant less efficient.

- Potentially more POC LSBs might be needed for signalling of POC values, which may affect coding efficiency.

- How would these affect delta POC based scaling of MVs and sample values?

- When merging different sub-pictures layers into one layer, rewriting of slice header would be needed due to the use of different POC values for sub-pictures originated from the same picture.

- Can different sub-pictures originated from the same picture have different values of picture_output_flag? It seems they can be just required to be the same.

- How about HDR behavior? From the CPB behavior point of view, the single-layer process would conceptually work, just the picture rate is n times as great as if no sub-pictures and no layering were used. From the DPB point of view, it could be similarly as the CPB behavior, and in addition to that, the output process for the entire picture after merging the decoded sub-pictures could be specified normatively; this is currently not addressed.

- Some encoder constraints are needed to enable sub-picture based sub-bitstream extraction.

- It was commented that the proposal could be applied to any type of layers, not just sub-picture layers.

- The reference picture marking needs to be layer based, i.e., layer ID is not just metadata.

- It was commented that if we decide to have layered based approach for sub-pictures, we need to consider whether the layered approach here would also be used for scalability.

- This approach does not need a change to any low-level decoding process.

- With a layered approach, there is no way to enable in-loop filtering across the boundaries of the sub-pictures. This could be an issue for some use cases and not for some other use cases.

- If a layered approach is specified, would we still envision the support of decoding based on the original spatial resolution and picture rate?

- Document O0497, O0393 are related to the approach in O0334.

- Can sub-pictures originated from the same picture have different tile/brick configurations? No, the intent is that they have the same tile/brick configuration, but currently there is no constraint for this.

  - Why?

- How is the cropping window defined? Do they refer to the same SPS or different SPSs?

The proponent and other interested participants were requested to work on completing the layered approach based on O0334, taking into account the comments noted above, possibly taking into account related aspects in O0497 and O0393. Revisit after the output of the offline work is available.

On single-layer based approaches:

- The sub-picture sizes would be less flexible than a layered approach, because sub-pictures not at the picture boundaries must contain complete CTUs.

  - Is that flexibility useful? For evenly dividing a CMP picture into sub-pictures for the 6 faces.

    - Note that the CMP face seam issue related to evenly dividing into 6 faces have been addressed by the virtual boundaries.

    - It seems that there is no other technical benefit for evenly dividing a CMP picture into sub-pictures for the 6 faces.

  - On disadvantage of this flexibly for the layered approach is that this disallows merging the coded sub-pictures of different layers into one layer.

Is there a coding efficiency impact for the layered approach if there are many sub-pictures (e.g., 96) and they do not share parameter sets? Same question for POC and RPL signalling.

It was commented that we need a single layer approach anyway, because allowing in-loop filtering across sub-picture boundaries is needed, and in addition to that, we should investigate decoding of multiple layers as if they were in the same layer.

  - to have the normative approach based on other approaches (none)

- To code sequence-level independent regions of one picture in one layer or in multiple layers?

  - One layer, all sub-pictures of one picture are in one layer (O0138, O0141, O0176, O0182, O0183, O0394, O0493, O0555, O0700)

  - Multiple layers, each sub-picture region is in its own layer (O0334, O0473)

General discussion on signalling of sequence-level independent regions

- Where to signal location, ID, and size of sequence-level independent region?

  - In the SPS (O0141, O0555)

  - In the PPS (O0176)

  - In the VPS (O0182, O0334)

  - In a new decoding control NAL unit (O0394)

  - In a new parameter set (O0473)

  - In a new SEI message (O0700)

It was firstly agreed to exclude b, c, and f.

On sub-picture location and size (not considering scalability, ARC/RPR, GRA):

Agreed to signal sub-picture location and size in the SPS unless a sub-picture based GRA solution is agreed in the CE11 discussion.

The information on sub-pictures and the sub-picture location and size for each sub-picture in the SPS describes the exactly complete pictures referring to the SPS.

On sub-picture ID:

The discussion and decision of this piece was chaired by S. Deshpande.

In O0141 and O0555, the sub-picture ID is mainly used for sub-bitstream extraction purpose.

There was no consensus to include the signalling of sub-picture IDs in the sub-picture design in the output VVC draft text from this meeting.

On the syntax for signalling of sub-picture location and size in the SPS:

It was agreed to use the SPS syntax in O0555 for this purpose as a starting point, with the sub-picture ID replaced with grid index (not considering scalability, ARC/RPR, GRA).

Regrading items 2) and 3) below:

The BoG agreed that it is generally useful to have some information signalling conformance points for (sets of) sub-pictures. Details are for further study.

- If to specify conformance for sequence-level independent regions, where to signal the conformance region (e.g., output sub-picture sets)?

  - In the SPS (O0141, O0555)

  - In the VPS (O0183)

- For conformance for sequence-level independent regions, to explicitly signal the PTL information or to indicate that the level for any sub-picture or sub-picture set can be derived based on scaling of the picture size relative to the sub-picture or sub-picture set sample size, respectively, when not possible, signal PTL in an SEI message?

  - To explicitly signal (O0141)

  - To indicate that the level can be derived, and if not, signal in an SEI message (O0555)

- To signal a sub-picture ID in the slice header? Not for now.

  - Yes (O0141, O0555)

General discussion on other aspects covered in these contributions

- Signal maximum decoded picture width and height in DPS and decoded picture width and height in PPS, and remove pic_width_in_luma_samples and pic_height_in_luma_samples from SPS? (O0182) Leave this for the ARC/RPR discussion.

- Update the sub-bitstream extraction process to support sub-picture extraction and reposition across multiple layers (O0183)? See notes elsewhere.

- Provide the possibility to specify partitioning to tiles, bricks, and rectangular slices in SPS, and when the partitioning is specified in SPS, it is not present in the PPS? (O0394) No action for now.

- Add a new SEI message for nesting of SEI messages for output sub-picture sets? (O0493) Not for now.

- Use a grid to indicate the sub-picture size and position? (O0555) Yes, see notes above.

- Use neighboring information to indicate neighbors of a layer and thus its position in the total layout? (O0473)

JVET-O0138 AHG12: Treating region boundaries as picture boundaries [Hendry, S. Hong, J. Chen, Y.-K. Wang (Futurewei)]

JVET-O0141 AHG12: Sub-picture based motion-constrained independent regions [Y.-K. Wang, Hendry, J. Chen (Futurewei), R. Skupin, Y. Sanchez, K. Suehring (HHI)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposal 6 in the contribution belongs to this agenda item.

JVET-O0182 AHG12: On picture and sub-picture signaling [Y. He, A. Hamza (InterDigital)]

JVET-O0183 AHG12: On layer-based sub-picture extraction and reposition [Y. He, A. Hamza (InterDigital)]

JVET-O0334 AHG8/AHG12: On sub-picture partitioning support with layers [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-O0394 AHG12: On independently coded picture regions [M. M. Hannuksela (Nokia)]

JVET-O0471 AHG8/AHG12: Layer concepts clarifications and improvements for immersive media use cases [E. Thomas, A. Gabriel (TNO)]

JVET-O0473 AHG8/AHG12: On generating final output picture from independent VVC layers [E. Thomas, A. Gabriel (TNO)] [late]

JVET-O0493 AHG12: On independent regions and output sub-picture sets [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-O0555 AHG12: Sub-pictures and sub-picture sets with level derivation [J. Boyce, L. Xu (Intel)]

JVET-O0700 AHG12: On sub-picture info SEI message [Y. He, A. Hamza (InterDigital)] [late]

### Mixed NAL unit types within a picture (2)

Chaired by J. Boyce 3 July.

From JVET-O0800:

List of design questions

- To support mixed NAL unit types within a picture or not?

  - Yes (O0140, O0498)

- If yes to 1),

  - Based on a PPS flag that specifies whether all VCL NAL units of a picture have the same NAL unit type (O0140)? Or

  - Base on allowing slices within a picture to have different sets of reference pictures drived from the RPLs and changing the reference marking process such that marking does not happen with the first VCL NAL unit when slices within a picture do have different sets of reference pictures derived from the RPLs (O0498)

- If yes to 1), add the following constraint?

  - For any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type (O0140)

What are the implications for decoder implementations in a mixed NAL unit case?

IRAP vs. non-IRAP type impacts parsing and decoder process.

POC process depends on IRAP vs non-IRAP. For a mixed pictures, use the non-IRAP style numbering.

Bitstream merging was mentioned as a possible use case that would benefit, to avoid the need to have to rewrite NUT. Other use cases also discussed Independent decoder regions like sub-pictures are an intended use case, with mix of IRAP and non-IRAP pictures. 360 video in particular is a use case, where some regions have more frequent IRAP than other regions.

Question about if prediction is restricted to enable the some random access behavior. How would such a constraint be expressed?

If this feature is used, how does a decoder determine that a picture is decodable? Would the restriction be imposed in an application spec or in the VVC spec?

The JVET-O0140 contribution requires that the mixed flag be set only when a picture uses both IRAP and non-IRAP VCL NAL units in the same picture.

Question about interaction of leading pictures with this feature. May want to disable using leading pictures when this feature is used.

It was suggested that this feature is most useful when sub-pictures or MCTS is used.

Revisit after sub-pictures contributions have been reviewed.

JVET-O0140 AHG12: On mixed NAL unit types within a picture [Y.-K. Wang, Hendry (Futurewei)]

See notes above.

JVET-O0498 AHG17: On marking process and unequal NALU types per AU [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

This contribution proposes changes to the current specification as follows:

- Allow VCL NAL unit within the same AU to have different NAL unit types.

- Allow VCL NAL units within the same AU to have different Reference Picture Lists

- Define a marking process that does not happen with the first VCL NAL unit, when different RPLs are present for different VCL NAL units.

This approach doesn’t require mixed VCL NAL units to be used when the flag is set. This could be a problem for decoders to know when they are able to start decoding.

A question raised about HRD behavior.

This contribution requires changes to the decoder for reference picture list handling.

### Slice, tile, and brick signalling in PPS and slice header (15)

JVET-O0143 AHG12: Signalling of addresses for rectangular slices [R. Sjöberg, M. Damghanian, M. Pettersson, J. Ström (Ericsson), Hendry, Y.-K. Wang (Futurewei)]

JVET-O0156 AHG12: On brick signaling [W. Lim, G. Bang, H. Y. Kim (ETRI)]

JVET-O0172 On rectangular slice address signaling [B.-K. Lee (Xris)]

JVET-O0173 On brick information signaling [B.-K. Lee (Xris)]

JVET-O0176 On Tiles, Bricks and Slices [S. Deshpande (Sharp)]

Only the proposals 1 to 4 in the contribution belongs to this agenda item.

JVET-O0181 Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

Only the proposal 3 in the contribution belongs to this agenda item.

JVET-O0199 AHG12: On rectangular slices [M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0225 AHG17: On signalling of bottom right brick for rectangular slices [V. Drugeon, T. Nishi (Panasonic)]

JVET-O0269 AHG17: Suggested syntax improvement on PPS [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0338 AHG12: On signaling for slice, tile and brick [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-O0403 AHG17: On brick index signalling [P. Bordes, M. Kerdranvat (InterDigital)]

JVET-O0452 AHG12: On tile and brick partition [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0453 AHG12: On rectangular slice signaling [Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-O0454 AHG12: On sub-bitstream extraction [Y.-J. Chang, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0610 AHG17: Association of VCL NAL units to coded pictures [B. Heng, W. Wan (Broadcom)]

Proposes to add first_slice_in_pic_flag at the beginning of the slice header, unconditioned.

Discussions:

It was commented that this would not work well with sub-bitstream extraction without the need of changing the slice header, as the extracted sub-bitstream may contain only one slice, in which case the value of this flag may change.

One solution is to mandate AU delimiter, which seems no problem.

Agreed to mandate AU delimiter, and this would also enable us to remove all the AU detection text.

### Signalling of brick/WPP entries (4)

JVET-O0144 AHG12: On end_of_subset_one_bit, end_of_brick_one_bit, and byte alignment in the slice data syntax [Hendry, Y.-K. Wang (Futurewei)]

Chaired by M. M. Hannuksela, July 7th.

This contribution proposes a modification to the slice data syntax, related to the syntax elements end_of_subset_one_bit and end_of_brick_one_bit as well as the byte alignment syntax. It is asserted that the current signalling has a problem when wavefront parallel processing (WPP) is used for a slice that contains more than one brick.

Agreed.

JVET-O0145 AHG12: Miscellaneous AHG12 topics [Y.-K. Wang, Hendry (Futurewei)]

Chaired firstly by M. M. Hannuksela and then by K. Sühring , July 7th.

The following aspects are proposed in this contribution:

- Remove the signalling of num_entry_point_offsets.

- Move offset_len_minus1 from the slice header syntax to the PPS syntax.

It was commented that merging might affect the selection of the selection of the offset_len_minus1 value.

It was commented that the bit count savings are likely not essential.

On aspect 2, it was commented that it is easier for parsing if the length field immediately precedes the syntax element that it affects.

On the first aspect, it was later that the value range of num_entry_point_offsets needs to be fixed. However, then the derivation proposed in the first aspect is needed to fix that.

Revisit in track on the first aspect, together with the second aspect of JVET-O0215.

No action on the second aspect.

JVET-O0215 AHG12: Clean up on wavefront parallel processing signaling [T. Ikai, E. Sasaki, T. Chujoh (Sharp)]

Proposes a syntax restriction and provides range of num_entry_point_offsets as follows:

1) entropy_coding_sync_enabled_flag is only signalled when rect_slice_flag is equal to 1

No action.

2) num_entry_point_offsets shall be 0 or total number of CTU row minus 1

Revisit in track, together with the first aspect in JVET-O0145.

JVET-O0306 AHG17: Entry point offsets without start code emulation [K. Abe, T. Toma, V. Drugeon (Panasonic)]

### New tile/brick/WPP partitioning schemes (6)

JVET-O0198 AHG12: On brick partitioning [M. Coban, A. Ramasubramonian, Y.-J. Chan, V. Seregin, M. Karczewicz (Qualcomm)]

In the example, wit"This contribution proposes a separated wavefront parallel processing for Luma and Chroma CTU in Dual Tree. The first test proposes a separation of CABAC processing for luma and chroma. The second test proposes a separation of WPP for Luma and Chroma without context propagation between Luma and Chroma. The third test proposes to separate the coding of SAO syntax for Luma and Chroma on top of test 1."

hout the proposed features, this could be done by vertically splitting tile 0 into two tiles. That boundary can be non-motion-constrained.

It was commented that vertically splitting a tile into bricks imposes additionally complexities in hardware implementation. It was counter argued that vertically spitting into more tiles would be the same.

No action.

JVET-O0358 AHG12: On uniform tile spacing [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

To be presented.

JVET-O0390 AHG17: On tile and brick signalling [M. M. Hannuksela (Nokia)]

See notes under O1075.

JVET-O0511 AHG12: On tiles and bricks partitioning [N. Ouedraogo, E. Nassor (Canon)]

See notes under O1075.

JVET-O0462 AHG12: Separated Luma and Chroma wavefront for dual tree [T. Poirier, F. Le Léannec, F. Galpin (InterDigital)]

"This contribution proposes a separated wavefront parallel processing for Luma and Chroma CTU in Dual Tree. The first test proposes a separation of CABAC processing for luma and chroma. The second test proposes a separation of WPP for Luma and Chroma without context propagation between Luma and Chroma. The third test proposes to separate the coding of SAO syntax for Luma and Chroma on top of test 1."

It was commented that the processing of the chroma components would be faster than the luma, thus there can be a bit waste of computing resources.

How useful is it to enable parallel parsing of luma and chroma while the actually the processing is not parallel?

No action.

JVET-O1075 AHG17: AHG12: On tile and brick signalling (combination of JVET-O0390 and JVET-O0511) [M. M. Hannuksela (Nokia), N. Ouedraogo, E. Nassor, F. Denoual (Canon)]

In some cases, scan order of bricks have changed, which can be avoided by adding an explicit signalling that is O0390.

A concern was expressed on the inference of horizontal tile boundaries.

There is one syntax element used in a loop syntax while the flag itself is signalled in the loop. This seems to be a syntax bug.

It was commented that this introduces more parsing dependency of PPS on SPS.

It was commented that O0173 is related.

It was commented that the existing syntax is more straightforward.

Three proposal pieces:

- Skipping tile row related syntax and directly indicating bricks per tile column

- Having a single signalling mode instead of separate uniform and explicit tile/brick spacing modes

- Two options to reuse the same brick split pattern for several tile columns

Revisited on afternoon Saturday July 6th based on v2 of this contribution.

Version 2 updates the contribution as a response to the comments in the HLS BoG, on July 5th, as follows:

- It is explicitly indicated whether a chain of brick boundaries crossing the entire picture is a tile row boundary (initially in JVET-00390).

- The parsing dependency of PPS on SPS is avoided by adding CTB size signalling into the PPS (as proposed when discussing JVET-O0236). The CTB size signalled in the PPS shall be equal to that signalled in the SPS.

This not only avoids the parsing dependency but also enables deriving the tile-and-brick partitioning in units of samples by parsing the PPS only.

- A syntax bug was fixed.

- The results were updated to match the syntax changes in v2. When compared to VVC Draft 5 + the PPS parsing dependency fix of JVET-O0236, this contribution is asserted to reduce:

- the number of syntax elements from 14 to 9,

- the number of syntax table rows from 28 to 23,

- the length of the semantics,

- the length of the scanning process in clause 6.5.1, and

- the bit count spent for tile and brick partitioning of the effective 6K schemes of OMAF by 46% to 58%.

It was commented that the syntax is still a bit complicated. It was asserted by proponents that if the syntax can use "break", then some parts of the syntax could be simplified.

It was commented PPS rewriting gets more difficult with the proposed syntax. It was asserted by proponents that PPS rewriting is actually straightforward.

There was some support of feature 2 (single spacing mode). Revisit after text with only this feature is available, and make decision together with JVET-O0173.

### Loop filtering across boundaries of independently coded regions (5)

JVET-O0090 CE5-4: alternative luma filter sets and alternative chroma filters for ALF [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

JVET-O0142 AHG12: On turning off ALF filtering at brick and slice boundaries [Y.-K. Wang, J. Chen, Hendry (Futurewei)]

Also included in the agenda item 6.5.

JVET-O0494 AHG12: On filtering of independently coded region [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Also included in the agenda item 6.5.

JVET-O0654 AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

Also included in the agenda item 6.5.

JVET-O0858 Crosscheck of JVET-O0654 (AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter) [H. Liu (Bytedance)]

JVET-O0662 Non-CE5: Modified ALF filtering for Slice, Brick and Virtual boundaries [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

Also included in the agenda item 6.5.

## AHG8: layered coding and resolution adaptivity (28)

### General (1)

JVET-O1040 AHG8: Summary of Resolution Adaptivity related proposals [S. Wenger (Tencent), Y.-K. Wang (Futurewei), M. M. Hannuksela (Nokia)]

The design questions provided in this contribution were used in the HLS BoG to structure the discussions of the proposals on reference picture resampling (RPR).

JVET-O1123 AHG8: Summary of layered coding proposals [S. Wenger (Tencent), Y.-K. Wang (Futurewei), M. M. Hannuksela (Nokia)]

The design questions provided in this contribution were used in the HLS BoG to structure the discussions of the proposals on scalability.

### Reference picture resampling (RPR) (13)

Contributions in this category were discussed on 5 July 9000–1300 in the HLS BoG, chaired by S. Wenger.

General discussion on signalling of picture sizes (and conformance windows)

Two solutions:

- List of sizes in SPS, referred to by index in PPS (O0133, O0332)

- Maximum size in SPS, actual size in PPS (O0182, O0204, O0303 O0319, O0395)

It was commented that, based on decision on DPB management, the difference between the two proposal groups appears to be insignificant, except for the conformance window aspect.

Maximum size in DPS (O0182, O0332)?

The following was agreed:

- Signal the maximum size in the SPS.

- Signal the actual picture size in the PPS, using the same coding as currently for the SPS. Need to move some derivations equations that rely on active resolution currently in the SPS to the PPS.

- Move the conformance window as currently in the SPS to the PPS.

How is the scaling factor calculated? Based on conformance window or decoded picture size?

There was agreement in the room that when resampling only based on decoded picture size there is unnecessary padding and resulting mismatch in the location.

When the cropping window offset is unequal to zero, the padding issue needs to be addressed (problem #1). Also needed for spatial scalability (ROI spatial problem, which is problem #2).

It is agreed that O0204 includes one possible solution to the problem #1.

We did not have another solution to problem #1 on the table.

We did not have a solution for problem #2 on the table.

It was commented that the same functionality could be realized by explicitly signalling the resampling ratio.

Agreed on the cropping-window based solution in JVET-O0204. Detailed study required by low-level coding tools experts.

General discussion on DPB management

Per picture, irrespective of size

Based on picture size (O0133)

Hardware people prefer the simplicity of using per picture granularity, worst case.

May influence level designs and interpretation.

This is not about changing the number of reference pictures.

Agreed that DPB is managed based on the maximum picture size (i.e., each picture store is of the size of a decoded picture of the maximum spatial resolution signalled in the SPS, regardless of the actual picture size).

General discussion on resampling filter(s)

Which filter to use?

- Existing interpolation filter for upsampling/downsampling (O0134 (spec text in 133, details and software in O0134, O0303, O0242)

- For downsampling, two sets of integer versions of 8-tap Lanczos filters optimized for 2:1 and 4:1 (O0240)

- For downsampling, 9 tap (7 tab complexity). For upsampling, 4 tap, 32 phases (SHVC upsampler Table H.2, extended to 32 phases) (O0319)

- SHVC resampling process (O0395)

Picture-based or block-based specification of the filtering process? Agreed to specify a block-based resampling process.

If picture-based specification of the filtering process, specify (in SPS) the maximum number of reference pictures that need to be resampled to be used as reference for any coded picture (O0395)

Possibility to turn on "wraparound" selection sample location of input samples for filtering (for equirectangular projection) (O0395)

A brainstorming discussion ensued:

- Can we decide between the four resampling filter proposals?

- Perhaps decide on the upsampler now.  Perhaps both.

- two sequences, test with compression?

- Picture based vs. block based

- Choose something simple baseline, then punt to CE

BOG to pick one as anchor for the CE, recommend to track, CE to next meeting.

Pick one for the CD? Number 1? Bilinear? Existing SHVC filters (not in VVC)?

Based on review of 0134 spec text, this text seems to be OK. All three proposals including 303 and 242 under Number 1 have approximately the same spec text. Probably half a page of text.

Pick Number 4? Different from Number 1, because it’s picture based. Needs several additional pages of text, but that text could be battle-proof from SHVC.

Placeholder should have minimum spec text and design (only for the baseline).

It was agreed to include the resampling filter based on the existing VVC interpolation filter for both upsampling and downsampling into the VVC CD text. The details and software were provided in JVET-O0134 (for which the spec text is included in JVET-O0133), JVET-O0303, and JVET-O0242.

Agreed to have a CE on resampling filters, coordinated by V. Seregin.

Regarding wrap-around: re-use sps_ref_wraparound_enabled_flag? Yes. Number 1 solution/spec text may already take care of that, as the existing interpolation filters may take care of it. If this were not yet in the spec, it probably should be a design goal.

General discussion on constraints on coding tools

The following was agreed:

- TMVP generally allowed but the collocated picture shall have the same resolution as the current picture. [JVET-O0134, JVET-O0242, JVET-O0303, JVET-O0395]

- DMVR generally allowed but disabled for a coding block when referring to different resolution. [JVET-O0134, JVET-O0242, JVET-O0303]

- BDOF generally allowed but disabled for a coding block when referring to different resolution. [JVET-O0134, JVET-O0242, JVET-O0303]

Where is the text coming from? This BOG recommends to track RPR feature design based on adoptions here. Coordinator to pick based on review.

On the proposal of a single flag to allow disabling all TMVP, DMVR, BDOF per slice, presence gated by SPS-level flag (O0184), it was concluded that this proposal is not specific for RPR, and based on the above agreement specific for RPR, the BOG suggested to discuss under the CE4-related agenda item.

On the proposal that the existing affine fallback mode shall be triggered for a coding block when referring to different resolution (O0303), the BoG recommended to revisit in track.

The draft spec text for all the agreed items noted above is to be prepared. V. Seregin to coordinate.

JVET-O0133 AHG8: Support for reference picture resampling - handling of picture size signalling, conformance windows, and DPB management [Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

See notes above.

JVET-O0134 AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF [Y.-C Sun, T.-S Chang, J. Lou (Alibaba), Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei)]

See notes above.

JVET-O0757 Crosscheck of JVET-O0134 (AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF) [H.-J. Jhu (Kwai Inc.)]

See notes above.

JVET-O0184 AHG8: On ARC indicator [Y. He, Y. He, A. Hamza (InterDigital)]

See notes above.

JVET-O0204 AHG8: On Adaptive Resolution Change (ARC) High-Level Syntax (HLS) [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

See notes above.

JVET-O0240 AHG8: Adaptive Resolution Change (ARC) with downsampling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

See notes above.

JVET-O0242 AHG8: Enabling BDOF and DMVR for reference picture resampling [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

See notes above.

JVET-O0303 AHG8: Adaptive Resolution Change [P. Chen, T. Hellman, B. Heng, W. Wan, M. Zhou (Broadcom)]

See notes above.

JVET-O0701 Crosscheck of JVET-O0303 (AHG8: Adaptive Resolution Change) [Y. He (InterDigital)]

See notes above.

JVET-O0319 AHG8: On Adaptive Resolution Change [P. Topiwala, M- Krishnan, W. Dai (FastVDO)]

See notes above.

JVET-O0332 AHG8: Signaling and filtering for Reference Picture Resampling (RPR) [B. Choi, S. Wenger, S. Liu (Tencent)]

See notes above.

JVET-O0395 AHG8: On adaptive resolution changing and scalable coding [M. M. Hannuksela, A. Aminlou (Nokia)]

Only the RPR part in the contribution belongs to this agenda item.

See notes above.

JVET-O0641 AHG8: AHG 8: JVET Common test conditions for adaptive resolution change (ARC) [M. G. Sarwer, R. -L Liao, J. Chen, J. Luo, Y. Ye (Alibaba)]

This contribution proposes the common test conditions for adding adaptive resolution change support into VVC. It is asserted that establishing the adaptive resolution change CTC enables the JVET to select coding tools with better performance vs. complexity trade-offs. This contribution further suggests to test both modes of resampling, i.e. picture-based and block-based, in a core experiment before selecting the solution that offers better performance vs. complexity trade-off.

The proponent was asked to upload a revision of his document including the slide deck.

Selection of resampling ratio? Agreed to test 2:1, 1.5:1, and optionally 4:1, and use same ratio in both dimensions.

Per test sequence?

Resampling time instance? Agreed to test 1) low delay, fixed in time, like 0./5 sec low, 0.5 sec high, and in addition 2) random access, I picture only.

First picture of the GOP header transition (this is not in the proposal)? For offline study.

Do you change QP (for fair comparison)? Not proposed.

PSNR computation method? For offline study.

Agreed to establish the CTC for RPR, based on the following details, coordinated by J. Luo:

- Resampling ratios: 2:1, 1.5:1, and optionally 4:1, use same ratio in both dimensions.

- Test configurations: 1) low delay, fixed in time, like 0./5 sec low, 0.5 sec high, and 2) random access, I picture only.

### Scalabitliy (13)

Contributions in this category were discussed on 7 July 9000–1300 in the HLS BoG, chaired by S. Wenger.

General discussion on overview/introduction/architecture

### RPS/RPR based scalability Presentations

Note Herein, RPS standards for Reference Picture Selection, unless otherwise noted.

O0045 (conceptual, scalability using a combination of RPS/RPR, version 1)

O0135 (advocating RPS/RPR + SSEI, version 1 and no scalable profile)

O0185 (advocating RPS/RPR + VPS, version 1 and no scalable profile)

O0395 (advocating RPS/RPR, version 1 and no scalable profile)

It was agreed to specify a scalability support in VVC edition 1.

### Question re version and profile

Any reason not to consider all this for version 1? None identified.

Profiling discussion—are there details we should bring to responsible parent body attention?

It was agreed that, since from technical perspective there is no undue burden to non-scalable applications when adding scalability, it can be assumed that scalability ought to be in baseline/main (non-scalable profiles).

As a note, in previous standards spatial scalability did incur an undue burden in the sense of above statement.

### Region-based Scalability Presentations

O0335 (use case discussion advocating region-based scalability, very brief presentation)

O0336 (region-wise scalability, presentation with focus on concept, not syntax details)

Discussions:

One case mentioned: In CMP video, one of the six faces have spatial scalability, while other faces don't.

It was suggested that to focus on a basic scalability design first, then discuss enhancements and new functionalities.

The BoG agreed with the following statement: If you do this with separate streams/layers, less an issue. But if you do it for full picture, tricky.

Proponent: no low level impact.

Suggest focus on per-picture scalability only.

Revisit/discuss in track the following documents: JVET-O0335, JVET-O0336, JVET-O0471, JVET-O0473.

### Questions re region-based scalability

Is the use case within our interpretation of agreed requirements? BoG answer: Yes.

If yes, can we deal with region-based scalability in version 1? BoG answer: unclear.

If yes, can/should region-based scalability technology be part of broad profile (“main” profile)? BoG answer: unclear.

General discussion on layer dependency and scalability information signalling

O0135 (SSEI)

Make sense to do scalability in V1, non-scalable profile, minimum impact desirable, that’s the reason for SSEI.

O0136: See presentation notes below.

O0185 (VPS and VPS extension?)

SHVC-style dependency signalling, plus a simulcast flag in VPS.

O0243 (VPS)

SHVC-style dependency signalling in VPS

O0333 (VPS)

SHVC-style dependency signalling in VPS

SSEI considered bug in H.264/SVC

VPS was introduced for system design reasons

SSEI less impactful

VPS makes more sense

Parameter sets are for decoding process, SEI metadata. Decide based on necessity for the decoder

It was agreed to signal the layer dependency information in the VPS.

Does the VPS need to be present in non-scalable present? But VPS is something we need to pay attention to.

It was agreed that VPS needs to be included in non-scalable bitstream.

General discussion on AU/POC/RPM-related

O0136 (proposes four options for POC/RPM in an order of signaling efficiency, all with spec text. Preference order expressed in the order of decreasing signalling efficiency, i.e., the one with the highest signalling efficiency is the most preferred)

|  | POC | RPL syntax | RPM |
| --- | --- | --- | --- |
| Approach 1 | Syntax and semantics not changed. Pictures in an access unit have the same POC value. | Changed to involve layer ID. | A reference picture may only be marked as "unused for reference" when decoding a picture of the same layer. |
| Approach 2 | Added a VPS syntax element. Pictures in an access unit have the same intra-layer POC values but different cross-layer POC values. | Not changed. | Same as approach 1. |
| Approach 3 | No syntax change. Pictures in an access unit have different POC values. | Not changed. | Same as approach 1. |
| Approach 4 | Same as approach 3. | Not changed. | A reference picture may be marked as "unused for reference" when decoding a picture of the same layer or a lower layer. |

Presentation:

Options: it is possible to disable scalability support using profiling.  It is similar possible to punt that disabling to constraint flags or application standards.

All approaches in descending order of syntax/spec impact and descending efficiency in signaling.

Approach 1.

Overhead: For each entry in the reference picture list there is one additional flag.  It is asserted that despite this change, approach 1 is still the most efficient approach of the four approaches of this doc (for multilayer obviously).  For single, no impact beyond a bit in the SPS.

In approach 1, spatial and temporal scalability can co-exist due to the basic design of RPS+RPR

It was remarked that approach 1 does not impose a significant implementation burden .  Pretty straightforwqard/easy.  Perhaps no undue burden

Approach 2.

No undue burden

Non-proponent prefers approach 1 (after presentation of approaches 1 and 2)

Approach 3.

No syntax change.

No undue burden

Approach 4:

Like approach 3, different reference picture marking

No undue burden

A non-proponent has a slight preference for either approach 2 ro 3 over approach 1.

A non-proponent prefers clean design, hence approach 1.

The “no undue burden” remark reflects primarily the POC/RPL and RPM aspects.  It is understood that additional syntax will be required even if the notes above indicate “no syntax change”.  It is also understood that for aspects such  as HRD, layer-dependcy signaling, sclabaility information signaling, etc. syntax additions are required.

It was remarked that the devil can be in the details, such as dirty random access (non-sychronized irap and so on).  Approach 1 seems to be the approach most aligned with the SHVC approach, where we have previous experience.

It was remarked that approach 3 or 4 allow POC to be different for different pics within an AU, hence less POC troubvle in the future.

A proponent of a different proposal prefers approach 4.

A lot of SHVC problems came from retrofitting scalability to non-scalabel version 1 in HEVC/SHVC.

O0243

Similar to approach 1, layer dependency in VPS

Does this layer dependency info affect decoding process? Yes.

Layer dependency signalling like SHVC

Two approaches, first similar to 3 and 4 (identical?)

Second approach similar to approach 1, two minor differences (1) layer index based on layer dependency signaling, (2) directly signal as long-term pictures

Similar in terms of undueness

O0333

Almost the same as approach #3 in JVET-O0136, with the only difference being adding a constraint that the POC value delta between two pictures of the same layer shall be the same for different AUs.

Different frame rate in different layers? There should be no problem for that.

Signaling of layer dependency in VPS, but not used for decoding process.

O0395 is close but not identical to the third approach for handling of POC, RPL, and RPM of O0136. O0395 can be summarized as follows:

- No change in the POC derivation process.

- Pictures within one CVS have different POC values and are identified by POC values only.

- The RPL signalling syntax remains unchanged.

- The RPM process remains unchanged.

  - O0395 presents a single-layer approach where "layers" are represented as sub-layers. Thus, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same or lower sub-layer.

  - O0395 could be combined with O0334 to use different NuhLayerId values. In this case, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same layer.

Single layer ID value, could be adapted to use different layer ID values as well.

Similar to approach 3.

Uses sub-layers as layers.  May want to move some bits from layer_id to sub-layer ID.

A different proponent asserted that spec languages changes are necessary beyond the spec text in the doc.  The proponent indicated that he continues to believe that no text change are required.

An asserted Advatage: HRD just works because this proposal is sub-layer based, temporal_id as sub-layer ID

Assignment of temporal ID

SEI for mapping of temporal ID to spatio-temporal layers

Following this approach, the layer_id would be relevant only for the (spatial) sub-layers, as having simulcasted several layers full screen may make little sense.

Different temporal bitstream extraction process would be needed that needs to understand the SEI.  SEI current not defined but asserted to be trivial.

“Output of the highest layer”

In some cases, output layer should be the highest only.  IN other cases, we need to output all (Multiview).  Output decided based on POC period.

It was agreed that support of outputting only the highest layer (for typical scalability) and support of outputting all layers (for multiview) are both needed.

It was asserted that a subset of output pictures is also useful certain scenarios.

The four proponents of POC related documents were requested to collaborate in creating unified proposals with as few alternatives as possible, for revisit in track/plenary. At least one unified proposal should be based on Approach 3, modified as required. Complete spec text was requested.

General discussion on layered random access

O0337 (conceptual)

Information. For further study.

O0471 (start picture alignment)

To track. See above.

General discussion on output of the highest layer only (for spatial and quality scalability)

Is it specified normatively that the decoder shall output only the highest layer (for spatial and quality scalability) or all layers (view scalability etc.)?

- Yes (O0395)

If yes, how to control the decoder output?

- For approaches where ppictures within one CVS have different POC values and are identified by POC values only:

  - O0395: poc_output_period_minus1 is added in the SPS syntax to specify a POC output period in terms of adjacent POC values. Pictures of the same POC output period represent the same capture time. Out of the decoded pictures of the same POC output period, the decoded pictures that precede the last decoded picture in decoding order are not output. poc_output_period_minus1 greater than 0 is used for spatial and quality scalability to avoid outputting more than one picture of the same capture time.

General discussion on temporal scalability, VUI

O0436 VUI, shutter angle

Should be in SEI message. Not VVC specific

Future study.

JVET-O0045 AHG8: Spatial Scalability using Reference Picture Resampling [S. Wenger, B. D. Choi, S. C. Han, X. Li, S. Liu (Tencent)]

See notes above.

JVET-O0135 AHG8: Scalability for VVC – general [Y.-K. Wang, Hendry, J. Chen (Futurewei)]

See notes above.

JVET-O0136 AHG8: Scalability for VVC - handling of POC, RPL, and RPM [Y.-K. Wang, Hendry, J. Chen (Futurewei)]

See notes above.

JVET-O0185 AHG8/AHG17: On layer dependency signaling [Y. He, A. Hamza (InterDigital)]

See notes above.

JVET-O0243 AHG8: On inter-layer reference for scalability support [V. Seregin, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

See notes above.

JVET-O0333 AHG8: On spatial scalability support with reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

See notes above.

JVET-O0335 AHG8: E-Sport Use cases of VVC region-wise scalability and its system support [B. Choi, S. Zhao, I. Sodagar, S. Wenger, S. Liu (Tencent)]

See notes above.

JVET-O0336 AHG8: Region-wise scalability support with reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

See notes above.

JVET-O0337 AHG8/AHG17: Layered Random Access point (LRA) [B. Choi, S. Wenger, S. Liu (Tencent)] [miss] [late]

See notes above.

JVET-O0395 AHG8: On adaptive resolution changing and scalable coding [M. M. Hannuksela, A. Aminlou (Nokia)]

Only the scalability part in the contribution belongs to this agenda item.

See notes above.

JVET-O0436 AHG8/AHG17: indication of shutter angle for variable frame rate application [S. McCarthy, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen (Dolby)]

See notes above.

JVET-O0471 AHG8/AHG12: Layer concepts clarifications and improvements for immersive media use cases [E. Thomas, A. Gabriel (TNO)]

See notes above.

JVET-O0473 AHG8: On HRD operations for layers [E. Thomas, A. Gabriel (TNO)] [miss] [late]

See notes above.