| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O01087-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-O0652 (CE4-related: Line buffer size reduction method for switchable interpolation filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Martin Winken | Email: | martin.winken@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The cross checker reports that the source code provided by Huawei was carefully analyzed and found to be consistent with the description in the input document JVET-O0652.  The cross checker could exactly reproduce the results reported in JVET-O0652 for the test on top of CE4-1.2 for the RA configuration and sequence classes C and D.  The measured encoder and decoder run time ratios are similar to those reported in JVET-O0652.  The variant on top of CE4-1.3 has not been run by the cross checker.

# Source Code Analysis

The source code was carefully analyzed. It is confirmed that the implementation matches the description.

# Simulation results

Simulations were conducted for Random Access and Low Delay B configurations and test sequences specified in the Common Test Conditions.  Only the variant of JVET-O0652 which is based on CE4-1.2 has been cross checked, the variant based in CE4-1.3 has not been cross checked.  The simulation results are provided in the accompanying Excel documents. A summary of the results is shown in the following tables.  The bit rates and PSNR values exactly match the results reported in JVET-O0652. Note that the run-times obtained in the cross check might not be reliable.

Table 1: Coding results on top of CE4-1.2

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | -0.03% | 102% | 102% |
| Class A2 | 0.03% | 0.06% | -0.03% | 101% | 100% |
| Class B | 0.01% | 0.06% | 0.08% | 101% | 101% |
| Class C | 0.03% | -0.01% | 0.02% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.02% | 101% | 101% |
| Class D | 0.01% | 0.02% | -0.02% | 102% | 102% |
| Class F | 0.00% | 0.02% | 0.04% | 101% | 99% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | 0.34% | 100% | 95% |
| Class C | 0.06% | 0.16% | 0.17% | 101% | 97% |
| Class E | 0.00% | 0.16% | 0.35% | 102% | 101% |
| Overall | 0.03% | -0.01% | 0.29% | 101% | 97% |
| Class D | 0.03% | 0.51% | -0.24% | 101% | 101% |
| Class F | 0.02% | 0.71% | 0.26% | 101% | 97% |

During crosschecking of CE4-1, minor software problem was noticed.

The following table shows the results with the bug-fix applied.

Table 2: Coding results on top of CE4-1.2 with bug-fix

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.2_withBugFix |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.05% | 100% | 102% |
| Class A2 | 0.02% | 0.00% | 0.01% | 100% | 101% |
| Class B | 0.05% | 0.16% | 0.15% | 100% | 100% |
| Class C | 0.03% | 0.03% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.05% | 100% | 100% |
| Class D | 0.00% | 0.03% | -0.06% | 100% | 100% |
| Class F | 0.01% | 0.01% | -0.01% | 100% | 103% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.2_bugFix |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.12% | 0.30% | 99% | 98% |
| Class C | 0.03% | -0.25% | -0.10% | 101% | 102% |
| Class E | 0.01% | -0.45% | 0.12% | 100% | 101% |
| Overall | 0.01% | -0.25% | 0.12% | 100% | 100% |
| Class D | 0.06% | -0.07% | -0.04% | 98% | 97% |
| Class F | 0.07% | 0.29% | 0.30% | 100% | 100% |