| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1093-v1 |
| --- | --- |

| Title: | Report of BoG on intra prediction and mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera | Email: | geertv@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met during following hours:

Thu 4 July: 14:30-19:45

# Notes

## MIP-related contributions

Overview of common aspects of MIP-related contributions:

- 8-bit MIP: proposed in JVET-O0084, JVET-O0481, JVET-O0925

- MIP MPM:

JVET-O0169, JVET-O0312, JVET-O0171, JVET-O0401: replace MIP MPM construction with mode index coded TB

JVET-O0170, JVET-O0255: simplify MIP MPM list to 3 default lists, one for each size index

JVET-O0469, JVET-O0255, JVET-O0598, JVET-O0171, JVET-O0202, JVET-O0396, JVET-O0170, JVET-O0793 (*), JVET-O0312, JVET-O0187(**): simplifies MIP by removing the mapping table from regular intra mode to MIP mode (default mode is used for each size index) for MIP MPM construction and also the mapping table from MIP mode to regular mode for non-MIP MPM list construction (use planar mode), and also DM chroma mode mapping is removed in case of MIP luma (use planar mode)

(*) this method has slightly different approach for MIP MPM construction

(**) suggests removing mapping table from MIP to regular mode, and DM mode

JVET-O0202, JVET-O0464: suggests aligning L and A of MIP MPM construction with regular MPM construction

JVET-O0755: signalling matrix parameters

JVET-O0160, JVET-O0276, JVET-0407, JVET-O0408, JVET-O0168: suggest to move clipping to subsampled prediction

JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0276: suggest to apply VVC reference sample construction process to MIP

JVET-O0261, JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0161, JVET-O0345, JVET-O0276: suggest using unfiltered reference sample for upsampling stage

JVET-O0571: suggests to use fixed order for upsampling

JVET-O0161: suggests downsampling filter change for alignment

JVET-O0848: suggests 2D upsampling

Approximate MIP matrices, or reduce number of matrices

JVET-O0523, JVET-O0139, JVET-O0397, JVET-O0187, JVET-O0793, JVET-O0621, JVET-O0547, JVET-O0203, JVET-O0345

JVET-O0084 8-bit implementation and simplification of MIP [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

At the 14th JVET meeting, held in Geneva from March 19th to March 27th 2019, matrix-based intra prediction (MIP) was adopted. Here, the coefficients of the involved matrices and offset vectors were stored in 10-bit precision. In this document, as requested at the Geneva Meeting, a version of MIP where the coefficients are stored in 8-bit precision is presented. In the course of this asserted simplification, the number of coefficients needed for MIP is reported to be reduced: The addition of the offset vector is removed and for the largest matrices, one column can be removed. As a result, the overall memory requirement of MIP is now 5.188 Kilobyte, both in hardware and software, whereas the MIP version in VTM-5.0 had a memory requirement of 7.92 Kilobyte.  	
Furthermore, two modifications of MIP are proposed in the present document; these are asserted to simplify the MIP design. First, it is proposed to remove the averaging step in the derivation of the boundary samples that are used for the linear interpolation of the reduced prediction signal and to remove here the extra handling of negative values in the rounding. Second, it is proposed to derive unavailable reference samples using the same padding method as the conventional intra prediction and, in the derivation of the most probable modes for MIP, it is proposed to treat a neighbouring CIIP-block exactly as neighbouring CIIP-blocks are treated in the derivation of the most probable modes of the conventional intra prediction. 	
The proposed changes of MIP were integrated into the VTM-5.0 software. No changes concerning the encoder control were made in comparison to VTM-5.0. The reported results for the presented 8-bit implementation of MIP are -0.05% luma BD-rate gain for the AI and -0.03% luma BD-rate gain for the RA configuration, compared to the VTM-5.0 anchor. The encoding times are reported to be 100%, the decoding times are reported to be 100% . The tool-off test results for the presented 8-bit version of MIP are reported to be 0.41% for AI and 0.30% for RA. The encoding times are reported to be 88% and 95 %, the decoding times are reported to be 101% and 102% .

JVET-O0139 CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP) [M. Salehifar, S. Kim (LGE)]

This contribution proposes several modification to the current MIP design in VTM5. Currently three different MIP index defined based on the block size (index 0: for W=H=4 index 1: for maxW,H=8, and index 2: formaxW,H>8). Also there are 19, 10, and 6 kernels which represent 35, 19, and 11 modes for each index 0,1, and 2 respectively. These MIP modes are divideded into MPM and non-MPM for signaling, where 3 MPM modes are coded as TU and non-MPM coded as fixed length code. This proposal suggests following modifications:

- Removing MIP for block size index 0 and 1.

- Reducing number of the kernels for MIP index 2 from 6 to 3.

- Encoder modification

- Removing the MPM derivation, and coding MIP mode with truncated binary code.

| Test # | Modification |
| --- | --- |
| Test 1 | a |
| Test 2 | a, c |
| Test 3 | a, b, c, d |

The reported result for test 1 is 0.08 %, 0.07 % and 0.04 % luma BD-rates differences with 94 %, 98 %, and 100 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 2 is 0.00 %, 0.02 % and 0.03 % luma BD-rates differences with 101 %, 101 %, and 101 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 3 is 0.03 %, 0.03 % and 0.05 % luma BD-rates differences with 98 %, 99 %, and 99 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~2 KB memory (75% less memory).

The proponent requests to study the aspects a and b in a core experiment. It was commented that the group needs to agree whether reducing the kernels is needed and that this may be due to the characteristics of the training set. If other content is tested the BD-rate loss may be different. It was further commented that hardware implementations are ok with the current memory use of MIP. If an experiment is defined then the reduced set of kernels would need to be retrained (not on CTC set). It was further commented that there does not seem to be a complexity issue with 4x4 blocks, for example.

BoG recommends further study outside CE. It is suggested to bring evidence on an independent test set to prove that the kernel reduction is general enough. It is also suggested in addition to CTC to study low-Qp range where 4x4 blocks will be used more.

JVET-O0721 Crosscheck of JVET-O0139 (CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP)) [T. Biatek (Qualcomm)]

JVET-O0957 Crosscheck of JVET-O0139 (CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP)) [J. Pfaff, B. Stallenberger (HHI)]

JVET-O0160 Non-CE3: Sample value clipping on MIP reduced prediction [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson)]

See comments JVET-O0925

JVET-O0961 Crosscheck of JVET-O0160 (Non-CE3: Sample value clipping on MIP reduced prediction) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0161 Non-CE3: Simplification on MIP boundary sample downsampling process [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson)]

This contribution presents a method to simplify the matrix based intra prediction (MIP) boundary sample downsampling process when the current block is predicted by MIP. The proposed method 1) enable parallel derivation of the reduced boundary for matrix multiplication and the reduced boundary for interpolation; 2) reduced the number of samples to be averaged to derive one reduced boundary sample. For the first aspect, the proposed method derives the reduced boundary for interpolation without averaging. For the second aspect, the proposed method simplifies the derivation of reduced boundary for matrix multiplication by averaging two original boundary samples to derive one reduced boundary sample. The proposed method has a luma BDR impact for AI / RA / LDB of -0.01% / -0.01% / 0.01% compared to VTM5.0 and decoding time impact for AI / RA / LDB of 100% / 100% /100% compared to VTM5.0.

It was commented that from the experiment it seems that the averaging of for example eight or sixteen samples does not differ much from the suggested two samples, but that averaging more samples may be more stable. In this case, retraining may be necessary. It suggested that another filter may be more appropriate.

In case a core experiment is established for studying MIP related methods, the BoG recommends to include this method (aspect 2). A variation of the downsampling filtering could be studied as well.

JVET-O0960 Crosscheck of JVET-O0161 (Non-CE3: Simplification on MIP boundary sample downsampling process) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0168 Non-CE3: MIP low resolution clipping [T. Biatek, G. Van der Auwera, M. Karczewicz (Qualcomm)]

See comments JVET-O0925

JVET-O0882 Crosscheck of JVET-O0168 (Non-CE3: MIP low resolution clipping) [Z. Zhang (Ericsson)]

JVET-O0169 CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP) [M. Salehifar, S. Kim (LGE)]

See comments JVET-O0925

JVET-O0722 Crosscheck of JVET-O0169, (CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP)) [T. Biatek (Qualcomm)]

JVET-O0958 Crosscheck of JEVT-O169 (CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP)) [J. Pfaff, B. Stallenberger (HHI)]

JVET-O0170 Non-CE3: Simplification on Matrix-based intra prediction [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, E. Alshina (Huawei)]

See comments JVET-O0925

JVET-O0883 Crosscheck of JVET-O0170 (Non-CE3: Simplifications of Intra Mode Coding for Matrix-based Intra Prediction) [Z. Zhang (Ericsson)]

JVET-O0171 Non-CE3: Simplifications for MIP [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

See comments JVET-O0925

JVET-O0850 Crosscheck of JVET-O0171 (Non-CE3: Simplifications for MIP) [Jie Yao (Fujitsu)]

JVET-O0943 Crosscheck of JVET-O0171 (Non-CE3: Simplifications for MIP) [T.-C. Ma (Kwai Inc.)]

JVET-O0187 Non-CE3: MIP simplifications [A. K. Ramasubramonian, G. Van der Auwera, T. Biatek, L. Pham van, M. Karczewicz (Qualcomm)]

See comments JVET-O0925

JVET-O0202 Non-CE3: MIP Modifications [Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

This contribution proposes two modifications of matrix-weighted intra prediction (MIP). The first modification is changing the positions of the neighboring blocks to be referred deriving MIP MPM. The second modification is removing inverse mapping table which maps MIP matrix modes to regular intra modes. This contribution reports the performances on the two tests below:

- Test 1: modified positions for deriving MIP MPM candidates

- Test 2: remove inverse mapping table that assigns MIP modes to regular intra prediction modes

The experiment results for test 1 reportedly show 0.00% and 0.XX% luma BD-rates performance with 101% and, XX% encoding time for the AI and RA, respectively.

The experiment results for test 2 reportedly show 0.01% and 0.XX% luma BD-rates performance with 99% and, XX% encoding time for the AI and RA, respectively.

Aspect 1 is not considered because it is understood that the MIP MPM construction can be removed and replaced with mode index coding using TB. For aspect 2 see comments under JVET-O0925.

JVET-O1024 Crosscheck of JVET-O0202 (Non-CE3: MIP Modifications) [Z.-Y. Lin (MediaTek)]

JVET-O0203 CE3-related: Simplification of Matrix-Based Intra Prediction (MIP) [A. Filippov, V. Rufitskiy, B. Wang, S. Esenlik, E. Alshina (Huawei)]

In this document, we propose a set of mechanisms for reducing computational complexity and memory consumption of Matrix-based Intra Prediction (MIP). In Test 1, we align the width of MIP multipliers with the 16-bit multipliers used for interpolation filtering in the conventional intra prediction mechanism. In addition to reducing the multiplier width, multiplication by the MIP coefficients used in Test 1 can be implemented using add-and-shift technique (with no more than either 1 addition or 1 subtraction operation). Test 1 reportedly shows the following overall results over VTM-5.0: 0.03% (Y), -0.02% (U), 0.03% (V) with encoding time of 100% and decoding time of 100% for AI configuration, 0.02% (Y), -0.01% (U), 0.00% (V) with encoding time of 99% and decoding time of 100% for RA configuration. Besides, we test MIP with 6-bit multipliers on top of JVET-O0084. This test (Test 2) reportedly shows the following overall results over VTM-5.0: -0.05% (Y), -0.10% (U), -0.09% (V) with encoding time of 100% and decoding time of 99% for AI configuration, -0.03% (Y), -0.17% (U), -0.07% (V) with encoding time of 100% and decoding time of 99% for RA configuration.

The method is tested on VTM5, JVET-O0084, JVET-O0481.

It is commented that the matrix coefficients are quantized by the method.

The proponent mentions that there are two implementations, either storing approximated matrices or computing approximation on the fly. In addition, there is a multiplication free method proposed which is tested with special tuning on VTM5. The other results are obtained with multiplications and without tuning.

A question is raised that the input samples could be downshifted to obtain 16-bit multiplications.

It is commented that the benefit will be larger if the method is applied on 10-bit coefficients as opposed to 8 bit. It may also be better to test such a method after stability of the MIP method is achieved.

BoG recommends to further study the method outside of CE. Additional feedback from hardware experts may be requested to get feedback on the importance of the method.

JVET-O0811 Crosscheck of JVET-O0203 (CE3-related: Simplification of Matrix-based Intra Prediction (MIP)) [T.-H. Li, J.-Y. Deng (Foxconn)]

JVET-O0255 Non-CE3: Fixed MPMs for MIP [K. Zhang, Z. Deng, L. Zhang, H. Liu, N. Zhang (Bytedance)]

It is understood that the MIP MPM may be replaced by mode index coded with TB (see comments under JVET-O0925). Therefore, the BOG recommends no action on the fixed MPM part.

For other aspects (removal of mapping tables) see comments under JVET-O0925.

JVET-O1005 Crosscheck of JVET-O0255 (Non-CE3: Fixed MPMs for MIP) [J. Choi (LGE)]

JVET-O0261 Non-CE3: MIP with one-stage boundary sample downsampling [Z. Deng, H. Liu, K. Zhang, L. Zhang, Y. Wang (Bytedance)]

See comments under JVET-O0925

JVET-O0826 Crosscheck of JVET-O0261 (Non-CE3: MIP with one-stage boundary sample downsampling) [H. Chen (Huawei)]

JVET-O0276 CE3-related: Simplifications for MIP [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

See comments under JVET-O0925

JVET-O0827 Crosscheck of JVET-O0276 (CE3-related: Simplifications for MIP) [H. Chen (Huawei)]

JVET-O0312 Non-CE3: Simplifications of MIP [P.-H. Lin (Foxconn)]

See comments under JVET-O0925

JVET-O1001 Crosscheck of JVET-O0312 (Non-CE3: Simplifications of MIP) [C.-C. Kuo, C.-H. YauLin, C.-L. Lin (ITRI)]

JVET-O0321 Non-CE3: Removal of MIP mapping table [J.-Y. Huo, Y.-Z. Ma, X.-W. Li, H.-X. Wang, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

See comments under JVET-O0925

JVET-O1007 Crosscheck of JVET-O0321 (Non-CE3: Removal of MIP mapping table) [J. Choi (LGE)]

JVET-O0323 Non-CE3: Unification of shifting for MIP mode [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), H.-X. Wang, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

This document proposes modification of matrix-based intra prediction (MIP). In current VVC, the weight shift may be equal to 8 or 9. This contribution unifies weight shift into 8 in MIP prediction calculation.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM5.0:

For AI configuration: 0.00%, -0.05%, and -0.02%, with 100% EncT, 100% DecT;

For RA configuration: 0.00%, -0.06%, and -0.09%, with 100% EncT, 100% DecT;

Benefit is that lookup table for sW is not needed.

It is commented that in the 8-bit MIP method the downshift is 6.

In case a core experiment is established, the BoG recommends studying this method with the 8-bit MIP algorithm.

It is commented that JVET-O0408 proposes the identical method.

JVET-O0717 Crosscheck of JVET-O0323 (CE3-related: Unification of shifting for MIP mode) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0324 Non-CE3: MIP Simplification [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

See comments JVET-O0139.

JVET-O0718 Crosscheck of JVET-O0324 (Non-CE3: MIP Simplification) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0345 Non-CE3: MIP simplification [C.-H. Yau, C.-C. Lin, C.-L. Lin (ITRI)]

Method 1: removal of 4x4 kernel (see comments under JVET-O0139)

Method 2: use unfiltered samples in upsampling stage (see comments JVET-O0925)

JVET-O0971 Crosscheck of JVET-O0345 (Non-CE3: MIP simplification) [M.-S. Chiang (MediaTek)]

JVET-O0396 Non-CE3: Simplified mode mapping in MIP [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

See comments JVET-O0925.

JVET-O0750 Crosscheck of JVET-O0396 (Non-CE3: Simplified mode mapping in MIP) [?? (??)]

JVET-O0397 Non-CE3: MIP mode reduction [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

In matrix intra prediction (MIP), the block sizes are classified into three categories and each category supports 35/19/11 modes, and it requires 18/10/6 matrices, respectively.This contribution proposes to reduce the number of modes in MIP. The proposed method reduces the number of MIP modes and matrices as follows:

- Method 1: each category uses 11/11/11 modes, and it requires 6/6/6 matrices.

- Method 2: each category uses 6/7/8 modes, and it requires 3/4/4 matrices

With the method 1, the memory of MIP matrix is reduced to 75.7% with negligible BD-rate change. In the method 2, the memory of MIP matrix is reduced to 49.2%, compared to the VTM5.0.

Experimental results for Method 1 show 0.01%, -0.03%, and 0.00% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and Method 2 shows 0.04%, -0.03%, and -0.03% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

BoG recommends further study outside CE. It is suggested to bring evidence on an independent test set to prove that the kernel reduction is general enough. It is also suggested in addition to CTC to study low-Qp range where 4x4 blocks will be used more.

JVET-O0918 Crosscheck of JVET-O0397 (Non-CE3: MIP mode reduction) [K. Zhang (Bytedance)]

JVET-O0401 Non-CE3: MIP mode simplifications [C. Rosewarne, J. Gan (Canon)]

4x4 removal: see comments JVET-O0139

MIP MPM table removal: see comments JVET-O0925

JVET-O0407 Non-CE3: On adaptive rounding offset of MIP [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

See comments JVET-O0925

JVET-O1006 Crosscheck of JVET-O0407 (Non-CE3: On adaptive rounding offset of MIP) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0408 Non-CE3: On rounding shift of MIP [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

See comments JVET-O0323

JVET-O1011 Crosscheck of JVET-O0408 (Non-CE3: On rounding shift of MIP) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0469 Non-CE3: Simplification of MIP MPM Derivation [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

Method 1: use planar mode in case MIP is neigbouring block in regular MPM (see JVET-O0925)

Method 2: remove mapping table regular intra mode to MIP for MIP MPM construction (see JVET-O0925)

JVET-O0481 Variations of the 8-bit implementation of MIP [P. Helle, J. Pfaff, T. Hinz, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

At the 14th JVET meeting, held in Geneva from March 19th to March 27th 2019, matrix-based intra prediction (MIP) was adopted. Here, the coefficients of the involved matrices and offset vectors were stored in 10-bit precision. At the Geneva Meeting, it was requested to present a version of MIP where the coefficients are stored in 8-bit precision. In JVET-O0084, such an 8-bit version is proposed. Responding to feedback that was given to the proponents of JVET-O0084, in the present document, two modifications of the 8-bit implementation of JVET-O0084 are proposed. The first proposed modification is the 7-bit version of JVET-O0084. If one implements this proposed method as an 8-bit method, the matrix offsets fW, introduced in JVET-O0084, can be removed. The second proposed modification is proposed on top of the first proposed modification, namely replacing the mean value of the reduced boundary in the algorithm of JVET-O0084 by the first entry of the reduced boundary. Therefore, in comparison to JVET-O0084, the computation of the mean value of the reduced boundary vector can be avoided. Note, that both proposed methods as well as the method of JVET-O0084 are different 8-bit (resp. 7-bit) integer realizations of the same floating point MIP-models. Moreover, both proposed methods are combined with the MIP design simplifications proposed in section 2 of JVET-O0084. The reported results of the first proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration for the luma-component. The tool-off test for MIP of the first proposed method is 0.4% in the AI and 0.3% in the RA configuration. The reported results of the second proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration. The tool-off test for MIP of the second proposed method is 0.4% in the AI and 0.3% in the RA configuration.

It was commented by the cross-checker that the cleanups proposed do not have any BD-rate impact, therefore, the change in integer representation has minor gain -0.04% (AI), -0.03% (RA).

Blocks 16x16 and larger, matrix size 8x64 changed to 7x64; from SW memory access point of view there is little benefit of this change.

Regarding DC value computation in JVET-O0084, using one value in JVET-O0481 is preferable.

JVET-O0485 Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

No need to present. Included in JVET-O0925.

JVET-O0885 Crosscheck of JVET-O0485 (Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST) [Z. Zhang (Ericsson)]

JVET-O0486 Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

See comments JVET-O0139

JVET-O0886 Crosscheck of JVET-O0486 (Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes) [Z. Zhang (Ericsson)]

JVET-O0514 Non-CE3: Analysis of Matrix-based Intra Prediction (Informative) [S. Keating, K. Sharman (Sony)]

Informational. No need to present (proponent).

JVET-O0523 CE3-related: simplification of Matrix Intra Prediction [T. Dumas, F. Galpin, G. Rath, F. Racapé (InterDigital)]

This contribution reports the result of adding one Matrix-based intra prediction mode per block size, called Simplified Matrix Intra Prediction (SMIP) mode, in addition to the classical intra modes. Compared to the Matrix-based Intra Prediction (MIP) implemented in VTM-5.0, the memory footprint is reduced from~8 kilobytes to 1 kilobyte. In addition, the decoding process does not require any extra MPM list derivation anymore, nor any mapping table between conventional modes and MIP modes. A loss of 0.14% in AI, and a loss of 0.10% in RA is reported for the luminance, when the proposed mode is not combined with MTS.

Proponent has retrained matrices for the proposed reduction of kernels. Aspects related MIP MPM (table removal) see comments JVET-O0925. Other simplification aspects: see comments JVET-O0139.

JVET-O0547 Non-CE: Modified MIP Matrices for Fast Implementation [K. Naser, L. Kerofsky, F. Le Léannec, T. Poirier (InterDigital)]

Recently, MIP was adopted to VTM5.0 as an additional prediction mode that is based on partial multiplication of the reference samples by a prediction matrix. Unlike the ordinary intra prediction mode, MIP requires matrix-vector multiplication that can be time consuming.

This contribution proposes a slight modification to the MIP matrices so that faster implementation can be achieved. The modification allows dual implementation support, in which an MIP transform can be either implemented by matrix multiplications or fast algorithm to yield exactly the same results.

Experimental results show some negligible RD loss, while the number of multiplications is significantly reduced to 18% and 7% of the original number.

Implemented on JVET-O0084.

It was commented that the formula for approximation changes row by row of the matrix, which would need to be implemented without reuse. Proponent asserts that this is similar to a proposal by the proponent for DCT-8 implementation. However, in the DCT-8 case the data is more regular.

It is commented that it is more appropriate to have such a method after MIP kernels have stabilised.

BoG recommends to further study the method outside of CE.

JVET-O0962 Crosscheck of JVET-O0547 (Non-CE: Modified MIP Matrices for Fast Implementation) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0571 Non-CE3: Up-sampling with a fixed order in MIP [H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance)]

When the reduced prediction signal in MIP needs to be up-sampled in both directions, the up-sampling is always firstly performed along the shorter side of a CU. Afterwards, the up-sampling along the longer side is performed. Therefore, the up-sampling order in MIP varies with different CUs. This contribution proposes to perform the up-sampling in a fixed up-sampling order for the prediction signal up-sampling process in MIP. Simulation results reportedly show that the proposed method has negligible impact on the coding performance.

It is commented that both upsampling processes are implemented and, therefore, the benefit of having the same upsampling order is rather a cleanup of the algorithm.

In case a core experiment is established, the BoG recommends studying the method on the 8-bit MIP algorithm.

JVET-O0843 Crosscheck of JVET-O0571: Non-CE3: Up-sampling with a fixed order in MIP [J. Chen (Alibaba)]

JVET-O0598 CE3-related: Simplification on MIP Look-up Table [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

See comments JVET-O0925 (removal of mapping table)

JVET-O0693 Crosscheck of JVET-O0598 (CE3-related: Simplification on MIP Look-up Table) [L. Zhao (Tencent)]

JVET-O0599 CE3-related: Simplification on MIP flag context model [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

This contribution proposes to reduce context model from 3 to 1 and remove selection logic when signaling 1-bit MIP flag. In this proposal, two tests and results are provided as following:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

It is commented that context reduction is better performed at a later stage of the VVC standardization when MIP algorithm stability has been achieved. BoG recommends no action at this stage.

JVET-O0815 Crosscheck of JVET-O0599 (CE3-related: Simplification on MIP flag context model) [J. Heo, L. Li (LGE)]

JVET-O0602 Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution proposes to reduces the line buffer of Matrix Intra Prediction (MIP)/Intra Block Copy (IBC) mode flag storage. For MIP mode flag storage, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. The proposed method is compared with both VTM5 JVET-O0084, with simulation results as follows:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are 0.00%, 0.02%, and 0.02% for AI, RA and LD, respectively, without increasing encoding and decoding time.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are -0.05%, -0.03%, and -0.01% for AI, RA and LD, respectively, without increasing encoding and decoding time.

For IBC mode flag storage, the context selection for the CABAC coding of IBC flag is not depending on the IBC flag of above CU when the above CU is not located in the same CTU as current CU. Furthermore, the above neighbor candidate for IBC AMVP/merge/skip mode prediction is not used when the above CU is not in the same CTU as current CU. Compared to CE8 anchors, for Class SCC sequences, The BD-rate results are 0.11%, 0.1%, and 0.05% for AI, RA and LD, respectively, without increasing encoding and decoding time.

It is commented that there are many other syntax elements that access line buffer memory for flag retrieval and that it is appropriate to study the proposed cases together with other cases in a comprehensive proposal at a later stage of standardization of VVC (BoG recommendation).

JVET-O0813 Crosscheck of JVET-O0602 (Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer) [T.-H. Li, J.-Y. Deng (Foxconn)]

JVET-O0621 Non-CE3: MIP simplification [Y. Yasugi, T. Ikai (Sharp)] [miss] [late]

See comments JVET-O0139

JVET-O0855 Crosscheck of JVET-O0621 (Non-CE3: MIP simplification) [Z. Deng (Bytedance)]

JVET-O0755 Non-CE3: On signalling of MIP parameters [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, T. Biatek, L. Pham Van, M. Karczewicz (Qualcomm)] [late]

To be presented.

JVET-O0793 CE3-related: MIP with simplified mode index coding [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

In this proposal, three methods are proposed to simplify the VTM5.0 matrix-based intra prediction (MIP). Method 1 removes mapping between non-MIP and MIP modes and uses power-of-2 modes in each MIP group. To remove the non-MIP-to-MIP mapping table, the MPMs in MIP mode are removed, and MIP mode indices are coded using fixed length binarization without using any context variables. To remove MIP-to-non-MIP mapping table, during the non-MIP MPMs derivation and the direct mode (DM) derivation of a chroma intra block, MIP mode index is mapped to planar mode index. To efficiently utilize the fixed length binarization codewords, only power-of-2 modes are used in each MIP group. The number of MIP modes in the three MIP groups are reduced from (35, 19, 11) to (32, 16, 8). Based on method 1, method 2 further simplifies the matrix index derivation by changing the composition of the MIP mode index from “matrixIndex + (numberOfMode/2)*isTranspose” to “2*matrixIndex + isTranspose.” Therefore, the matrix index can be easily derived by right shifting the MIP mode index by one bit, and whether the MIP mode is transposed mode is derived by checking the parity bit of the MIP mode index. Method 3 further simplifies method 2 by reducing the numbers of MIP modes in the three MIP mode groups from (32, 16, 8) to (8, 8, 8).

Test results of method 2 are reported as follows.
VTM5.0-AI: {BD-rates = 0.03% (Y), -0.03% (Cb), 0.00% (Cr); EncT = 98%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.02% (Y), -0.08% (Cb), 0.09% (Cr); EncT = 99%; DecT = 99%}
VTM5.0-LB: {BD-rates = -0.01% (Y), 0.32% (Cb), 0.15% (Cr); EncT = 99%; DecT = 99%}

Test results of method 3 are reported as follows.
VTM5.0-AI: {BD-rates = 0.05% (Y), -0.01% (Cb), 0.02% (Cr); EncT = 97%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.04% (Y), 0.02% (Cb), 0.03% (Cr); EncT = 99%; DecT = 97%}
VTM5.0-LB: {BD-rates = 0.06% (Y), 0.28% (Cb), 0.10% (Cr); EncT = 99%; DecT = 99%}

Regarding MIP MPM construction removal (table mappings): see comments JVET-O0925.

Regarding reduction of kernels: see comments JVET-O0139.

Regarding MIP mode index coding with fixed length coding, the BoG recommends to study the method in a core experiment (if established).

JVET-O0848 Non CE3: Modification of MIP interpolation [B.-J. Fuh, C.-H. Yau, C.-C. Lin, C.-C. Kuo, C.-L. Lin (ITRI)] [late]

This contribution proposed the modified interpolation of MIP. The 1D linear interpolation is replaced by a 2D linear interpolation by an interpolation unit to improve the flexibility for the hardware and reduce the interpolation process latency. The results of the proposed method show 0.00%(Y), -0.01%(U) and -0.01%(V) coding gain.

It is commented that the number of operations is increased. It is further commented that with current separable interpolation a similar benefit can be achieved if smaller areas are interpolated consecutively. It is further commented that a similar argument holds for other separable processing.

The BoG recommends to further study the method outside of CE and it is recommended to request feedback on the latency versus complexity trade-off from, for example, hardware experts.

JVET-O0925 Non-CE3: Simplifications of MIP [J. Pfaff, P. Merkle, P. Helle, H. Schwarz, D. Marpe, T. Wiegand (HHI), A. K. Ramasubramonian, T. Biatek, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm), J. Choi, J. Heo, J. Lim, M. Salehifar, S. Kim (LGE), K. Kondo, M. Ikeda, T. Suzuki (Sony), Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten (Ericsson)] [late]

It proposes to adopt the following changes of MIP:

- MIP is implemented in 8 bit, as proposed in JVET-O0481, Method 2.

- The reference sample derivation for MIP is performed exactly as for the conventional intra prediction modes. The identical method is proposed in JVET-O0187 (proposal 3), JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0276.

- For the upsampling step used in the MIP-prediction, original reference samples are used instead of downsampled ones. The identical method is proposed in JVET-O0161 (aspect 1), JVET-O0261, JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0345, JVET-O0276.

- The extra handling of negative values is removed from the upsampling. The identical method is proposed  in JVET-O0160,  JVET-O0407, JVET-O0084, JVET-O0481, JVET-O0485.

- Clipping is performed before upsampling and not after upsampling. The identical method is proposed in JVET-O0160, JVET-O0168, JVET-O0276,  .

- The mapping tables from MIP modes to conventional intra prediction modes are removed. Instead, MIP modes are always mapped to the planar mode. Method is also proposed in JVET-O0187 ( Proposal 1), JVET-O0396 (Method 1a), JVET-O0469, JVET-O0255, JVET-O0598, JVET-O0171, JVET-O0202, JVET-O0396, JVET-O0170, JVET-O0793 (*), JVET-O0312, JVET-O0187(**), JVET-O0321:

  - (*) this method has slightly different approach for MIP MPM construction

  - (**) suggests removing mapping table from MIP to regular mode, and DM mode

- For the coding of the MIP-modes, the MIP-MPMs are no longer used and the mapping from conventional intra prediction modes to MIP modes is removed. Instead, the MIP modes are coded using truncated binary code. The same method is proposed in JVET-O0169, JVET-O0312, JVET-O0171, JVET-O0401.

- MIP is harmonized with unified MPM-coding: In the list of the five non-planar MPMs, the DC mode is replaced with an angular mode if one of the neighboring modes is angular, as proposed in JVET-O0485.

- LFNST is enabled for MIP on large blocks. Here, the LFNST transforms corresponding to the planar mode are used, as proposed in JVET-O0485.

It was commented that 8-bit MIP change has 0.04% gain, UMPM change has some gain also, but overall there appears to be an element that has some loss. This is reportedly due to the encoder speedup (7% reduction for AI).

It was commented that an adopted LFNST proposal may reduce the gain from applying the LFNST to MIP large blocks.

A question was raised about the sequence set for training the new matrices. The proponent shared the training code, but did not share the sequences. Proponent of JVET-O0523 is willing to also share a training set. For future contributions, it would be useful to have such a set available that is different from the CTC sequences.

It was commented that the cross-check is pending as well as some results in the proposal are missing.

The proponent asserts that the draft spec text is finalized and can be studied.

The proponent of JVET-O0203 suggests that 16-bit implementation of multiplications is desirable and agreed by the proponent and a non-proponent. This means that the result of the multiplication is in the 16-bit range after clipping and before downshifting.

It was commented that ‘MIP harmonization with unified MPM-coding’ affects regular intra mode MPM construction and is not necessarily related to MIP.

BoG recommends adoption of this contribution, subject to the finalization of the cross-check, completion of results, studying of software and working draft text, review of other proposals on this topic.

## Intra mode coding

JVET-O0111 Non-CE3: Modified CABAC Coding for Intra Prediction Mode [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, E. Alshina (Huawei)]

JVET-O1003 Crosscheck of JVET-O0111 (Non-CE3: Modified CABAC Coding for Intra Prediction Mode) [L. Li (LGE)]

JVET-O0192 Non-CE3: MPM flag first signaling for intra luma mode coding [Jie Yao, Jianqing Zhu, Wenting Cai, Kimihiko Kazui (Fujitsu)]

JVET-O0768 Crosscheck of JVET-O0192: Non-CE3: MPM flag first signaling for intra luma mode coding [L. Pham Van, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0197 Non-CE3: cleanup for ISP flag and MRL index signalling [H. Liu, L. Zhang, K. Zhang, N. Zhang, J. Xu, Y. Wang (Bytedance)]

JVET-O0851 Crosscheck of JVET-O0197 (Non-CE3: Cleanup for ISP flag and MRL index signalling) [Jie Yao (Fujitsu)]

JVET-O0224 CE3-related: Further simplifications of the unified MPM list for intra mode coding [G. Rath, F. Racapé, F. Urban (InterDigital)]

JVET-O0968 Crosscheck of JVET-O0224 (Non-CE3: Further simplifications of the unified MPM list for intra mode coding) [S. De-Luxán-Hernández (HHI)]

JVET-O0275 CE3-related: Chroma DM derivation for IBC and PCM [M.-S. Chiang, O. Chubach, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0823 Crosscheck of JVET-O0275 (CE3-related: Chroma DM derivation for IBC and PCM) [S. Yoo, J. Lim (LGE)]

JVET-O0363 CE3-related: Context reduction for intra MPM related syntax element [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-O0816 Crosscheck of JVET-O0363 (CE3-related: Context reduction for intra MPM related syntax element) [T. Ikai (Sharp)]

JVET-O0369 CE3-related: Context reduction for ISP related syntax element [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-O0817 Crosscheck of JVET-O0369 (CE3-related: Context reduction for ISP related syntax element) [T. Ikai (Sharp)]

JVET-O0377 CE3-related: Context reduction for CCLM syntax element [L. Xu, X. Cao, F. Chen (Hikvision)]

JVET-O0818 Crosscheck of JVET-O0377 (CE3-related: Context reduction for CCLM syntax element) [T. Ikai (Sharp)]

JVET-O0416 Non-CE3: Removal of CCLM context derivation dependency [J. Q. Zhang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

JVET-O0841 Crosscheck of JVET-O0416: Non-CE3: Removal of CCLM context derivation dependency [J. Chen (Alibaba)]

JVET-O0464 Non-CE3: Unification of Neighbouring Location for MPM List Construction [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-O0467 Non-CE3: simplified MRL and ISP mode coding [F. Le Léannec, Y. Chen, T. Poirier (InterDigital)]

JVET-O0856 Crosscheck of JVET-O0467 (Non-CE3: simplified MRL and ISP mode coding) [H. Liu (Bytedance)]

JVET-O0477 Non-CE3: Simplified entropy coding for intra prediction mode [D. Park, Y.-U. Yoon, J. Do, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

JVET-O1009 Crosscheck of JVET-O0477 (Non-CE3: Simplified entropy coding for intra prediction mode) [J. Choi (LGE)]

JVET-O0484 CE3-related: Improved intra mode coding based on unified MPM list generation [J. Pfaff, T. Hinz, P. Merkle, P. Helle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI), B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-O0938 Crosscheck of JVET-O0484 (CE3-related: Improved intra mode coding based on unified MPM list generation) [M. Salehifar (LGE)]

JVET-O0502 Non-CE3: Proposed ISP cleanup [S. De-Luxán-Hernández, V. George, G. Venugopal, J. Brandenburg, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-O0944 Crosscheck of JVET-O0502 (Non-CE3: Proposed ISP cleanup) [T.-C. Ma (Kwai Inc.)]

JVET-O0509 Non-CE3: MPM flag signaling first [D. Y. Lee, W. Gwun, W. Lee, T. H. Kim, G. H. Park (KHU), J. Lee, J. Kang (ETRI)]

JVET-O0512 Non-CE3: CCLM Coding bug fix [S. Keating, K. Sharman (Sony)]

JVET-O0716 Crosscheck of JVET-O0512 (Non-CE3: CCLM Coding bug fix) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0515 Non CE3: Intra mode coding of non-MPM modes [S. Blasi, G. Kulupana, A. Seixas Dias (BBC)]

JVET-O0606 Non-CE3: Context Reduction for MPM Signaling [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-O1020 Crosscheck of JVET-O0606 (Non-CE3: Context Reduction for MPM Signaling) [?? (??)]

JVET-O0647 Non-CE3: On signalling of intra coded blocks [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)]

JVET-O0651 Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0694 Crosscheck of JVET-O0651 (Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma) [L. Zhao (Tencent)]

JVET-O0655 Non-CE3: On chroma DM derivation table for 4:2:2 chroma format [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-O0687 Crosscheck of JVET-O0655 (Non-CE3: On chroma DM derivation table for 4:2:2 chroma format) [T. Chujoh (Sharp)]

JVET-O0671 Non-CE3: Intra chroma mode coding simplification [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0748 Non-CE3: On signalling for intra coding mode in the MPM list [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)] [late]

## Intra prediction

JVET-O0229 CE3-related: Simplifications of PDPC for diagonal and adjacent diagonal directions [G. Rath, F. Racapé, F. Urban (InterDigital)]

JVET-O0533 Crosscheck of JVET-O0229 (CE3-related: Simplifications of PDPC for diagonal and adjacent diagonal directions)) [L. Zhao (Tencent)] [miss] [late]

JVET-O0230 CE3-related: Interpolation filtering for intra prediction in non-diagonal directions [G. Rath, F. Racapé, F. Urban, F. Le Léannec (InterDigital)]

JVET-O1030 Crosscheck of JVET-O0230 (Non-CE3: Interpolation filtering for intra prediction in non-diagonal directions) [J. Lee, S.-C. Lim, H. Lee, J. Kang (ETRI)]

JVET-O0260 Non-CE3: Rounding offset removal for intra prediction [Z. Deng, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-O0862 Cross-check of JVET-O0260: "Non-CE3: Rounding offset removal for intra prediction" [F. Le Léannec (Interdigital)]

JVET-O0277 CE3-related: Simplification and unification for intra reference sample filtering [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0825 Crosscheck of JVET-O0277 (CE3-related: Simplification and unification for intra reference sample filtering) [J. Shingala (Ittiam)]

JVET-O0311 CE3-related: Intra sub-partition improvement for low decoding latency and hardware implementation efficiency [H. Yang, Y. He (InterDigital)]

JVET-O0974 Crosscheck of JVET-O0311 (CE3-related: Intra sub-partition improvement for low decoding latency and hardware implementation efficiency) [L. Zhao (Tencent)]

JVET-O0316 Non-CE3：Unification of ISP and Non-ISP [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0317 CE3-related: MPM List Modification for Zero Reference Line [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0318 Non-CE3: Intra Combined predict [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0320 Non-CE3: CCLM simplification [J.-Y. Huo, Y.-Z. Ma, X.-W. Li, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

JVET-O0978 Crosscheck of JVET-O0320 (Non-CE3: CCLM simplification) [X. Ma (Huawei)]

JVET-O0322 Non-CE3: Simplification of PDPC for chroma component [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

JVET-O0980 Crosscheck of JVET-O0322 (Non-CE3: Simplification of PDPC for chroma component) [X. Ma (Huawei)]

JVET-O0341 Non-CE3: Unification of intra interpolation filter selection [J. Lee, J. Kang, H. Lee, S.-C. Lim, H. Y. Kim (ETRI)]

JVET-O0765 Crosscheck of JVET-O0341: Non-CE3: Unification of intra interpolation filter selection [F. Racapé, G. Rath (Interdigital)]

JVET-O0343 CE3-related: Simplification on CCLM process [D.-Y. Kim, J.-O. Lee (Chips&Media), S.-C. Lim, J. Lee, J. Kang (ETRI)]

JVET-O0845 Crosscheck of JVET-O0343 (CE3-related: Simplification on CCLM process) [M. Ikeda (Sony)]

JVET-O0351 Non-CE3: Clean-up intra type signaling [J. Heo, J. Nam, J. Choi, J. Lim, S. Kim (LGE)]

JVET-O0353 Non-CE3: Simplification on transform selection for multiple reference line (MRL) [J. Heo, M. Koo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-O0707 Crosscheck of JVET-O0353 (Non-CE3: Simplification on transform selection for multiple reference line (MRL)) [L. Zhao (Tencent)]

JVET-O0355 Non-CE3: MRL restriction [J. Heo, J. Choi, H. Jang, J. Lim, S. Kim (LGE)]

JVET-O0942 Crosscheck of JVET-O0355 (Non-CE3: MRL restriction) [T.-C. Ma (Kwai Inc.)]

JVET-O0356 Non-CE3: Restriction on WAIP for MRL and ISP intra prediction [J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-O0836 Crosscheck of JVET-O0356 (Non-CE3: Restriction on WAIP for MRL and ISP intra prediction) [A. Tamse (Samsung)]

JVET-O0364 On general intra sample prediction [F. Bossen (Sharp)]

JVET-O0365 CE3-related: Simplification of Multi-reference line prediction scheme [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-O0749 Crosscheck of JVET-O0365 (CE3-related: Simplification of Multi-reference line prediction scheme) [?? (??)]

JVET-O0371 CE3-related: On 1xN and 2xN sub-partition in ISP [Y. Sun, L. Xu, X. Cao (Hikvision)]

JVET-O0771 Crosscheck of JVET-O0371 (CE3-related: On 1xN and 2xN sub-partition in ISP) [Y.-W. Chen (Kwai Inc.)]

JVET-O0399 Non-CE3: CCLM block size restriction [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-O1014 Crosscheck of JVET-O0399 (Non-CE3: CCLM block size restriction) [F. Chen (Hikvision)]

JVET-O0404 Non-CE3: Linear Model-based Intra Prediction [R. Ghaznavi-Youvalari (Nokia)]

JVET-O0412 CE3-related: Sampling point extension for CCLM [M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [miss] [late]

JVET-O0833 Crosscheck of JVET-O0412 (CE3-related: Sampling point extension for CCLM) [S.-C. Lim, J. Lee, H. Lee, J. Kang (ETRI)]

JVET-O0426 Non-CE3: DC intra prediction mode alignment [A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm)]

JVET-O0762 Crosscheck of JVET-O0426 (Non-CE3: DC intra prediction mode alignment) [H.-J. Jhu (Kwai Inc.)]

JVET-O0449 Non-CE3: Decoder-side Intra Mode Derivation (DIMD) with prediction fusion using Planar [M. Abdoli, T. Guionnet, E. Mora, M. Raulet (ATEME), S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-O0720 Crosscheck of JVET-O0449 Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar [V. Drugeon (Panasonic)]

JVET-O0507 Non-CE3: Intra-sub-partition coding mode only performing on non-square blocks [W. Gwun, W. Lee, D. Y. Lee, T. H. Kim, G. H. Park (KHU), G. Bang, W. Lim, H.Y. Kim (ETRI)]

JVET-O0988 Cross-check of Non-CE3: Intra-sub-partition coding mode only performing on non-square blocks (JVET-O0507) [M. W. Park (Samsung)]

JVET-O0510 CE3-related: Extensions to CCLM reference sample filtering in CE3-3.2.1 [S. Keating, K. Sharman (Sony)]

JVET-O0535 Non-CE3: Unification of PDPC for diagonal intra prediction modes [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-O0766 Crosscheck of JVET-O0535: Non-CE3: Unification of PDPC for diagonal intra prediction modes [F. Racapé, G. Rath (Interdigital)]

JVET-O0536 CE3-related: Redundant bit saving for CE3-2.2 [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-O0779 Crosscheck of JVET-O0536: CE3-related: Redundant bit saving for CE3-2.2 [L. Pham Van, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0632 Non-CE3: WAIP restriction for square sub-CU blocks partitioned by ISP [A. Kumar, S. Shrestha, B. Lee (Chosun Univ), Y. Lee, J. Park (Humax)]

JVET-O0640 CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0649 Non-CE3: Simplification of reference sample smoothing in intra prediction [A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0672 Non-CE3: MRL intra prediction for chroma DM [C.-C. Kuo, C.-C. Lin, C.-L. Lin (ITRI)]

JVET-O0673 Harmonized MIP/CCLM boundary downsampling [T. Biatek, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0674 Non-CE3: Multiple reference set for CCLM [P.-H. Lin, J.-Y. Deng (Foxconn)] [late]

JVET-O0760 Crosscheck of JVET-O0674 (Non-CE3: Multiple reference set for CCLM) [H.-J. Jhu (Kwai Inc.)]