| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1096 |
| --- | --- |

| Title: | Cross-check of JVET-O0515: Non CE3: Intra mode coding of non-MPM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabien Racapé, Tangi Poirier, Gagan Rath | Tel: Email: | (+1) 510-359-7732 fabien.racape@interdigital.com Tangi.poirier@interdigital.com Gagan.rath@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This document is a cross-check report of the contribution JVET-O0515 on the intra mode coding of non-MPM modes. It is reported that the partial average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests. The execution times are not reliable since the anchor and tests were run on heterogeneous cluster. The proposed modifications and the evaluation of the software are coherent with a minor impact on runtimes, as reported in JVET-O0515.

# Description

JVET-O0515 reports the results of removing the dependency between the coding of remaining modes and the MPM list. It aims at enabling the decoding of modes that are not in the MPM list without building the MPM list.

The maximum value of intra_luma_mpm_remainder is changed from 60 to 64, which includes all the 67 available modes except Planar and DC.

# Experimental Results

Experiments were performed based on VTM-5.0, in All Intra configuration and Random access.

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.10% | -0.02% | 84% | 86% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.05% | -0.03% | 0.02% | 86% | 92% |
| Class C | 0.05% | 0.02% | 0.03% | 86% | 94% |
| Class E | 0.06% | -0.02% | 0.06% | 85% | 92% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.05% | 0.14% | 0.02% | 86% | 93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Patent rights declaration

Interdigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).