| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1106 |
| --- | --- |

| Title: | BoG report on CE-8 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu | Tel: Email: | xiaozhongxu@tencent.com |
| ource: | BoG |  |  |

_____________________________

- Abstract

The BoG report provides summary of recommendations from the discussion and detailed comments for each identified CE8 related contributions.

Summary of the BoG on SCC tools

List of recommendations for adoption into VVC/VTM

Recommendation: (Bugfix) adopt JVET-O0162 method 2/JVET-O0331 method 3/JVET-O0480 solution 1/JVET-O0574 solution 2. Signal mvp flag for IBC only when MaxNumMergeCand>1

Recommendation: (HLS) adopt JVET-O0455, the first aspect, having a separate signaling of max number of merge size for IBC at slice header. The range of the size will be the same as the one of inter merge, which is [1, 6].

Recommendation: (Simplification) adopt JVET-O0626 method 1. Inside the shared merge region for IBC, remove the 2 spatial merge candidates from candidate list.

List of items for revisit

Recommendation: Revisit. This proposal contains changes to the binarization of BVD coding. One syntax is conditionally signaled based on the other and the ibc flag value. Conclusion of this proposal will be better made after the discussion of CE8-1.x discussion on BVD/MVD coding.

Recommendation: Revisit. The proponents of these three proposals will form a joint contribution, using the same encoder to test various masks used for TPM blending off. Report back also the syntax changes relative to current design in the joint contribution.

Detailed discussions are based on the following individual contributions.

IBC related contributions:

JVET-O0162 Non-CE8: On IBC motion vector list construction [H. Chen, H. Yang (Huawei)]

In current VTM, a unified motion vector candidate list is used for IBC merge and MVP mode. According to the working draft JVET-N1001, when MaxNumMergeCand is set equal to 1, the length of MVP candidate list may be equal to 1 or 2, depending on the number of available spatial motion vector candidates. If mvp_l0_flag for the decoded IBC block is equal to 1, the motion vector for the IBC block may be undefined. Therefore, this contribution presents 3 methods to solve this issue. First method is to set the second candidate to be zero motion vector when MaxNumMergeCand is 1, which is aligned with the software. Second method is to infer mvp_l0_flag to 0 for IBC mode. Third method is to derive max(2, MaxNumMergeCand) candidates for IBC motion vector prediction.

The proposed method 2 reportedly shows 0.64%, 0.49% luma BD-rates for Class F in AI and RA settings, respectively, and shows 0.15%, 0.11% luma BD-rates for Class SCC in AI and RA settings, respectively.

The proposed method 3 reportedly shows -0.91%, -0.46% luma BD-rates for Class F in AI and RA settings, respectively, and shows -0.32%, -0.20% luma BD-rates for Class SCC in AI and RA settings, respectively, for Class SCC.

There are several solutions on the table from this proposal and from several other contributions.

It was commented that the proposed method 2 in this contribution (signal the mvp_l0_flag for IBC only when MaxNumMergeCand>1) is the one that has the least changes to the current spec. while keeps the spirit of having a unified candidate list for merge and amvp mode of IBC (which was the adoption from the last meeting).

Recommendation: adopt JVET-O0162 method 2/JVET-O0331 method 3/JVET-O0480 solution 1/JVET-O0574 solution 2.

JVET-O0331 Non-CE8: On IBC BVP Candidate List Derivation [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

In this contribution, it was proposed to fix an issue in IBC BVP candidate list derivation. The issue is that when the maximum number of merge candidates is set to 1, the second candidate could be unavailable in AMPV mode. The proposed changes have no impact under CTC test conditions (maximum number of merge candidates > 1). Compared to VTM-5.0 with IBC and MaxNumMergeCand = 1, the proposed method 1 reportedly shows that there are no BD-rate changes. The proposed method 2 reportedly shows that average luma BD-rate change is -0.01%, -0.02% and -0.05% for AI, RA and LDB, respectively. For class F, it was observed that average luma BD-rate change is -0.32%, -0.20% and -0.10% for AI, RA and LDB, respectively. For TGM sequences, it was observed that average luma BD-rate change is -0.91%, -0.46% and -0.20% for AI, RA and LDB, respectively. The proposed method 3 reportedly shows that average luma BD-rate change is 0.02%, 0.01% and 0.00% for AI, RA and LDB, respectively. For class F, it was observed that average luma BD-rate change is 0.15%, 0.11% and 0.07% for AI, RA and LDB, respectively. For TGM sequences, it was observed that average luma BD-rate change is 0.64%, 0.49% and 0.40% for AI, RA and LDB, respectively.

See notes under JVET-O0162

JVET-O0480 Non-CE8: On signaling of IBC block vector predictor list size [X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, an issue in the unified block vector prediction list for IBC mode is identified, where the list size for merge mode can range from 1 to 6 (controlled by MaxNumMergeCand). However, for IBC AMVP mode, the size is always assumed to be 2. When MaxNumMergeCand is signaled to be 1, there will be no second candidate generated from the candidate list for the AMVP mode.

Three solutions are proposed to fix the issue.

In solution 1, when MaxNumMergeCand is equal to 1, the mvp_l0_flag for IBC will not be signaled and inferred to be 0. Simulation results report that, by using the proposed method:

- -0.05%/-0.06%/-0.06% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

- -0.07%/-0.05%/-0.30% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

In solution 2, MaxNumMergeCand for IBC mode is clipped to be at least 2. Simulation results report that, by using the proposed method:

- -0.50%/-0.46%/-0.23% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

- -1.64%/-0.85%/-1.13% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

In solution 3, a separate variable MaxNumIbcCand for IBC mode candidate list size is signaled, which ranges from 2 to 6.

See notes under JVET-O0162

JVET-O0860 Crosscheck of JVET-O0480: Non-CE8: On signaling of IBC block vector predictor list size [X. Xiu (Kwai Inc.)]

JVET-O0574 Non-CE8: Fixes of IBC BV candidate list [L. Zhang, J. Xu, K. Zhang, H. Liu, W. Zhu (Bytedance)]

In the last JVET meeting, the single motion candidate list for IBC AMVP and IBC merge mode is adopted. The list size indicated by the maximum number of merge candidates (i.e., MaxNumMergeCand) is signalled in the slice header. However, the BV predictor of the IBC AMVP mode could only be selected from two candidates. Therefore, when the BV candidate list size is 1, how to handle IBC AMVP mode is undefined. This contribution provides two solutions to fix this issue. In the first solution, the termination of BV candidate list construction process is dependent on whether the block is coded with AMVP or merge mode. In the second solution, signalling of the BV predictor index for IBC AMVP mode is under the condition that MaxNumMergeCand is greater than 1. This contribution proposes modifications on the signalling of BV predictor index. No performance changes are observed under the common test conditions.

See notes under JVET-O0162

JVET-O0325 CE8-related: Simple BVD coding scheme without adding more context [Y. Sun, F. Chen, L. Wang (Hikvision)]

This contribution proposes a simple BVD coding scheme reusing the context of MVD coding. In method 1, zero BVD is disabled because of the redundancy caused by ABVP list construction modification adopted in last meeting, which reuse the merge list for ABVP mode. In method 2, contains method 1 and the binarization process [1] for abs_mvd_minus2 syntax, which is coded by Exp-Golomb order K equal to 1 (EG1), is changed to use EG3. From experiment results, following BD rate changes are observed over VTM-5.0 with IBC:

Method 1,(won’t parse abs_mvd_greater0_flag[0] when abs_mvd_greater0_flag[1] = 0)

- AI: -0.63% (CTC), -0.12% (Class F), -0.43% (TGM)

- RA: 0.01% (CTC), -0.12% (Class F), -0.18% (TGM)

Method 2,(method 1 + EG3 coding for abs_mvd_minus2)

- AI: -0.64% (CTC), -0.26% (Class F), -0.74% (TGM)

- RA: 0.02% (CTC), -0.20% (Class F), -0.41% (TGM)

A binarization change is proposed to BVD coding module, where the case of (0, 0) BVD is disallowed (method 1). This is done by signaling abs_mvd_greater0_flag[1] first, and conditional signal the abs_mvd_greater0_flag[0] based on abs_mvd_greater0_flag[1] and ibc_flag. In addition, the binarization of abs_mvd_minus2 is changed (method 2). The range of gain for class F and TGM AI is 0.12%/0.43% for method 1, and 0.26%/0.74% for method 1+ method 2.

It is claimed that the proposed methods would not have higher complexity as compared to the current VTM-5 design (in terms of # of context coded bins).

Recommendation: revisit after the discussion of CE8-1.x discussion on BVD/MVD coding.

JVET-O0709 Crosscheck of JVET-O0325 (CE8-related: Simple BVD coding scheme without adding more context) [J. Nam (LGE)]

JVET-O0326 CE8-related: JVET-O0325 combined with CE8 BVD coding scheme [Y. Sun, F. Chen, L. Wang (Hikvision)]

This contribution combines JVET-O0325 method 1 with CE8-1.1, CE8-1.2 and CE8-1.3. JVET-O0325 proposes a simple BVD coding scheme reusing the context of MVD coding. Combination of JVET-O0325 and CE8-1.1 is tested in test 1. Combination of JVET-O0325 method 1 and CE8-1.2 is tested in test 2. Combination of JVET-O0325 and CE8-1.3 is tested in test 3. From experiment results, following BD rate changes are observed over VTM-5.0 with IBC:

Test 1,(JVET-O0325 method 1 + CE8-1.1)

- AI: -0.68% (CTC), -0.52% (Class F), -1.32% (TGM)

- RA: -0.04% (CTC), -0.39% (Class F), -1.12% (TGM)

Test 2,( JVET-O0325 method 1 + CE8-1.2a)

- AI: -0.65% (CTC), -0.32% (Class F), -0.91% (TGM)

- RA: -0.02% (CTC), -0.27% (Class F), -0.81% (TGM)

Test 3,( JVET-O0325 method 1 + CE8-1.2b )

- AI: -0.64% (CTC), -0.23% (Class F), -0.74% (TGM)

- RA: 0.00% (CTC), -0.18% (Class F), -0.55% (TGM)

Test 4,( JVET-O0325 method 1 + CE8-1.3a )

- AI: -0.64% (CTC), -0.36% (Class F), -0.99% (TGM)

- RA: 0.00% (CTC), -0.33% (Class F), -0.61% (TGM)

The proposed method 1 in JVET-O0325 was tested in combination of several tests in CE8-1.x. The additional benefit of having the change is in the range of 0.1%~0.3% for TGM AI case for different tests.

This contribution provides combination results of the proposal in JVET-O0325 with existing CE8 tests.

See notes under JVET-O0325.

JVET-O0710 Crosscheck of JVET-O0326 (CE8-related: JVET-O0325 combined with CE8 BVD coding scheme) [J. Nam (LGE)]

JVET-O0327 CE8-related: Default filling method for IBC mode [S. Ye, Y. Sun, F. Chen, L. Wang (Hikvision)]

This contribution presents a default filling method for IBC mode in order to replace invalid zero block vectors. The reported BD rate changes from the VTM5.0 with IBC enable are as follows:

Method 1(Filling the merge list with (-2*cbWidth, 0) block vector):

- 0.05%/0.06%/0.11% gain in AI/RA/LDB configuration for class TGM, separately.

Method 2(Filling the merge list with a default block vector list):

- 0.09%/0.16%/0.17% gain in AI/RA/LDB configuration for class TGM, separately.

This contribution proposed to use block size dependent default bv predictor, to replace (0, 0) vector which is used in the current VTM-5.

No action, as similar proposals have been proposed before, and no support for inclusion was made.

JVET-O0840 Crosscheck of JVET-O0327: CE8-related: Default filling method for IBC mode [J. Chen (Alibaba)]

JVET-O0908 Crosscheck of JVET-O0327: CE8-related: Default filling method for IBC mode [W. Zhu (Bytedance)]

JVET-O0455 Non-CE8: On IBC merge mode [Y.-J. Chang, Y. Zhang, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

The current VVC includes 4 different types of merge modes, e.g. regular merge mode, subblock merge mode, triangle merge mode, and IBC merge mode, to address different contents such as translational motion, motions in triangle shape, intra-picture motion (for screen content use case), and general motion. It was already agreed to utilize three independent syntax elements to separately specify the maximum number of merging candidates for regular merge mode, subblock merge mode, and triangle merge mode. This proposal further adds one more syntax element in slice header to independently specify the maximum number of merging candidates for IBC merge mode, as the same as what regular merge mode, subblock merge mode, and triangle merge mode already do.

First aspect, it is proposed a separate merge candidate size signaling at slice level, for IBC.

The motivation is to have separate control of IBC merge list size. There are already several different merge types, including regular merge, triangle merge and subblock merge modes. The sizes of these modes are controlled separately in the slice header.

The maximum number of the size is proposed to be 5. And for I slice, the minimum number is set to be 1 while for P/B slices, the minimum number is set to be 0.

| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| --- | --- |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| five_minus_max_num_ibc_merge_cand | ue(v) |
| … |  |

It is questioned that if merge size for IBC in P/B slices is set to be 0 (disabled), will there still be candidate(s) generated for IBC amvp mode? It seems that current design does not define the behavior for this. There is more complication than simply setting a value in the slice header. So the minimum number of merge candidate size for IBC should be 1 for all cases.

It was discussed whether we need to make the maximum number to be 5 (reduced from 6 currently). The motivation was to reduce complexity. It was commented that currently IBC merge list construction has fewer spatial candidates than inter and less pruning (comparison) operations. Adding a 6th element (from HMVP buffer, or being (0, 0) vector, without pruning) to the list does not seem to have extra complexity. So it is better to stay unchanged (being 6 as maximum number).

The second aspect, it is proposed to have either separate syntax element of IBC merge idx or, separate context of this syntax element, to switch between IBC and inter.

The simulation results of having separate syntax element of merge idx for IBC showed the gain for class F/TGM is in the range of 0.1%, it does not seem to be big impact of having shared syntax/context for IBC and inter. It is preferred to be stay unchanged.

It was commented that if ibc merge size and inter merge size are different (as recommended in aspect 1), there would be potential interference between ibc merge idx and inter merge idx ( no results yet).

Recommendation: adopt JVET-O0455, the first aspect, having a separate signaling of max number of merge size for IBC at slice header. The range of the size will be the same of inter merge, which is [1, 6].

JVET-O0626 Non-CE8: Simplified IBC BV candidate list construction process [L. Zhang, K. Zhang, H. Liu, J. Xu, Z. Deng, N. Zhang (Bytedance)]

In this contribution, two methods are provided which are asserted to further simplify the IBC BV candidate list construction process. In the first method, the derivation of spatial merge candidates is skipped for coding units under a shared parent node. In the second method, the shared merge list is totally removed and the derivation of spatial merge candidates for 4×4 blocks is skipped. In both methods, the pruning operations among BV candidates and derivation of spatial neighboring blocks are totally removed. It is asserted that both proposed methods could be benefit for the MV throughput. Simulation results reportedly show that the first method has {0.04%, 0.06%, -0.11%} and {0.12%, 0.07%, 0.07%} BD-rate increase in {AI, RA, LDB} configurations for class F and class TGM, respectively; the second method has {0.04%, 0.04%, -0.14%} and {0.07%, -0.02%, 0.02%} BD-rate increase in {AI, RA, LDB} configurations for class F and class TGM, respectively.

This contribution proposed to remove 2 spatial candidates in the candidate list for IBC, if the blocks are under the Shared Merge List. This effectively removes 3 pruning comparisons in the candidate list derivation process. The reported coding difference as compared to the VTM-5 is marginal. The reduction of complexity applies to small blocks, for which it is commented that the bottleneck of throughput will be alleviated.

Two tests are performed, first one is with Shared Merge List concept while the second one also removes the Shared Merge List. In that case, spatial candidates are removed from 4x4 blocks.

It was commented that if Shared Merge List is removed (in method 2), the worst-case scenario would be having three different lists in the region consist of 4x4, 8x4 and 4x4 blocks. It would make the parsing more complex than in method 1. It is therefore desirable to keep the Shared Merge List unchanged.

Recommendation: adopt JVET-O0626 method 1.

Other SCC related contributions:

JVET-O0513 Non-CE4: Adaptive blending filtering for TPM [S. Esenlik, H. Gao, B. Wang, A. Kotra, Z. Zhao, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

It is asserted that the blending filtering of TPM causes over-smoothing when the content is known to include sharp edges, hence rendering TPM less useful for screen content application. It is proposed to include a control flag in tile group header or picture parameter set to enabling turning blending filtering of TPM off. The goal of the proposed modification is to retain sharp details that are common especially in SCC material. For TGM class, the simulation results demonstrate 0.59% and %X luma BD-rate reduction for RA and LDB configurations respectively.

Triangle blending across the diagonal boundary is not useful for screen content, where the shapes are usually sharp without transitioning area.

In the proposal, the mask to assign different motion vectors has been changed from left one to the right one.

A slice level flag is used to disable the TPM blending.

Similar proposals are presented in JVET-0563 aspect 1 and JVET-O0645, where some differences are identified.

In the following two proposals, an sps level flag is also used to enable this TPM blending-off feature.

In JVET-O0645, the proposed mask is slightly different as follows (exactly along the edge):

In JVET-O0563 aspect 1, two additional sharp masks (whether the diagonal edge uses weight 8 or 0) are switchable depending on which partition has the smaller MV of the two partitions. The divider is along the diagonal edge exactly.

Different encoder algorithms were used, which partially explain the RD performance difference across the three proposals.

Some questions raised: 1) if it is desirable to have this low-level tool changes for SCC purpose. 2) would subjective benefits observable from the proposal methods. 3) what is the impacts of the differences among these three proposals, if a common encoder decision is used as slice level.

Recommendation: Revisit. The proponents of these three proposals form a joint contribution, using the same encoder to test various masks used for blending off. Report back also the syntax changes relative to current design in the joint contribution.

JVET-O0921 Crosscheck of JVET-O0513: Non-CE4: Adaptive blending filtering for TPM [X. Xiu (Kwai Inc.)]

JVET-O0563 Non-CE8: disabling TPM blending [W. Zhu, L. Zhang, J. Xu, K. Zhang (Bytedance)]

It is proposed to disable the blending process in TPM based on a flag signaled at the slice level in this document. When the blending process is disabled, prediction samples for the two partitions will be directly copied from their own motion compensated blocks. The samples located on the diagonal line or anti-diagonal line will be determined by comparing the motion vectors of two partitions. Motion storage for TPM is modified so that only one uni-directional MV is stored for each 4x4 block. Simulation results report that the proposed scheme can achieve -0.83% and -0.84% BD bitrate changes compared with VTM-5.0 on class TGM. On 4:4:4 sequences, the proposed scheme -0.85% and -1.11% BD bitrate changes compared with VTM-5.0 on TGM 1080p.

Two aspects are discussed in this proposal.

Aspect 1 about blending off for TPM at slice level.

See notes under JVET-O0513.

Aspect 2 about a separate storage mechanism for 4x4 blocks along diagonal line of the TPM block.

The benefit of the 2nd aspect along is:

For 420/444 cases, 0.3%/0.1% gain in RA/LDB for TGM 1080p.

Need to discuss together with the outcome from Aspect 1 of this proposal. Revisit JVET-O0513.

JVET-O0922 Crosscheck of JVET-O0563 (Non-CE8: disabling TPM blending) [S. Ye (Hikvision)]

JVET-O0645 Non-CE4/8: Blending-off Switch for TPM Mode [C.-C. Chen, Y. Zhang, K. Reuze, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This proposal disables the blending operation of Triangle Prediction Merge (TPM) mode for screen content sequences. With it, the weighting values in the blending area are all clipped to be either 0/8 or 8/8, depending on whether they are closer to 0/8 or 8/8. Thus, none of the final prediction samples are generated by weighted-averaging two different ones which would blur out certain video contents with extremely sharp signals (e.g., screen contents). It is noted that a slice-level flag is introduced to indicate whether the blending operation of TPM is enabled or not, and a sequence-level flag is introduced to indicate whether the mentioned slice-level flag is present is the bitstream. Experiments were conducted on top of the VTM-5.0 with Random Access (RA) and Low Delay B (LDB) testing conditions for both CTC and CE8 testing sequences. Performance results show that the average BD-rate impact on CTC sequences is around 0.01%, while Class TGM1080p444 achieves 0.60% and 0.91% BD-rate gain in RA and LDB, respectively.

See notes under JVET-O0513.

JVET-O0912 Crosscheck of JVET-O0645: Non-CE4/8: Blending-off Switch for TPM Mode [W. Zhu (Bytedance)]

JVET-O0646 Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode [C.-C. Chen, K. Reuze, Y. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes to combine the two TPM-related methods as in JVET-O0645 (Non-CE4/8: Blending-off Switch for TPM Mode) and JVET-O0411 (Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage). The former introduces a slice-level switch to disable the blending operation during the motion compensation stage of TPM mode, and the latter removes the derivation process of bi-prediction MV by taking only one MV from the two triangle partitions when both MVs of the selected TPM merge candidate point to the same reference picture list. Experiments were conducted on top of the VTM-5.0 with Random Access (RA) and Low Delay B (LDB) testing conditions for both CTC and CE8 testing sequences. Experimental results of the top performing one show that Class TGM1080p444 achieves 0.62% and 1.05% BD-rate gain in RA and LDB, respectively.

This proposal provides additional information when JVET-O0645 is combined with JVET-O0411.

JVET-O0763 Crosscheck of JVET-O0646 (Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode) [H. Yang (InterDigital)]