| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1109-v3 |
| --- | --- |

| Title: | Non-CE2: unification of chroma residual scaling design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ruling Liao, Yan Ye, Jiancong Luo (Alibaba) Zhipin Deng, Li Zhang (ByteDance) Taoran Lu, Peng Yin (Dolby) Edouard François, Franck Hiron (InterDigital) Jie Zhao, Seethal Paluri, Seung Hwan Kim (LG Electronics) Zhi-Yi Lin, Tzu-Der Chuang (MediaTek) | Email: | jiechen.cj@alibaba-inc.com {zhipin.deng,lizhang.idm}@bytedance.com {pyin,tlu}@dolby.com {jie.zhao,seunghwan3.kim}@lge.com edouard.francois@interdigital.com {zhi-yi.lin,peter.chuang} @mediatek.com |
| Source: | Alibaba, ByteDance, Dolby, InterDigital, LG Electronics, MediaTek Inc |  |  |

_____________________________

# Abstract

In this contribution, a unified chroma residual scaling design for both dual tree and single tree is proposed. In the proposed solution, a high-level switch in SPS enables switching between explicit signalling or implicit derivation of the scaling values. The explicit signalling spatial granularity can be controlled from CTU-scale to slice-scale. The implicit derivation is applied per VPDU using neighboring reconstructed luma samples of the VPDU. The proposed solution applies both for Dual Tree and Single Tree cases. It is asserted that the proposed solution removes pipeline latency for chroma residual scaling. In addition, explicit signalling removes the luma-chroma dependency too.

The simulation results of the proposed solution are reported as follows:

For SDR:

- AI: 	Y/U/V	0.07%, -0.85%, -0.95%

- RA: 	Y/U/V	0.05%, -1.19%, -1.02%

- LDB:	Y/U/V

For HDR-PQ:

- HDR-PQ AI: 	DE/L100/wPSNRY/U/V	1.77%, -1.70%, -1.24%, 5.54%, 1.17%

- HDR-PQ RA: 	DE/L100/wPSNRY/U/V	2.07%, -1.50%, -1.31%, 7.04%, 3.04%

It is asserted that the proposed design significantly improves chroma performance compared to the VTM5.0 anchor for both SDR and HDR-PQ content with negligible encoding and decoding runtime change.

# Introduction

Chroma residual scaling (CRS) is a component of LMCS. CRS scales chroma residual values by a factor derived from the average value of the corresponding luma prediction block. VTM5.0 design of CRS results in a pipeline latency. Several non-CE2 contributions intend to address this issue.

Two categories of solutions were identified from non-CE2 contributions. Category 1 consists of solutions implementing an explicit signalling of the CRS values, with various levels of spatial granularity. Category 2 consists of solutions implementing an implicit derivation of the CRS values, with various levels of spatial granularity (CTU or VPDU). The implicit derivation is performed using the luma samples neighboring the VPDU, with different variants depending on the number of used samples.

Explicit signalling has the advantage of removing any chroma-to-luma dependency, but signalling can only apply at minimum CTU-level to limit the signalling cost overhead. Using one single CRS value per slice was observed to perform well for SDR CTC content. Implicit derivation approaches benefit from a better localization of the CRS values, which is preferable for content presenting high luminance variations (such as HDR PQ content). A flexible design taking benefits from both designs is desirable.

This contribution proposes a unified design from JVET-O0232, JVET-O0271, JVET-O0429, JVET-O0618, and JVET-O0627, aiming at taking benefits from the Category 1 and 2 designs.

# Proposal

The proposed unified design can be summarized as follows. A SPS-level flag is introduced to indicate if explicit CRS signalling or implicit CRS derivation is used. Signalling of 16-pieces LM model is performed in APS, as in VTM5.0. At slice level, the following signalling applies:

- If explicit signalling is used

  - the CRS values are signalled per n×m CTUs

    - n, m are signalled in the slice header. This enables controlling the granularity of the CRS values signalling and application (from slice-scale to CTU-scale).

- Otherwise (implicit derivation is used)

  - the CRS values are inferred from luma at VPDU level

In case of explicit signalling, one CRS value is decoded per n×m CTUs. The chroma residual scaling applies using this decoded CRS value for all chroma blocks belonging to the n×m CTU area. The CRS value coding process is the same as for delta QP.

In case of implicit derivation, the CRS factor for each VPDU is derived from luma sample values from previously reconstructed VPDU neighbors. If the CTU partition is inter 128x128, inter 128x64 and inter 64x128, then the CRS factor derived for the CU associated with the first VPDU  is used for all chroma TBs in that CU. For each VPDU, the left and top neighboring reconstructed luma samples are used. First, the average value of M left neighboring luma samples and M top neighboring luma samples which are the closest to the top-left position, as shown in Figure 1, avgY, is calculated. M = min(CTUSize, 64). Then the same method in VVC draft 5 is used to derive the chroma scaling factor based on avgY. The factor derived on VPDU level is used for all the chroma blocks in this VPDU. There is no additional dependency between luma and chroma when decoding the current VPDU.

Figure 1. VPDU neighboring samples used in chroma scaling factor derivation

On the picture boundary, for the rightmost column VPDU, if the upper neighboring samples within the picture is less than M, the picture boundary sample is padded until M; for the bottom row VPDU, if the left neighboring samples within the picture is less than M, the picture boundary samples is padded until M. By doing it, the number of averaged samples is always 2M where M is set as power of 2, so division operation is avoided. For the leftmost column VPDU, only M top neighboring samples are used for average value; for the top row VPDU, only M left neighboring samples are used for average value; for the left-top VPDU of the picture, avgY is directly set as 1<<(bitdepth-1).

# Simulation results

The proposed modifications are implemented on top of VTM5.0. Tests are reported for SDR and HDR PQ content.

The following configuration is tested:

- For SDR, slice-level explicit signalling (nxm > #CTUs in one slice) is used

- For HDR-PQ, implicit derivation on VPDU level is used

Table 1. Results of SDR

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.06% | -0.23% | 101% | 100% |
| Class A2 | -0.14% | 1.02% | 1.33% | 101% | 101% |
| Class B | 0.08% | -0.73% | -0.81% | 101% | 101% |
| Class C | 0.22% | -1.96% | -1.96% | 102% | 101% |
| Class E | 0.20% | -2.23% | -2.85% | 101% | 100% |
| Overall | 0.07% | -0.85% | -0.95% | 101% | 101% |
| Class D | 0.11% | -0.63% | -0.98% | 101% | 103% |
| Class F | 0.39% | -1.84% | -1.98% | 102% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -1.09% | -0.52% | 100% | 101% |
| Class A2 | 0.04% | -0.46% | -0.38% | 100% | 101% |
| Class B | 0.04% | -0.86% | -0.78% | 101% | 101% |
| Class C | 0.06% | -2.23% | -2.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -1.19% | -1.02% | 100% | 101% |
| Class D | 0.08% | -1.44% | -1.65% | 100% | 101% |
| Class F | 0.33% | -1.70% | -1.61% | 101% | 102% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.04% | -0.19% | 0.02% | 100% | 101% |
| Class E | -0.15% | -0.76% | -1.23% | 100% | 102% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.43% | 0.77% | 100% | 102% |
| Class F |  |  |  |  |  |
|  |  |  | All Intra |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.22% | -1.96% | -1.96% | 102% | 101% |
| Class E | 0.20% | -2.23% | -2.85% | 101% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.11% | -0.63% | -0.98% | 101% | 103% |
| Class F |  |  |  |  |  |

The HDR CTC anchor uses Luma-based QP adaptation (LPQ=1) and LMCS is off (LMCS=0).

In the performed tests for HDR, LMCS is activated and LQP is deactivated.

Two sub-tests are performed for HDR PQ:

- HDR1: LQP 0, LMCS on / CRS not activated in DualTree

- HDR2.: LQP 0, LMCS on / implicit signalling on VPDU level in DualTree & SingleTree

The first test (HDR1) is included to show the performance of LMCS as is for HDR PQ content.

Table 3. Results of HDR1: LQP off/LMCS on versus LQP off/LMCS on

|  | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 8.08% | -2.06% | -1.64% | 14.15% | 8.35% |
| RA | 6.26% | -1.79% | -1.59% | 14.12% | 6.85% |

Table 5. Results of HDR2

|  | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 1.77% | -1.70% | -1.24% | 5.54% | 1.17% |
| RA | 2.07% | -1.50% | -1.31% | 7.04% | 3.04% |

# Conclusion

In this contribution, a unified design is proposed to eliminate pipeline dependency for chroma residual scaling for Dual Tree. The solution enables the support of explicit signalling and implicit derivation of the CRS values. This flexibility allows taking benefits from these two options depending on the content. The proposed solution applies both for Dual Tree and Single Tree cases. The proposed design significantly improves chroma performance compared to the VTM5.0 anchor, while solving CRS latency issue.

It is suggested to adopt it in the next version of VVC draft.

# Syntax

Table 6. SPS parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| if ( sps_lmcs_enabled_flag ) |  |
| sps_cs_explicit_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 6. slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(5) |
| if( !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| if ( sps_cs_explicit_flag && slice_chroma_residual_scale_flag ) { |  |
| slice_cs_horiz_size_minus1 | ue(v) |
| slice_cs_vertic_size_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Table 6. coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| … |  |
| if( sps_cs_explicit_flag && slice_chroma_residual_scale_flag ) { |  |
| if ( ( xCtb >> CtbLog2SizeY ) % CsHorizSize = = 0 && |  |
| ( yCtb >> CtbLog2SizeY ) % CsVerticSize = = 0 ) { |  |
| cu_crs_delta_abs | ae(v) |
| if( cu_crs_delta_abs ) |  |
| cu_crs_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Semantics of added syntax elements

slice_cs_horiz_size_minus1 specifies the horizontal number of CTUs sharing the same chroma residual scaling index. When slice_cs_horiz_size_minus1 is not present, it is inferred to be equal to 0.

The variable CsHorizSize is derived as ( slice_cs_horiz_size_minus1 + 1 ).

slice_cs_vertic_size_minus1 specifies the vertical number of CTUs sharing the same chroma residual scaling index. When slice_cs_vertic_size_minus1 is not present, it is inferred to be equal to 0.

The variable CsVerticSize is derived as ( slice_cs_vertic_size_minus1 + 1 ).

# Patent rights declaration

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ByteDance may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediatek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).