| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1113-v2 |
| --- | --- |

| Title: | BoG report on CE2: luma-chroma dependency reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jie Chen Edouard François Peng Yin | Email: | jiechen.cj@alibaba-inc.com edouard.francois@interdigital.com pyin@dolby.com |
| Source: | BoG |  |  |

_____________________________

Introduction

The sessions were held 0940~1300, 1440~21:30 on Jul. 4th and 09:00~13:10 on Jul. 7th to discuss 22 technical contributions in 3 categories:

- Luma-chroma latency reduction for CCLM (7)

- Luma-chroma latency reduction for CRS (8)

- LMCS simplification/modification (7)

Recommendations

Recommendations of adoption

Luma-chroma dependency reduction for CCLM

- JVET-O1124 Draft text for CCLM restriction to reduce luma-chroma latency for chroma separate tree [Yin Zhao, Haitao Yang (Huawei), Chia-Ming Tsai, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei (MediaTek)]

Same as JVET-O0196

when the 64x64 luma coding tree node is partitioned with Not Split (and ISP is not used for the 64x64 CU) or QT, the chroma CUs in 32x32 / 32x16 chroma coding tree node are allowed to use CCLM in the following way:

- if the 32x32 chroma node is partitioned with Not Split or QT split, all chroma CUs in the 32x32 node can use CCLM

- if the 32x32 chroma node is partitioned with HBT, and the 32x16 child node does not split or uses VBT split, all chroma CUs in the 32x16 chroma node can use CCLM.

In other luma and chroma coding tree split conditions, CCLM is not allowed for chroma CU.

Luma-chroma dependency reduction for CR

- JVET-O0429 Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree [T. Lu, P. Yin (Dolby), E. François, F. Hiron (InterDigital)]

Only for encoder bug fix part

An encoder bug fix related to a rounding operation in CRS lambda calculation

Coding performance impact: 0.02%/-0.44%/-0.38% in RA, 0.02%/-0.25%/-0.21% in LDB

- JVET-O1109 Non-CE2: unification of chroma residual scaling design [J. Chen, R. Liao, Y. Ye, J. Luo (Alibaba), Z. Deng, L. Zhang (ByteDance), T. Lu, P. Yin (Dolby), E. François, F. Hiron (InterDigital), J. Zhao, S. Paluri, S. H. Kim (LGE), Z.-Y. Lin, T.-D. Chuang (MediaTek)]

A unified chroma residual scaling design for both dual tree and single tree is proposed. In the proposed solution, a high-level switch in SPS enables switching between explicit signalling or implicit derivation of the scaling values. The explicit signalling spatial granularity can be controlled from CTU-scale to slice-scale. The implicit derivation is applied per VPDU using neighboring reconstructed luma samples of the VPDU. The proposed solution applies both for Dual Tree and Single Tree cases.

LMCS modification & simplification

- JVET-O0272 CE2-related: Simplified inverse luma mapping [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

one encoder constraint is imposed as “For the variable LmcsPivot[ i ] with i = 0..15, it is a requirement of bitstream conformance that if LmcsPivot[ i ] is not multiples of 32, then (LmcsPivot[ i ] >> 5) shall not be equal to  (LmcsPivot[ i + 1 ]>>5).”

- JVET-O0428 AHG16/AHG17: LMCS related clean-ups [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Zhao, S.H. Kim (LGE)]

- Bug fix in the process of identifying the index of inverse piecewise function (clause 8.8.2.3);

- LMCS code changes to improve readability, compactness, and software-specification correspondence without affecting coding performance;

- Constraints on LMCS APS,

  - The total number of LMCS APS per CVS shall be less than or equal to 4,

  - Only one LMCS model per picture shall be allowed.

Summary

Luma-chroma latency reduction for CCLM

There are 2 categories of proposals.

- Category 1 – partitioning restrictions, CCLM always enabled for all CBs.

- Category 2 – CCLM enabling restrictions based on partitioning, no partitioning restriction.

Regarding category 1:

- It was agreed that ISP shall be disabled at VDPU level.

- The simplest approach common in 3 contributions (O0273 method 1, O0274 method 1, O0524) consists in:

- restricting luma & chroma VDPU partitioning to not split or QT split

Performance of this approach (O0273 method 1 results):

|  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.18% | 2.35% | 2.30% | 0.05% | 0.76% | 0.84% | 0.03% | 0.61% | 0.71% |

- Other methods add more partitioning options. The most performing proposal (O0273 method 2) adds enabling BT split for chroma CB at VDPU level, and implicit TB split. The intra reference samples availability check is also modified.

Performance of this approach is O0273 method 2:

|  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.04% | 1.19% | 1.17% | 0.03% | 0.46% | 0.36% | 0.05% | 0.32% | 0.54% |

- For category 1, it should be clarified if these restrictions also apply when CCLM is disabled at SPS level.

- As a general question, it should be clarified if the latency issue depends on the CTU size, i.e. if the latency should be at most ¼ of the CTU size.

Regarding category 2:

- It was agreed that CCLM shall not be used when ISP is used at VDPU level.

- Common aspects from O0129, O0196, O0274 are:

- CCLM enabled if luma & chroma VDPU is not split or QT split

- CCLM enabled if luma VDPU is not split or QT split, and chroma is split by BT horizontal and further split shall be only vertical split, or no split

O0196 corresponds to this design. Performance is:

|  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.07% | 1.06% | 1.07% | 0.02% | 0.41% | 0.29% |  |  |  |

- O0273 method 3 is the closest category 1 solution to category 2 solution O0196. Performance is:

|  | AI |  |  | RA |  |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10% | 1.53% | 1.49% | 0.03% | 0.46% | 0.44% |  |  |  |

Further discussion was made on Sunday, July 7, 2019

It was commented that the latency issue happens in the combination with dual tree and CCLM. Category 1 solutions which impose partitioning constraints, potentially impact other intra coding tools.

It was commented by a hardware expert that JVET-O0274-method 2 allows any partitioning when luma VPDU is not split, which potentially has implementation issues, but it doesn’t show the sufficient coding performance benefit compared with JVET-O0196 which has no such implementation issue, so it is suggested to remove JVET-O0274-method 2 option from consideration.

Two solutions were considered for recommended adoption during the discussion: 1) JVET-O0274 method 1, which is the simplest solution in category 2 with performance {0.18%,2.43%,2.40%} for AI, {0.04%,0.76%,0.84%} for RA; 2) JVET-O0196 which has the best performance solution in category 2 ({0.07%,1.06%,1.07%} for AI, {0.02%,0.39%,0.35%} for RA).

JVET-O0274 method 1 is considered to give too much coding performance loss. Thus, JVET-O0196 is the candidate for adoption.

It is noted that the newly submitted contribution JVET-O1124 provides the text and complexity analysis of JVET-O0196. So based coding performance and complexity, it was agreed to recommend JVET-O1124 for adoption.

Recommendation: adopt JVET-O1124

Luma-chroma latency reduction for CRS (8)

Discussion

- All proposals are considered to reduce or solve the pipeline latency issue.

- It is agreed that a unified design for SDR and HDR is desirable.

- Two key aspects have to be considered for the design of chroma residual scaling:

- explicit signalling or implicit derivation

- spatial granularity for the application of CRS

  - if CRS is derived at e.g. VDPU or CTU scale, derivation of CRS scale from neighboring luma samples enables solving the pipeline latency issue. Sufficient number of neighboring luma samples should be used.

Contributions relevant to these two key aspects are summarized as:

- CRS usage / signalling (addressing dual tree and single tree cases)

- 1 idx per LMCS Model (O0232 method 2.2)

- 1 idx per n×n CTU (O0429 Option1)

- CRS implicit derivation – various granularity (addressing dual tree and single tree cases)

- 1 factor inferred per LMCS model (O0232 method 1, O0618)

- 1 factor inferred per CTU from several (4, or 16) top / left neighbor reco luma samples (O0627)

- 1 factor inferred per VDPU from top-left reco neighbor reco luma sample (O0271 method 4 and method 3, O0429 option2 DT&ST)

The coding performance are:

- SDR

|  |  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CRS signalling | 1 idx per LMCS Model | 0.06% | -0.62% | -0.75% | 100% | 100% | 0.01% | -0.61% | -0.57% | 100% | 101% |
|  | 1 idx per nxn CTU | 0.03% | -0.23% | -0.33% | 99% | 100% | 0.01% | -0.53% | -0.54% | 100% | 100% |
| CRS derivation | 1 factor inferred per LMCS model | 0.06% | -0.62% | -0.75% | 100% | 100% | 0.01% | -0.61% | -0.57% | 100% | 101% |
|  | 1 factor inferred per LMCS model | -0.01% | -0.05% | -0.14% | 103% | 104% | 0.02% | -0.77% | -0.75% | 102% | 103% |
|  | 1 factor inferred per CTU | 0.05% | -0.53% | -0.76% | 100% | 100% | 0.07% | -1.27% | -1.25% | 99% | 100% |
|  | 1 factor inferred per VDPU | 0.03% | -0.37% | -0.49% | 101% | 101% | 0.08% | -1.24% | -1.22% | 101% | 100% |

- HDR PQ

|  |  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNRL | wPNSRY | wPSNRU | wPSNRV | DE100 | PSNRL | wPNSRY | wPSNRU | wPSNRV |
| CRS signalling | 1 idx per LMCS Model | 5.42% | -1.61% | -1.14% | 7.02% | -0.97% | 3.59% | -1.40% | -1.20% | 5.74% | -3.51% |
|  | 1 idx per nxn CTU | 2.22% | -1.71% | -1.26% | 6.16% | -0.20% | 2.01% | -1.49% | -1.30% | 6.69% | -0.58% |
| CRS derivation | 1 factor inferred per LMCS model | 5.42% | -1.61% | -1.14% | 7.02% | -0.97% | 3.59% | -1.40% | -1.20% | 5.74% | -3.51% |
|  | 1 factor inferred per LMCS model | 0.60% | -0.70% | -0.20% | -2.68% | -12.06% | -0.10% | -0.70% | -0.48% | -4.14% | -13.74% |
|  | 1 factor inferred per CTU | 2.70% | -1.49% | -1.03% | 5.41% | 1.20% | 2.82% | -1.32% | -1.12% | 7.02% | 3.21% |
|  | 1 factor inferred per VDPU | 1.94% | -1.66% | -1.23% | 5.77% | 1.42% | 2.14% | -1.48% | -1.30% | 7.27% | 3.26% |

- Other contributions which only address single tree case

- Simplified derivation process of CRS for CIIP case (O0271 method 1 & 2)

- Simplified derivation process of CRS for DMVR, BDOF and CIIP (method1) or just using uni-dir pred samples (method2) (O0608)

- Alternate derivation process of CRS for intra blocks (aspects 1&2) and inter blocks (aspect 3) (O0550)

- It is recommended to adopt the encoder bug-fix from JVET-O0429.

- Open issues:resolved

For the open issues, the agreement is achieved by recommendation of adopting JVET-O01109 (see section 4.1). Therefore there is no CE in this category.

Luma-chroma latency reduction for CRS (8)

See section 4.3

CE2 related contributions

Luma-chroma latency reduction for CCLM

JVET-O0129 Non-CE2: CCLM with Chroma Separate Tree [C.-W. Kuo, J. Li, C. Lim (Panasonic)]

In this contribution, it is proposed to use VPDU level partitioning information for CCLM enabling condition checks to reduce the chroma prediction latency to ¼ luma VPDU reconstruction process (equals to 32x32 luma samples in VTM-5.0).

Method 1: 0.26/2.94/2.97, 0.05/1.22/1.19, and 0.01/0.13/0.50 Y/U/V for AI, RA, and LB, respectively.

Method 2: 0.08/1.29/1.25, 0.02/0.56/0.42 and 0.05/0.30/0.60 Y/U/V for AI, RA, and LB, respectively.

Method 3: 0.07/1.04/1.02, 0.02/0.41/0.29 and 0.01/0.21/0.82 Y/U/V for AI, RA, and LB, respectively.

In method 1, the QT conditions guarantee the reconstruction scan order and achieve 32x32 latency. The no split condition is the same case as single tree, which provides coding gains.

In method 2 luma QT with chroma no split case, it is asserted that chroma prediction can be executed just after each luma 32x32 by subpart processing, and the CCLM parameter can be derived without waiting additional luma block reconstruction. The BT_H case guarantees the scan order.

In method 3, luma no split with chroma QT is further considered. If given more decoder complexity, luma VPDU could provide QT subpart reconstruction for chroma prediction.

The proposed restrictions apply only for Dual Tree case.

Restrictions on CCLM usage are applied depending on the VDPU luma & chroma partitioning.

Method 1 enables CCLM if Luma & chroma VDPU are both not split, or both QT split. Method 2 and 3 add possibility to use BT horizontal split (then possibly vertical split) at VDPU level.

It was commented that the issue of dependency on the partition information made for CE2-3.1 proposal is strongly reduced by the proposal (compared to CE2-3.1).

The proponent commented that if the CTU size is 32x32 then the maximum latency is reduced to16x16 luma block.

Since BoG recommends to adopt JVET-O1124, no need to take action on this contribution

JVET-O0797 Crosscheck of JVET-O0129 on CCLM with Chroma Separate Tree [C. Chevance (Interdigital)]

JVET-O0196 CE2-related: CCLM for dual tree with 32x32 latency [Y. Zhao, H. Yang (Huawei)]

This contribution proposes to conditionally allow CCLM for chroma CUs in dual tree, by which the luma-chroma latency is reduced from 64x64 to 32x32. The CCLM allowing condition is based on the split mode of 64x64 luma coding tree node and the split mode of 32x32 chroma tree node and that of 32x16 chroma tree node, which is of low complexity and can be determined in parsing of coding_tree(). It is reported that the coding performance is 0.07%/1.06%/1.07% and 0.02%/0.39%/0.35% Y/Cb/Cr BD-rates in AI and RA configurations, respectively.

when the 64x64 luma coding tree node is partitioned with Not Split (and ISP is not used for the 64x64 CU) or QT, the chroma CUs in 32x32 / 32x16 chroma coding tree node are allowed to use CCLM in the following way:

- if the 32x32 chroma node is partitioned with Not Split or QT split, all chroma CUs in the 32x32 node can use CCLM

- if the 32x32 chroma node is partitioned with HBT, and the 32x16 child node does not split or uses VBT split, all chroma CUs in the 32x16 chroma node can use CCLM.

In other luma and chroma coding tree split conditions, CCLM is not allowed for chroma CU.

The proposed restrictions apply only for Dual Tree case.

Proposal has commonalities with JVET-O0129 method3.

Differences are:

- a condition is added on the usage of ISP (CCLM disabled if ISP is used at VDPU level)

- CCLM is disabled in case of BT horizontal split of luma VDPU (which does not have impacts on results because current SDR CTCs do not allow BT split at VDPU level).

It was commented that the ISP constraint t VDPU level is needed to reduce the latency.

The proposed method is further submitted as JVET-O1124.

JVET-O0805 Crosscheck of JVET-O0196 (CE2-related: CCLM for dual tree with 32x32 latency) [C.-M. Tsai (MediaTek)]

JVET-O0611 CE2-3 related: On CCLM restriction for 32x32 luma latency [K. Kawamura, K. Unno, S. Naito (KDDI)]

This contribution proposes the CCLM restriction to reduce the chrome decoding latency from 64x64 luma samples to 32x32. For instance, CCLM enabled flag is introduced and set when the luma block size is equaled to 32x32 and the luma / chroma partitioning is the same. The flag is not changed at the remaining partitioning. This contribution also proposes an encoder optimization of chroma RDO to reduce coding loss. Y/U/V BD-rate performances are 0.32% / 2.81% / 2.94% and 0.20% / 2.74% / 2.88% for the all intra and the random access conditions, respectively.

In this contribution, CCLM enabled flag is introduced into block level and determined at the specific CU depth. Two options are provided:

Option 1: 32x32 luma sample block latency

CCLM enabled flag is set equal to TRUE when the following conditions are true.

- The luma block is partitioned into 32x32 sample blocks by a series of quad trees.

- The chroma block is partitioned into 16x16 sample blocks by a series of quad trees.

Further split is allowed but the flag is not changed. This condition check is applied to both single tree and chroma separate tree.

Option 2: two 32x32 luma sample block latency

When two 32x32 luma sample block latency is acceptable, coding loss caused by the partitioning restriction can be reduced. Condition check is, however, more complicated than the option 1.

It is noted that encoder optimization is conducted in this contribution. Two sequential QTs check of luma/chroma block and chroma block is always applied for single and separate tree case, respectively.

The proposed restrictions apply for Dual Tree and Single Tree.

Preferred option from the proponent is Option1.

One comment was that it may not be required for reducing the latency to impose the chroma block to be 16x16. Latency for a chroma block 32x32 and 4 luma blocks of size 32x32 is still a 32x32 luma block.

No action

JVET-O0708 Crosscheck of JVET-O0611 (CE2-3 related: On CCLM restriction for 32x32 luma latency) [K. Abe (Panasonic)]

JVET-O0273 CE2-related: Luma-chroma latency reduction for chroma separate tree [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this proposal, three methods are proposed to reduce the latency to 32x32 luma samples.

Method 1 imposes split constraints on both luma and chroma CBs. if luma CB size is 64x64-L, only no split and quadtree (QT) split are allowed. If chroma CB size is 32x32-C, only no split and QT split are allowed.

Method 2 is based on method 1 and allows one horizontal binary tree (BT) split option and other split options with implicit transform block (TB) split and intra prediction boundary samples constraints for chroma CBs.

Method 3 is also based on method 1 and allows more horizontal BT split options.

Besides, to avoid the worse-case latency increased by intra sub-partitions (ISP) mode, ISP is disallowed for 64x64 luma CB when CST is enabled in all proposed methods.

It is asserted that the proposed methods preserve most coding gain of CST. Results are reported as follows.

Method 1 – AI: BD-rates = 0.18% (Y), 2.35% (Cb), 2.30% (Cr); EncT=93%; DecT=101%
RA: BD-rates = 0.05% (Y), 0.76% (Cb), 0.84% (Cr); EncT=99%; DecT=98%

Method 2 – AI: BD-rates = 0.04% (Y), 1.19% (Cb), 1.17% (Cr); EncT=99%; DecT=100%
RA: BD-rates = 0.03% (Y), 0.46% (Cb), 0.36% (Cr); EncT=100%; DecT=99%

Method 3 – AI: BD-rates = 0.10% (Y), 1.53% (Cb), 1.49% (Cr); EncT=96%; DecT=100%
RA: BD-rates = 0.03% (Y), 0.46% (Cb), 0.44% (Cr); EncT=99%; DecT=98%

The proposed restrictions apply for Dual Tree only.

Partitioning restrictions are introduced, and CCLM is always enabled.

In addition, ISP is disabled at VDPU level for the Dual Tree case.

Method 1 is similar to JVET-O0524.

Method 2 adds BT split possibilities for chroma, with in some cases implicit TB split of chroma rectangular blocks. Additional neighboring samples availability condition checks are introduced.

It was commented by the proponent that option 3 of method 2 (vertical BT split with implicit TU split) involves vertical scanning order for the parsing, but the reconstruction of 16x16 chroma blocks can still be performed in “normal” scanning order.

Method 3 does not allow vertical BT split of chroma VDPU.

No action

JVET-O1013 Crosscheck of JVET-O0273: CE2-related: Luma-chroma latency reduction for chroma separate tree  [T. Poirier (InterDigital)]

JVET-O0274 CE2-related: Luma-chroma dependency reduction for chroma separate tree by constraining CCLM usage [C.-M. Tsai, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this proposal, two methods are proposed to reduce the latency to 32x32 luma samples.

Method 1 imposes split constraints on both luma and chroma CB. if luma CB size is 64x64-L, only no split and quadtree (QT) split are allowed. If chroma CB size is 32x32-C, only no split and QT split are allowed.

Method 2 imposes constraints only on the enabling of CCLM mode to allow more split options and is implemented as two versions: encoder-side normative constraints and decoder normative changes.

Besides, if intra sub-partitions (ISP) mode is used in the corresponding 64x64 luma CB, CCLM is disallowed in all proposed methods. The results are summarized as follows.

Method 1
AI: BD-rates = 0.18% (Y), 2.43% (Cb), 2.40% (Cr); EncT=93%; DecT=100%
RA: BD-rates = 0.04% (Y), 0.76% (Cb), 0.84% (Cr); EncT=100%; DecT=99%

Method 2, encoder-side normative constraints
AI: BD-rates = 0.10% (Y), 1.21% (Cb), 1.16% (Cr); EncT=98%; DecT=100%
RA: BD-rates = 0.04% (Y), 0.44% (Cb), 0.38% (Cr); EncT=99%; DecT=99%

Method 2, decoder normative changes
AI: BD-rates = 0.07% (Y), 1.03% (Cb), 1.03% (Cr); EncT=98%; DecT=100%
RA: BD-rates = 0.02% (Y), 0.34% (Cb), 0.34% (Cr); EncT=99%; DecT=100%

The same ISP restriction as in JVET-O0196 is introduced (when ISP is used for 64x64 luma CB, CCLM is disabled).

Method 1 introduces the same partitioning restrictions as in JVET-O0273 and JVET-O0524 (CCLM enables only if no split or QT split at VDPU level).

Method 2 is built on top of method 1 and introduces CCLM usage restrictions based on partitioning.

Method 2 adds possibility of any split of chroma 32x32 CB when luma 64x64 Cb is not split.

It was commented that this may involve implementation issues because all luma prediction modules should be adapted to chroma partitioning/split.

Method 2 is tested either with encoder constraints, or with normative decoder changes.

Spec text is not available yet.

No action

JVET-O0754 Crosscheck of JVET-O0274 (CE2-related: Luma-chroma dependency reduction for chroma separate tree by constraining CCLM usage) [	Y. Zhao (Huawei)]

JVET-O0524 Non-CE2: alternative solutions for reducing the luma-chroma latency [T. Poirier, E. François, F. Le Léannec (InterDigital)]

This contribution proposes partitioning restrictions to reduce the latency due to chroma-to-luma dependency of CCLM and Chroma Residual Coding (CRS) coding tools. The proposal consists in enabling BT and TT partitioning only for luma blocks of size lower than or equal to 32x32 samples, and for chroma blocks of size lower than or equal to 16x16 samples. Above those dimensions, only NoSplit or QT split are enabled. These restrictions reportedly reduce the chroma reconstruction latency from one 64x64 luma block to one 32x32 luma block.

Three tests are reported. In Test1, the partitioning restrictions are applied without other change to VTM5. In Test2, the partitioning restrictions are applied, and CRS is activated in dual tree with the CRS factor derived from collocated reconstructed luma samples. In Test3, the partitioning restrictions are applied, and CRS is activated in dual tree with the CRS factor derived from reconstructed luma samples neighboring the luma block colocated with the chroma block.

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| SDR test1 | 0.17% | 2.35% | 2.30% | 0.04% | 0.77% | 0.78% | 0.04% | 0.57% | 0.50% |
| SDR test2 | 0.18% | 2.11% | 1.86% | 0.07% | 0.15% | 0.18% | 0.03% | 0.04% | 0.28% |
| SDR test3 | 0.18% | 2.09% | 1.84% | 0.07% | 0.15% | 0.17% | 0.01% | 0.03% | 0.06% |

For HDR PQ content, the CTC anchors do not use LMCS. One additional test is reported with LMCS on and without using CRS in DualTree (“HDR LMCS”). Reported BD-rate DE100, PSNR-L100, wPSNR-Y, U and V variations for HDR PQ are summarized in the table below. Using CRS in DualTree (Test2 and Test3) is reported to be beneficial for the luma / chroma balance when LMCS is activated.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL | wY | wU | wV | DE100 | PSNRL | wY | wU | wV |
| HDR Test1 | 5.82% | 0.24% | 0.23% | 5.89% | 8.70% | 4.70% | 0.48% | 0.13% | 3.21% | 4.73% |
| HDR LMCS | 14.36% | -1.84% | -1.41% | 20.50% | 17.61% | 10.94% | -1.69% | -1.51% | 17.77% | 11.81% |
| HDR Test2 | 6.42% | -1.48% | -1.03% | 9.70% | 4.40% | 4.54% | -1.46% | -1.24% | 8.18% | 1.70% |
| HDR Test3 | 6.52% | -1.47% | -1.01% | 9.72% | 4.72% | 4.79% | -1.45% | -1.23% | 8.16% | 1.80% |

The main difference from JVET-O0129/JVET-0196/JVET-O0611 is that it restricts partitioning in dual tree and always allows CCLM to reduce latency.

It was asserted by the proponent that the latency issue in both CCLM and CRS is addressed by the proposed constraints.

It was commented that compared with contributions with CCLM enabling condition, it suffers slightly bigger loss.

It was commented that disallowing the binary split in VPDU level doesn’t affect the coding performance in CTC, but loss potential gain from VPDU level binary splitting.

No action

JVET-O0806 Crosscheck of JVET-O0524 (Non-CE2: Alternative solutions for reducing the luma-chroma latency) [C.-M. Tsai (MediaTek)]

JVET-O0668 Non-CE2: CCLM latency reduction for single tree [T. Biatek, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes block-size dependant CCLM activation for single tree case. It enables to comply with 32x32 luma reconstruction latency during CLLM process. Results show limited losses over single-tree anchor with 0.05%(Y)/0.55%(U)/0.53%(V) BD-Rate in all-intra configuration

The proposed method enables CCLM for a chroma CU only when the luma CU dimension (W,H) respects the following condition:

W*H≤256

It guarantees that chroma block reconstruction using CCLM can be started after reconstruction of 32*32 luma samples.

The proposal focuses on the chroma-to-luma latency issue for the single tree.

Tests were performed by disabling Dual Tree for all I-slices (not as in SDR CTCs) to evaluate the impact of the change in Single Tree only case.

It was observed that a loss of around 0.2% in luma and 2% in chroma is observed in class 1, mostly because of Tango sequence.

Whether the chroma-to-luma latency is an issue in single tree has to be confirmed.

The cross-check document is not yet uploaded. The cross-checker is in the room and confirms mapping results.

No action

JVET-O1124 Draft text for CCLM restriction to reduce luma-chroma latency for chroma separate tree [Yin Zhao, Haitao Yang (Huawei), Chia-Ming Tsai, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei (MediaTek)]

This contribution provides the text and complexity analysis of JVET-O0196.

In the Sunday discussion, it is agreed to recommend adoption of this contribution.

Luma-chroma latency reduction for CRS

JVET-O0130 Non-CE2: CRS with Chroma Separate Tree [C.-W. Kuo, J. Li, C. Lim (Panasonic)]

This contribution proposes to enable CRS with chroma separate tree and introduce constraints on CRS to reduce the chroma reconstruction latency to ¼ luma VPDU reconstruction process (equals to 32x32 luma samples in VTM-5.0).

Simulation results report in method 1, 0.02/-0.32/-0.4, 0.02/-0.48/-0.52, Y/U/V BD-rate change in AI, RA,, respectively. In method 2, 0.03/-0.31/-0.45, 0.02/-0.52/-0.61 Y/U/V BD-rate change in AI, RA, respectively.

In this contribution, it is proposed to use VPDU level partitioning information for CRS enabling condition checks.

In method 1, only if both luma VPDU and chroma VPDU is not split or quad split, CRS is enabled.

In method 2, CRS is enabled when 1)both luma VPDU and chroma VPDU is not split; or 2)luma VPDU is quad split, chroma VPDU is no split or quad split or horizontally bi-split(no further split or followed by vertical bi-split).

In method 3, luma no split with chroma QT is further considered. If given more decoder complexity, luma VPDU could provide QT subpart reconstruction for chroma prediction. The case luma no split with chroma BT_H or BT_V is similar. The BT_H and BT_V are supported for luma VPDU size in the current VVC draft, so luma BT_H and BT_V are syntax-wise considered

As in JVET-N0389, collocated luma area is used for DC calculation in both single tree and dual tree cases

It was commented that in JVET-O0524 CRS is enabled with the constraint on partitioning and that constraint is the same as method 1 in this contribution, only difference is that JVET-O0524 imposes constraint on partitioning while this contribution imposes CRS enabling condition

It is noted that HDR PQ results are not available.

No action

JVET-O0723 Crosscheck of JVET-O0130 (Non-CE2: CRS with Chroma Separate Tree) [T. Biatek (Qualcomm)]

JVET-O0798 Crosscheck of JVET-O0130 on CRS with Chroma Separate Tree [C. Chevance (Interdigital)]

JVET-O0232 CE2-related: No-latency LMCS Chroma Residual Scaling [J. Zhao, S. Kim (LGE)]

In this contribution, a simplified chroma scaling method is introduced to remove the dependency of LMCS chroma residual scaling on corresponding luma block by using a single scale factor per picture, and therefore eliminates the latency of chroma residual scaling.

This single chroma scale factor may be inferred at decoder by linear approximation of the luma LMCS model or signalled from bitstream. Two sets of results are provided:
(1) Results of only applying to Inter (single tree) slices are: -0.02%, -0.08%, -0.04% for Y,U,V respectively in RA configuration, and in 0.00%, 0.16%, 0.07% for Y,U,V in LD configuration.

(2) Results of applying to both Inter and Intra (dual tree) slices are: 0.06%, -0.62%, -0.75% in AI configuration; 0.01%, -0.61%, -0.57% in RA configuration and -0.02%, -0.36%, -0.34% in LD configuration. It balances the bit allocation better between luma and chroma for dual tree cases.

According to HDR PQ test, current LMCS has loss in chroma because CS is enabled in intra slice.

HDR PQ results of applying linear model both to single tree and dual tree:

AI:

| DE | L100 | wpsnrY | wpsnrU | wpsnrV |
| --- | --- | --- | --- | --- |
| 5.42% | -1.61% | -1.14% | 7.02% | -0.97% |

RA:

| DE | L100 | wpsnrY | wpsnrU | wpsnrV |
| --- | --- | --- | --- | --- |
| 3.59% | -1.40% | -1.20% | 5.74% | -3.51% |

It was observed by the proponent that the slopes of the different pieces in LM model are quite similar to each other for SDR sequences. Based on this observation, one-piece linear model is proposed in this contribution.

It was commented that for SDR, using picture-level, CTU-level or VPDU level factor doesn’t impact coding performance much. But it was also commented for HDR, the results are different. Localized scaling factor is better than global factor

It was questioned that whether using only one factor for chroma scaling for the whole picture will introduce any subjective artifacts.It was commented it should be carefully check the perceptual quality especially in HDR sequences before we change the localized scaling to global factor.

It was commented that the performance should be compared with changing chroma QP.

it was commented that between two options of explicitly signaling and implicit derivation, explicitly signaling is preferable.

It was commented that it is better to keep one solution both for SDR and HDR for hardware implementation.

No action

JVET-O0745 Crosscheck of JVET-O0232 (CE2-related: No-latency LMCS Chroma Residual Scaling) [T. Lu (Dolby)]

JVET-O0271 CE2-related: Chroma scaling for CIIP [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, three methods are proposed to reduce the luma-chroma dependency of the chroma scaling.

In method 1, for each chroma transform block (TB) of intra coding unit (CU), the top-left reconstructed boundary sample of the collocated luma TB is used as the reference luma sample. For each chroma TB of non-intra (inter or intra block copy) CU, the top-left prediction sample of the collocated luma TB is used as the reference luma sample.

In method 2, for each chroma TB of intra CU, the top-left reference boundary sample (i.e., reconstructed boundary sample after smoothing) of the collocated luma TB is used as the reference luma sample. For each chroma TB of non-intra CU, the top-left prediction sample of the collocated luma TB is used as the reference luma sample.

For the above 2 methods, two schemes are provided. In scheme 1, CIIP is treated as inter prediction mode for chroma scaling, and the reference luma sample is the top-left inter prediction sample of the collocated luma TB. In scheme 2, CIIP is treated as intra prediction mode for chroma scaling, and the reference luma sample is the top-left reconstructed/reference boundary sample of the collocated luma TB.

BD-rate results of method 1.1 (method 1 with scheme 1) are reported as follows.
VTM5.0-RA: {BD-rates = 0.01% (Y), -0.05% (Cb), -0.05% (Cr)}
VTM5.0-LB: {BD-rates = -0.01% (Y), 0.04% (Cb), -0.12% (Cr)}

BD-rate results of method 1.2 (method 1 with scheme 2) are reported as follows.
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.02% (Cb), -0.11% (Cr)}
VTM5.0-LB: {BD-rates = 0.02% (Y), 0.06% (Cb), 0.01% (Cr)}

BD-rate results of method 2.1 (method 2 with scheme 1) are reported as follows.
VTM5.0-RA: {BD-rates = 0.01% (Y), -0.05% (Cb), -0.03% (Cr)}
VTM5.0-LB: {BD-rates = 0.03% (Y), -0.05% (Cb), 0.02% (Cr)}

BD-rate results of method 2.2 (method 2 with scheme 2) are reported as follows.
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.03% (Cb), -0.07% (Cr)}
VTM5.0-LB: {BD-rates = 0.01% (Y), 0.08% (Cb), -0.16% (Cr)}

In method 3, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB regardless of the current CU mode, and the chroma scaling factor of the the first chroma TB in one 64x64-luma/32x32-chroma (64x64-L/32x32-C) unit is shared in the entire 64x64-L/32x32-C unit.

BD-rate results of method 3 are reported as follows.
VTM5.0-RA: {BD-rates = -0.01% (Y), 0.10% (Cb), 0.08% (Cr)}
VTM5.0-LB: {BD-rates = 0.00% (Y), 0.09% (Cb), -0.11% (Cr)}

In method 4, based on the method 3, chroma scaling is also applied to dual tree. In both dual tree and single tree, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB regardless of the current CU mode, and the chroma scaling factor of the the first chroma TB in one 64x64-L/32x32-C unit is shared in the entire 64x64-L/32x32-C unit.

BD-rate results of method 4 are reported as follows.
VTM5.0-AI : {BD-rates = 0.03% (Y), -0.37% (Cb), -0.51% (Cr)}
VTM5.0-RA: {BD-rates = 0.02% (Y), -0.39% (Cb), -0.41% (Cr)}
VTM5.0-LB: {BD-rates = 0.04% (Y), -0.12% (Cb), -0.47% (Cr)}

It was commented that method 4 is similar as JVET-O0429 option 2. The difference is method 4 only uses one reconstructed sample and JVET-O0429 option 2 uses averaged value of multiple reconstructed samples. The coding performance of using multiple reconstructed samples (0.84% chroma gain estimated by reduction bug fixing gain from total gain of JVET-O0429 option 2) is better than using only one sample (0.39% chroma gain), which indicates only using one samples may not be accurate enough. And from the CE2 HDR test results (CE2-2.1a vs. CE2-2.1.e), it can also be observed that using more samples does give benefit in term of coding performance.

HDR PQ results are not provided.

No action

JVET-O0781 Crosscheck of JVET-O0271: CE2-related: Chroma scaling for CIIP [X. Xiu (Kwai Inc.)]

JVET-O0429 Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree [T. Lu, P. Yin (Dolby), E. François, F. Hiron (InterDigital)]

In this contribution, two options are proposed to enable chroma residual scaling (CRS) for separate luma/chroma tree (DualTree). The two options can also apply on single/shared tree case (SingleTree) to provide a unified CRS approach for both DualTree and SingleTree.

In Option 1, the index of the CRS factor is explicitly coded/decoded per CTU, thus the chroma-to-luma dependency and pipeline latency are completely removed.

In Option 2, the CRS factor is derived per VPDU from reconstructed luma samples from a neighboring VPDU, thus removing the pipeline latency.

An encoder bug fix related to a rounding operation in CRS lambda calculation was found in the original LMCS encoder implementation and is fixed in the tests.

For enabling CRS in DualTree (with SingleTree remaining the same as the current CRS design in VTM):

For Option 1 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI: 		Y/U/V {0.05%, -0.17%, -0.31%} EncT:   99% DecT: 99%

  - RA: 	Y/U/V {0.05%, -0.71%, -0.78%} EncT:   101% DecT: 100%

  - LDB:	Y/U/V {0.03%, -0.47%, -0.86%} EncT:   99% DecT: 99%

  - HDR-PQ AI: 	DE/L100/wPSNRY {2.22%, -1.71%, -1.26%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.07%, -1.57%, -1.37%}

For Option 2 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI:   	Y/U/V { 0.03%, -0.37%, -0.49%} EncT: 101% DecT: 101%

  - RA:   	Y/U/V {0.05%, -1.27%, -1.22%} EncT: 100% DecT: 100%

  - LDB: 	Y/U/V {0.03%, -0.26%, -0.72%} EncT:   99% DecT: 100%

  - HDR-PQ AI:  	DE/L100/wPSNRY {1.94%, -1.66%, -1.23%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.14%, -1.48%, -1.30%}

For unified CRS in both DualTree and SingleTree:

For Option 1 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI: 		Y/U/V {0.03%, -0.23%, -0.33%} EncT:   99% DecT: 100%

  - RA: 	Y/U/V {0.01%, -0.53%, -0.54%} EncT:   100% DecT: 100%

  - LDB:

  - HDR-PQ AI: 	DE/L100/wPSNRY {2.22%, -1.71%, -1.26%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.01%, -1.49%, -1.30%}

For Option 2 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI:   	Y/U/V { 0.03%	-0.37%	-0.49%} EncT: 101% DecT: 101%

  - RA:   	Y/U/V {0.08%	-1.24%	-1.22%} EncT: 101% DecT: 100%

  - LDB: 	Y/U/V {0.01%	-0.46%	-0.62%} EncT: 100% DecT: 99%

  - HDR-PQ AI:  	DE/L100/wPSNRY {1.94%, -1.66%, -1.23%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.08%, -1.47%, -1.30%}

It was commented that in option 1, the signaling overhead is a little big if both apply to single tree and dual tree according to the test results.

It was commented that the coding gain of chroma is significant if CRS is enabled in dual tree according to HDR PQ results.

The contribution includes a bug fix to remove unnecessary floating point rounding during lambda derivation in encoder.

The bug fix gives -0.44% chroma gain in RA for SDR, but has very slight loss in HDR sequence.

Recommendation: adopt (SW) bug fix part (encoder lambda rounding bug fix), since the fixing is technically reasonable.

JVET-O0744 Crosscheck of JVET-O0429 (Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree) [K. Zhang (Bytedance)]

JVET-O1044 Crosscheck of JVET-O0429 (Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree) [Y. Zhang, D. Rusanovskyy (Qualcomm)]

JVET-O0618 CE2-related: Removal of luma-chroma dependency in LMCS [Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang (Bytedance)]

In this contribution, a fixed chroma scaling factor for each LMCS model is used. This fixed chroma scaling factor is calculated once according to a representative luma sample, and then used for all blocks that refer to the same LMCS model. Two methods are provided to select the representative luma sample. In method #1, the fixed chroma scaling factor is derived as the median piece-wise function index of the LMCS model (e.g., the median value among the signaled LMCS minimum bin index and the signaled LMCS maximum bin index). In method #2, the fixed chroma scaling factor is derived as the piece-wise function index calculated from the median value of reshaped luma values.

For SDR content under the CTC, the overall BD-rate changes for {Y, Cb, Cr} are listed as follows.

| Method #1 | RA | -0.01% | -0.03% | -0.03% |
| --- | --- | --- | --- | --- |
|  | LDB | 0.01% | 0.06% | -0.11% |
| Method #2 | RA | -0.01% | -0.08% | -0.07% |
|  | LDB | 0.01% | 0.09% | -0.14% |

For HDR

Method #1 AI -1.64%(wPSNRY), 14.15%(wPSNRU), 8.34%(wPSNRV)

RA-1.61%(wPSNRY), 13.87%(wPSNRU).5.89%(wPSNRV)

Method #2 AI -0.20%(wPSNRY), -2.68%(wPSNRU),-12.07%(wPSNRV)

RA-0.49%(wPSNRY), -4.14%(wPSNRU),-13.73%(wPSNRV)

It was commented that two methods have different directions for luma-chroma performance trade-off , it is because HDR curve is not linear, so picking slope of different of pieces may lead to quite different coding performance. And it was also commented it is interesting to see the subjective quality difference between two methods.

It is agreed that this contribution (same as JVET-O0232) totally removes luma-chroma dependency.

It was commented that explicitly signaling of scaling factor is preferable

It was commented the proposal can be thought of as a solution at different spatial scales, for example, a single value for the entire slice, alternatively value at VDPU/CTU/CU/TU level. It is agreed that picture/slice level scaling factor is simplest solution. But the question is it may be too simple, because it normatively loses flexibility to adapt chroma on local level.

It is commented that a unified design for HDR and SDR is desirable .

It is commented that explicitly signaling is more interesting.

No action

JVET-O0767 Crosscheck of JVET-O0618: CE2-related: Removal of luma-chroma dependency in LMCS [J. Zhao (LGE)]

JVET-O0627 CE2-related: luma-chroma dependency reduction on chroma scaling [Alibaba]

This contribution proposes to use CTU neighboring reconstructed samples to derive chroma scaling factor to solve the dependency issue. For each CTU, the left and top neighboring reconstructed luma samples are used. First, the average value of M left neighboring luma samples and M top neighboring luma samples which are the closest to the top-left position avgY, is calculated. And then the same method in VVC draft 5 is used to derive the chroma scaling factor based on avgY. The factor derived on CTU level is used for all the chroma blocks in this CTU.

It is reported that the performance impact for {Y, U, V} is {0.00%, -0.07%, -0.06%} for RA and {0.02%, 0.00%, -0.20%} for LDB in the case of using 4 left and 4 up neighbors, and {0.00%, -0.09%, -0.13%} for RA and {-0.01%, -0.07%, -0.19%} for LDB in the case of using 16 left and 16 up neighbors if the CTU-level derivation method is applied only in case of single tree. And by additionally applying the proposed method in the case of dual tree, and enabling the chroma scaling in dual tree, the total coding performance is {0.05%, -0.49%, -0.65%} for AI, {0.03%, -0.65%, -0.66%} for RA and {-0.02%, -0.59%, -0.51%} for LDB in the case of using 4 left and 4 up neighbors, and {0.04%, -0.43%, -0.59%} for AI, {0.03%, -0.67%, -0.67%} for RA and {0.02%, -0.47%, -0.53%} for LDB in the case of using 16 left and 16 up neighbors.

No action

JVET-O0691 Crosscheck of JVET-O0627 (CE2-related: luma-chroma dependency reduction on chroma scaling) [Y. Sun, S. Ye, F. Chen, L. Wang (Hikvision)]

JVET-O0747 Crosscheck of JVET-O0627 (CE2-related: luma-chroma dependency reduction on chroma scaling) [T. Lu (Dolby)]

JVET-O0550 CE2-related: Further simplification of chroma residual scaling factors derivation [M. Sychev, E. Alshina (Huawei)]

The first aspect of Intra simplification proposes to use luma DC prediction value instead of using an average value of luma samples for chroma scaling factor derivation. The second aspect about using shifted reference line with offset depended on directional intra prediction angle. Third aspect related to pipeline latency reduction in Inter processing by using reference samples corresponded to displacement with integer motion vector of current block.

Aspect 1: {0.01%, -0.06%,-0.10%} in RA

Aspect 2: {0.01%,-0.01%,-0.08%} in RA

Aspect 3: {}

It is reported by crosschecker that mismatches happen in aspect 2 & 3

Considering adoption of JVET-O1124 which uses VPDU neighboring reconstructed samples derive the factor, aspect 1 & 2 are not relevant.

Aspect 3 is considered to be further studied in CE if there is next round CE on CRS.

Finally, it is decided not to run the next round CE

No action

JVET-O1064 Crosscheck of JVET-O0550 (CE2-related: Further simplification of chroma residual scaling factors derivation) [S.-T. Hsiang (MediaTek)]

JVET-O0608 CE2-related: On latency reduction for chroma residual scaling [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, S. Ye, X. Wang (Kwai Inc.)]

This contribution proposes two methods to reduce the latency for the chroma residual scaling. In the first method, it is proposed to use the luma prediction samples obtained prior to decoder-side motion refinement (DMVR), bidirectional optical flow (BDOF) and combined inter and intra prediction (CIIP) to calculate the chroma residual scaling factor. In the second method, it is proposed to always use luma uni-prediction samples to derive the scaling factor for chroma residuals. When the current CU is bi-predicted, the L0 prediction samples are always used in the second method.

Compared to the VTM-5.0 anchors, the first method reportedly provides {Y, U, V} BD-rate variations of {0.00%, -0.03%, -0.06%} and {0.02%, 0.16%, -0.07%} for RA and LD configurations, respectively. The corresponding {Y, U, V} BD-rate variations of the second method are {0.00%, -0.04%, -0.05%} and {0.02%, 0.16%, -0.07%}.

Additionally, complementary experiments are performed to combine the proposed two methods with the subblock based chroma residual scaling method in CE2-1.1. After such combination, for inter CUs, only up to 256 top-left luma prediction samples, which are obtained before the DMVR, the BDOF and the CIIP (Method #1) or obtained from the initial uni-prediction, are used to calculate the chroma residual scaling factors.

It was commented that this proposal reduces the current pipeline latency, but not completely remove the pipeline latency.

The proposed method is similar with JVET-O0271 method 1/method2 in which inter predictors before intra combination (if CIIP) are used to derive the scaling factor.

It is suggested by one expert that if implicitly derivation of the chroma residue scaling factor is relevant, this proposal and other relevant contributions need to be further studied in CE.

As discussed in Sunday, since VPDU level derivation is recommended to adoption, no need to further study this contribution in CE

No action.

JVET-O1026 	Crosscheck of JVET-O0608 (CE2-related: On latency reduction for chroma residual scaling) [Z.-Y. Lin (MediaTek)]

JVET-O1109 Non-CE2: unification of chroma residual scaling design[J. Chen, R. Liao, Y. Ye, J. Luo (Alibaba), Z. Deng, L. Zhang (ByteDance), T. Lu, P. Yin (Dolby), E. François, F. Hiron (InterDigital), J. Zhao, S. Paluri, S. H. Kim (LGE), Z.-Y. Lin, T.-D. Chuang (MediaTek)]

In this contribution, a unified chroma residual scaling design for both dual tree and single tree is proposed. In the proposed solution, a high-level switch in SPS enables switching between explicit signalling or implicit derivation of the scaling values. The explicit signalling spatial granularity can be controlled from CTU-scale to slice-scale. The implicit derivation is applied per VPDU using neighboring reconstructed luma samples of the VPDU. The proposed solution applies both for Dual Tree and Single Tree cases. It is asserted that the proposed solution removes pipeline latency for chroma residual scaling. In addition, explicit signalling removes the luma-chroma dependency too.

The simulation results of the proposed solution are reported as follows:

For SDR:

- AI: 	Y/U/V	0.07%, -0.85%, -0.95%

- RA: 	Y/U/V	0.05%, -1.19%, -1.02%

- LDB:	Y/U/V

For HDR-PQ:

- HDR-PQ AI: 	DE/L100/wPSNRY/U/V	1.77%, -1.70%, -1.24%, 5.54%, 1.17%

- HDR-PQ RA: 	DE/L100/wPSNRY/U/V	2.07%, -1.50%, -1.31%, 7.04%, 3.04%

It is asserted that the proposed design significantly improves chroma performance compared to the VTM5.0 anchor for both SDR and HDR-PQ content with negligible encoding and decoding runtime change.

(chaired by G. Van der Auwera)

SPS level flag indicates whether impl or expl is used.

Applies for both dual and single tree cases.

It was asked for implicit case, at VPDU level 16 top and left samples are used. The value 16 is a typo in the slides and the value is actually determined by VPDU size. The latency problem is primarily for the Dual Tree case. Comparing the explicit with implicit cases, for SDR implicit option has very similar coding performance than explicit (AI implicit: 0.03%/-0.37%/-0.49%; AI explicit 0.07%/-0.85%/-0.95%; RA implicit 0.08%/-1.24%/-1.22%; RA explicit :0.05%/-1.19%/-0.95%). For HDR PQ case, there is no clear evidence that implicit case is better (explicit : DE100 2.22% loss; implicit : 1.77% loss; while L100 metric values are similar).

It is commented that it better to have only one case as suggested by two non-proponents from an implementation perspective, although it is commented that the explicit case does not have much additional complexity compared with implicit case and implementing both is doable. However, implicit would give more reliability, which would make VVC usable as is with high quality while offering the option of encoder optimization with the explicit option.

One more comment is that local chroma QP adaptation (discussion is ongoing still – JVET-O0737 is to be revisited) being considered and this may interact with the proposal, so that there may be multiple approaches that achieve similar result. The chroma QP adaptation has additional use cases compared with chroma residual scaling. It is generally agreed that the granularity of chroma residual scaling is smaller than what is achievable with QP adaptation. The spatial granularity of chroma residual scaling is larger than the granularity of chroma QP adaptation. The granularity at which chroma residual scaling is applied for implicit and explicit cases is different. For the explicit case, the finest granularity is at CTU level, while for the implicit case the granularity is at the VPDU level.

BoG recommends adoption.

LMCS simplification/modification

JVET-O0212 On an enable condition for LMCS [T.Chujoh, T.Ikai (Sharp)]

in this contribution, an enable condition on the syntax of LMCS is proposed. If any flag of WP (weighted prediction), which is an effective tool for fading sequences, is not true, LMCS can be applied.

Performance is provided on fade sequences which are generated with a fading tool in JVET-L0201-v1

It was commented there is no need to do normative change. Encoder can turn off LMCS simply by setting sps_lmcs_enable_flag to zero when weighted prediction is on. On the other hand, normative change forces LMCS be disabled when weighted prediction is on which may lose potential coding gain if a smart encoder can have LMCS algorithm for fade sequences.

No action.

JVET-O0715 Crosscheck of JVET-O0212 (On an enable condition for LMCS) [P. Bordes (InterDigital)]

JVET-O0233 CE2-related: Linear LMCS [J. Zhao, S. Paluri, S. Kim (LGE)]

This contribution proposes a simplified version of LMCS for SDR, referred as linear LMCS, with only one linear piece between the start and end flat pieces. With linear LMCS, it achieves performance close to LMCS in VTM5. Results are 0.05%, 0.93%, 1.05% for Y,U,V respectively in AI configuration; 0.04%, -0.05%, -0.07% for Y, U, V respectively in RA configuration, and 0.11%, -0.16%, -0.29% for Y, U, V respectively in LD configuration.

The linear approximation is done as following. Keep p_min the same, and only adjust the mapped value at p_max, the pivot point after adjustment is denoted as pL. PL is determined by minimizing the weighted error between the PWL pivot points and the virtual pivot point if connecting the p_min and p_max. The weight is based on histogram of a piece. Histogram is already known during LMCS SDR statistics collection. The linear approximation line is then equally divided into (LmcsMaxBinIdx- lums_min_bin_idx +1) pieces, then it can easily reuse existing 16-piece LMCS structure.

HDR results are not provided.

It was commented that the loss (0.11%) in LDB is relatively large if we consider that the gain of LMCS is only about 0.6%.

It was commented that by adding a linear model, decoder need to both support piece-wise linear model and linear model. So it is not a real simplification. However, it was also commented that by using linear model, encoder can change LMCS model more frequently without big burden of model signaling since only one codeword need to be signaled in linear model. And that may give some potential coding gain.

Further discussion on Sunday

Since JVET-O1109 is recommended to adopt, there is no need to study this contribution in CE

No action

JVET-O0796 Crosscheck of JVET-O0233 on Linear LMCS [C. Chevance (Interdigital)]

JVET-O0262 Non-CE2: Removal of division operations in LMCS [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)]

This contribution proposes to replace the division operation in LMCS by a lookup table. The lookup table has 64 entries and each stores an 8-bit integer. Simulation results reportedly show 0.00% and 0.00% BD-rate changing under AI and RA configurations, respectively, with no impact on encoding/decoding time.

The crosschecker confirms that the modification also has no big coding performance impact on HDR QP sequences.

It is claimed that the proponent that further reduction of lookup table size was tested but suffered signification loss. So the current lookup table size is a good tradeoff.

Recommendation: adopt

(The recommendation was reverted in track B on Saturday)

JVET-O0746 Crosscheck of JVET-O0262 (Non-CE2: Removal of division operations in LMCS) [T. Lu (Dolby)]

JVET-O0272 CE2-related: Simplified inverse luma mapping [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In the inverse luma mapping of the VTM5.0 luma mapping chroma scaling (LMCS), for each luma sample, at most 15 comparisons are required to allocate the interval of the luma sample. Since these 15 comparisons are done for each mapped luma sample. In this contribution, one method is proposed to solve the above issue. The value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range. For each mapped luma sample, its segment index can be easily derived by right shifting the mapped luma sample value by five bits, and the interval index of the mapped luma sample is derived by comparing its value with a threshold pre-calculated for the segment. Only one comparison is needed for each luma sample. It is reported that the proposed method shows no BD-rate changes with negligible runtime changes under the common test conditions (CTC).

It was commented that this contribution introduces a 32-entries lookup table for inv-mapping. It is noted that when the LMCS was adopted, 32 piece-wise linear model was changed to 16 piece-wise linear model to reduce the size of lookup table.

It is asserted by the proponent that the impact of this contribution to the spec. is only adding an encoder constraint.

It is asserted by the proponent that by adoption of this encoder constraint, the decoder can implement inv-mapping in an alternative way with a 32 equal entries lookup but only one comparison.

It is requested to run the HDR PQ sequences.

The text is provided on Sunday discussion.

Only one encoder constraint is imposed as “For the variable LmcsPivot[ i ] with i = 0..15, it is a requirement of bitstream conformance that if LmcsPivot[ i ] is not multiples of 32, then (LmcsPivot[ i ] >> 5) shall not be equal to  (LmcsPivot[ i + 1 ]>>5).”

HDR results will be provided before track. If there is no impact on HDR, BoG recommend to adopt.

JVET-O0910 Crosscheck of JVET-O0272: CE2-related: Simplified inverse luma mapping [W. Zhu (Bytedance)]

JVET-O0428 AHG16/AHG17: LMCS related clean-ups [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Zhao, S.H. Kim (LGE)]

In this contribution three groups of software and specification changes are proposed to fix a bug, make the code more compact and readable, and add constraints on LMCS APSs to ease implementation. The proposed changes are:

- Bug fix in the process of identifying the index of inverse piecewise function (clause 8.8.2.3);

- LMCS code changes to improve readability, compactness, and software-specification correspondence without affecting coding performance;

- Constraints on LMCS APS,

  - The total number of LMCS APS per CVS shall be less than or equal to 4,

  - Only one LMCS model per picture shall be allowed.

Only the first aspect changes the coding performance. The performance impact is {0.00%, 0.00%, 0.00%} for AI, {0.0%,0.00%,0.00%} for RA and {0.00%, -0.01%,0.00%} for LDB

It is asserted by the proponent that the coding performance impact on HDR PQ sequences of aspect 1 also negligible, and aspect 2 doesn’t impact coding performance.

It is noted that the aspect 3 is requested by the hardware experts.

Recommendation: adopt aspect 1 & aspect 2 & aspect 3. And further confirm text of aspect 3 with HLS experts in track.

JVET-O0842 Crosscheck of JVET-O0428: AHG16/AHG17: LMCS related clean-ups [J. Chen (Alibaba)]

JVET-O0448 Non-CE2: Inverse LUT carriage for LMCS [P. Andrivon, E. François, C. Chevance, H. Hiron, R. Jullian (InterDigital)]

This contribution relates to the luma mapping with chroma scaling (LMCS). LMCS involves at the decoder an inverse mapping as well as a chroma residual scaling. In addition, in inter slices a forward mapping is also applied to luma prediction samples. By design, in current VTM, the inverse mapping curve is non-uniformly sampled, while the forward mapping curve is. Consequently, the inverse mapping requires extra-processing such as a binary search algorithm also employed for chroma residual scaling. This contribution proposes to carry a uniformly sampled inverse mapping function. The forward mapping function is derived from carried inverse mapping function. It is asserted that the proposition simplifies the intra picture decoding, inverse mapping and chroma residual scaling processes. Performance for both SDR and HDR is reported to be comparable to the current LMCS design using a carried forward mapping function. Encoding and decoding times are also reportedly similar to VTM-5.0 anchor with LMCS enabled

Option 1: only signal uniform inverse mapping

Option 2: adding a flag to switch between signaling uniform inverse mapping or signaling uniform forward mapping

Option 3: option 1/2 + last pivot point extension and clipping

It is commented that forward mapping is conducted much more frequently than inverse mapping in encoder side.

It is commented that in case of intra only encoding, no forward mapping is needed.

Need to check with hardware people if it is acceptable to use non-uniform forward mapping in prediction loop.

Revisit on Sunday, July 7, 2019

It is confirmed by the hardware expert, there is no benefits to hardware implementation by doing it.

No action.

JVET-O0992 Crosscheck of JVET-O0448 (Non-CE2: Inverse LUT carriage for LMCS) [C.-W. Kuo (Panasonic)

]

JVET-O0603 CE2-related: Prediction dependent luma residual scaling for adaptive in-loop re-shaper [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, S. Ye, X. Wang (Kwai Inc.)]

this contribution proposes to extend the existing chroma residual scaling method to luma component in which the scaling factor of luma residuals are calculated based on the corresponding luma prediction samples. It is asserted that after applying the proposed method all the in-loop mapping of the predicted and reconstructed luma samples in the existing LMCS design can be removed. Compared to the VTM-5.0 anchors, the proposed method reportedly leads to BD-rate variations {0.04%, 0.06%, 0.12%} and {-0.05%, -0.12%, -0.13%} for AI and RA configurations, respectively.

In this contribution, except for the residual decoding modules (e.g., the inverse quantization and the inverse transform), all the other decoding modules (including intra/inter prediction, reconstruction and all in-loop filters) are operating in the original sample domain.

instead of deriving the scaling factor merely from the segment of the piecewise linear model where one luma prediction sample belongs to, the scaling factor is derived as the average of two neighboring segments.

It is asserted by the proponent that using averaged value of two segments instead of only one segment increase accuracy of the residual scaling, especially in the case of large residuals.

It is clarified that in the proposed design each luma pixel has an individual scaling factor, and it is mentioned by the proponent that 4x4-pixel scaling factor suffers some coding performance loss.

It is asserted by the proponent that the signaling is not changed.

It is suggested to check subjective quality.

HDR results are not provided.

And HDR PQ shall be tested

Further discussion in Sunday

The next round CE is not conducted.

No action on this contribution

JVET-O0844 Crosscheck of JVET-O0603: CE2-related: Prediction dependent luma residual scaling for adaptive in-loop re-shaper [J. Chen (Alibaba)]