| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1115-v1 |
| --- | --- |

| Title: | BoG report on CE6 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | BoG |  |  |

_____________________________

# Summary

This BoG is mandated to review the CE6 related contributions of the 15th JVET meeting. Totally 10 contributions have been reviewed, which are categorized as follows,

- Context reduction for transform signalling (10 Proposals)

- Simplification on LFNST (24 Proposals)

- Transform kernel selection (12 Proposals)

- Maximum TU size for chroma format 4:2:2 and 4:4:4 (1 proposal)

- Mismatch between text specification and reference software on MTS for IBC-coded block (1 proposal)

- Configurable maximum transform size (1 proposal)

- Modified DCT8/DST7 Transform Matrices for Faster Implementation (1 proposal)

The BoG met on Friday July 05 from 08:40pm to 10:54pm and Sunday July 06 from xx:xx to xx:xx.

## Recommendations

### Recommended adoptions into VVC/VTM

### Recommended CE study

## Open issues

### Revisit in track/plenary

# Contributions

The following CE6 related proposals are categorized and reviewed in this BoG, see notes below each proposal.

## Context reduction for transform signalling (10 Proposals)

### Context reduction for MTS signalling (7 Proposals)

CTC:

|  | Bin 0/1/2/3 | Total Ctx | AI | RA | LDB |
| --- | --- | --- | --- | --- | --- |
| VTM | 0~5(QTDepth)/6/7/8 | 9 | - | - | - |
| O0294, method 1 | 0~5(luma CB area)/6/7/8 | 9 | 0.00%/-0.02%/-0.01% | 0.01%/-0.05%/-0.01% | N/A |
| O0294, method 2 | 011223 (luma CB area)/4/5/6 | 7 | -0.01%/-0.05%/0.01% | 0.00%/-0.05%/-0.04% | N/A |
| O0294, method 3 | 0/1/2/3 | 4 | 0.00%/-0.04%/0.01% | N/A | N/A |
| O0354, proposal 1 (init value retrain?) | 0/1/BP/BP | 2 | 0.01%/-0.02%/0.00% | -0.01%/-0.10%/-0.02% | 0.02%/0.12%/0.29% |
| O0354, proposal 2 | max(w, h) > 4 ? 1 : 0/2/BP/BP | 3 | 0.00%/-0.04%/0.01% | -0.01%/-0.14%/-0.08% | 0.01%/0.10%/-0.13% |
| O0372* (*retrained init values) | 0/1/2/3 | 4 | 0.00%/-0.06%/-0.01% | -0.01%/-0.12%/-0.05% | 0.02%/0.04%/ 0.22% |
| O0380, method 1 | 0/1/2/3 | 4 | 0.00%/-0.04%/0.01% | -0.02%/-0.06%/-0.07% | 0.03%/0.25%0.22% |
| O0380, method 2 | 0/1/BP/BP | 2 | 0.00%/-0.03%/0.01% | -0.02%/-0.08%/-0.09% | 0.04%/0.27%/0.07% |
| O0387, method 1 | 0/1/1/1 | 2 | 0.00%/-0.03%/0.03% | -0.01%/-0.09%/-0.05% | 0.04%/0.19%/0.13% |
| O0387, method 2 | 0/1/BP/BP | 2 | 0.00%/-0.02%/0.03% | 0.00%/-0.10%/-0.08% | 0.04%/0.06%/-0.04% |
| O0402 | 0/1/BP/BP | 2 | 0.00%/-0.02%/0.03% | 0.00%/-0.10%/-0.08% | 0.04%/0.06%/-0.04% |
| O0576 | 0/1/BP/BP | 2 | 0.00%/-0.02%/0.03% | 0.00%/-0.10%/-0.08% | N/A |

InterMTS on:

| O0380, method 1 | 0/1/2/3 | 4 | - | -0.02%/-0.04%/-0.03% | -0.03%/-0.08%/0.17% |
| --- | --- | --- | --- | --- | --- |
| O0380, method 2 | 0/1/BP/BP | 2 | - | -0.03%/-0.02%/-0.02% | -0.03%/0.21%/0.26% |
| O0402 | 0/1/BP/BP | 2 | - | -0.01%/0.00%/-0.04% | -0.02%/-0.04%/0.3% |

It is agreed that the yellow highlighted solution has a lower complexity than the blue highlighted solution. However, it is mentioned that changing the last two bins of mts_index to bypass coded bins may affect the coding performance for a different encoder design which may select more DST-7/DCT-8.

It is commented that the current encoder with default CTC configuration rarely check the last 2 (or 3) MTS candidates. It is commented that no critical issues have been solved by the above listed contributions, it is also commented by another proponent that there is clear complexity reduction by changing two context coded bins in mts_index into bypass coded bins.

It is commented that it may be dangerous to change context coded bins as bypass coded bins, with results only available for very limited encoder options.

Recommendation: revisit the above yellow, purple and blue highlighted solutions in track.

JVET-O0294 CE6-related: Context modelling for signalling MTS index [O. Chubach, M.-S. Chiang, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution is related to the context modelling for signalling the index of multiple transform set (MTS). In VVC Draft 5, the quadtree depth (QTDepth) of one coding block (CB) is used to select the context variable when signalling the first bin of MTS index (tu_mts_idx). However, the QTDepth cannot always represent the CB area, because of additional usage of binary tree (BT) and ternary tree (TT) splitting. In this contribution, three methods are proposed to replace QTDepth in the context modelling of the first bin of tu_mtx_idx by CB area. The current method in VTM5.0 and method 1 use six context variables, while method 2 uses four context variables. In method 3, only one context variable is used for signalling the first bin of tu_mtx_idx. It is reported that the proposed context modelling results in AI, RA, and LB Y/Cb/Cr BD-rates as 0.00%/-0.02%/-0.01, 0.01%/-0.05%/-0.01%, and x.xx%/x.xx%/x.xx% for method 1, respectively, -0.01%/-0.05%/0.01%, 0.00%/-0.05%/-0.04%, and x.xx%/x.xx%/x.xx% for method 2, respectively, and 0.00%/-0.04%/0.01, x.xx%/x.xx%/x.xx%, and x.xx%/x.xx%/x.xx% for method 3, respectively.

In addition, a change for the VVC draft specification is also proposed using transform_not_skip_flag instead of transform_skip_flag to allow using truncated unary code to binarize the syntax element, (transform_not_skip_flag, tu_mts_idx).

It is asked that for the first and second methods, whether the computation of luma block area size adds complexity for context derivation, it is mentioned that the block area size is available in both SW and Spec, so actually no added complexity. It is commented by proponent that the context derivation based on QTDepth is considered as a bug in current VVC, because it is a legacy before BT/TT.

It is mentioned that an editorial improvement on VVC spec text is also proposed, which change the naming of syntax element transform_skip_flag to transform_not_skip_flag.

Recommendation: suggest the proposed editorial improvement to Editors.

JVET-O0917 Crosscheck of JVET-O0294: CE6-related: Context modelling for signalling MTS index [C.-H. Hung (Qualcomm)]

JVET-O0354 Non-CE6: Context modeling for MTS index coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

This contribution proposes two simplified context modeling for MTS index coding to reduce the number of context variables and context-coded bins in worst case: For proposal#1, MTS index (tu_mts_idx) is binarized with Truncated Unary coding (TU), bins with binIdx=0..1 of MTS index are context-coded with single context variable per binIdx and the reminder of bins (binIdx=2..3) are bypassed. For proposal#2, 1st bin (binIdx=0) is context-coded with two context variables based on transform block size, and the 2nd bin (binIdx=1) is context-coded with single context variable, and the reminder of bins is bypassed.

It is reported that proposal#1 reduces 7 context variables (9 to 2) and 2 context-coded bins (4 to 2) for MTS index coding with average BD-rate differences of 0.01%, -0.01% and 0.02% for AI, RA and LB; Proposal#2 reduces 6 context variables (9 to 3) and 2 context-coded bin (4 to 2) with average BD-rate differences of 0.00%, -0.01% and 0.01% for AI, RA and LB.

It is commented that the proposed methods are complexity reduction, which reduced context coded bins.

JVET-O0711 Crosscheck of JVET-O0354 (Non-CE6: Context modeling for MTS index coding) [J. Nam (LGE)]

JVET-O0372 Non-CE6: A Simplification for MTS Index Signaling [A. Nalci, H. E. Egilmez, A. Said, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes a simplification for CABAC contexts when signaling multiple transform selection (MTS) index in VTM-5.0. Specifically, dependency between the MTS index signaling and the CU level quad-tree depth (cqtDepth) is removed. This reduces the number of total CABAC contexts by 5. Negligible gains/losses are observed for both luma and chroma, with average BD rates of  (0.00%, -0.06% -0.01%) for all intra (AI), (-0.01%, -0.12% -0.05%) for random access (RA), and (0.02%, 0.04% 0.22%) for low delay B (LDB) under common test conditions (CTC).

It is commented that the proposed method is identical to the method #3 in JVET-O0294 and method #1 in JVET-O0380, but with retrained initialization value for context.

JVET-O0380 Non-CE6: Simplification on MTS index coding [X. Cao, F. Chen, L. Wang (Hikvision)]

Two methods proposed, for method #1, see comments under O0372, method # is identical to JVET-O0354 proposal 1, without retraining on the init value of context.

JVET-O0387 Non-CE6: MTS Index Signaling Simplification [A. Tamse, M. W. Park, K. Choi (Samsung)]

See comments in above summarization table.

JVET-O0852 Crosscheck of JVET-O0387 (Non-CE6: MTS Index Signaling Simplification) [Z. Deng (Bytedance)]

JVET-O0402 Non-CE6: Simplification of context modeling on MTS index [J. Nam, M. Koo, J. Lim, S. Kim (LGE)]

See comments in above summarization table.

JVET-O0877 Crosscheck of JVET-O0402 (Non-CE6: Simplification of context modeling on MTS index) [T. Tsukuba (Sony)]

JVET-O0576 Non-CE6: Reduced contexts and context coded bins for MTS index coding [Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang (Bytedance)]

See comments in above summarization table.

JVET-O0837 Crosscheck of JVET-O0576 (Non-CE6: Reduced contexts and context coded bins for MTS index coding) [A. Tamse (Samsung)]

### Context reduction for SBT signalling (3 Proposals)

|  | cu_sbt_flag | cu_sbt_quad_flag | cu_sbt_horizontal_flag | cu_sbt_pos_flag | Total contexts |
| --- | --- | --- | --- | --- | --- |
| VTM | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | 0 | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | 0 | 7 |
| JVET-O0381 Method 1 | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | BP | BP | BP | 2 |
| JVET-O0381 Method 2 | 0 | BP | BP | BP | 1 |
| JVET-O0465 | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | BP | BP | BP | 2 |
| JVET-O0475 | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | 0 | ( cbWidth < cbHeight ) ? 0 : 1 | 0 | 6 |

|  | RA | LDB |
| --- | --- | --- |
| JVET-O0381, Method 1 | 0.01%/-0.08%/-0.01% | 0.02%/0.04%/0.00% |
| JVET-O0381, Method 2 | 0.01%/0.01%/-0.01% | 0.00%/0.07%/-0.20% |
| JVET-O0465 | 0.01%/-0.08%/-0.01% | 0.02%/0.04%/0.00% |
| JVET-O0475 | 0.01%/-0.02%/-0.02% | 0.04%/0.04%-0.13% |

It is commented it is not right time to reduce context or context coded bins, such effort should be made at the later stage of VVC standardization.

It is recommended to keep the current context derivation for SBT related syntaxes. There is no support from non-proponent to consider the proposed solution at this moment.

JVET-O0381 Non-CE6: Simplification on SBT coding [X. Cao, F. Chen, L. Wang (Hikvision)]

This contribution present two methods on simplification for SBT coding, both them reduced context coded bin for SBT.

The first method reduces 3 context coded bin by using bypass coding to replace CABAC based coding, and 5 contexts are removed, the BD-rate are 0.01% and 0.02% for RA and LB configuration respectively.

The second method reduces 3 context coded bin by using bypass coding to replace CABAC based coding, and 6 contexts are removed, the BD-rate are 0.01% and 0.00% for RA and LB configuration respectively.

There is no context retraining with the proposed method.

It is commented by the proponent of SBT that the last two bins of sbt syntaxes, i.e., cu_sbt_horizontal_flag and cu_sbt_pos_flag, whether they are context coded may have little impact on the coding performance. It is commented that the results are reported under CTC, for different video content or encoder design, a penalty of the coding performance is possible.

JVET-O0752 Crosscheck of JVET-O0381 (Non-CE6: Simplification on SBT coding) [Y. Zhao (Huawei)]

JVET-O0465 Non-CE6: simplified SBT mode coding [F. Le Léannec, Y. Chen, T. Poirier (InterDigital)]

See comments in above summarization table.

JVET-O0853 Crosscheck of JVET-O0465 (Non-CE6: Simplified SBT mode coding) [Z. Deng (Bytedance)]

JVET-O0475 Non-CE6: Context Simplification for Subblock Transform (SBT) [J. Park, B. Jeon (SKKU)]

It is commented by the proponent that it may be not right time to change context derivation design at this moment.

JVET-O0764 Crosscheck of JVET-O0475: Non-CE6: Context Simplification for Subblock Transform (SBT) [L. Pham Van, G. Van der Auwera, M. Karczewicz (Qualcomm)]

### Context reduction for LFNST signalling (1 Proposal)

JVET-O0373 Non-CE6: An Improved Context Modeling for LFNST [A. Nalci, H. E. Egilmez, A. Said, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

|  | binIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| VTM-5.0 | ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass |
| JVET-O0373 | ( tu_mts_idx[ x0 ][ y0 ] = = 0 && cIdx != 0) ? 2 : ( ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE && cIdx == 0) ? 1 : 0 ) | bypass |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.39% | -0.51% | 100% | 102% |
| Class A2 | 0.01% | -0.24% | -0.23% | 102% | 103% |
| Class B | 0.01% | -0.24% | -0.23% | 100% | 98% |
| Class C | 0.00% | -0.11% | -0.14% | 102% | 107% |
| Class E | 0.03% | -0.20% | -0.33% | 103% | 111% |
| Overall | 0.01% | -0.23% | -0.27% | 101% | 104% |
| Class D | 0.02% | 0.04% | -0.09% | 100% | 100% |
| Class F | 0.01% | -0.10% | -0.10% | 101% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.36% | -0.06% | 101% | 102% |
| Class A2 | 0.00% | 0.10% | -0.15% | 98% | 97% |
| Class B | 0.02% | -0.13% | -0.36% | 101% | 101% |
| Class C | -0.05% | -0.20% | -0.08% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.15% | -0.18% | 100% | 100% |
| Class D | 0.02% | -0.01% | 0.06% | 100% | 102% |
| Class F | -0.01% | -0.05% | 0.04% | 98% | 98% |

Recommendation: further study in CE, to see the performance on top of the CE6-2.1 adoption.

## Simplification on LFNST (24 Proposals)

### LFNST signalling (14 Proposals)

Limit LFNST up to 64x64:

TU-level LFNST (modified location of LFNST index signalling):

JVET-O0213 Non-CE6: Block size restriction of LFNST [M. Koo, J. Nam, J. Lim, M. Salehifar, S. Kim (LGE)]

In the 14th JVET meeting, Low Frequency Non-Separable Transform (LFNST) was adopted, where an LFNST kernel is chosen by LFNST index signaling. Considering that a large CU greater than 64x64 is implicitly split (TU tiling) due to the existing maximum transform size restriction (64x64), an LFNST index search could increase data buffering by four times for a certain number of decode pipeline stages so that it is desirable to disable the LFNST index derivation across 64x64 VPDU boundaries.

Therefore, two suggestions are proposed as follows:

Method 1: the maximum size that LFNST is allowed is restricted to 64x64,

Method 2: the LFNST index signaling is performed in TU-level which guarantees that LFNST index derivation must be performed within maximum size TU.

The experimental results of BD-rate/encoding time/decoding time changes due to explicit size limitation to 64x64 for LFNST are reported to be 0.00%/99%/99% (AI) and 0.00%/100%/100% (RA), and those due to the TU-level signaling of LFNST index 0.00%/100%/100% (AI) and 0.00%/101%/100% (RA).

JVET-O0884 Crosscheck of JVET-O0213 (Non-CE6: Block size restriction of LFNST) [Z. Zhang (Ericsson)]

JVET-O0291 CE6-related: Constraint and simplification for LFNST signalling [M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

The signalling of low frequency non-separable transform (LFNST) needs to count nonzero quantized transfrom coefficients of the entire luma coding unit (CU) and the LFNST index is signalled after all transform blocks (TBs) are signalled. This causes a serious processing issue for 64x64-luma/32x32-chroma (64x64-L/32x32-C) decoder pipelining when the CU is 128x128-luma/64x64-chroma (128x128-L/64x64-C) with four forced split transform units (TUs) of 64x64-L/32x32-C in inter slices. In this case, the first three 64x64-L/32x32-C TUs cannot be processed before the parsing of the fourth 64x64-L/32x32-C TU is done, which breaks the 64x64-L/32x32-C decoder pipelining. Three methods are proposed in this contribution to solve the issue as follows.

- Method 1: Disabling intra coding for CUs larger than 64x64-L/32x32-C samples by not signalling pred_mode_flag and inferring it to be 0 (MODE_INTER)

- Method 2: Disabling LFNST for CUs larger than 64x64-L/32x32-C samples

- Method 3: Signalling LFNST at TU level (three variants of method 3 are proposed)

It is reported that, compared to VTM5.0, the proposed method 1 results in very tiny (0.00% after rounding) BD-rate changes. For method 2 and method 3, BD-rates are not changed under the current common test conditions (CTC).

JVET-O0293 CE6-related: Latency reduction for LFNST signalling [M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM5.0, the signalling of low frequency non-separable transform (LFNST) needs to count the number of the nonzero coefficients of each transform block (TB) of transform units (TUs) in one coding unit (CU), which causes a latency issue when one TU contains multiple TBs. Two methods are proposed in this contribution to solve this issue.

- Method 1: For chroma separate tree (CST), signal LFNST index at the first TB with coefficients and share LFNST index with the TB in the same TU; for shared tree, signal LFNST index at luma TB, infer LFNST index to be 0 in case of no nonzero coefficients in luma TB, and share LFNST index with the chroma TBs in the same TU.

- Method 2: Signal LFNST index at TU level before signalling the coefficients of the first TB, which includes signalling the position of last significant coefficient of Cb and Cr before signalling the luma coefficients, and then to decide whether to signal LFNST index according to the positions of last significant coefficients of three colour components.

Compared to VTM5.0, results of the proposed methods are reported as follows.

Method 1:
AI: BD-rates = 0.07% (Y), 0.13% (Cb), 0.19% (Cr); EncT=100%; DecT=101%
RA: BD-rates = 0.01% (Y), 0.12% (Cb), 0.09% (Cr); EncT=100%; DecT=98%

Method 2:
AI: BD-rates = 0.09% (Y), -0.11% (Cb), -0.07% (Cr); EncT=101%; DecT=100%
RA: BD-rates = 0.03% (Y), -0.06% (Cb), -0.10% (Cr); EncT=100%; DecT=97%

JVET-O0352 Non-CE6: TU/TB-level LFNST index coding [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

This contribution proposes two LFNSTLFNST index coding to resolve the parsing dependency (latency problem) at 128x128 CU: a) TU-level LFNST index coding, and b) TB-level LFNST index coding rather than CU-level LFNST index coding. For TU-level LFNST index coding, lfnst_idx is coded at the end of transform unit. For TB-level LFNST index coding, lfnst_idx is coded per component at the end of residual coding.

It is reported that TU-level LFNST index coding provides average BD-rate differences (Y, U, V) of (-0.01 %,0.01% and 0.02%) for AI, (-0.02%, 0.02%, -0.07%) for RA, and (0.00%, 0.00%, 0.00%) for LB with negligible runtime impacts; TB-level LFNST index coding provides average BD-rate differences (Y, U, V) of (0.15%,0.53% and 0.60%) for AI, (0.02%, 0.29%, 0.30%) for RA, and (0.XX%, 0.XX%, 0.XX%) for LB with negligible runtime impacts.

It is recommended to adopt TU-level LFNST index signalling into the next VTM/VVC WD and study TB-level LFNST index coding in CE for LFNST control flexibility per component.

JVET-O0743 Crosscheck of JVET-O0352 (Non-CE6: TU/TB-level LFNST index coding) [K. Zhang (Bytedance)]

JVET-O0368 Non-CE6: An MTS-based Restriction for LFNST beyond Transform Skip [H. E. Egilmez, A. K. Ramasubramonian, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

In VTM-5.0, the Low-Frequency Non-separable Transformation (LFNST) is disabled for the transform skip, and it can be used with all other MTS candidates. This contribution proposes a further restriction on LFNST, where it is enabled with 2-D DCT-2 only (i.e., only when MTS index is 0). It is asserted that the transformation process is simplified with the proposed method, and the experimental results show that the proposed changes provide slight coding gains in RA configuration and do not lead to a significant loss in AI configuration. On average, the proposed normative restrictions lead to coding loss of 0.07% in AI and gain of 0.04% in RA over VTM-5.0 under common test conditions (CTC). With further encoder changes, the BD-rate performance of 0.00% in AI and gain of 0.05% in RA is achieved.

JVET-O0863 Crosscheck of JVET-O0368: "Non-CE6: An MTS-based Restriction for LFNST beyond Transform Skip" [F. Le Léannec (Interdigital)]

JVET-O0410 CE6-related: Signalling of Low Frequency Non-Separable Transform [Y. Kidani, K. Unno, K. Kawamura, S. Naito (KDDI)]

This contribution proposes the signalling of Low Frequency Non-Separable Transform (LFNST) which was adopted into VVC WD5 and VTM-5.0 in the previous meeting. In the current design, the necessity of decoding lfnst_idx, which indicates a kernel type of LFNST, is determined according to the two type of the numbers of non-zero transform coefficients in a coding block. These counters are, however, utilized only for LFNST signalling. In this contribution, it is proposed to remove the whole counters from the condition check by using existing syntax elements, i.e., cbf, last_sig_coeff and coded_sub_block_flag. In addition, it is also proposed that the block size restriction is added to LFNST to meet VPDU constraints. Simulation results of the proposed method implemented on top of VTM-5.0 reportedly show that the overall BD-rates changing for luma in AI/RA/LDB are 0.02%/0.00%/0.00% comparing VTM-5.0. In addition, the results of the proposed method implemented on top of CE6-2.1a show the overall BD-rates changing for luma in AI/RA/LDB are 0.08%/0.01%/0.00% comparing VTM-5.0.

JVET-O0444 Non-CE6: On LFNST index coding [C. Hollmann, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

This contribution proposes to modify the coding of the LFNST index, so that it no longer is part of the coding unit syntax, but part of the transform unit syntax. It is claimed that this change increases readability of the standard while having no impact on compression performance.

JVET-O0873 Crosscheck of JVET-O0444 (Non-CE6: On LFNST index coding) [J. Lainema (Nokia)]

JVET-O0466 Non-CE6: interaction between MTS and LFNST [F. Le Léannec, F. Urban, K. Naser, T. Poirier (InterDigital)]

This contribution proposes to limit the usage of LFNST according to the primary transform used for a given block. Two options are proposed. First option disallows LFNST for a block with MTS index different from 0 (MTS_DCT2_DCT2). Second option forbids LFNST if MTS index is different from 0 and different from 2 (MTS_DST7_DST7). These 2 restrictions are added to already existing the LFNST restrictions in VTM-5.0. The MTS-LFNST interaction option used is signaled in the SPS.

The first option reportedly brings -0.04% Luma BD-rate gain in RA and 0.07% Luma BD-rate loss in AI over VTM-5.0. The second option reportedly brings 0.00% Luma BD-rate in RA and 0.00% in AI.

It is suggested to adopt the first option in RA coding test conditions and the second option in AI.

JVET-O0915 Crosscheck of JVET-O0466: Non-CE6: interaction between MTS and LFNST [C. Hollmann (Ericsson)]

JVET-O0472 Non-CE6: Simplified LFNST signalling [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

In this contribution, a simplified LFNST (Low Frequency Non-Separable Transform) signalling method is proposed where one of dependencies on the number of significant coefficients is removed. In VTM-5.0, a CU-level LFNST index can be conditionally signalled when the number of significant coefficients in the CU exceeds a predefined threshold. Therefore, a counter is needed to count the number of significant coefficients in the whole CU region and it may increase the processing cost for residual coding. It is proposed to code LFNST index based on position of the last significant coefficient, instead of the number of significant coefficients. Experimental results show that the proposed method can achieve 0.00% and 0.xx% luma BD-rates for AI and RA configurations, respectively. It is asserted that the proposed method makes residual coding process much simpler than that of the current specification.

JVET-O0476 Non-CE6: Syntax change for LFNST [D. Park, Y.-U. Yoon, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

This contribution proposes an alternative syntax structure that moves the syntax element of the low-frequency non-separable transform (lfnst_idx) from at the end of the coding unit (CU) syntax to the end of the transform unit (TU) syntax. In addition, lfnst_idx is modified to be parsed only at the first TU of the CU. Two different tests are evaluated. Test 1 is applying the LFNST for the first TU only, and Test 2 is sharing the lfnst_idx with all TUs. The experimental results for Test1 reportedly show 0.00%, 0.00%, and 0.00% luma BD-rates performance for AI, RA, and LDB, respectively. The experimental results for Test2 also reportedly show 0.XX%, and 0.XX% luma BD-rates performance for RA, and LDB, respectively.

JVET-O0521 Non-CE6: Removing LFNST when MTS larger than zero [G. Kulupana, S. Blasi, A. Seixas Dias (BBC)]

This contribution presents a modification to the Low Frequency Non-Separable Transform (LFNST) usage in VVC. In the current VVC draft specification, LFNST is used regardless of the primary transform type. This contribution proposes to remove the usage of LFNST when the MTS index is different than 0 (transform different than DCT2-DCT2). This allows shorter encoding time as well as removal of one context model in the current VVC draft specification. Consequently the average BD-rates of 0.xx% (Y), -0.xx% (U) and 0.xx% (V) are reported for AI with 9x% average encoding time and 9x% decoding time with respect to VTM 5.0.

JVET-O0569 Non-CE6: LFNST Signaling at the TU Level [A. Nalci, H. E. Egilmez, A. Said, M. Coban, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes a new signaling approach for Low-frequency Non-Separable Transform (LFNST). In VTM-5.0, signaling of the LFNST index is done at the coding unit (CU) level after primary transform and before quantization at encoder side. Likewise, at decoder LFNST index is read after dequantization and inverse primary transform.

This contribution proposes to: (1) move LFNST index signaling from CU level to transform unit (TU) level, and (2) signal LFNST index only for the first TU in a given CU to reduce possible signaling overhead. The proposed approach is an inherent simplification considering that LFNST is a complementary secondary transform to the primary transform (e.g. primary transform is signaled at the TU level). Note that we perform LFNST on all TUs as in VTM-5.0 regardless of the proposed signaling.

To warrant the proposed simplification, 4 separate tests were conducted for both (1) and (2), and under common test conditions (CTC) and under non-CTC conditions in VTM-5.0.

Under CTC conditions signaling of LNST index can be moved to TU level without any losses with average BD-rates of  YUV = (0%, 0%, 0%) for all intra (AI) and (0%, 0%, 0%) for random access (RA). This is partly because in CTC conditions a TU is usually equivalent to a CU as 128x128 CU splitting case does not appear often.

Under non-CTC conditions (i.e. where DualTree and LCTUFAST are disabled) LNST index signaling can be moved to TU level with average BD rates of  (0.00%, 0.02%, 0.02%) for AI and (0.00%, 0.05%, 0.04%) for RA.

JVET-O0642 Non-CE6: Reducing secondary transform kernel for specific residual block size [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

In this contribution, reduced secondary transform for 32×4 and 4×32 TU blocks are proposed. Instead of using 16×16 secondary transform kernel, we apply the 8×16  secondary transform kernel, which is taken from 16×16 secondary transform kernel for left or upper parts of 32x4 and 4x32 TU blocks. With reduced kernel, 4×4 secondary transform block can produced 4×2 secondary transformed block.

### LFNST calculation (10 Proposals)

JVET-O0195 Non-CE6: Simplification on LFNST [Y. Zhao, H. Yang (Huawei)]

This contribution proposes to simplify LFNST from three aspects: 1) for 4xN and Nx4 TU (N>8), LFNST is applied to only the top-left 4x4 sub-block; 2) limit the sum of non-zero coefficient check in LFNST index parsing to only the top-left 2x2 coefficients; 3) reduce four transform sets to three transform sets, which saves 25% memory for storing LFNST matrices, i.e., 2KB.

It is reported that aspect 1 results in 0.02%/0.01%/0.03% Y/Cb/Cr BD-rates in AI configuration. Aspect 1 and 2 together result in 0.03%/-0.20%/-0.12% and 0.03%/-0.12%/-0.07% Y/Cb/Cr BD-rates in AI and RA configuration, respectively. The three aspects together result in 0.10%/-0.15%/-0.05% and 0.04%/-0.25%/-0.17% Y/Cb/Cr BD-rates in AI and RA configuration, respectively.

In version 2, the coding performance of Test 2 with threshold equal to 1 for single tree is tested, which results in 0.00%/-0.06%/-0.11% Y/Cb/Cr BD-rates in RA configuration (AI is the same as Test 2). Besides, the aspect 3 alone performance is provided for additional information, which is 0.07%/-0.01%/0.04% and 0.02%/-0.19%/-0.11% Y/Cb/Cr BD-rates in AI and RA configuration, respectively.

JVET-O0209 Non-CE6: LFNST simplification based on the methods proposed in CE6-2.1a [M. Koo, J. Nam, J. Lim, M. Salehifar, S. Kim (LGE)]

In the 14th JVET meeting, Low Frequency Non-Separable Transform (LFNST) was adopted and also some LFNST simplification methods are being considered in CE6-2.1. The proposal introduces several configurations/variations of LFNST simplification based on the methods proposed in CE6-2.1 by considering 4 aspects as follows:

(a) Max. 16 LFNST coefficients

(b) Additional zero-out of primary transform coefficients when 4x4 LFNST is applied

(c) Additional zero-out of primary transform coefficients when 8x8 LFNST is applied

(d) Change of the region where non-zero coefficients are counted

In CE6-2.1, all of (a), (b), (c), and (d) are incorporated on top of VTM 5.0 LFNST.

In this contribution, several combinations of the four features are tested, of which descriptions and experimental results are summarized as follows:

| Test | Description | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate | EncT | DecT | BD-rate | EncT | DecT |
| CE6-2.1a | (a)+(b)+(c)+(d) | 0.06% | 90% | 99% | 0.02% | 97% | 100% |
| Test 1 | (a) | 0.02% | 100% | 100% | 0.01% | 100% | 100% |
| Test 2 | (c)+(d) | 0.02% | 93% | 98% | 0.01% | 99% | 100% |
| Test 3 | (c) | 0.03% | 94% | 100% | 0.02% | 99% | 100% |
| Test 4 | (a)+(c)+(d) | 0.04% | 92% | 98% | 0.03% | 99% | 100% |
| Test 5 | (a)+(c) | 0.05% | 93% | 100% | 0.02% | 99% | 100% |

The results of CE6-2.1a in the above table are those in its contribution document. Specifically, Test 3 and Test 5 does not require any normative changes relative to VTM 5.0 and Test 1, respectively.

JVET-O0292 CE6-related: Simplifications for LFNST [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes two methods to simplify transform for intra coding unit (CU) as follows.

- Method 1 is to disable low frequency non-separable transform (LFNST) when multiple transform set (MTS) is used (tu_mtx_idx > 0).

- Method 2 is to remove the option of LFNST index equal to 2 and select the LFNST kernel using the intra prediction mode when LFNST index is equal to 1.

Results of the proposed methods compared with VTM5.0 are reported as follows.

Method 1:
AI: BD-rates = 0.07% (Y), 0.06% (Cb), 0.10% (Cr); EncT=94%; DecT=101%
RA: BD-rates = -0.04% (Y), -0.07% (Cb), -0.02% (Cr); EncT=98%; DecT=97%

Method 2:
AI: BD-rates = 0.12% (Y), 0.82% (Cb), 0.88% (Cr); EncT=82%; DecT=100%
RA: BD-rates = 0.09% (Y), 0.45% (Cb), 0.51% (Cr); EncT=94%; DecT=98%

JVET-O0266 Non-CE6: Simplifications on LFNST [K. Zhang, L. Zhang, Z. Deng, H. Liu, N. Zhang (Bytedance)]

Three modifications on LFNST are proposed. First, it is proposed to disable LFNST when the width or height of a CU is larger than the maximum transform size. Second, it is proposed that the number is counted only for significant coefficients in the LFNST applied 4×4 blocks, and the threshold of the number of significant coefficients to apply LFNST is unified to be 1 for both dual-tree and single-tree coding structure. Third, it is proposed to remove the LFNST for the 2nd 4×4 block for a CU with sizes N×4 or 4×N, N > 8. Therefore, LFNST is only applicable to the top-left 4×4 block for all CUs. The average BD-rate changes are reported as below:

Modification #1: 			AI: 0.00%,	100%, 99%;	RA: 0.00%, 99%, 100%.

Modification #2: 			AI: 0.02%,	100%, 99%;	RA: 0.00%, 100%, 101%.

Modification #3: 			AI: 0.02%,	100%, 100%;	RA: 0.01%, 100%, 100%.

Modification #1+#2+#3:	AI: 0.03%,	100%, 99%;	RA: -0.01%, 100%, 100%.

JVET-O0876 Crosscheck of JVET-O0266 (Non-CE6: Simplifications on LFNST) [T. Tsukuba (Sony)]

JVET-O0349 CE6-related: Simplified LFNST [J. Lainema (Nokia)]

This contribution proposes to simplify the LFNST (Low Frequency Non-Separable Transform) process as described in CE6-2.1a. That is, it is proposed to modify the VTM-5 LFNST design as follows:

- When LFNST is applied, all transform coefficients outside the LFNST area are zeroed out (in VTM-5 coefficients X…47 in scan order are zeroed out, but there may be “high frequency primary transform left-overs” in later positions [X = 8 for 4x4/8x8; and X = 16 otherwise)

- Applying only one secondary transform within one transform block (in VTM-5 there are two secondary transforms for 4xN and Nx4 transform blocks when N > 4)

It is asserted that the simplifications reduce encoder runtimes and decoder complexity with minor impact on coding efficiency. Similarly to JVET-O0209, it is pointed out that in the case of these modifications the check for coefficients violating the LFNST area restriction can be done either by using the lastScanPos variable or checking locations of individual coefficients.

The BD-rate impact with respect to VTM-5 is +0.06 % and +0.02 % for AI and RA configurations, respectively. In AI configuration the encoder and decoder runtimes with respect to VTM-5 are reported to be 90 % and 99 %, respectively.

JVET-O0350 CE6-related: LFNST with one mode [J. Lainema (Nokia)]

This contribution proposes to simplify the LFNST (Low Frequency Non-Separable Transform) process. In VTM-5 there are two alternative LFNST transforms for each intra prediction mode. Selection of the LFNST transform type is done by the encoder and signalled in the bitstream with an additional CU level flag. It is proposed to omit this signalling and define a single transform for each intra prediction mode by alternating between the available two transforms for consecutive intra prediction modes.

It is asserted that the simplification reduces encoder runtimes with minor impact on coding efficiency.

It is reported that BD-rate impacts with respect to VTM-5 are +0.08 for both AI and RA configurations. The encoder runtimes with respect to VTM-5 are reported to be 83 % and 95 % for AI and RA configurations, respectively.

JVET-O0370 CE6-related: Applying LFNST Only for Top-Left Subblocks [H. E. Egilmez, A. Nalci, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

In VTM-5.0, depending on the block size, the Low-Frequency Non-separable Transformation (LFNST) can be either applied to a single subblock located at the top-left or applied to two subblocks (i.e., top-left blocks and its adjacent block). This contribution document proposes to remove the latter case, so the LFNST is applied to a single top-left subblock only. The experimental results show that the proposed changes do not lead to a significant loss in coding efficiency. On average, 0.02% AI and 0.01% RA luma coding losses are observed over VTM-5.0 under common test conditions (CTC).

JVET-O0539 CE6-related: Unified LFNST using block size independent kernel [X. Zhao, X. Li, S. Liu (Tencent)]

The Low Frequency Non-Separable Transform (LFNST) in VVC Draft v5 applies two different sizes of non-separable secondary transform kernel, the one with size 16x16 is applied for blocks with either width or height smaller than 8, the other one with size 16x48 is applied for blocks with both width or height greater than or equal to 8. To unify the LFNST design, in this contribution, it is proposed to use only the 16x48 kernels as in VTM-5.0 for LFNST. The reported benefits include: 1) the memory for storing the 16x16 LFNST kernels, i.e., 2KB memory, can be saved, 2) the calculation of LFNST is simplified because the switching of transform sizes is not needed, and 3) both VTM-5.0 and Spec text can be simplified. Experimental results reportedly show that, compared to VTM-5.0, the proposed method gives 0.05%, 0.01% luma BD-rate changes for All Intra (AI) and Random Access (RA) configurations, respectively, with 3% overall encoding time saving for AI. Compared to CE6-2.1a, the proposed method gives 0.00%, -0.01% luma BD-rate changes for AI and RA configurations, respectively.

JVET-O0899 Crosscheck of JVET-O0539 (CE6-related: Unified LFNST using block size independent kernel) [M. Siekmann (HHI)]

JVET-O0934 Crosscheck of JVET-O0539: CE6-related: Unified LFNST using block size independent kernel [C. Hollmann (Ericsson)]

JVET-O0544 Non-CE6: Modified LFNST Transform Matrices for Fast Implementation [K. Naser, L. Kerofsky, F. Le Léannec, T. Poirier (InterDigital)]

Recently, LFNST was adopted to VTM5.0 as an additional transform process in non-separable manner. LFNST requires the multiplication of transform matrices (either 16x48 or 16x16) by a vector of input data (of length 16 or 48). Unlike the ordinary transforms of DCT2, DST7 and DCT8, LFNST is a learned transform whose basis functions do not have specific properties that can be used fast implementation.

This contribution proposes a slight modification to the LFNST matrices so that faster implementation can be achieved. Similar to the ordinary transform, the proposed modification allows dual implementation support, in which an LFNST transform can be either implemented by matrix multiplication or a fast algorithm to yield exactly the same results.

Experimental results show 0.03% and 0.02% RD loss in AI and RA respectively, while the number of multiplications is significantly reduced in the fast algorithms.

JVET-O0620 Non-CE6: Simplification with new LFNST transform basis [T. Zhou, T. Hashimoto, T. Ikai (Sharp)] [miss] [late]

The Low-frequency Non-Separable Transform (LFNST) has been adopted in VVC-5. However, in the current design, LFNST requires 8KB table sizes for storing transform matrices. This contribution proposes the reduce LFNST matrices and LFNST computation by replacing 16x48 matrices with new 16x36 matrices. The scan order is switched by lfnstIdx to keep coding efficiency. It is reported that the proposed method achieves about 19 % table size reduction with 0.03 % loss in CTC IO condition.

JVET-O0719 Crosscheck of JVET-O0620 (Non-CE6: Simplification with new LFNST transform basis) [L. Xu, F. Chen, L. Wang (Hikvision)]

## Transform kernel selection (12 Proposals)

### Primary transform kernel selection (10 Proposals)

#### Harmonization among implicit MTS, MIP and LFNST.

Anchor: VTM-5.0, --MTS=0 --MTSImplicit=0

|  | Doc # | AI | RA | LDB |
| --- | --- | --- | --- | --- |
| Disable implicit MTS for LFNST | JVET-O0214 JVET-O0540 | -0.55 % | -0.28 % | -0.11 % |
| Mode-dependent implicit MTS for ISP | JVET-O0382 | N/A | N/A | N/A |
| Disable implicit MTS for LFNST and MIP | JVET-O0529 JVET-O0540 | -0.60% | -0.30% | -0.14% |

Anchor: VTM-5.0, --MTS=0 --MTSImplicit=1

|  | Doc # | AI | RA | LDB |
| --- | --- | --- | --- | --- |
| Disable implicit MTS for LFNST | JVET-O0214 JVET-O0540 | -0.74 % | N/A | N/A |
| Mode-dependent implicit MTS for ISP | JVET-O0382 | -0.63% | -0.27% | -0.04% |
| Disable implicit MTS for LFNST and MIP | JVET-O0529 JVET-O0540 | -0.79% | -0.35% | -0.03% |

Anchor: VTM-5.0, CTC

|  | Doc # | AI | RA | LDB |
| --- | --- | --- | --- | --- |
| Mode-dependent implicit MTS for ISP | JVET-O0382 | -0.01% | -0.01% | 0.00% |

JVET-O0214 Non-CE6: Harmonization of implicit MTS and LFNST [T. Ikai (Sharp)]

JVET-O0382 Non-CE6: Unification between implicit MTS and ISP [X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-O0529 Non-CE6 Interaction between Implicit MTS and LFNST and MIP [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

JVET-O0540 Non-CE6: Harmonization of LFNST, MIP and implicit MTS [X. Zhao, X. Li, S. Liu (Tencent)]

#### Reduced MTS candidates

JVET-O0264 CE6-related: On DCT-8 in VVC [K. Zhang, L. Zhang, Z. Deng, H. Liu, N. Zhang (Bytedance)]

In this contribution, two methods are proposed to restrict the usage of DCT-8. In both methods, DCT-8 is removed from MTS. Three context coding bins and three arithmetic coding contexts for MTS can be removed accordingly. In method #1, only 4-point DCT-8 is kept. DCT-8 is replaced by DCT-2 for SBT when the block size is larger than 4. In test #2, only 4-point and 8-point DCT-8 are kept. DCT-8 is replaced by DCT-2 for SBT when the block size is larger than 8. The simulation results reportedly show that the BD-rate changes are -0.01%/0.05%, and -0.01%/0.01% for AI/RA in method #1 and #2, respectively.

JVET-O0916 Crosscheck of JVET-O0264: CE6-related: On DCT-8 in VVC [C. Hollmann (Ericsson)]

JVET-O0446 Non-CE6: Reduced MTS [C. Hollmann, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

In this document it is proposed to remove the DCT-8 from the MTS tool, leaving the encoder with two options for the primary transform: DCT-2 and DST-7. It is claimed that this can be done without any loss in compression efficiency. Test results are reported as -0.01% (AI-Y), -0.01% (RA-Y) and 0.06% (LD-Y) without a measurable difference in encoder and decoder run times for AI and RA, while the encoder run time for LD is reduced to 98%. As a consequence, it is claimed that DCT-8 of size 32 can be removed from the standard.

An additional test with MTS allowed for inter predicted blocks results reportedly in 0.11% (RA-Y) and 0.13% (LD-Y). In the additional test the encoder run times are reported to be 96% (RA) and 92% (LD).

JVET-O0808 Crosscheck of JVET-O0446 (Non-CE6: Reduced MTS) [C.-H. Hung (Qualcomm)]

#### JVET-O0528

JVET-O0528 Non-CE6: Simplified Transform Selection For ISP [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

In Geneva meeting, a unified transform selection for both ISP and Implicit MTS was adopted JVET-N0866. On one hand, this unification is not applicable anymore due to the adoption of LFNST and MIP (JVET-O0529). On the other hand, the ISP transform design adds new transform pairs, compared to MTS pairs, which result from the combination of DCT2 and DST7. This increases the overall design complexity and leads to additional difficulties for both future computational and perceptual optimization.

This contribution proposes a new design of ISP transform, which is unified with MTS design. This comes with 0.02% and 0.00% luma loss in AI and RA configuration.

#### JVET-O0538

JVET-O0538 Non-CE6: An SPS Control of ISP and SBT Transform Selection [K. Naser, F. Le Léannec, M. Kerdranvat (InterDigital)]

ISP and SBT are TU tiling tools used for intra and inter coding (resp.), which provide significant coding gain. Their transform selection is Implicitly composed of combination of DCT2, DST7 and DCT8. Unlike the normal transform blocks, non-DCT2 transforms cannot be controlled by the MTS SPS flag (sps_mts_enabled_flag) in the case of ISP and SBT. Therefore, in the case of a light-weight encoder that is not supported with non-DCT2 transform, ISP and SBT must be disabled, leading to large BD-rate loss.

This contribution solves this problem by enabling an ISP and SBT with DCT2 when sps_mts_enabled_flag is disabled. In other words, if non-DCT2 transform is not enabled for the normal transform blocks, it is also not enabled for ISP or SBT.

Experimental results show significant coding gain when ISP and SBT are enabled with DCT2, compared to the anchor case when they are disabled.

#### JVET-O0541

JVET-O0541 Non-CE6: Modified high-level syntax control on implicit transform [S. Liu, X. Zhao, X. Li (Tencent)]

For intra prediction residuals which are not coded by Intra Sub-Partitioning (ISP) mode, the current implicit MTS design in VVC Draft v5 can be only enabled when MTS is disabled for both intra and inter coded blocks. However, the implicit MTS does not conflict with MTS for inter coded blocks, and it is not necessary to disable implicit MTS when MTS is only enabled for inter coded blocks. To resolve this issue, it is proposed to modify the condition, and whether implicit transform can be applied no longer depends on whether MTS is disabled for inter coded blocks.

#### JVET-O0546

JVET-O0546 Non-CE3/6: Transform selection for MRL [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

In this contribution, it is proposed to use only DCT-2 transform when a non-zero reference line is selected for intra prediction to reduce encoder complexity. It is reported that the proposed method obtains 0.00% and 0.00% luma BD-rate change for the AI and RA configurations on top of VTM-5.0 with 98% and 99% encoding time, respectively.

JVET-O0680 Crosscheck of JVET-O0546 (Non-CE3/6: Transform selection for MRL) [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

### Secondary transform kernel selection (2 Proposals)

JVET-O0219 Non-CE6: On LFNST transform set selection for a CCLM coded block [Z. Zhang, R. Sjöberg, R. Yu (Ericsson)]

This contribution presents a method to improve chroma coding efficiency by improving LFNST (Low Frequency Non-Separable Transform) transform set selection when one of three CCLM (cross-component linear model) modes is used for a chroma block. The proposed method utilizes the co-located luma intra prediction mode as the transform set selection table index to select the LFNST transform set for the chroma block. The proposed method has Y / Cb / Cr BDR impact for AI of -0.01% / -0.40% / -0.34% compared to VTM5.0 and Y / Cb / Cr BDR impact for RA of 0.00% / -0.37% / -0.33% compared to VTM5.0 and decoding time impact for AI / RA of 100% / 100% compared to VTM5.0.

JVET-O0518 Non-CE6: LFNST selection with no intra-mode dependency [G. Kulupana, A. Seixas Dias, S. Blasi (BBC)][withdrawn]

## Maximum TU size for chroma format 4:2:2 and 4:4:4 (1 Proposal)

JVET-O0357 Non-CE6: Maximum TU size for chroma format 4:2:2 and 4:4:4 [L. Li, J. Nam, J. Lim, S. Kim (LGE)]

The contribution proposes maximum TU size considering chroma formats such as 4:2:2 and 4:4:4. It is proposed to separate maximum TU size for chroma maxTbSizeC into two components: maxTbWidthC and maxTbHeightC so that VPDU is supported.

## Mismatch between text specification and reference software on MTS for IBC-coded block

JVET-O0474 Non-CE6: Mismatch between text specification and reference software on MTS for IBC-coded block [J. Jung, D. Kim, G Ko, J. Son, J. Kwak (WILUS)]

This contribution reports a mismatch between text specification and reference software on MTS (Multiple Transform Selection) for IBC (Intra Block Copy) coded block. In the current specification, MTS can be applied to IBC-coded block when MTS for inter is enabled in SPS-level, while MTS is not applied to IBC-coded block in VTM-5.0 even though the SPS flag is equal to 1. In this proposal, test results of enabling MTS for IBC-coded block are presented. Experimental results show that aligning VTM-5.0 with the current spec text achieves

- For Class F with AI: -0.01% (Y), 111% (EncT), 100% (DecT)

- For Class F with RA: 0.02% (Y), 102% (EncT), 100% (DecT)

- For Class F with LDB: 0.02% (Y), 101% (EncT), 101% (DecT)

- For TGM 4:2:0 with AI: -0.03% (Y), 106% (EncT), 100% (DecT)

- For TGM 4:2:0 with RA: 0.02% (Y), 101% (EncT), 100% (DecT)

- For TGM 4:2:0 with LBD: 0.15% (Y), 102% (EncT), 100% (DecT)

Considering the test results, it is shown that the current MTS scheme is not quite beneficial to IBC-coded block. Therefore, it is proposed to change the specification text as disabling MTS for IBC-coded block to align with the VTM reference software.

## Configurable maximum transform size in VVC

JVET-O0545 Non-CE6: Configurable maximum transform size in VVC [X. Zhao, X. Li, S. Liu (Tencent)]

This contribution proposes to include configurable max transform size in VVC. With the proposed SW and spec text fixes, the encoder has the flexibility to choose only implementing up to 16-length, 32-length, or 64-length transform, without changing the max coding unit size.

JVET-O0941 Crosscheck of JVET-O0545 (Non-CE6: Configurable maximum transform size in VVC) [T.-C. Ma (Kwai Inc.)]

## Modified DCT8/DST7 Transform Matrices for Faster Implementation

JVET-O0699 Non-CE6: Modified DCT8/DST7 Transform Matrices for Faster Implementation [K. Naser, L. Kerofsky, F. Le Léannec, T. Poirier (InterDigital)] [late]

This contribution proposes a slight modification to the DCT8/DST7 matrices so that faster implementation can be achieved. Similar to the current fast transform design, the proposed modification allows dual implementation support, in which the transform can be either implemented by matrix multiplication or a fast algorithm to yield exactly the same results.

Experimental results reportedly show XX% and XX% RD loss in RA and AI respectively, while the number of multiplications is reduced by XX% in the fast algorithms.