| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1117-v2 |
| --- | --- |

| Title: | BoG report on CE7-related and Non-CE7 contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz | Email: | heiko.schwarz@hhi.fraunhofer.de |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This BoG met on 5th July 2019 from 9:30 to 17:45 to review the CE7-related and Non-CE7 contributions submitted to the 15th JVET meeting. In total, 27 contributions have been reviewed, which are categorized as follows:

- Transform skip residual coding (12 contributions)

- Coding of coded block flag (4 contributions)

- Context-coded bins & number of contexts (5 contributions)

- Scaling matrices (3 contributions)

- Joint chroma residual coding (3 contributions)

The following contribution have not been discussed in the BoG, but are related to CE7 topics:

- JVET-O0068 (study of bin-to-bit ratio): No presenter available, information only

- JVET-O0223, JVET-O0527 (scaling matrices): Already discussed in track A

- JVET-O0376: One aspect relates to CE7 (high-level flag for joint chroma coding)

- JVET-O1098: Reduction of number of context models (same as JVET-M0107 submitted to 13th JVET meeting). – Could be tested together with other proposals in a corresponding CE.

# BoG Recommendations

The BoG recommends the adoption of the following aspects:

- 3-pass version of residual coding for transform skip (syntax only change, JVET-O0623 method 3.2, JVET-O0619 test 2):

  - Pass 1: sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, par_level_flag

  - Pass 2: abs_level_gt3_flag, abs_level_gt5_flag, abs_level_gt7_flag, abs_level_gt9_flag

  - Pass 3: abs_remainder

WD text available in JVET-O0623 and JVET-O0619.

- Align counting of context-coded bins in transform skip residual coding and regular transform coefficient coding: Exclude coded_subblock_flag’s in transform skip residual coding (JVET-O0409, but without modifying the threshold).

WD text available in JVET-O0409.

- QP clipping in scaling process for transform skip (in order to prevent cases in which the effective quantization step size for transform skip becomes smaller than 1) as proposed in JVET-O0919 (JVET-O0405 is conceptually similar). The minimum allowed QP is basically given by QP* = 4 – 6*(inputBitDepth – 8). In the scaling process for transform skip, the quantization parameter is clipped according to

qP = Max( QpTsMin, Qp’Y ).

The minimum QP value QpTsMin is derived from an SPS syntax element qp_ts_minimum_minus4 according to

QpTsMin = 4 + qp_ts_minimum_minus4.

Benjamin agreed to provide WD text.

- Simplify context derivation for coded block flag by removing the dependency on transform depth (JVET-OO0193, JVET-O0375). WD text available in JVET-O0193 and JVET-O0375.

- Context model reduction for sig_coeff_flag (JVET-O0617). Removes 30 out of 90 context models with straightforward modification of context derivation. WD text available in JVET-O0617.

- Enable additional joint chroma coding modes (for adopted CE7-2.1) only in I slices (JVET-O0543). WD text available in JVET-O0543.

- Software only fix regarding position of chroma cbfs for SBT TUs (in order to align software with WD text). Proponent of JVET-O0596 will provide fix for software.

The BoG recommends a further study of the following aspects in a CE:

- Multiple variants of organizing the syntax elements for transform skip residual coding into scan passes, including 2-pass, 4-pass variants as well as variants with re-positioning of the parity flag (JVET-O0295, JVET-O0623, JVET-O0619).

- Proposed concepts for restricting the switch to bypass mode (similar as in regular transform coefficient coding): JVET-O0619

- Advanced context modelling for syntax elements in transform skip residual coding (JVET-O0295)

- QP dependent binarization of quantization indexes in transform skip (JVET-O0559): Maximum dynamic range is divided into 4 intervals, interval index and remainder within the interval is transmitted.

- Modified signalling of chroma cbf’s and joint chroma residual coding flag (JVET-O0231).

For the following aspects, the BoG request a discussion in the track or JVET plenary:

- Scaling for transform blocks with LFNST. Contribution JVET-O0383 asserts that transform coefficients after LFNST do not relate to spatial frequencies. It is proposed to modify the scaling for such transform blocks as follows:

  - Scaling with flat scaling matrix (before LFNST)

  - Additional scaling with scaling matrix (after LFNST)

- Complexity impact of using different scans in transform skip residual coding and regular transform coefficient coding (forward vs backward scan): JVET-O0619

- With the adoption of CE7-1.1, we use a different maximum number of context-coded bins for regular transform coefficient coding and transform skip residual coding (1.75 vs 2.0 context-coded bins per coefficient). Is that desirable? – Might be worth the further study the impact on coding efficiency.

# Review of Contributions

## Transform Skip Residual Coding (12)

JVET-O0295 CE7-related: Modified subblock coding passes for coding TS residual [S.-T. Hsiang, S.-M. Lei (MediaTek)]

In VVC Draft 5, each subblock may need to go through six subblock coding passes for entropy coding the associated coefficient levels. This contribution proposes modified subblock coding passes for coding transform skip (TS) residual. The proposed method integrates multiple subblock coding passes for signalling syntax element abs_level_gtx_flag[n][j], j = 0..4, into one single subblock coding pass, for entropy coding each coefficient at scanning position n in a current subblock. The proposed method further moves coding par_level_flag[n] from the 1st subblock coding pass to a separate subblock coding pass after signaling abs_level_gtx_flag[n][j]. The number of the subblock coding passes is thus reduced to four from six in the proposed method.

The proposed method reportedly provides -0.05%, -0.05%, and 0.01% luma BD-rates for the AI, RA, and LB settings, respectively, for Class F and provides -0.01%, -0.12%, and -0.07% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 SCC common test conditions (CTC).

In version 2, the further experimental results are provided for employing additional context variables for entropy coding abs_level_gtx_flag[n][j], j = 0..4, and par_level_flag[n]. The proposed method reportedly provides -0.23%, -0.16%, and -0.10% luma BD-rates for the AI, RA, and LB settings, respectively, for Class F and provides -0.45%, -0.31%, and -0.20% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 SCC common test conditions (CTC) while increasing the number of context variables by 7.

See notes after JVET-O0619.

JVET-O0937 Crosscheck of JVET-O0295 (CE7-related: Modified subblock coding passes for coding TS residual) [M. Salehifar (LGE)]

JVET-O0623 CE7-related: Interleaved coefficient coding for transform-skip mode [M. Karczewicz, H. Wang, M. Coban, Y.-H. Chao (Qualcomm)]

This contribution presents 3 methods of reducing the number of passes when encoding transform-skip coefficients. Instead of encoding the abs_level_gtX_flags (X = 2,3,4,5) of a coding group (CG) as 4 separate bit-planes, the method 1 combines all the components except the abs_remainder into a single pass, and the method 2 combines all the components including abs_remainder the into a single pass and the method 3 combines GTX(X=2,3,4,5) and parity into a single pass. Compared to VTM-5.0 with the bugfix introduced in merge request 613 (for ticket #330), the performance of the proposed method is summarized as follows:

- Method 1:

  - Overall all: 0.00% (AI), -0.02% (RA), -0.03% (LB)

  - Class F -0.01% (AI), 0.00% (RA) 0.04% (LB)

  - Class TGM 0.02%(AI), 0.06% (RA) 0.05% (LB)

- Method 2:

  - Overall all: 0.00% (AI), 0.00% (RA), -0.03%(LB)

  - Class F -0.06% (AI), -0.07% (RA), 0.01% (LB)

  - Class TGM -0.03%(AI), -0.05% (RA) -0.08% (LB)

- Method 3:

  - Overall all: 0.00% (AI), 0.01% (RA), -0.03(LB)

  - Class F -0.01% (AI), 0.02% (RA), 0.01% (LB)

  - Class TGM -0.04%(AI), -0.04% (RA) 0.02% (LB)

See notes after JVET-O0619.

JVET-O0690 Crosscheck of JVET-O0623: Interleaved coefficient coding for transform-skip mode [T. Nguyen (HHI)]

JVET-O0619 Non-CE7: Simplification of transform skip residual coding [M. G. Sarwer, Y. Ye (Alibaba)]

In order to unify the coefficient scanning order between transform residual and transform skip residual coding, this contribution proposes reverse scanning order for residual coding of the transform skip (TS) residual block. It also proposes two options of a single pass coding of gt3, gt5, gt7 and gt9 flag of TS residual block in order to reduce the number of coding passes from 6 to 3. In addition to the coding pass reduction options, a unified by-pass coding method between transform and TS residual coding is also proposed. Following results (RA) are reported.

- Test 1: Reverse scanning order:

  - 0.13%/0.13%/0.04% Y/Cb/Cr BD-rate for TGM

- Test 2: Forward scanning order + single pass gt1 coding – Method 1

  - -0.09%/-0.02%/-0.05% Y/Cb/Cr BD-rate for TGM

- Test 3: Forward scanning order + single pass gt1 coding – Method 2

  - 0.00%/0.04%/-0.09% Y/Cb/Cr BD-rate for TGM

- Test 4: Test 2 + unified by-pass coding:

  - 0.15%/0.23%/0.12% Y/Cb/Cr BD-rate for TGM

Discussion on JVET-O0295, JVET-O0623, and JVET-O0619:

It was remarked that there are the following potential problems in the current transform skip residual coding (differences to the regular transform coefficient coding):

- Larger number of coding passes per coefficient group (6 instead of 2)

- Switch to bypass mode can occur at any bin (instead of at the start of a scan position)

- Scan order is reversed (uses forward instead of backward scan)

There was an agreement in the BoG that it is preferable for implementation to reduced the number of scan passes. According to the presented results, the coding efficiency impact is very minor. The following methods have been proposed in JVET-O0295, JVET-O0619, JVET-O0623:

- 2-pass variant (JVET-O0623):

  - First pass: All individual bins (significance, sign, parity, greaterX); can switch to bypass coding at any bin;

  - Second pass: Remainder (always bypass coded)

  - TGM results: 0.02% (AI), 0.06% (RA), 0.05% (LB)

- 2-pass variant with switch to Rice-Golomb code (JVET-O0623):

  - Basically the same as the 2-pass variant above, but after a switch to bypass mode occurred, the remaining samples are coded using a significance flag, a sign flag, and a single Golomb-Rice code for abs(coeff) ˗ 1

  - TGM results: -0.09% (AI), -0.08% (RA), -0.08% (LB)

- 3-pass variant (JVET-O0623):

  - First pass: Sig flag, sign flag, gt1, and parity (same as in the VTM-5)

  - Second pass: All remaining gtx flags (gt3, gt5, gt7, gt9)

  - Third pass: Remainder

  - TGM results: 0.00% (AI), 0.04% (RA), 0.00% (LB)

- 3-pass variant with re-positioning of parity flag (JVET-O0623):

  - First pass: Sig flag, sign flag, gt1 flag

  - Second pass: gt3, gt5, gt7, gt9, parity

  - Third pass: Remainder

  - TGM results: -0.04% (AI), -0.04% (RA), 0.02% (LB)

Note: All results for JVET-O0623 were generated using the RDOQ bug fix that was integrated after the VTM-5 release (in both the tested version and the reference).

- 3-pass variant  (JVET-O0619):

  - First pass: Sig flag, sign flag, gt1, and parity (same as in the VTM-5)

  - Second pass: All remaining gtx flags (gt3, gt5, gt7, gt9)

  - Third pass: Remainder

  - TGM results: 0.01% (AI), -0.09% (RA)

- 3-pass variant with swapped positions of gt3 and parity flag (JVET-O0619)

  - First pass: Sig flag, sign flag, gt1 flag, gt3 flag

  - Second pass: parity, gt5, gt7, gt9 flags

  - Third pass: Remainder

  - TGM results: -0.04% (AI), 0.00% (RA)

- 4-pass variant (JVET-O0295):

  - First pass: Sig flag, sign flag, gt1

  - Second pass: gt3, gt5, gt7, gt9

  - Third pass: parity flag

  - Forth pass: Remainder

  - TGM results: -0.05% (AI), -0.11% (RA), -0.08% (LB)

- 4-pass variant with re-positioning of gt1 flag (JVET-O0295):

  - First pass: Sig flag, sign flag

  - Second pass: gt1, gt3, gt5, gt7, gt9

  - Third pass: parity flag

  - Forth pass: Remainder

  - TGM results: -0.06% (AI), -0.14% (RA),  0.00% (LB)

BoG recommendation:

- Adopt the straightforward 3-pass version (JVET-O0623 method 3.2, JVET-O0619 test 2).

- Further study the other approaches in a CE (including the position of the parity flag).

In JVET-O0619, it was also proposed to use the same concept for switching to bypass mode as in regular transform coefficient coding. The proposed method (based on the 3-pass variant) results in some coding efficiency losses (0.34% AI, 0.15% RA for TGM).

It was remarked that the limit for context-coded bins was reduced from 2 per coefficients to 1.75 per coefficient for the regular transform coefficient coding.

It was remarked by a proponent that it is desirable to align the concept of switching from context-coded to bypass-coded bins with the regular transform coefficient coding. The current transform skip residual coding can switch to bypass mode at any flag, which is undesirable for implementations.

It was also remarked that it should be studied whether the maximum number of context-coded bins could be aligned with the regular transform coefficient coding without losing coding efficiency.

BoG recommendation: Study this aspect in a CE (complexity vs coding efficiency trade-off)

In O0619, it was proposed to use the backward scanning order as in regular transform coefficient coding. This yielded some coding efficiency losses for SCC sequences. It is not completely clear whether this is a problem. It was requested to get some opinions from the hardware people.

In O0295, additional modifications of the context modelling are proposed:

- 2 instead of 1 context models for abs_gtx_flag’s (based on neighbouring coefficients)

- 3 instead of 1 context models for parity flag (based on gtx flags for same scan position) – Requires that the parity flag is coded after the gtx flags.

- Both modification together show a BD rate gain of roughly 0.4% AI, 0.2% RA, 0.1% LB for TGM.

BoG recommendation: Study this aspect in a CE.

JVET-O0878 Crosscheck of JVET-O0619 (Non-CE7: Simplification of transform skip residual coding) [T. Tsukuba (Sony)]

JVET-O0558 CE7-related: Reduction of Gtx passes in transform skip residual coding [M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban (Qualcomm)]

In this proposal, it is proposed to simplify the transform skip residual coding in JVET-O0122 by reducing the number of passes in coding the absolute coefficient levels, i.e. syntax element abs_level_gtx_flag. By reducing the number of passes for abs_level_gtx_flag from 5 to 2 in CE7-3.10 (sign context modeling and level mapping), the results show Class F: -0.53% AI, -0.46% RA, -0.30% LB BD-Rate, and Class SCC: -0.60% AI, -0.52% RA, -0.41% LB BD-Rate, versus the VTM-5.0 anchor under CTC. Compared to CE7-3.10, the proposed scheme show: Class F: 0.23% AI, 0.13% RA, 0.08% LB BD-Rate, and Class SCC: 0.66% AI, 0.27% RA, 0.26% LB BD-Rate.

The contribution investigates the impact of removing 3 gtx flags (reducing the passes from 5 to 2). The removal of gtx flags results in coding efficiency losses.

BoG recommendation: No action.

JVET-O0775 Crosscheck of JVET-O0558 (CE7-related: Reduction of Gtx passes in transform skip residual coding) [Y.-W. Chen (Kwai Inc.)]

Reported in this document is the cross-check results for the reduction of Gtx passes in transform skip residual coding as reported in JVET-O0558. The cross-check results match the coding performance reported by the proponent.

JVET-O0406 Non-CE7: Unification of syntaxes after CCB count exceeds the maximum number between transform residual and transform skip residual [Y. Kato, K. Abe, T.Toma (Panasonic)]

In this contribution, it is proposed to unify the syntax design after CCB count exceeds the maximum number between transform residual and transform skip residual. Two methods are proposed. In method1, the same processing as that of transform residual is used in transform skip residual. In method2, two modifications are added on top of method1, which is to disable poszero mapping and derive rice-paramter by the sum of 2 neighbor coefficients.

Simulation results reportedly are shown as follows:

Method1:

ClassF:  0.13% AI,  0.11% RA,  0.01% LB  ClassTGM:  0.35% AI,  0.18% RA,  0.13%LB

Method1(Low QP):

ClassF:  -0.41%AI,  -1.94%RA,  -0.17%LB  ClassTGM:  -1.03%AI,  -0.40%RA,  -0.78%LB

Method2

ClassF:  0.01% AI,  0.00% RA,  0.05% LB  ClassTGM:  0.17% AI,  0.02% RA,  0.04%LB

Method2(LowQP):

ClassF:  -0.53%AI,  -1.66%RA,  -0.34%LB  ClassTGM:  -1.36%AI,  -1.42%RA,  -1.16%LB

The contribution proposes to use a similar concept as in regular transform coefficient coding after the maximum number of context-coded bins is reached.  It is related to aspects in JVET-O0623 and JEVT-O0619.

BoG recommendation: Further study.

JVET-O0788 Crosscheck of JVET-O0406 (Non-CE7: Unification of syntaxes after CCB count exceeds the maximum number between transform residual and transform skip residual) [C.-Y. Lai (MediaTek)]

JVET-O0409 Non-CE7: Unification of CCB count method between transform residual and transform skip residual [Y. Kato, K. Abe, T. Toma (Panasonic)]

In this contribution, it is proposed to exclude coded_sub_block_flag from CCB count in transskip residual mode so that syntax used for CCB count is unified between trans and transskip residual modes, i.e. the coded_sub_block_flag is not included CCB count and CCB count starts from coeffloop. Simulation results reportedly show ClassF: 0.00% AI, 0.03% RA, -0.02% LB BD-Rate, and ClassTGM: 0.02% AI, 0.01% RA, 0.05%LB BD-Rate, over the VTM5.0.

In the current design, there are differences in counting the context-coded bin in transform skip residual coding and regular transform coefficient coding. While in transform skip residual coding, the coded subblock flags are counted, these are excluded in the regular transform coefficient coding. In this contribution, it is proposed to align the counting of context-coded bins. It is suggested the exclude the coded subblock flags from counting the context-coded bins in transform skip residual coding.

BoG recommendation: Exclude coded subblock flags from counting context-coded bins, but do not modify the limit.

JVET-O0613 CE7-related: Unification of Rice parameter tables in residual coding [C. Auyeung, X. Li, S. Liu (Tencent)]

In VVC Draft 5, two Rice Parameter tables are used for residual coding, one table is used when transform skip is on, the other table is used when transform skip is off. This contribution proposes a method to use one Rice parameter table independently on the status of transform skip. With this method, one lookup table is removed and the amount of text description is reduced.

Compared to VTM-5.0 at CTC QPs with QP in [22, 27, 32, 37], experimental results show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

Compared to VTM-5.0 at low QPs with QP in [2, 7, 12, 17], experimental results show 0.00% AI, 0.00% RA, 0.00% LB luma BD-rate.

It is proposed to remove the Rice parameter table for transform skip residual coding and use the table for regular transform coefficient coding (with some additional arithmetic). A non-proponent remarked that the table only contains a few different values and may not be implemented as a table. It was further remarked that there is no real issue with the current design in this respect.

BoG recommendation: No action

JVET-O0758 Crosscheck of JVET-O0613 (CE7-related: Unification of Rice parameter tables in residual coding) [H.-J. Jhu (Kwai Inc.)]

Reported in this document is the cross-check results for the Unification of Rice parameter tables in residual coding as reported in the CTC-QP test conditions of JVET-O0613. The cross-check results match the coding performance reported by the proponent.

JVET-O0557 Non-CE7: Context modeling unification in significant flag of coefficient group [Y.-H. Chao, M. Coban, M. Karczewicz (Qualcomm)]

In this proposal, it is proposed to unify the context modeling of significant coefficient group (CG) flag, syntax element coded_sub_block_flag, in regular coefficient coding and transform skip residual coding. The unification shows minor difference versus VTM5.0: Class F: 0.01% AI, -0.04% RA, -0.01% LB BD-Rate, and Class SCC: 0.03% AI, -0.08% RA, -0.10% LB BD-Rate.

The contribution proposed to align the context derivation for the coded subblock flag in transform skip residual coding and regular transform coefficient coding. It was remarked that there is no real problem with the current design.

BoG recommendation: No action.

JVET-O0777 Crosscheck of JVET-O0557 (Non-CE7: Context modeling unification in significant flag of coefficient group) [Y.-W. Chen (Kwai Inc.)]

Reported in this document is the cross-check results for the context modeling unification in significant flag of coefficient group as reported in JVET-O0557. The cross-check results match the coding performance reported by the proponent.

JVET-O0559 CE7-related: QP dependent binarization in TS residual coding [M. Karczewicz, Y.-H. Chao, M. Coban (Qualcomm)]

In this proposal, a new binarization method for transform skip residual coding is proposed, where the binarization of absolute coefficient levels is adjusted based on the dynamic range of the quantized residuals. In addition, a QPoffset is introduced which modifies QP to be QP – QPoffset if the corresponding CU uses RDPCM mode. The proposed method is implemented on top of JVET-O0556 and CE7-3.10 (with RDPCM). The results show: Class F: -0.28% AI, -0.61% RA, -0.34% LB BD-Rate, and Class SCC: -1.22% AI, -0.72% RA, -0.29% LB BD-Rate versus JVET-O0556+CE7-3.10 (with RDPCM). Compared to VTM5.0 anchor (with RDPCM), the results show Class F: -0.91% AI, -1.03% RA, -0.67% LB BD-Rate, and Class SCC: -2.34% AI, -1.50% RA, -0.90% LB BD-Rate.

The contribution suggests the following binarization: The maximum dynamic range (which depends on the QP value), is divided into 4 equally-sized intervals. The following binarization is used: The syntax elements sig_coeff_flag, abs_level_gt1_flag and coeff_sign_flag are transmitted in the first pass. When abs_level_gt1_flag is equal to 1, the interval index and the remainder inside the interval is transmitted.

In addition, the QP is decreased by 3 when the CU uses RDPCM mode.

The following coding efficiency improvements are reported for TGM (relative to adopted CE7-3.10): -1.22% (AI), -0.72% (RA), -0.29% (LB).  Virtually no coding efficiency impact for non-SCC content.

BoG Recommendation: Study in CE

JVET-O0776 Crosscheck of JVET-O0559 (CE7-related: QP dependent binarization in TS residual coding) [Y.-W. Chen (Kwai Inc.)]

Reported in this document is the cross-check results for the QP dependent binarization in TS residual coding as reported in JVET-O0559. The cross-check results match the coding performance reported by the proponent.

JVET-O0556 CE7-related: Alternative method to RDPCM with TS level mapping [M. Karczewicz, Y. Han, Y.-H. Chao, M. Coban (Qualcomm)]

In this proposal, it is proposed to apply the level mapping in JVET-O0122 on RDPCM mode, replacing the quantized level domain prediction in RDPCM mode with the level mapping proposed in JVET-O0122. The proposed scheme on top of CE7-3. in JVET-O0122 show Class F: -0.64% AI, -0.43% RA, -0.34% LB BD-Rate, and Class SCC: -1.13% AI, -0.79% RA, -0.62% LB BD-Rate, versus the VTM-5.0 anchor under CTC. Compared to CE7-3.10, the proposed scheme shows only minor losses: Class F: 0.12% AI, 0.17% RA, 0.07% LB BD-Rate, and Class SCC: 0.12% AI, 0.00% RA, 0.05% LB BD-Rate.

The contribution presents results with RDPCM disabled (but still transmitting the RDPCM flag and using the unfiltered horizontal/vertical intra prediction). If the level mapping of JVET-O0122 is used in both the reference and the test, only a very minor coding efficiency loss is observed.

No action requested.

JVET-O0774 Crosscheck of JVET-O0556 (CE7-related: Alternative method to RDPCM with TS level mapping) [Y.-W. Chen (Kwai Inc.)]

Reported in this document is the cross-check results for the alternative method to RDPCM with TS level mapping as reported in JVET-O0556. The cross-check results match the coding performance reported by the proponent.

JVET-O0405 Non-CE8: Minimum Allowed QP for Transform Skip Mode [T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This document proposes to clip the Quantization Parameter (QP) in case of Transform Skip Mode (TSM) so that the minimum allowed QP for TSM is equal to four. That is necessary since the lossless condition is already met by a QP equal to four. When the QP is lower than four for TSM, the quantization stage introduces rounding errors, which is, from the authors point-of-view, an unintended behaviour. The results of the proposed bugfix are as follows. Note that the runtimes are not reliable.

Low QPs 2 – 7	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			-0.18%	-1.51%	-1.60%	101%	100%	100%

Random-Access:	-0.00%	-0.05%	-1.72%	100%	104%	100%

Conceptually similar to JVET-O0919. No presentation requested.

JVET-O0919 Non-CE8: Minimum QP for Transform Skip Mode [M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban (Qualcomm)] [late]

This document proposes to limit the minimum allowed Quantization Parameter (QP) in transform skip when the nearly lossless condition is reached. Specifically, the baseQP for transform skip is clipped to the value 4 + 6*(internalBitDepth - inputBitDepth). The results of the proposed bugfix are as follows.

Low QPs 2 – 7	(Luma BD-rates for CTC, Class F, TGM)

- Overall: -0.18%(AI) -0.01%(RA) -0.10%(LB)

- Class F: -1.51%(AI) -0.05%(RA) -0.40%(LB)

- Class TGM: -1.60%(AI) -1.72%(RA) 0.09%(LB)

It is reported that in lowQP test for class F and TGM sequences, the PSNR for QP=2 is typically lower than for QP=7. This is related to the fact that virtually lossless coding is achieved if the QP is set equal to

QP* = 4 – 6*( inputBitDepth – 8)

For the 8-bit material in class F and TGM, this QP is equal to 4. For this QP, the quantization step size is equal to 1 and thus nearly lossless coding is achieved. When reducing the QP, i.e., QP<QP*, the quantization step size becomes smaller than 1, and the distortion increases.

The BoG agreed that it is desirable to fix this issue. In order to do so, we need to transmit additional information. There are different possibilities, including the following:

- Input bit depth

- Clipping QP for transform skip

- Difference of original QP and minimum allowed

Given the minimum allowed QP for transform skip, the QP is clipped in the scaling process.

BoG recommendation: Adopt clipping with coding of QP difference

Benjamin agreed to provide working draft text.

## Coded Block Flag (4)

JVET-O0193 CE7-related: Remove transform depth in cbf coding [Y. Zhao, H. Yang (Huawei)]

This contribution proposes to remove transform depth in the context modeling of tu_cbf_luma and tu_cbf_cb. It is reported that this change results in 0.00%/0.00%/0.00%, 0.00%/-0.03%/-0.03% and 0.0x%/0.0x%/0.0x% Y/Cb/Cr BD-rates in AI, RA and LB configuration, respectively. Together with CE7-4.1, the simplification on cbf coding results in 0.0x%/0.0x%/0.0x%, 0.0x%/0.0x%/0.0x% and 0.0x%/0.0x%/0.0x% Y/Cb/Cr BD-rates in AI, RA and LB configuration, respectively

The contribution proposed to remove the dependency on transform depth for cbf coding; and it additionally removes one context. This doesn’t have any impact on coding efficiency.

BoG recommendation: Adoption

JVET-O0773 Crosscheck of JVET-O0193 (CE7-related: Remove transform depth in cbf coding) [Y.-W. Chen (Kwai Inc.)]

Reported in this document is the cross-check results for the remove reansform depth in cbf coding as reported in JVET-O0193. The cross-check results match the coding performance reported by the proponent.

JVET-O0375 Non-CE7: A Simplified CBF Coding for Luma and Chroma [A. Nalci, H. E. Egilmez, A. Said, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes to remove the transform tree depth (i.e. trDepth) dependency in coded block flag (CBF) coding. In VTM-5.0 a total of 12 CABAC contexts for CBF coding for both luma and chroma. In this contribution, it is proposed to remove trDepth dependency when coding the CBF flag. As a result, the total number of CBF related contexts for luma and chroma is reduced by 2 (e.g. 1 for luma and 1 for Chroma). This approach results in BD rates of (0.00%, 0.00% 0.00%) for all intra (AI), (0.00%, -0.03% -0.03%) for random access (RA), and (0.03%, 0.17% 0.02%) for LDB under common test conditions (CTC).

See notes on JVET-O0193.

JVET-O0231 Non-CE7: Modified chroma coded block flag signalling [S. Esenlik, Y. Zhao, A. M. Kotra, H. Gao, B. Wang, E. Alshina (Huawei)]

The current contribution proposes an alternative way of coding the coded block flags for Cb and Cr blocks. Based on the statistical observation, a first flag (tu_cbf_cb_or_cr) indicating whether Cb or Cr  blocks have residual is signalled. If the flag tu_cbf_cb_or_cr is true then a second flag (tu_joint_cbcr_residual) is signalled, which when true indicates that both Cb and Cr blocks have residual and also the condition “cb_residual = -cr_residual” is valid. If tu_joint_cbcr_residual flag is false, then a third flag (tu_cbf_cb_and_cr) indicating that both Cb and Cr blocks have residual is signalled. If the flag tu_cbf_cb_and_cr is false then (tu_cbf_cb) is signalled which when true indicates tu_cbf_cb is true (Cb has residual) and when false indicates tu_cbf_cr is true (Cr has residual).

The objective results with the above “cbf” coding scheme are indicated in the table below

| Config. | Y | U | V | EncT | DecT | (6Y+U+V)/8 |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.13% | −1.19% | -1.21% | 101% | 100% | -0.20% |
| RA | 0.21% | -1.67% | -2.76% | 101% | 101% | -0.39% |
| LDB | 0.13% | -0.38% | -3.65% | 101% | 103% | -0.41% |
| LDP | 0.12% | -0.41% | -3.92% | 101% | 101% | -0.45% |

The contribution proposes an alternative way of coding the chroma cbf flags and the joint chroma coding flag. It shows improvements in chroma BD rate, but losses in luma BD rate. It was remarked that the syntax was changed with the adoption of CE7-2.1. One aspect is that a first flag indicates whether or not, there is any residual for chroma blocks. It was suggested to further study this approach with the adopted modification of chroma residual coding (CE test CE7-2.1).

BoG Recommendation: Study in CE

JVET-O0872 Crosscheck of JVET-O0231 (Non-CE7: Modified chroma coded block flag signalling) [J. Lainema (Nokia)]

JVET-O0596 CE7-related: On chroma CBF signaling of subblock transform mode [X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

Subblock transform (SBT) mode is included in the VVC working draft 5 to enhance the transform coding efficiency of inter coding units (CUs). It is reported that there is a mismatch between the VVC working draft 5 and the test model software VTM-5.0 on signaling the coded block flags (CBFs) of chroma components for the SBT mode. Fixes are provided in this contribution to align the test model software with the text specification. Simulation results reportedly show the BD-rate impact of the proposed fixes is negligible.

There two issues reported (mismatch between spec text and software):

- Position of chroma cbfs for SBT TUs (needs to be fixed in the software)

- Context derivation for SBT chroma blocks (no change would be required if we adopt O0193 as recommended by the BoG)

BoG recommendation: Proponent provides fix for the software

JVET-O0753 Crosscheck of JVET-O0596 (CE7-related: On chroma CBF signaling of subblock transform mode) [Y. Zhao (Huawei)]

## Reduction of Context-Coded Bins & Number of Contexts (5)

JVET-O0068 AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0 [M. Zhou (Broadcom)]

This contribution reports the bin to bit ratio of the VTM5.0 for both the CTC and low QP setting (QP =2, 7, 12, 17).  The summary results are as follows:

- On average for  CTC AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM5.0 is 19.71%/20.88% /16.99% higher than the HM16.19, up from 15.72%/17.39%/13.34% in the VTM4.0, respectively.

  - the unweighted bin to bit ratio in the VTM5.0 is 14.21%/15.25%/12.88% higher than the HM16.19, up from 12.38%/13.75%/11.25% in the VTM4.0, respectively.

- On average for low-QP AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM5.0 is 20.24%/19.00%/19.67% higher than the HM16.19,  up from 18.11%/17.17%/17.67% in the VTM4.0, respectively.

  - the unweighted bin to bit ratio in the VTM5.0  is 11.40%/8.39%/10.28% higher than the HM16.19, up from 10.45%/7.63%/9.54% in the VTM4.0, respectively.

The newly adopted residual coding for transform skip mode has significantly increased the bin to bit ratio for screen content coding. From VTM4.0 to VTM5.0, the weighted bin to bit ratio (averaged over AI/RA/LD_B) in class F sequences is increased by 11.10% for CTC and 5.61% for low-QP setting, respectively. The same average number for other classes is 3.71% for CTC and 1.98% for low-QP setting, respectively.

The BD-rate reduction of the VTM5.0 relative to the HM16.19 at low QPs (QP =2, 7, 12, 17) is -12.99%, -14.46%, -13.77% and -14.94% in AI, RA, LD_B and LD_P configurations, respectively. When compared to HM16.19, the incremental BD-rate reduction from VTM4.0 to VTM5.0 is -0.60% in AI, -0.47% in RA, -0.08% in LD_B and -0.25%, respectively.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1). All the percentage numbers are measured against the HM16.19.

Presenter was not available.

JVET-O0400 CE7-related: Context-coded bin restriction [J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim (LGE)]

This contribution proposes to further restrict the total number of context-coded bins for coefficient coding using the intra sub-partitions (ISP) mode. The restriction for the ISP mode is set to number of coefficients in a certain unit (coefficient group (CG) or transform unit (TU)) times the limit of the context-coded bin count (1 bin/coefficient). The experimental results show X.XX% AI, X.XX% RA, X.XX% LB BD-Rate versus the VTM-5.0 anchor under common test conditions.

The contribution proposes to use a maximum number of context-coded bins for ISP blocks. It was remarked that in the CE7 discussion, there was an agreement that using different ratios for different block types is undesirable. In that spirit, in the CE7 discussion, it was agreed that the same value should be used for luma and chroma blocks (see discussion on CE7-1.1).

BoG recommendation: No action.

JVET-O0909 Crosscheck of JVET-O0400: CE7-related: Context-coded bin restriction [W. Zhu (Bytedance)]

JVET-O0575 CE7-related: Reduction for Context Coded Bins in Transform Skip Mode [W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu (Bytedance)]

In VTM-5.0, the bin2bit ratio for classF sequences are higher than other classes, which brings a throughput issue for screen content coding. It is proposed to limit the number of context coded bins for TS mode based on the block size, with less context coded bins limits for small blocks. Compared with VTM-5.0, the proposed method has {0.53%, 0.31%} and {0.25%, 0.18%} BD-rate changes on TGM sequences and class F sequences under the AI/RA configurations with common test conditions (CTC QPs). For low QPs, {0.92%, -2.67%} and {0.29%, 0.75%} BD-rate changes on TGM and class F sequences are reported. On TGM sequences, average transform coefficient bin to bit ratio under CTC shows the proposed method achieve -2.09%/-1.43% changes and -2.98%/-2.41% change under AI/RA configurations with CTC and low QPs respectively. On class F sequences, average transform coefficient bin to bit ratio under CTC shows the proposed method achieve -1.03%/-0.96% changes and -1.37%/-1.32% change under AI/RA configurations with CTC and low QPs respectively.

It was suggested to use different values for deriving the maximum number of context coded bins. A value of 1.5 was suggested for IBC and BDPCM with block sizes larger than 16 samples, and 1.75 otherwise.

It was remarked that in the CE7 discussion, it was agreed not use different values for different block types. The proposed change results in some coding efficiency losses for SCC (0.53% for AI TGM, 0.31% for RA TGM). It was reported that the b2b ratio was decreased by 2.2% AI TGM, 2.4% RA TGM.

It was remarked that the b2b ratio might not be a big problem for low bit rates as they are typically observed for SCC sequences.

Question to JVET: Should the bins per coefficient be aligned between transform coefficient coding and TSkip residual coding?

BoG recommendation: Further study (outside CE).

JVET-O0617 CE7-related: Context reduction of transform coefficient significance flag [C. Auyeung, X. Zhao, X. Li, S. Liu (Tencent)]

This contribution presents three proposals to reduce the number of sig_coeff_flag context models from 90 contexts to 75, 60 and 48, respectively. The experiments were conducted following CE7 testing conditions.

For the 48 contexts (47% reduction) proposal, experimental results show CTC QP: 0.07% AI, 0.08% RA, 0.03% LB luma BD-rate and Low QP: 0.04% AI, 0.08% RA, 0.09% LB luma BD-rate.

For the 60 contexts (30% reduction) proposal, experimental results show CTC QP: 0.00% AI, 0.00% RA, -0.01% LB luma BD-rate and Low QP: 0.00% AI, -0.01% RA, -0.01% LB luma BD-rate.

For the 75 contexts (17% reduction) proposal, experimental results show CTC QP: 0.00% AI, 0.01% RA, -0.04% LB luma BD-rate and Low QP: -0.01% AI, -0.01% RA, -0.02% LB luma BD-rate.

See discussion in JVET-O0928.

JVET-O0936 Crosscheck of JVET-O0617 (CE7-related: Context reduction of transform coefficient significance flag) [H. Schwarz (Fraunhofer HHI)]

The cross checker reports that the source code provided by Tencent was carefully analyzed and found to be consistent with the description in the input document JVET-O0617.  The cross checker could exactly reproduce the results reported in JVET-O0617.  The measured encoder and decoder run time ratios are similar to those reported in JVET-O0617.

Note: Some results (CTC/LB/class B and lowQP/RA/classes A1+A2) are not finished and will be updated in a revision of the document.

JVET-O0928 CE7-related: Context reduction for entropy coding sig_coeff_flag [S.-T. Hsiang, S.-M. Lei (MediaTek)] [late] [miss]

In VVC Draft 5, the total number of the context variables for entropy coding the syntax element sig_coeff_flag is significantly increased by allocating three separate sets of context variables for different dependent quantization states. This contribution proposes to reduce the number of context variables in the two additional context sets associated with dependent quantization states 2 and 3, motivated by the observation that the context variables associated with the local sum of the partially reconstructed level values locSumAbsPass1 > 0 in a context subset corresponding to the same transform block zone often exhibit similar probability distribution. The proposed method reportedly provides 0.01%, 0.00%, and 0.xx% luma BD-rates for the AI, RA, and LB settings, respectively, under the common test conditions (CTC) while achieving reduction of context variables by 24 or 40% of the total number of context variables associated with dependent quantization states 2 and 3.

The contributions JVET-O0617 and JVET-O0928 propose a reduction of the number of context models for the coeff_sig_flag. JVET-O0617 reports results for 3 different configurations. The configuration that removes 30 context models seems to provide the best trade-off; it does not impact coding efficiency, is a rather straightforward modification of the current design.

The contribution JVET-O0928 proposes the removal of 24 context models for quantization stages 2 and 3. It was remarked that both JVET-O0617 and JVET-O0928 could be combined, but no results are available for this combination.

BoG Recommendation: Adopt JVET-O0617, variant with removal of 30 context models

Another contribution O1098 was submitted late (same topic as 928, saves 24 contexts). This contribution is the same as JVET-M0107, which was submitted to the 13th JVET meeting.

## Scaling Matrices (3)

JVET-O0223 Non-CE7: Quantization matrices with single identifier and prediction from larger ones [P. de Lagrange, F. Le Léannec, E. François, K. Naser (InterDigital)]

This contribution proposes to simplify quantization matrix (QM) signalling and prediction, while enhancing them so that any matrix can be predicted from any previously signalled one, by: identifying a QM with a single index, transmitting QMs in decreasing block size order, and specifying the prediction process as either a copy or decimation process.

It also describes a unified QM derivation process that encompasses selecting a decoded QM for a given transform block, up-sampling for blocks larger that 8x8, and down-sampling for rectangular blocks.

These modifications are said to reduce by half the text needed to support QMs, and to save about 50% of the bits needed to signal QMs from a provided test set.

The contribution was discussed in Track A

JVET-O0924 Crosscheck of JVET-O0223 (Non-CE7: Quantization matrices with single identifier and prediction from larger ones) [J. Le Tanou (MediaKind)]

This input document reports crosscheck results of JVET-O0223 on Quantization matrices with single identifier and prediction from larger ones from Interdigital. JVET-O0223 proposes to modify quantization matrix (QM) signaling and prediction mechanism, initially adopted in VVC WD5 (as per JVET-N0847). Authors in JVET-O0223 provided draft text changes on top of current VVC WD5, as well as text count statistics for highlighting text simplification. Draft text and statistics have been verified and are reported consistent with the proposed changes. In addition, a GNU Octave source code, a test set of popular QMs and a test procedure for running code were provided by Authors to measure the bit cost reduction of the proposed QMs signaling and prediction process in comparison with HEVC and VVC WD5 (as per JVET-N0847) mechanisms. The current crosscheck reports that the GNU Octave source code is only implementing the encoding processes for bit generations and cost measurements. Decoding processes for compliance verification were not provided, nor crosschecked. However, the implemented encoding processes seem consistent with the proposed modifications in JVET-O0223, as well with the related specification texts in HEVC and in VVC WD5 being used for comparison. Following the provided test procedure, code has been successfully run. The bit costs and relative bit savings results are reported to match the ones in JVET-O0223. No implementation or test results on top of VTM-5.0 were provided nor cross-checked for performance and compliance verification purposes.

JVET-O0383 Non-CE7: Harmonization of scaling matrix and LFNST [T. Hashimoto, E. Sasaki, T. Chujoh, T. Ikai (Sharp)]

Scaling matrix and LFNST have been adopted in VVC-5 draft. In VVC-5, scaling matrices are applied even in the case of LFNST mode, which is asserted to be wrong since LFNST’s coefficient does not correspond to conventional transform coefficients in spatial frequency meaning. This contribution proposes a fix for this problem which performs scaling matrices in LFNST case as well at the normal transform coefficient domain. Specifically this proposal applies scaling matrices after secondary transform in decoder side. The proposed scheme provides transform coefficient control capability in all conventional and LFNST transform cases. It is also reported that experimental result confirmed LFNST coding gain in scaling matrices is similar to that of non-scaling matrices.

The proponent remarked that the usage of scaling matrices is not suitable for block in which LFNST is used, since the transform coefficients are no longer related to spatial frequencies.

The proposal suggest the following modification for decoding process of transform blocks that use LFNST:

- In the transform coefficient scaling, set the scaling factor equal to 16.

- Then, apply LFNST

- After LFNST, apply another scaling with the scaling matrix

c[x][y] = ( s[x][y] * d[x][y] + 8 ) >> 4

with s[x][y] being the scaling matrix, d[x][y] being the reconstructed intermediate coefficients after LFNST, and c[x][y] representing the input samples for the inverse DCT/DST.

BoG recommendation: Discuss further in track

JVET-O0791 Crosscheck of JVET-O0383 (Non-CE7: Harmonization of scaling matrix and LFNST) [Y.-L. Hsiao (MediaTek)]

This document reports the results of crosschecking JVET-O0383 proposed by Sharp. JVET-O0383 proposes a method to fix for the problem that scaling matrices are not applied in the case of LFNST mode where flat scaling is always used even if scaling matrices is applied in other transform mode which performs scaling matrices in LFNST case as well at the normal transform coefficient domain. The verification task was done, and the BD-rate results match exactly with those in JVET-O0383.

JVET-O0527 Non-CE7: Use INTER quantization matrices for IBC [P. de Lagrange, Y. Chen (InterDigital)]

This contribution proposes to use INTER quantization matrices to IBC-coded blocks, instead of INTRA quantization matrices. This is said to align the design with IBC prediction mode being closer to INTER than INTRA.

A similar contribution (which also proposes to use INTER quantization matrices for IBC) was discussed in Track A.

## Joint Chroma Residual Coding (3)

JVET-O0543 CE7-related: Alternative configuration for joint chroma residual coding [C. Helmrich, H. Schwarz, T. Nguyen, C. Rudat, D. Marpe, T. Wiegand (Fraunhofer HHI)]

This document reports simulation results for an alternative configuration of the CE test CE7-2.1.  The test CE7-2.1 extends the joint coding mode for chroma residuals; it supports three different joint coding modes (instead of one in VTM-5) for chroma residuals (which are indicated by the chroma cbfs).  The Cb and Cr residuals are derived from a single coded transform block.  In the configuration proposed in this contribution, the two additional joint chroma coding modes are only supported in intra slices; in inter slices, only one of the joint coding modes is supported (the VTM-5 mode).

For the SDR common test conditions, the following average BD rates are reported relative to VTM-5.0:

AI:		–0.18% (Y),    –0.57% (Cb),     0.07% (Cr) 
RA:	–0.14% (Y),    –0.42% (Cb),     0.65% (Cr) 
LB:		  0.00% (Y),    –0.02% (Cb),     0.30% (Cr)

For the HDR common test conditions, the following average BD rates (in %) are reported relative to VTM-5.0:

AI:		(DE100) –3.05,  (L100) –0.05,  (wPSNR) –0.06, –4.26,   0.75,  (PSNR) –0.07, –3.33,   0.42
RA:	(DE100) –2.36,  (L100) –0.01,  (wPSNR) –0.04, –3.51,   1.71,  (PSNR) –0.06, –2.74,   1.04

For the low-QP configuration (QP values 2, 7, 12, 17 and 100 coded frames) for SDR, the following BD rates are reported relative to VTM-5.0:

AI:		–0.05% (Y),    –0.14% (Cb),     0.10% (Cr) 
RA:	  0.01% (Y),    –0.20% (Cb),   –0.07% (Cr) 
LB:		  0.01% (Y),    –0.15% (Cb),   –0.19% (Cr)

In this contribution additional results based on CE test CE7-2.1 are reported. It was asserted that the additional joint chroma coding modes are mainly advantageous for intra slices. The contribution proposes to enable the additional chroma modes only for intra slices. – It corresponds to a syntax change only (more precisely, the condition for the presence of the joint chroma coding flag is modified). – Relative to CE7-2.1, the results are virtually the same for AI and RA, but improvements for LB are reported.

BoG recommendation: Adopt

JVET-O0874 Crosscheck of JVET-O0543 (CE7-related: Alternative configuration for joint chroma residual coding) [J. Lainema (Nokia)]

JVET-O0670 CE7-related: Alternative joint coding of chroma residuals [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution reports an alternative version of joint coding of chroma residuals, where the method described in JVET-N0347, which is studied in test CE7-2.3, is only applied for intra blocks, and for non-intra blocks, the joint chroma coding is kept unchanged, i.e., the method in VTM-5.0 that was adopted from JVET-N0054. The results for CTC and lowQP test conditions compared to VTM-5.0 are reported below:

Table-1: Average BD rate results for CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.12% | -0.05% | 0.46% | 102% | 99% |
| RA | -0.02% | -0.66% | 0.37% | 102% | 102% |
| LD | 0.03% | 0.10% | 0.34% | 99% | 101% |

Table-2: Average BD rate results for lowQP (Pending)

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI |  |  |  |  |  |
| RA |  |  |  |  |  |
| LD |  |  |  |  |  |

No presentation requested.

JVET-O0875 Crosscheck of JVET-O0670 (CE7-related: Alternative joint coding of chroma residuals) [J. Lainema (Nokia)]

JVET-O0935 CE7-related: Alternative configuration of CE7-2.2 joint chroma residual coding [C. Helmrich, H. Schwarz, T. Nguyen, C. Rudat, D. Marpe, T. Wiegand (Fraunhofer HHI, B. Ray, G. Van der Auwera, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)] [late]

This document reports simulation results for an alternative configuration of the CE test CE7-2.2.  The test CE7-2.2 extends the joint coding mode for chroma residuals; it supports three different joint coding modes (instead of one in VTM-5) for chroma residuals (which are indicated by the chroma cbfs).  The Cb and Cr residuals are derived from a one or two coded transform block.  In the configuration proposed in this contribution, the two additional joint chroma coding modes are only supported in intra slices; in inter slices, only one of the joint coding modes is supported (the VTM-5 mode).

For the SDR common test conditions, the following average BD rates are reported relative to VTM-5.0:

AI:		–0.22% (Y),    –0.42% (Cb),   –0.06% (Cr) 
RA:	–0.15% (Y),    –0.31% (Cb),     0.47% (Cr) 
LB:		  0.03% (Y),      0.00% (Cb),     0.12% (Cr)

For the HDR common test conditions, the following average BD rates (in %) are reported relative to VTM-5.0:

AI:		(DE100) –2.98,  (L100) –0.06,  (wPSNR) –0.04, –3.85, –0.85,  (PSNR) –0.05, –2.81, –0.96
RA:	(DE100) –2.71,  (L100) –0.04,  (wPSNR) –0.04, –3.75,   0.14,  (PSNR) –0.06, –2.66, –0.11

For the low-QP configuration (QP values 2, 7, 12, 17 and 100 coded frames) for SDR, the following BD rates are reported relative to VTM-5.0:

AI:		–0.18% (Y),      0.00% (Cb),   –0.04% (Cr) 
RA:	  x.xx% (Y),      x.xx% (Cb),     x.xx% (Cr) 
LB:		  0.01% (Y),    –0.12% (Cb),   –0.22% (Cr)

No presentation requested.