| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1118 |
| --- | --- |

| Title: | Core Experiment Viewing Test Procedure and Results |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Lehrstuhl für Bildverarbeitung RWTH Aachen University | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

This contribution provides a report on the subjective testing performed at the 15th JVET meeting. Subjective tests have been conducted for CE5-2, CE5-3 (CE report JVET-O0025), and for document JVET-O0548. Upon completion, the results were provided to the chairs for consideration in the discussion of the respective CEs. The report includes a description of the evaluation method, the testing procedure and the test results for the above-mentioned CEs. MOS plots for the results of all performed tests are reported in the document. The corresponding data is provided in the accompanying Excel sheet.

# Introduction

Expert viewing tests were conducted during the 15th JVET meeting at Gothenburg in July 2019 for the core experiments CE5-2, CE5-3 (CE report JVET-O0025), and by request during the meeting also for bilateral filtering proposed in document JVET-O0548. CE5-2 contained 5 sub-experiments. CE5-3 and JVET-O0548 comprised a comparison of one technical proposal to the VTM as the anchor. Calls for participation in the subjective viewing were issued on the JVET reflector and verbally during JVET Track A and B sessions. For each test campaign, at least 12 experts were invited from the list of experts who volunteered. The dates and timing of the viewing sessions are reported in the results section of this document. The resulting MOS values are provided in the accompanying Excel sheet. The testing procedure follows the method used for visual testing during the 13th and 14th JVET meetings [1][2].

# Test arrangement

The sequences were presented on an LG OLED55C8PLA display using a Linux PC (Ubuntu 16.04) with a pre-installed video player capable of playing UHD 60 Hz video at full frame rate and stable output. Three expert viewers were seated in front of the screen at a distance of 1.6 times the height of the screen. The room did not have windows. Before the viewing sessions the light in the room was turned off.

The YUV files for the tests were decoded on the test PC, with the proponents and cross-check partners verifying the correctness of the decoded YUV files and the assigned YUV file names by comparison of MD5 sums.

CE description JVET-N1025 [3] defined test sets with a low number of sequences at various resolutions (720p, 1080p, 4K). It was decided to present all sequences scaled to the full UHD resolution of the screen. Nearest neighbor interpolation was used for up-scaling.

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM 5.0 anchor.

In the following, video A and video B denote either the anchor or the proposal for the same sequence and same QP for a test point of the core experiment under evaluation. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. A viewing session consisted of 12 to 15 test cells, with each test cell constructed as follows:

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

For a video sequence duration of 10 s, the duration of one test cell thereby accumulates to 47 s.

The sequence of test cells included each test cell once. The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The sheet records the CE, test session, ID of the test subject and position in front of the screen.

Fig. 1: Voting sheet

In each viewing test, the order of the video sequences was fixed and looped over the available test points (QP values, sub-CEs). Thereby it was asserted that not two test points for a video sequence would be presented subsequently. The order of the available test points was randomized over the session.

Before the start of the test session, the viewers were shown example video sequences encoded with VTM 5.0 at QP22 demonstrating banding artifacts visible on the display, which were assumed to be induced by 8-bit playout over HDMI and the applied up-scaling method. The viewers were instructed not to consider these artifacts when rating the test sequences. The viewers were further shown two example test cells in order to familiarize them with the test procedure.

## Session arrangement

The viewing tests were carried out with groups of three experts. The tests were drafted to have a maximum assessment time below 15 min per session. For tests with more than one session, the assessment of two groups was interleaved: While one group was under test, the other group had a break. In the break, the groups were advised to enter the voting results of their previous session into a prepared Excel sheet and verify the correctness of this transfer. This arrangement was set up to minimize the test duration for the participating experts while providing them sufficient recreation over the testing period.

## Voting scores

For evaluation, the votes of the test subjects were mapped to numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality.

# Test results

## CE5-2 Deblocking at 4x4 block boundaries

The description of the core experiment can be found in JVET-N1025 [3]. CE5-2 contains 5 subtests. The following sequences and QPs were used for subjective performance comparisons. By decision of JVET in the Track B session on Thu. 2019-07-04, only the ALF-on configuration was assessed.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |
| 06 | EBULupoAndCandleLight YCbCr 4:4:4 | 1920x1080p | 50fps | 500 | LDB | 32, 37 |

The viewing was conducted on Fri. 2019-07-05 19:00h, Sat. 2019-07-06 09:00h, Sat. 2019-07-06 19:30h, with results of all subtests CE5-2.1 to CE5-2.5 assessed in comparison to the anchor. The test comprised 60 test cells, split into 15 cells per session. 18 test subjects in six groups participated in the viewing. The viewing time per session was approximately 11:45 min.

### MOS Plots

## CE5-3 MV difference threshold in deblocking decisions

The description of the core experiment can be found in JVET-N1025 [3]. CE5-3 contains one subtest. The following sequences and QPs were used for subjective performance comparisons. By decision of JVET in the Track B session on Thu. 2019-07-04, only the ALF-on configuration was assessed.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| 02 | Campfire | 3840x2160p | 30fps | 300 | RA | 37, 32 |
| 03 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 06 | Johnny | 1280x720p | 60fps | 600 | LDB | 32, 37 |

The viewing was conducted in the morning of Fri. 2019-07-05, with the results assessed in comparison to the anchor. The test comprised 12 test cells in a single session. 15 test subjects in five groups participated in the viewing. The viewing time per session was approximately 09:30 min.

### MOS Plots

## Bilateral Filtering (JVET- O0548)

The description of the proposed filtering is provided in JVET-O0548 [4]. The following sequences were selected by the proponents. The test point with QP32 and QP37 were assed.

| # | Class | Sequence name | Frame rate | Cod.str. |
| --- | --- | --- | --- | --- |
| S01 | B | Cactus | 50 | RA |
| S02 | F | ArenaOfValor | 60 | LD |
| S04 | A2 | CatRobot | 60 | RA |
| S05 | E | KristenAndSara | 60 | LD |
| S06 | B | BQTerrace | 60 | RA |
| S08 | A2 | DaylightRoad2 | 60 | RA |

The viewing was conducted on Sun. 2019-07-07 at 09:30h, with the results assessed in comparison to the anchor. The test comprised 12 test cells in a single session. 15 test subjects in five groups participated in the viewing. The viewing time per session was approximately 09:30 min.

### MOS Plots

# Thanks

- Thanks to all volunteers who participated in the tests.

- Thanks to Andrew Segall for providing the test PC, video player, and support.

- Thanks to Andrey Norkin and Christian Helmrich for checking the results.

- Thanks to Ericsson for providing OLED display.

- Thanks to Vittorio Baroncini for his guidance.

# Conclusions

With respect to the conduction of visual tests conducted as this meeting, the following aspects are noted:

- Viewing sessions may need to include sequences of different resolution if the CE definition includes a corresponding sequence selection. At this meeting, all sequences were presented full-screen, with nearest-neighbor interpolation used for eventually required up-scaling. This aspect should be considered when drafting the CE definition.

- It would be helpful to have the proponents recommend one representative best-case test point for each test to be made (test sequence and QP with best performance relative to the anchor). This comparison could be used in the training of the test subjects to calibrate the expectable range of changes during the test session.

- CE participants should upload bitstreams, decoder binaries, and a decoding script to the dedicated CE directory on the JVET ftp site. The binaries and scripts should be adapted to the operating system of the test PC (e.g., Ubuntu 16.04 with gcc 5.4.0 at this meeting). The data should be available at least three days before the meeting to allow for upfront transfer to the test PC. 
The recommended YUV file name convention is as follows, with “P” denoting the proposal, and “A” denoting the anchor. The subCE identification should only use dashes:

CE<xx-y-z>_S<ii>_QP<jj>_{P|A}.yuv, 		e.g.: CE05-1-2_S02_QP34_P.yuv

The script should include the generation of a md5sum log-file for verification of the decoded results. The correctness of the assignment of the filenames should be verified and reported by the cross-checkers of the respective experiment. It is further suggested to always activate the picture hash SEI message in the uploaded bitstreams to help identification.

# References

- V. Baroncini, A. Norkin, A. M. Kotra, K. R. Andersson, K. Misra, H. Jang, C.M. Tsai, D. Rusanovskyy, “Subjective assessment of CE11 proposals,“ Document JVET-M0906, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 13th meeting, Jan. 2019.

- M. Wien, P. Hanhart, “Core Experiment Viewing Test Procedure and Results,” Document JVET-N0835, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, 14th meeting, Mar. 2019.

- C.-Y. Chen, A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering,” Document JVET-N1025, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, 14th meeting, Mar. 2019.

- J. Ström, P. Wennersten, J. Enhorn, D. Liu, K. Andersson, R. Sjöberg, “Combined bilateral/SAO loop filter,” Document JVET-O0548, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, 15th meeting, Jul. 2019.