| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1120 |
| --- | --- |

| Title: | Non-CE5: Hadamard transform domain in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Alexander Karabutov | Tel: Email: | +7-906-035-18-23 sergey.ikonin@huawei.com stepin.victor@huawei.com karabutov.alexander@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports performance of Hadamard transform domain filter from JVET-N0479 on top of VTM5.0 and with deblocking RDO option enabled. enabled option as well combination of the filter with SAO.

Being applied as conventional in-loop the filter provides -0.39%/-0.45% of the luma BD-rate change with 107%/103% of encoding time and 107%/102% of decoding time for AI/RA configurations correspondingly.

Additionally it is proposed to apply the filter in parallel with SAO using output of deblocking filter is an input with following averaging of filter and SAO outputs. In this mode the filter provides --0.36%/-0.44% of the luma BD-rate change with 107%/110% of encoding time and 107%/105% of decoding time for AI/RA configurations correspondingly.

# Introduction

During discussion of JVET-N0479 proposal at Geneva meeting it was remarked that demonstrating filter performance over “a non-CTC anchor enabling the RDO-optimized deblocking would be welcome, to understand if such an additional processing element in the loop would have benefit by itself”. Suggested experiment was performed over VTM5.0 and it was noted that most of the gain is kept and performance drop followed by enabling encoder deblocking RDO is around 0.1%.

# Effect of deblocking RDO encoder option

## Test 1

The following table represents performance of Hadamard transform domain filter (HTDF) described in JVET-N0479 over VTM5-0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.47% | 0.38% | 106% | 104% |
| Class A2 | -0.39% | 0.28% | 0.22% | 107% | 109% |
| Class B | -0.31% | 0.37% | 0.38% | 107% | 108% |
| Class C | -0.50% | 0.17% | 0.28% | 106% | 107% |
| Class E | -0.46% | 0.22% | 0.27% | 107% | 108% |
| Overall | -0.39% | 0.30% | 0.31% | 107% | 107% |
| Class D | -0.44% | 0.55% | 0.40% | 106% | 107% |
| Class F | -0.49% | 0.20% | 0.15% | 103% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.27% | 0.01% | 103% | 102% |
| Class A2 | -0.63% | -0.56% | -0.35% | 103% | 102% |
| Class B | -0.41% | 0.01% | -0.30% | 104% | 102% |
| Class C | -0.47% | -0.12% | -0.13% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.20% | -0.20% | 103% | 102% |
| Class D | -0.48% | 0.14% | -0.07% | 103% | 103% |
| Class F | -0.43% | 0.06% | 0.17% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.28% | 0.27% | 0.23% | 102% | 102% |
| Class E | -0.18% | 0.97% | 0.97% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.60% | 1.01% | 1.35% | 102% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Test 2

The same filter with deblocking RDO enabled has the following performance.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with DbkRDO enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.52% | 0.59% | 107% | 106% |
| Class A2 | -0.23% | 0.32% | 0.19% | 107% | 105% |
| Class B | -0.10% | 0.35% | 0.34% | 107% | 107% |
| Class C | -0.35% | 0.25% | 0.15% | 107% | 106% |
| Class E | -0.23% | 0.17% | 0.28% | 107% | 107% |
| Overall | -0.19% | 0.32% | 0.30% | 107% | 106% |
| Class D | -0.32% | 0.29% | 0.41% | 106% | 106% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 with DbkRDO enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.10% | 0.13% | 105% | 102% |
| Class A2* | -0.53% | -0.37% | -0.45% | 103% | 101% |
| Class B | -0.31% | 0.13% | -0.04% | 105% | 102% |
| Class C | -0.37% | -0.01% | -0.09% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.01% | -0.10% | 104% | 102% |
| Class D | -0.42% | -0.05% | 0.10% | 103% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| *) one QP 22 sequence extrapolated from anchor |  |  |  |  |  |

From obtained result the performance drop for RA is around 0.1% that save most of the coding gain of proposed filter

# HTDF/SAO combination

HTDF

SAO

DBF

Avg

ALF

Additionally the filter was tested when it’s applied in parallel with SAO using output of deblocking filter is an input. This configuration allows to exclude delay of additional loop filter stage due to performing filtering operations of two filters in parallel.

## Test 3

The following performance numbers were observed:

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -0.30% |  | 0.50% |  | 0.39% |  | 107% |  | 110% |  |
| Class A2 |  | -0.39% |  | 0.32% |  | 0.27% |  | 106% |  | 109% |  |
| Class B |  | -0.29% |  | 0.42% |  | 0.44% |  | 107% |  | 111% |  |
| Class C |  | -0.46% |  | 0.26% |  | 0.39% |  | 107% |  | 108% |  |
| Class E |  | -0.39% |  | 0.30% |  | 0.35% |  | 107% |  | 110% |  |
| Overall |  | -0.36% |  | 0.36% |  | 0.38% |  | 107% |  | 110% |  |
| Class D |  | -0.35% |  | 0.66% |  | 0.50% |  | 107% |  | 108% |  |
| Class F |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -0.29% |  | -0.29% |  | 0.09% |  | 103% |  | 104% |  |
| Class A2 |  | -0.63% |  | -0.46% |  | -0.25% |  | 104% |  | 104% |  |
| Class B |  | -0.39% |  | 0.16% |  | -0.16% |  | 104% |  | 105% |  |
| Class C |  | -0.48% |  | 0.00% |  | -0.02% |  | 103% |  | 105% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.44% |  | -0.09% |  | -0.09% |  | 104% |  | 105% |  |
| Class D |  | -0.37% |  | 0.28% |  | 0.24% |  | 103% |  | 105% |  |
| Class F |  | -0.25% |  | 0.15% |  | 0.33% |  | 101% |  | 102% |  |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class C |  | -0.29% |  | 0.53% |  | 0.53% |  | 101% |  | 98% |  |
| Class E |  | -0.12% |  | 1.30% |  | 1.03% |  | 100% |  | 103% |  |
| Overall |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Class D |  | -0.43% |  | 0.15% |  | 1.08% |  | 102% |  | 102% |  |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  |

## Test 4

The same filter with deblocking RDO enabled has the following performance.

The following results were obtained with encoder deblocking RDO option enabled both in anchor and test:

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with DbkRDO enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.66% | 0.59% | 107% | 111% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.09% | 0.50% | 0.48% | 107% | 109% |
| Class C | -0.29% | 0.28% | 0.29% | 107% | 109% |
| Class E | -0.17% | 0.40% | 0.44% | 108% | 110% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.22% | 0.41% | 0.49% | 106% | 106% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 with DbkRDO enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.32% | 0.36% | 0.24% | 105% | 103% |
| Class C | -0.33% | 0.26% | 0.16% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.38% | -0.09% | 0.28% | 104% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

Considering that proposed filter demonstrates substantial coding performance over several iterations of VTM development we suggest to consider its adoption to next release of VVC.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).