| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1124-v1 |
| --- | --- |

| Title: | Draft text for CCLM restriction to reduce luma-chroma latency for chroma separate tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310000, China Chia-Ming Tsai, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Email: | {yin.zhao, haitao.yang} @huawei.com {chia-ming.tsai, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Huawei Technologies Co., Ltd MediaTek Inc. |  |  |

_____________________________

- Abstract

Draft text for JVET-O0196 based on VVC draft 5 is provided in this contribution. During CE2 BoG discussion, JVET-O0196 was considered the commonality of three proposals JVET-O0129, JVET-O0196 and JVET-O0274, and reduced luma-chroma latency in dual tree from 64x64 to 32x32 with minimized coding performance degradation via restricting the usage of CCLM based on split modes of luma and chroma coding tree nodes.

Draft text

Note: As the specification text improvement in JVET-O0194 was adopted, the variables CbWidth[x][y], CbHeight[x][y], CqtDepth[x][y] are extended to CbWidth[chType][x][y], CbHeight[chType][x][y], CqtDepth[chType][x][y], respectively, where chType equal to 0 denotes luma and chType equal to 1 denotes chroma.

    - 8.4.4	Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

The variable cclmEnabled is derived by invoking the process specified in clause 8.4.4.1 with the luma location ( xCb, yCb ) as input.

…

Table 8-5 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flagcclmEnabled is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flagcclmEnabled is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

### 8.4.4.1	Cross-component chroma intra prediction mode checking process

Input to this process is:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

Output to this process is:

- a flag cclmEnabled specifying if a cross-component chroma intra prediction mode is enabled or not for the current chroma coding block.

In this process, cclmEnabled is derived as follows.

- If sps_cclm_enabled_flag is equal to 0, cclmEnabled is set equal to 0.

- Otherwisse, if qtbtt_dual_tree_intra_flag is equal to 0 or CtbLog2SizeY is less than 6, cclmEnabled is set equal to 1.

- Otherwise the following applies.

  - The variables xCb64, yCb64, yCb32 are derived as follows.

xCb64 = (xCb >> 6) << 6

yCb64 = (yCb >> 6) << 6

yCb32 = (yCb >> 5) << 5

  - If one of the following conditions is true, the variable cclmEnabled is set equal to 1

    - CbWidth[ 1 ][ xCb64 ][ yCb64 ] is equal to 64 and CbHeight[ 1 ][ xCb64 ][ yCb64 ] is equal to 64.

    - CbWidth[ 1 ][ xCb64 ][ yCb32 ] is equal to 64 and CbHeight[ 1 ][ xCb64 ][ yCb32 ] is equal to 32

    - CqtDepth[ 1 ][ xCb64 ][ yCb64 ] is greater than CtbLog2SizeY – 6.

    - CqtDepth[ 1 ][ xCb64 ][ yCb64 ] is equal to CtbLog2SizeY – 6, MttSplitMode[ xCb64 ][ yCb64 ][ 0 ] is equal to SPLIT_BT_HOR, and MttSplitMode[ xCb64 ][ yCb32 ][ 1 ] is equal to SPLIT_BT_VER.

  - Otherwise, the variable cclmEnabled is set equal to 0.

  - If cclmEnabled is equal to 1 and one of the following conditions is true, cclmEnabled is set equal to 0

    - CbWidth[ 0 ][ xCb64 ][ yCb64 ] and CbHeight[ 0 ][ xCb64 ][ yCb64 ] are both equal to 64, and intra_subpartitions_mode_flag[ xCb64 ][ yCb64 ] is equal to 1.

    - CbWidth[ 0 ][ xCb64 ][ yCb64 ] or CbHeight[ 0 ][ xCb64 ][ yCb64 ] is less than 64, and CqtDepth[ 0 ][ xCb64 ][ yCb64 ] is equal to CtbLog2SizeY – 6.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).