| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1127-v1 |
| --- | --- |

| Title: | Non-CE3：Fixed downshifting for 8-bit MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Yanzhuo Ma, Fuzheng Yang (Xidian Univ.) Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki (Sony) Shuai Wan (NPU) Yuanfang Yu (OPPO) | Email: | jyhuo@mail.xidian.edu.cn, yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com, swan@nwpu.edu.cn yuyuanfang@oppo.com |
| Source: | Xidian University, Sony Corporation, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

Proposed is to fix downshifting operation to 6 binary digits in MIP mode.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over JVE-O0084:

For AI configuration: 0.00%, -0.01%, and 0.00%, with 100% EncT, 100% DecT;

For RA configuration: 0.00%, 0.01%, and -0.08%, with 100% EncT, 100% DecT;

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over 8-bit MIP in JVET-O0925: (partials results provided)

For AI configuration: xxx%, xxx%, and xxx%, with xxx% EncT, xxx% DecT;

For RA configuration: xxx%, xxx%, and xxx%, with xxx% EncT, xxx% DecT.

Introduction

In 8-bit MIP implementation [1], matrix-based intra prediction (MIP) is applied to derive the intra luma predicted samples as：

In JVET-O0084:

predMip[ x ][ y ] =

( ( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> sW )  + dcVal	(8-69)

where sW is defined in Table 8-8 as follows

Table 8-8 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 | 6 | 6 | 7 | 7 | 6 | 6 | 6 | 6 |
| 1 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |  |  |  |  |  |  |  |
| 2 | 7 | 7 | 7 | 7 | 6 | 7 |  |  |  |  |  |  |  |  |  |  |  |  |

In JVET-O0925:

predMip[ x ][ y ] =

( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> sW ) + pTemp[ 0 ]  (8-69)

wherein the weight shift sW depends on the different MipSizeId and modeId and is provided by a look-up table (i.e. Table 8-8 in [1]).

Table 8-8 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 |
| 1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 |  |  |  |  |  |  |  |  |
| 2 | 6 | 6 | 6 | 7 | 5 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |

Proposed Method

In this contribution, to reduce complexity and simplify the text of MIP mode, it is proposed to fix the value of sW to 6 and the look-up table for sW is removed. For the cases that weight shifts equal to 5 and 7 in the look-up table, the weight matrices and bias vectors are updated correspondingly. The intra luma samples are calculated as:

predMip[ x ][ y ] =

( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> 6 ) + pTemp[ 0 ]  (8-69)

Modified text based on 8-bit MIP implementation and VVC WD [2] is presented as below.

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

……

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-8.

- The variable fO is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-X.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) 32 – fO * (i = 0inSize - 1p i )	(8-65)

sB = BitDepthY − 1		(8-66)

incW = ( predC > mipW )  ?  2  :  1		(8-67)

incH = ( predC > mipH )  ?  2  :  1		(8-68)

predMip[ x ][ y ] = ( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> sW 6 ) + pTemp[ 0 ]		(8-69)

……

Table 8-8 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 |
| 1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 |  |  |  |  |  |  |  |  |
| 2 | 6 | 6 | 6 | 7 | 5 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |

Experimental Results

Experiments were performed based on 8-bit MIP implementation under JVET common test condition [3].Table 2 and Table 3 show experimental results of the proposed method compared to JVET-O0084 and JVET-O0925.

Table 2.  Results of the proposed method over JVET-O0084

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.09% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class B | 0.01% | -0.04% | 0.01% | 100% | 100% |
| Class C | 0.01% | 0.02% | -0.01% | 100% | 100% |
| Class E | -0.01% | 0.00% | -0.06% | 100% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.18% | -0.01% | 100% | 100% |
| Class F | 0.02% | -0.05% | 0.04% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.10% | 100% | 100% |
| Class A2 | 0.02% | 0.06% | -0.07% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.10% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.31% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.08% | 100% | 100% |
| Class D | 0.02% | 0.21% | -0.17% | 100% | 100% |
| Class F | 0.04% | 0.15% | 0.19% | 100% | 100% |

Table 3.  Results of the proposed method over JVET-O0925

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.04% | 0.01% | 89% | 100% |
| Class C | 0.00% | -0.02% | 0.07% | 107% | 100% |
| Class E | 0.00% | 0.00% | -0.07% | 117% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.07% | -0.08% | 109% | 100% |
| Class F | 0.02% | -0.10% | -0.03% | 134% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.14% | -0.01% | 118% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | -0.17% | 0.12% | 110% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this contribution, it is proposed to fix the value of sW to 6 in MIP mode and the look-up table for sW is removed. Experimental results confirmed that the BD-rate impact is negligible. It is recommended to adopt the method into VVC.

References

[1]  J. Pfaff et al., “Non-CE3: Simplifications of MIP”, JVET-O0925, Gothenburg, Sweden, July 2019.

[2]  B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, Geneva, Switzerland, March 2019.

[3]  F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019.

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).