| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1140-v1 |
| --- | --- |

| Title: | Unified syntax for JVET-O0184/O0250/O0504 on DMVR and BDOF flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yuwen He, Ahmed Hamza (InterDigital) Han Huang, Wei-Jung Chien, Vadim Seregin, Muhammed Coban and Marta Karczewicz (Qualcomm) Sriram Sethuraman (Ittiam) | Tel: Email: | Yong.He@InterDigital.com Hanhuang@qti.qualcomm.com Sriram.sethuraman@ittiam.com |
| Source: | InterDigital Communications, Inc., Qualcomm Incorporated, Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a slice header control flag for disabling DMVR and BDOF, which is an unified solution for JVET-O0184, JVET-O0250, and JVET-O0504.

# Introduction

Slice level on/off control of DMVR and BDOF is proposed in JVET-O0184, JVET-O0250, and JVET-O0504. This contribution provides a unified solution, wherein a slice header control flag slice slice_disable_bdof_dmvr_flag is introduced for disabling both DMVR and BDOF. A SPS flag sps_bdof_dmvr_slice_present_flag, which is signalled in the SPS when either of DMVR or BDOF sps level control flags are true, is used to indicate the presence of slice_disable_bdof_dmvr_flag. If present, the slice_disable_bdof_dmvr_flag is signaled after the fractional MMVD flag in the slice header.

slice_disable_bdof_dmvr_flag equal to 1 specifies that bidirectional optical flow inter prediction and decoder motion vector refinement based inter bi-prediction are both disabled in the current slice. slice_disable_bdof_dmvr_flag equal to 0 specifies that bidirectional optical flow inter prediction or decoder motion vector refinement based inter bi-prediction may or may not be disabled in the current slice. When slice_disable_bdof_dmvr_flag is not present, the value of slice_disable_bdof_dmvr_flag is inferred to be 0.

The derivation of dmvrFlag in the decoding process of a coding unit coded in inter prediction mode and the derivation of bdofFlag in the decoding process of inter blocks are modified as follows. The added text is highlighted in green.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If MergeTriangleFlag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index bcwIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag  is equal to 0

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

# Patent rights declaration(s)

InterDigital Communications, Inc., Qualcomm Inc.,  Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).