| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O01142 |
| --- | --- |

| Title: | CE4/9-related: Combined Test of JVET-O0593/JVET-O0252/JVET-O0281/JVET-O0615 on harmonization on BDOF and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang Han Huang, Wei-Jung Chien, Vadim Seregin, and Marta Karczewicz Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Yuwen He, Wei Chen | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com {yuwen.he, wei.chen} @interdigital.com |
| Source: | Kwai Inc., Qualcomm Incorporated, MediaTek Inc., InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes on unified design of the BDOF and the PROF. where the components of the proposed unified solution were jointly proposed independently proposed in JVET-O0593, JVET-O0252, JVET-O0281 and JVET-O0615:

- Unifying the representation precision of the motion refinements for the BDOF and the PROF as proposed in JVET-O0593, JVET-O0252 and JVET-O0281.

- Modifying the range of the BDOF motion refinement to be full dynamic range of signed 6-bit variable, as proposed in JVET-O0252 and JVET-O0281.

- Unifying the sample padding methods for the gradient calculation of the BDOF and the PROF as proposed in JVET-O0594, JVET-O0252, JVET-O0506, JVET-O0615 and JVET-O0624.

Compared to the VTM-5.0 anchors, the performance of the proposed joint solution is summarized as follows:

For 10-bit coding (CTC): {Y, U, V} BD-rate of {0.xx%, 0.xx%, 0.xx%} and {0.xx%, 0.xx%, 0.xx%} %} for RA and LD configurations.

For 8-bit coding:        {Y, U, V} BD-rate of {-0.70%, 0.35%, 0.15%} and {0.xx%, 0.xx%, 0.xx%} for RA and LD configurations.

For 12-bit coding:       {Y, U, V} BD-rate of {0.xx%, 0.xx%, 0.xx%} and {0.xx%, 0.xx%, 0.xx%} for RA and LD configurations

# Introduction

In the VVC working draft (WD) 5 [1], bidirectional optical flow (BDOF) is applied to enable the optical flow based motion refinement for bi-predicted coding units (CUs) that has both forward and backward predictions. In [2], prediction refinement with optical flow (PROF) is proposed to refine the subblock based affine motion compensation using the optical flow model. The method was adopted into the VVC at the 15-th JVET meeting.

# Proposal

Given that both the BDOF and the PROF are built upon the optical flow concept, it is highly desirable to maximally harmonize the BDOF and PROF designs to facilitate hardware implementations. In this contribution, the following modifications are proposed to modify the existing PROF design to be fully aligned with that of the BDOF.

## Alignment of motion refinement precisions of the BDOF and the PROF

It is proposed to align the representation precision of the PROF to be the same as that of the BDOF, i.e., in the range of [-32, 31].

#### Derivation process for motion vector arrays from affine control point motion vectors

- The variable dmvLimit is set to equal to 1 << Max (65, bitDepth - 67).

      - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 713 – Max(5, bitDepth - 7), and leftShift set equal to 0 as inputs and the rounded diffMv as output.

      - diffMv[ x ][ y ][ i ] = Clip3( -dmvLimit, dmvLimit - 1, diffMv[ x ][ y ][ i ])

#### 8.5.6.7 Prediction refinement with optical flow process

Variables bitDepth and shift1 are derived as follows:

- The variable shift1 is set equal to Max( 6, bitDepth − 6 ).

- The variable dILimit is set equal to 1 << (14 – 1).

For x =0.. sbWidth - 1, y =0.. sbHeight - 1, the following ordered steps apply:

  - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] − predSamples[ x ][ y ] ) >> shift1

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] − predSamples[ x ][ y ] ) >> shift1

  - The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]

  - Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1, ( dI + 1 ) >> 1 ) )dI

Additionally, it is proposed to change the range of the motion refinements for the BDOF to be fully dynamic range of signed 6-bit variable, i.e.,

#### 8.5.6.4 Bidirectional optical flow prediction process

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

Table 1 shows the precisions of the motion and sample refinements of the BDOF and the PROF after the proposed method is applied, when the internal bit-depth is ranging from 8-bit to 12-bit.

Table 1 The precisions of motion and sample refinements of the BDOF and the PROF

| BDOF |  | PROF |  |
| --- | --- | --- | --- |
| gradientH0, gradientH1, gradientV0, gradientV1 | 11 | gradientHL0, gradientHL1, gradientV0, gradientV1 | 11 |
| diffMv[0], diffMv[1] | 6 | diffMv0[0], diffMv0[1], diffMv1[0], diffMv1[1] | 6 |
| dI = ((diffMv[0]*(gradientH0 – gradientH1)) + (diffMv[1]*(gradientV0 – gradientV1)) +1)>>1 | 17 | dI0 = diffMv0[0]*gradientH0 + diffMv0[1]*gradientV0 dI1 = diffMv1[0]*gradientH1 + diffMv1[1]*gradientV1 | 17 |

## Alignment of sample padding methods of the BDOF and PROF

It is proposed to align the sample padding method of the BDOF to be the same as that of the PROF. Specifically, the integer samples that are closest to the fractional samples are used to pad the prediction samples in the extended region.

# Simulation results

The proposed methods were implemented based on the VTM-5.0 reference software [3] and tested when the internal bit-depth is set to 8-bit, 10-bit (CTC) and 12-bit.

Table 2 BD-rate performance of the proposed harmonization method compared to VTM-5.0 when the internal bit-depth is 8-bit

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.36% | -0.20% | -0.07% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.35% | -0.15% | 99% | 99% |
| Class D | -0.81% | -0.33% | -0.10% | 97% | 96% |
| Class F | -0.84% | -0.39% | -0.33% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | -0.20% | -0.39% | -0.08% | 104% | 104% |
| Overall | -0.26% | -0.16% | -0.41% | 101% | 101% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | -0.40% | -0.78% | 0.25% | 101% | 101% |

Table 3 BD-rate performance of the proposed harmonization method compared to VTM-5.0 when the internal bit-depth is 10-bit

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.64% | -0.25% | -0.23% | 101% | 101% |
| Class C | -0.41% | -0.17% | -0.24% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.83% | -0.48% | -0.20% | 100% | 100% |
| Class F | -0.86% | -0.45% | -0.39% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.20% | -0.03% | -0.10% | 98% | 97% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.42% | -0.11% | -0.63% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4 BD-rate performance of the proposed harmonization method compared to VTM-5.0 when the internal bit-depth is 12-bit

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.66% | -0.32% | -0.18% | 101% | 101% |
| Class C | -0.40% | -0.21% | -0.04% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.86% | -0.29% | -0.49% | 99% | 100% |
| Class F | -0.90% | -0.56% | -0.57% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.47% | -0.58% | -0.29% | 102% | 110% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

J. Chen, Y. Ye and S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

J. Luo, Y. He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, July 2019, Gothenburg, Sweden.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).