| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O1143-v1 |
| --- | --- |

| Title: | AHG12: Text for subpicture agreements integration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Jianle Chen Seungwook Hong Jill Boyce Lidong Xu Robert Skupin Yago Sanchez Karsten Suehring Miska M. Hannuksela | Tel: Email: | ye-kui.wang@futurewei.com fhendry@futurewei.com shong@futureweicom jianle.chen@futurewei.com jill.boyce@intel.com lidong.xu@intel.com {firstname.lastname}@hhi.fraunhofer.de firstname.lastname@nokia.com |
| Source: | Futurewei Technologies, Inc., Intel, Fraunhofer HHI, and Nokia |  |  |

_____________________________

# Abstract

This contribution provides proposed text for the agreed pieces of the subpicture design, based on the following notes in JVET-O_Notes_d5:

Decision pending text review: Adopt a sub-picture design based on the following pieces:

  - Normative changes to the low-level decoding processes for treating rectangular regional boundaries as picture boundaries proposed in JVET-O0138, JVET-O0141, and JVET-O0394. Apply these changes to the boundaries of sub-pictures, where each sub-picture consists of one or more rectangular-mode slices that collectively form a rectangular region. [JVET-O0138, JVET-O0141, JVET-O0394]

  - Adopt signalling sub-picture location and size in the SPS (unless a sub-picture based GRA solution is agreed in the CE11 discussion), using the SPS syntax in O0555 for this purpose as a starting point, with the sub-picture ID replaced with grid index (not considering scalability, ARC/RPR, GRA). [JVET-O0555]

  - Adopt the following, pending on review of spec text (J. Boyce to coordinate the work on the spec text):

    - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether the sub-picture boundaries are treated as picture boundaries. [JVET-O0141]

    - In the SPS syntax for signalling of sub-picture size and position, add a flag for each sub-picture to control whether in-loop filtering (deblocking, SAO, and ALF) across sub-picture boundaries are enabled. [JVET-O0141]

# Patent rights declaration(s)

Futurewei Technologies, Inc., Intel, Fraunhofer HHI, and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).