| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1150 |
| --- | --- |

| Title: | AHG8: Single-layer scalability approach |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an approach that uses a single layer (i.e. all NAL units have the same NuhLayerId value) for spatial, quality, view, and other types of scalability. It is asserted that no specification text changes beyond a new SEI message are needed, although changing the bit allocation in the NAL unit header to allocate a greater number of bits to TemporalId could be desirable for applications using both temporal and other types of scalability together. The following is proposed:

- Temporal identifier mapping SEI message with two parts:

  - Indicating temporal sub-layer dependencies. This part enables a more sophisticated pruning of sub-layers than simply removing sub-layers is descending order of TemporalId values.

  - Mapping of scalability types (temporal, multiview, spatial/quality) to TemporalId values

- Reallocation of bits in the NAL unit header

This contribution is a follow-up of the scalability part of JVET-O0395 (which additionally contained adaptive resolution changing parts).

Asserted benefits of the proposal include:

- No changes in the syntax, semantics, or decoding process in VVC Draft 5 are required (although new bit allocation in NAL unit header could be preferable).

- The proposal supports profile-tier-level (PTL) and HRD signalling through the existing sub-layer specific PTL and HRD signalling in VVC Draft 5.

# Introduction

This proposal enables using the decoding process specified in VVC Draft 5 without changes for decoding of a bitstream representing multiview, spatial/quality, or other types of scalability. Figure 1 illustrates a spatially scalable bitstream.

Figure 1 – Spatially scalable bitstream with two resolutions.

From POC, RPL, and RPM perspective, the proposal can be summarized with the following table:

| POC | RPL syntax | RPM |
| --- | --- | --- |
| Syntax and semantics not changed. No two pictures have the same POC. | Not changed. | The RPM process remains unchanged. "Layers" are represented as sub-layers. Thus, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same or lower sub-layer. |

# Temporal identifier mapping SEI message

## Indicating dependency

| tid_mapping( payloadSize ) { | Descriptor |
| --- | --- |
| max_tid_minus1 | u(X) |
| all_tid_present_flag | u(1) |
| for( i = 1; i <= max_tid_minus1; i++ ) { |  |
| if( !all_tid_present_flag ) |  |
| tid_present_flag[ i ] | u(1) |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ i ] ) |  |
| for( j = 1; j < i; j++ ) |  |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ j ] ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |

Note: X will be replaced by the number of bits for temporal_id_plus1 in the NAL unit header syntax.

max_tid_minus1 plus 1 specifies the maximum TemporalId value for which this SEI message provides dependency information.

all_tid_present_flag equal to 0 specifies that tid_present_flag[ i ] is present. all_tid_present_flag equal to 1 specifies that this SEI message provides the dependency information for all TemporalId values in the range of 1 to max_tid_minus1 + 1, inclusive.

tid_present_flag[ i ] equal to 0 specifies that no dependency information is provided for TemporalId equal to i. tid_present_flag[ i ] equal to 1 specifies that dependency information is provided for TemporalId equal to i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the sub-layer with TemporalId equal to i does not depend on the sub-layer with TemporalId equal to j. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the sub-layer with TemporalId equal to i may or may not depend on the sub-layer with TemporalId equal to j.

## Mapping of scalability types to TemporalId values

| tid_mapping( payloadSize ) { | Descriptor |
| --- | --- |
| … // append the following at the end of the SEI message |  |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 1; i <= max_tid_minus1; i++ ) { |  |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ i ] ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(X) |
| } |  |

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table XXX1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table XXX1 – Scalability dimensions

| Scalability mask index | Scalability dimension |
| --- | --- |
| 0 | Temporal |
| 1 | Multiview |
| 2 | Spatial/quality scalability |
| 3-15 | Reserved |

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th sub-layer.

# NAL unit header

In the HLS BOG review of JVET-O0395, it was commented that the TemporalId value space might be too tight for both temporal and spatial/quality scalability in some applications. It is proposed to reallocate the bits of the NAL unit header as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) u(5) |
| nal_unit_type_lsb | u(4) u(5) |
| nuh_layer_id_plus1 | u(7) u(5) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

The value of nuh_temporal_id_plus1 shall be less than 23 (to avoid MPEG-2 PES start code emulation).

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).