| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1150 |
| --- | --- |

| Title: | AHG8: Single-layer scalability approach |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an approach that uses a single layer (i.e. all NAL units have the same NuhLayerId value) for spatial, quality, view, and other types of scalability. It is asserted that no specification text changes beyond a new SEI message or VPS metadata are needed, although changing the bit allocation in the NAL unit header to allocate a greater number of bits to TemporalId could be desirable for applications using both temporal and other types of scalability together. The following is proposed:

- VPS metadata or SEI message with two parts:

  - Indicating temporal sub-layer dependencies. This part enables a more sophisticated pruning of sub-layers than simply removing sub-layers is descending order of TemporalId values.

  - Mapping of scalability types (temporal, multiview, spatial/quality) to TemporalId values

- Reallocation of bits in the NAL unit header

- poc_output_period_minus1 in the SPS syntax for controlling normatively whether the decoder outputs only the highest "layer" (for spatial and quality scalability) or all "layers" (view scalability etc.)

poc_output_period_minus1 is used to specify a POC output period in terms of adjacent POC values. Out of the decoded pictures of the same POC output period, the decoded pictures that precede the last decoded picture in decoding order are not output. poc_output_period_minus1 greater than 0 is used for spatial and quality scalability to avoid outputting more than one picture of the same capture time.

This contribution is a follow-up of the scalability part of JVET-O0395 (which additionally contained adaptive resolution changing parts).

Asserted benefits of the proposal include:

- No changes in the syntax, semantics, or decoding process in VVC Draft 5 are required (although new bit allocation in NAL unit header could be preferable).

- The proposal supports profile-tier-level (PTL) and HRD signalling through the existing sub-layer specific PTL and HRD signalling in VVC Draft 5.

# Introduction

This proposal enables using the decoding process specified in VVC Draft 5 without changes for decoding of a bitstream representing multiview, spatial/quality, or other types of scalability. Figure 1 illustrates a spatially scalable bitstream.

Figure 1 – Spatially scalable bitstream with two resolutions.

From POC, RPL, and RPM perspective, the proposal can be summarized with the following table:

| POC | RPL syntax | RPM |
| --- | --- | --- |
| Syntax and semantics not changed. No two pictures have the same POC. | Not changed. | The RPM process remains unchanged. "Layers" are represented as sub-layers. Thus, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same or lower sub-layer. |

Figure 1 presents an example of a spatially scalable bitstream with two resolutions. poc_output_period_minus1 is equal to 1, specifying that the POC output period consists of two adjacent POC values. When both resolutions are present in the bitstream, pictures with POC equal to 0, 2, 4, and so on are not output, whereas pictures with POC equal to 1, 3, 5, and so on are output. When the higher resolution (i.e., pictures with POC equal to 1, 3, 5, and so on) has been removed from the bitstream, pictures with POC equal to 0, 2, 4, and so on are output.

Figure 2 – Spatially scalable bitstream with two resolutions. poc_output_period_minus1 is equal to 1. 
The values within solid rectangles indicate the POC value of the picture

# VPS metadata / temporal identifier mapping SEI message

## Overview

Two sets of syntax elements are proposed:

- Indicating temporal sub-layer dependencies. This part enables a more sophisticated pruning of sub-layers than simply removing sub-layers is descending order of TemporalId values. For example, if there two spatial resolutions and two pictures rates (e.g. 30 and 60 Hz), the TemporalId values are assigned as follows:

TID 0: low resolution 30 Hz

TID 1: high resolution 30 Hz

TID 2: low resolution 60Hz, with dependency on TID 0, no dependency on TID1

TID 3: high resolution 60Hz

The dependency information would enable creating a sub-bitstream with only TIDs 0 and 2 included.

- Mapping of scalability types (temporal, multiview, spatial/quality) to TemporalId values

Both sets are not needed for decoding and hence could reside in an SEI message. However, it might be preferred to keep or scalability information in VPS.

## Indicating dependency

| tid_mapping( payloadSize ) { | Descriptor |
| --- | --- |
| max_tid_minus1 | u(X) |
| all_tid_present_flag | u(1) |
| for( i = 1; i <= max_tid_minus1; i++ ) { |  |
| if( !all_tid_present_flag ) |  |
| tid_present_flag[ i ] | u(1) |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ i ] ) |  |
| for( j = 1; j < i; j++ ) |  |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ j ] ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |

Note: X will be replaced by the number of bits for temporal_id_plus1 in the NAL unit header syntax.

max_tid_minus1 plus 1 specifies the maximum TemporalId value for which this SEI message provides dependency information.

all_tid_present_flag equal to 0 specifies that tid_present_flag[ i ] is present. all_tid_present_flag equal to 1 specifies that this SEI message provides the dependency information for all TemporalId values in the range of 1 to max_tid_minus1 + 1, inclusive.

tid_present_flag[ i ] equal to 0 specifies that no dependency information is provided for TemporalId equal to i. tid_present_flag[ i ] equal to 1 specifies that dependency information is provided for TemporalId equal to i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the sub-layer with TemporalId equal to i does not depend on the sub-layer with TemporalId equal to j. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the sub-layer with TemporalId equal to i may or may not depend on the sub-layer with TemporalId equal to j.

## Mapping of scalability types to TemporalId values

| tid_mapping( payloadSize ) { | Descriptor |
| --- | --- |
| … // append the following at the end of the SEI message |  |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 1; i <= max_tid_minus1; i++ ) { |  |
| if( all_tid_present_flag ¦ ¦ tid_present_flag[ i ] ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(X) |
| } |  |

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table XXX1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table XXX1 – Scalability dimensions

| Scalability mask index | Scalability dimension |
| --- | --- |
| 0 | Temporal |
| 1 | Multiview |
| 2 | Spatial/quality scalability |
| 3-15 | Reserved |

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th sub-layer.

# NAL unit header

In the HLS BOG review of JVET-O0395, it was commented that the TemporalId value space might be too tight for both temporal and spatial/quality scalability in some applications. It is proposed to reallocate the bits of the NAL unit header as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) u(5) |
| nal_unit_type_lsb | u(4) u(5) |
| nuh_layer_id_plus1 | u(7) u(5) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

The value of nuh_temporal_id_plus1 shall be less than 23 (to avoid MPEG-2 PES start code emulation).

## poc_output_period_minus1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| poc_output_period_minus1 | ue(v) |
| … |  |

poc_output_period_minus1 plus 1 specifies the number of adjacent POC values assigned to the same POC output period. Out of the decoded pictures of the same POC output period, the decoded pictures that precede the last decoded picture in decoding order are not output.

### 8.1.2	Decoding process for a coded picture

…

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the slice header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first slice of a picture.

–	At the beginning of the decoding process for each slice of a non-IDR picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice of a picture.

–	When the current picture is a CRA picture with NoIncorrectPicOutputFlag equal to 1 or GRA picture with NoIncorrectPicOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.4 is invoked, which needs to be invoked only for the first slice of a picture.

–	PictureOutputFlag for CurrPic is set as follows:

–	If one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	the current picture is a RASL picture and NoIncorrectPicOutputFlag of the associated IRAP picture is equal to 1.

–	gra_enabled_flag is equal to 1 and the current picture is GRA picture with NoIncorrectPicOutputFlag equal to 1.

–	gra_enabled_flag is equal to 1, the current picture is associated with a GRA picture with NoIncorrectPicOutputFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GRA picture.

–	Otherwise, PictureOutputFlag is set equal to 1.

–	The variable PocOutputPeriod for CurrPic is derived as follows:

–	If PicOrderCntVal is greater than or equal to 0, PocOutputPeriod is set equal to PicOrderCntVal / ( poc_output_period_minus1 + 1 ).

–	Otherwise, PocOutputPeriod is set equal to −( ( Abs( PicOrderCntVal ) − 1 ) / ( poc_output_period_minus1 + 1 ) + 1 ).

–	It is a requirement of bitstream conformance that the decoded picture preceding CurrPic in decoding order and having the same PocOutputPeriod value as CurrPic has not been output.

–	PictureOutputFlag is set equal to 0 for all decoded pictures having the same PocOutputPeriod value as CurrPic.

- [Ed. (YK): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.]

- After all slices of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).