| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1158-v1 |
| --- | --- |

| Title: | Cross-check of JVET-O0793 (CE3-related: MIP with simplified mode index coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of a cross-check of JVET-O0793-v1 is reported. It is reported that the cross-checker has conducted a cross-check of the bit-rates and the PSNRs reported for the AI and RA configurations, and the results obtained match the values reported by the proponent. The software implementation that was provided by the proponents to the cross-checkers was used for the cross-check. The cross-checker verified the software implementation and reported that the implementation matches the description of the method provided in JVET-O0793-v1.

# Introduction

Matrix intra prediction (MIP) uses several modes for prediction; based on the block size category, one of several MIP modes is signalled. The MIP mode is signalled using an MPM index or a remaining mode index. For derivation of regular modes, mode of neighbouring modes that are coded with MIP are derived using a mapping table. Three MPMs are derived for the MIP.

# Cross-checked test

The proposal JVET-O0793 proposes three simplifications to MIP, described below:

- Method 1: For regular MPM derivation, consider neighbouring MIP-coded blocks as Planar-coded blocks; remove MIP MPM derivation; reduce the MIP modes to next power of 2 (32, 16 and 8 for categories 0, 1, and 2, respectively).

- Method 2: Use the lower bit of MIP to determine if the mode matrix is to be transposed.

- Method 3: Reduce the number of MIP modes in each MIP mode category to 8.

The cross-checker tested two configurations of these simplifications:

- Method 1 + Method 2 (Described as Method 2 in the results section of JVET-O0793-v2).

- Method 1 + Method 2 + Method 3 (Described as Method 3 in the results section of JVET-O0793-v2).

# Software

No deviation was observed in the software implementation compared to the description provided in JVET-O0793-v1.

# Results

The PSNR numbers and bit rates numbers for AI and RA configurations were tested, and it is reported that they match the bitrates and PSNRs reported by the proponent.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).