| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O1159-v1 |
| --- | --- |

| Title: | AHG8: Scalability for VVC – Text for approach 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Jianle Chen (Futurewei) Vadim Seregin Muhammed Coban Adarsh K. Ramasubramonian Marta Karczewicz (Qualcomm) Yong He Ahmed Hamza (InterDigital) Miska M. Hannuksela (Nokia) Stephan Wenger Byeongdoo Choi (Tencent) | Email: | ye-kui.wang@futurewei.com fhendry@futurewei.com jianle.chen@futurewei.com vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com yong.he@interdigital.com ahmed.hamza@interdigital.com firstname.lastname@nokia.com stewe@stewe.org bdchoi@tencent.com |
| Source: | Futurewei Technologies, Inc., Qualcomm, Inc., InterDigital, Nokia, and Tencent |  |  |

_____________________________

# Abstract

This contribution provides proposed text for the suggested design of supporting scalability coding for VVC based on the discussion in HLS BoG with approach 1 from the 4 approach categories described in JVET-O0136. The proposal is the result of integration from contributions JVET-O0136, JVET-O0185, JVET-O0243, JVET-O0333, and JVET-O0395.

# Introduction

During the discussion in the HLS BoG on scalabilty support for VVC, the following items were agreed:

It was agreed to signal layer dependency information in the VPS.

Does the VPS need to be present in non-scalable present? But VPS is something we need to pay attention to.

It was agreed that VPS needs to be included in non-scalable bitstream.

Approach 1.

Overhead: For each entry in the reference picture list there is one additional flag. It is asserted that despite this change, approach 1 is still the most efficient approach of the four approaches of this doc (for multilayer obviously). For single, no impact beyond a bit in the SPS.

In approach 1, spatial and temporal scalability can co-exist due to the basic design of RPS+RPR

It was remarked that approach 1 does not impose a significant implementation burden. Pretty straightforward/easy. Perhaps no undue burden

It was agreed that support of output of only the highest layer (for typical scalability), and support of output of all layers (for multiview) are needed.

The four proponents of POC related documents were requested to collaborate in creating unified proposals with as few alternatives as possible, for revisit in track/plenary. At least one unified proposal should be based on Approach 3, modified as required. Complete spec text was requested.

The attached text is the result of an offline work to create a unified design for the direction of the approach 1, based on JVET-O0136, JVET-O0185, JVET-O0243, JVET-O0333, and JVET-O0395, following the above guidelines and agreed points. The offline work was coordinated by Hendry and Vadim Seregin.

# Conclusion

It is proposed to adopt the proposed text in the next version of VVC WD.

# Patent rights declaration(s)

Futurewei Technologies, Inc., Qualcomm, Inc., InterDigital, Nokia, and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).