| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1164-v1 |
| --- | --- |

| Title: | AHG8: Integrated Specification Text for Reference Picture Resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Chen, T. Hellman, B. Heng, W. Wan, M. Zhou (Broadcom) M. M. Hannuksela, A. Aminlou (Nokia) V. Seregin, M. Coban, M. Karczewicz (Qualcomm) T.-S Chang, Y.-C Sun, L. Zhu, J. Lou (Alibaba) Hendry, S. Hong, Y.-K. Wang, J. Chen (Futurewei) J. Samuelsson, S. Deshpande, A. Segall (Sharp) Y. He, A. Hamza (InterDigital) P. Topiwala, M. Krishnan, W. Dai (FastVDO) S. Wenger, B. Choi (Tencent) | Email: | peisong.chen@broadcom.com miska.hannuksela@nokia.com vseregin@qti.qualcomm.com tsuishan.cts@alibaba-inc.com fhendry@futurewei.com samuelssonj@sharplabs.com yong.he@interdigital.com pankajtva@gmail.com stewe@stewe.org |  |
| Source: | Broadcom, Nokia, Qualcomm Incorporated, Alibaba Group, Futurewei Technologies, Inc., Sharp Labs of America, Inc., InterDigital Communications, Inc., FastVDO, Tencent |  |  |

_____________________________

# Abstract

This contribution provides a specification draft for the agreed in HLS BoG approach for reference picture resampling by including elements of the following documents: JVET-O0133, JVET-O0134, JVET-O0182, JVET-O0204, JVET-O0242, JVET-O0303, JVET-O0319, and JVET-O0395.

It contains the following aspects:

- max picture size signaling in SPS,

- actual picture size and conformance window signalling moved to PPS,

- scaling ratio is defined based on the output picture size after cropping,

- resampling is done on a block basis using existed interpolation filters,

- TMVP is used only when current and collocated pictures have the same size,

- DMVR and BDOF are used only when current and reference pictures, selected in a CU for inter prediction, have the same size.

# Patent rights declaration(s)

Broadcom, Nokia, Qualcomm Incorporated, Alibaba Group, Futurewei Technologies, Inc., Sharp Labs of America, Inc., InterDigital Communications, Inc., FastVDO, Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).