| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1169 |
| --- | --- |

| Title: | CE8(7)-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to align the number of context coded bins per coefficient for transform skip residual coding and transform coefficient coding by assigning 1.75 bin/coefficient at TB-level.  The simulations results show Class F:0.16% AI, 0.10% RA, 0.xx% LB BD-Rate, and Class TGM: 0.27% AI, -0.22% RA, 0.xx% LB BD-Rate, versus  CE7-3.10 test result under common test conditions.

# Proposal

Newly adopted JVET-O0052 (CE7-1.1) limits the number of context coded bins per coefficient for luma and chroma coefficients to be 1.75 bin/coefficient at TB-level. VTM-5.0 transform skip (TS) residual coding sets this limit at TB-level to 2 bin/coefficient. It is proposed to align the number of context coded bins per coefficient at TB level for transform coefficient and TS residual coding schemes by setting the limit to 1.75 bin/coefficient.

The proposed VVC specification[1] change is highlighted below.

#### 7.3.7.10 Transform unit syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 (7 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight )) >> 2 |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |

# Results

The proposed scheme is implemented on top of CE7-3.10 software [2]. The simulations were carried out under the JVET common test conditions [3].

The test results for proposed scheme using CE7.3.10 as anchor is shown in Table 1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE7.3.10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.02% | 97% | 95% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.01% | -0.04% | 96% | 90% |
| Class C | 0.01% | 0.01% | 0.03% | 86% | 75% |
| Class E | -0.01% | 0.06% | 0.07% | 112% | 76% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.02% | 0.00% | 92% | 77% |
| Class F | 0.16% | 0.07% | 0.25% | 94% | 80% |
| Class TGM | 0.27% | 0.23% | 0.23% | 93% | 85% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE7.3.10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.10% | -0.25% | 97% | 91% |
| Class C | 0.03% | 0.16% | 0.40% | 82% | 74% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.07% | -0.14% | 91% | 81% |
| Class F | 0.10% | 0.12% | 0.01% | 91% | 81% |
| Class TGM | 0.22% | 0.23% | 0.25% | 90% | 83% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE7.3.10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | -0.18% | 0.09% | 83% | 81% |
| Class E | 0.07% | 0.64% | 0.19% | 98% | 81% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.26% | -0.46% | 92% | 83% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 1: Average Bjøntegaard delta rate savings of the proposed method relative to CE7-3.10.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet-n-ce7/VVCSoftware_VTM/tree/CE7-3.10

- “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).