| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–17 July 2019 | Document: JVET-O1172-v2 |
| --- | --- |

| Title: | Non-CE4/8: On disabling blending process in TPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Yan Zhang, Kevin Reuze, Yao-Jen Chang, Wei-Jung Chien, Marta Karczewicz (Qualcomm) Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang (Bytedance) M. Bläser, J. Sauer (RWTH Aachen University) | Email: | chunchic@qti.qualcomm.com weijia.zhu@bytedance.com blaeser@ient.rwth-aachen.de |
| Source: | Qualcomm Inc., Bytedance Inc., RWTH Aachen University, |  |  |

_____________________________

# Abstract

The contribution proposes solutions to disable the blending process in the Triangle Prediction Merge (TPM) mode. In common, these solutions introduce a sps- and a slice-level flag to indicate the whether the blending process of TPM is disabled or not. According to flag values, if the blending process is not indicated to be disabled, the blending process as for the current TPM is not changed; otherwise, this blending process is disabled. On top of the introduced flags, this proposal suggests two methods to fill the prediction samples for each triangle partition based on the respective motion-compensated blocks of the two TPM motion vectors. In suggested method 1 (JVET-O0645), the weighting values in the blending area are all clipped to be either 0/8 or 8/8, depending on whether they are closer to 0/8 or 8/8. In suggested method 2 (JVET-O0563), the samples located on the diagonal or anti-diagonal line will be determined by using the motion-compensated samples of the shorter MV. In suggested method 3 (JVET-O0513) the weighting values in the blending area are all clipped to be either 0/8 or 8/8, and so that both partition contain the same number of samples.

For each of these methods, two methods for motion vector storage are tested. In suggested method 1, when both motion vector used for triangle are identical, the motion vector from the bottom partition (Mv2) is stored. This is the method that was adopted during the Göteborg meeting. In suggested method 2, motion storage for TPM is modified so that the shorter Mv is stored, following the motion vector storage proposed in JVET-O0563.

Simulation results report that when compared with VTM-5.0 anchor,

Method A1 (adopted storage + JVET-O0645 blending off)

Class TGM1080p420: -0.51% (RA) / -0.78% (LDB)

Class TGM1080p444: -0.53% (RA) / -1.01% (LDB).

Method A2 (adopted storage + JVET-O0563 blending off)

Class TGM1080p420: -0.56% (RA) / -0.82% (LDB)

Class TGM1080p444: -0.73% (RA) / -1.09% (LDB).

Method A3 (adopted storage + JVET-O0513 blending off)

Class TGM1080p420: -0.61% (RA) / -0.71% (LDB)

Class TGM1080p444: -0.68% (RA) / -1.09% (LDB).

Method B1 (JVET-O0563 storage + JVET-O0645 blending off)

Class TGM1080p420: -0.72% (RA) / -0.71% (LDB)

Class TGM1080p444: -0.80% (RA) / -0.98% (LDB).

Method B2 (JVET-O0563 storage + JVET-O0563 blending off)

Class TGM1080p420: -0.83% (RA) / -0.84% (LDB)

Class TGM1080p444: -0.85% (RA) / -1.11% (LDB).

Method B3 (JVET-O0563 storage + JVET-O0563 blending off)

Class TGM1080p420: -0.81% (RA) / -0.85% (LDB)

Class TGM1080p444: -0.94% (RA) / -1.05% (LDB).

# Disabling blending in TPM

## Suggested method 1 (JVET-O0645)

In this suggested method, when the slice-level flag (slice_triangle_blending_off_flag) turns on, the weighting values (i.e., w) assigned to the samples sitting on the blending area are all clipped to be either 0/8 or 8/8, depending on whether they are closer to 0/8 or 8/8.

- if w ≠ 4/8, the value of w is clipped as follows:

  - If w < 4/8, the value of w is clipped to 0/8;

  - Otherwise, w is clipped to 8/8.

- Otherwise (if w = 4/8), the value of w is clipped as follows:

- If the CU is split diagonally, w is clipped to 8/8;

- Otherwise, w is clipped to 0/8.

The below figure at the left-hand side visualizes the map of sample-by-sample weighting values when blending process is enabled. The other figure at the right-hand side illustrates the map of sample-by-sample weighting values when blending operation is disabled.

| (a) VTM | (b) Proposed |
| --- | --- |

Figure 3. (a) Weighting map when blending is enabled; (b) weighting map when blending is disabled using the proposed method 1.

## Suggested method 2 (JVET-O0563)

In this suggested method, when the slice-level flag (slice_triangle_blending_off_flag) turns on, the two partitions as shown in the Figure 1 will copy its own prediction samples. The prediction values of samples located on the diagonal or anti-diagonal line are determined by comparing the motion vectors (MVs) of two partitions. Their values will copy from the partition with a smaller MV. Here, the magnitude of a MV is defined as its L-1 norm, i.e. if |MV1|1>|MV0|1, MV0 is smaller. When they have the same L-1 norm, we simply select MV0.

Figure 1. Diagonal and anti-diagonal partitions in TPM

## Method 3 (JVET-O0513)

In this suggested method, when the slice-level flag (slice_triangle_blending_off_flag) turns on, the sample weight for weighted combination can either be 0 or 8, with the following scheme:

Figure 3. Sample weights relating to partition 0 of the proposed method 3.

The scheme in JVET-O0513 partitions a rectangular block into two triangles which contain an equal number of samples in each triangle.

# Modified motion storage for TPM

## Method A (adopted in Göteborg)

The proposed method also proposes to modify the motion storage for the TPM mode to match the storage method adopted during the Göteborg meeting.

With the proposed method, when both motion vectors come from the same reference picture list, the stored motion vector will be the one from partition 2.

| VTM | Method A |
| --- | --- |

## Method B (JVET-O0563)

The proposed method also proposes to modify the motion storage for the TPM mode when the current tile group is considered as screen contents. In this method, the stored motion information will be from the partition with a smaller motion vector, as defined in 2.1. In addition, as shown in Figure 2, this method is only applied to 4x4 units that contain diagonal or anti-diagonal samples.

Figure 2. Motion storage of the proposed scheme

# Experimental Results

The experiments were conducted on top of the VTM-5.0 software with the common test conditions [1] and CE8 testing conditions [2] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio.

At the encoder, the sps flag (sps_tpm_blending_off_flag) is always set enabled for each test. The introduced slice-level flag (slice_triangle_blending_off_flag) is configured by using the same hash-ratio based function as MMVD uses to swap between fractional and integer distance offset tables and the threshold value is set to be 40 for all the tests for slice_triangle_blending_off_flag.

- Y BD-rate comparison among Test A1/A2/A3, simplified storage (JVET-O0411 method B) and TPM off.

|  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YUV420 | A1 | A2 | A3 | Simp. | TPMoff |  | A1 | A2 | A3 | Simp. | TPMoff |
| Class A1 | 0.01% | 0.01% | 0.01% | 0.00% | 0.36% |  | x | x | x | x | x |
| Class A2 | 0.02% | 0.02% | 0.02% | 0.02% | 0.47% |  | x | x | x | x | x |
| Class B | 0.02% |  | 0.02% | 0.02% | 0.45% |  | 0.08% |  |  | 0.08% | 0.80% |
| Class C | -0.02% | -0.02% | -0.02% | -0.03% | 0.87% |  | 0.10% | 0.10% | 0.10% | 0.09% | 1.31% |
| Class E | x | x | x | x | x |  | 0.04% | 0.04% | 0.04% | 0.03% | 1.34% |
| Overall | 0.01% |  |  | 0.00% | 0.55% |  | 0.08% |  |  | 0.07% | 1.10% |
| EncT | 101% |  |  | 100% | 92% |  | 101% |  |  | 100% | 89% |
| Dec T | 100% |  |  | 1005 | 100% |  | 99% |  |  | 102% | 100% |
| Class D | 0.02% | 0.02% | 0.02% | -0.02% | 0.54% |  | 0.13% | 0.13% | 0.13% | 0.12% | 1.19% |
| Class F | 0.00% | 0.00% | -0.02% | 0.00% | 0.66% |  | -0.08% |  | 0.01% | 0.09% | 0.58% |
| TGM1080 | -0.51% | -0.56% | -0.61% | -0.02% | 1.15% |  | -0.78% | -0.82% | -0.71% | -0.06% | 0.20% |
|  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |  |
| YUV444 | A1 | A2 | A3 | Simp. | TPMoff |  | A1 | A2 | A3 | Simp. | TPMoff |
| TGM1080 | -0.53% | -0.73% | -0.68% | -0.07% | 1.25% |  | -1.01% | -1.09% | -1.09% | 0.10% | 0.28% |
| TGM720 | -0.06% | -0.08% | -0.08% | 0.02% | 0.33% |  | -0.04% | -0.13% | -0.29% | 0.22% | 0.40% |
| Animation | -0.05% | -0.05% | -0.05% | -0.05% | 0.77% |  | 0.02% |  |  | 0.02% | 1.07% |
| Mixed | 0.00% | -0.07% | -0.04% | 0.09% | 0.23% |  | -0.09% | -0.25% | -0.28% | 0.02% | 0.47% |
| Camera | -0.02% | -0.02% | -0.02% | -0.03% | 0.25% |  | 0.13% | 0.13% | 0.13% | 0.12% | 0.78% |
| Overall | -0.17% | -0.24% | -0.22% | -0.01% | 0.60% |  | -0.28% |  |  | 0.11% | 0.52% |
| EncT | 100% | 97% | 98% | 101% | 93% |  | 101% |  |  | 101% | 88% |
| Dec T | 99% | 96% | 94% | 101% | 101% |  | 100% |  |  | 102% | 100% |

- Y BD-rate comparison among Test A1/A2/A3 and B1/B2/B3.

|  | Random Access |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YUV420 | A1 | A2 | A3 | B1 | B2 | B3 |  | A1 | A2 | A3 | B1 | B2 | B3 |
| Class A1 | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |  | x | x | x | x | x | x |
| Class A2 | 0.02% | 0.02% | 0.02% |  |  |  |  | x | x | x | x | x | x |
| Class B | 0.02% |  | 0.02% | 0.02% | 0.02% | 0.02% |  | 0.08% |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% |  | 0.10% | 0.10% | 0.10% | 0.10% | 0.10% | 0.10% |
| Class E | x | x | x | x | x | x |  | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% | 0.04% |
| Overall | 0.01% |  |  |  |  |  |  | 0.08% |  |  |  |  |  |
| EncT | 101% |  |  |  |  |  |  | 101% |  |  |  |  |  |
| Dec T | 100% |  |  |  |  |  |  | 99% |  |  |  |  |  |
| Class D | 0.02% | 0.02% | 0.02% | 0.02% | 0.02% | 0.02% |  | 0.13% | 0.13% | 0.13% | 0.13% | 0.13% | 0.13% |
| Class F | 0.00% | 0.00% | -0.02% | -0.01% | -0.02% | -0.03% |  | -0.08% |  | 0.01% |  |  |  |
| TGM1080 | -0.51% | -0.56% | -0.61% | -0.72% | -0.83% | -0.81% |  | -0.78% | -0.82% | -0.71% | -0.71% | -0.84% | -0.85% |
|  | Random Access |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |
| YUV444 | A1 | A2 | A3 | B1 | B2 | B3 |  | A1 | A2 | A3 | B1 | B2 | B3 |
| TGM1080 | -0.53% | -0.73% | -0.68% | -0.80% | -0.85% | -0.94% |  | -1.01% | -1.09% | -1.09% | -0.98% | -1.11% | -1.05% |
| TGM720 | -0.06% | -0.08% | -0.08% | -0.08% | -0.09% | -0.11% |  | -0.04% | -0.13% | -0.29% | -0.04% | -0.10% | -0.30 |
| Animation | -0.05% | -0.05% | -0.05% | -0.05% | -0.05% | -0.05% |  | 0.02% |  |  |  |  |  |
| Mixed | 0.00% | -0.07% | -0.04% | -0.05% | -0.10% | -0.06% |  | -0.09% | -0.25% | -0.28% | -0.13% | -0.19% | -0.18 |
| Camera | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% |  | 0.13% | 0.13% | 0.13% | 0.13% | 0.13% | 0.13% |
| Overall | -0.17% | -0.24% | -0.22% | -0.25% | -0.28% | -0.30% |  | -0.28% |  |  |  |  |  |
| EncT | 100% | 97% | 98% | 98% | 98% | 100% |  | 101% |  |  |  |  |  |
| Dec T | 99% | 96% | 94% | 96% | 96% | 96% |  | 100% |  |  |  |  |  |

- Performance results of suggested method A1.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 | 0.01% | -0.15% | 0.07% | 98% | 98% | x | x | x | x | x |  |
| Class A2 | 0.02% | 0.06% | 0.05% | 103% | 101% | x | x | x | x | x |  |
| Class B | 0.02% | -0.06% | 0.01% | 100% | 99% | 0.08% | 0.39% | 0.06% | 101% | 101% |  |
| Class C | -0.02% | -0.02% | -0.16% | 102% | 102% | 0.10% | 0.09% | 0.21% | 101% | 98% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 101% | 100% |  |
| Overall | 0.01% | -0.04% | -0.01% | 101% | 100% | 0.08% | 0.27% | 0.05% | 101% | 99% |  |
| Class D | 0.02% | 0.10% | 0.12% | 102% | 103% | 0.13% | -0.02% | 1.33% | 100% | 100% |  |
| Class F | 0.00% | 0.10% | 0.08% | 100% | 109% | -0.08% | -0.10% | 0.30% | 102% | 100% |  |
| Class SCC | -0.51% | -0.36% | -0.41% | 101% | 99% | -0.78% | -0.62% | -0.60% | 99% | 101% |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.53% | -0.48% | -0.50% | 100% | 98% | -1.01% | -0.92% | -0.89% | 101% | 100% |
| TGM720 | -0.06% | -0.03% | -0.01% | 101% | 99% | -0.04% | -0.29% | 0.01% | 101% | 100% |
| Animation | -0.05% | -0.02% | -0.03% | 101% | 101% | 0.02% | -0.08% | -0.07% | 100% | 100% |
| Mixed | 0.00% | 0.02% | -0.03% | 98% | 98% | -0.09% | -0.04% | 0.04% | 102% | 103% |
| Camera | -0.02% | -0.06% | 0.03% | 102% | 101% | 0.13% | 0.11% | 0.15% | 100% | 99% |
| Overall | -0.17% | -0.14% | -0.14% | 100% | 99% | -0.28% | -0.33% | -0.22% | 101% | 100% |

- Performance results of suggested method A2.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  | x | x | x | x | x |  |
| Class A2 |  |  |  |  |  | x | x | x | x | x |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.16% | 99% | 93% | 0.10% | 0.09% | 0.21% | 97% | 88% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 98% | 91% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.10% | 0.12% | 99% | 100% | 0.13% | -0.02% | 1.33% | 101% | 98% |  |
| Class F | 0.00% | 0.10% | 0.05% | 99% | 100% |  |  |  |  |  |  |
| Class SCC | -0.56% | -0.45% | -0.43% | 97% | 97% | -0.82% | -0.73% | -0.70% | 98% | 100% |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.73% | -0.57% | -0.59% | 98% | 99% |  |  |  |  |  |
| TGM720 | -0.08% | -0.06% | -0.05% | 97% | 92% | -0.13% | -0.49% | -0.12% | 94% | 90% |
| Animation | -0.05% | -0.02% | -0.03% | 97% | 92% |  |  |  |  |  |
| Mixed | -0.07% | -0.01% | -0.07% | 98% | 100% | -0.25% | -0.15% | -0.18% | 101% | 100% |
| Camera | -0.02% | -0.06% | 0.03% | 97% | 95% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |

- Performance results of suggested method A3.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  | x | x | x | x | x |  |
| Class A2 |  |  |  |  |  | x | x | x | x | x |  |
| Class B | 0.02% | -0.06% | 0.01% | 97% | 96% |  |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.16% | 99% | 91% | 0.10% | 0.09% | 0.21% | 102% | 95% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 103% | 99% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.10% | 0.12% | 98% | 101% | 0.13% | -0.02% | 1.33% | 103% | 101% |  |
| Class F | -0.02% | 0.07% | 0.07% | 104% | 103% |  |  |  |  |  |  |
| Class SCC | -0.61% | -0.45% | -0.46% | 97% | 94% | -0.71% | -0.52% | -0.57% | 98% | 93% |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.68% | -0.59% | -0.62% | 97% | 94% | -1.09% | -0.96% | -0.99% | 100% | 95% |
| TGM720 | -0.08% | -0.10% | -0.07% | 99% | 92% | -0.29% | -0.38% | -0.25% | 104% | 98% |
| Animation | -0.05% | -0.02% | -0.03% | 98% | 94% |  |  |  |  |  |
| Mixed | -0.04% | -0.01% | -0.04% | 100% | 98% | -0.28% | 0.00% | -0.17% | 102% | 99% |
| Camera | -0.02% | -0.06% | 0.03% | 97% | 92% | 0.13% | 0.11% | 0.15% | 99% | 94% |
| Overall | -0.22% | -0.19% | -0.19% | 98% | 94% |  |  |  |  |  |

- Performance results of suggested method B1.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  | x | x | x | x | x |  |
| Class A2 |  |  |  |  |  | x | x | x | x | x |  |
| Class B | 0.02% | -0.06% | 0.01% | 99% | 99% |  |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.16% | 98% | 92% | 0.10% | 0.09% | 0.21% | 98% | 90% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 98% | 92% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.10% | 0.12% | 98% | 99% | 0.13% | -0.02% | 1.33% | 101% | 99% |  |
| Class F | -0.01% | 0.09% | 0.07% | 100% | 100% |  |  |  |  |  |  |
| Class SCC | -0.72% | -0.51% | -0.52% | 99% | 99% |  |  |  |  |  |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.80% | -0.60% | -0.64% | 99% | 98% |  |  |  |  |  |
| TGM720 | -0.08% | -0.04% | -0.04% | 97% | 91% | -0.04% | -0.17% | -0.05% | 96% | 91% |
| Animation | -0.05% | -0.02% | -0.03% | 97% | 94% |  |  |  |  |  |
| Mixed | -0.05% | -0.05% | -0.10% | 99% | 100% | -0.13% | 0.00% | -0.02% | 100% | 98% |
| Camera | -0.02% | -0.06% | 0.03% | 97% | 95% |  |  |  |  |  |
| Overall | -0.25% | -0.19% | -0.20% | 98% | 96% |  |  |  |  |  |

- Performance results of suggested method B2.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  | x | x | x | x | x |  |
| Class A2 |  |  |  |  |  | x | x | x | x | x |  |
| Class B | 0.02% | -0.06% | 0.01% | 99% | 99% |  |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.16% | 98% | 92% | 0.10% | 0.09% | 0.21% | 98% | 91% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 98% | 93% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.10% | 0.12% | 98% | 100% | 0.13% | -0.02% | 1.33% | 99% | 97% |  |
| Class F | -0.02% | 0.10% | 0.05% | 100% | 100% |  |  |  |  |  |  |
| Class SCC | -0.83% | -0.61% | -0.58% | 99% | 98% |  |  |  |  |  |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.85% | -0.66% | -0.70% | 99% | 99% |  |  |  |  |  |
| TGM720 | -0.09% | -0.08% | -0.07% | 97% | 93% | -0.10% | -0.41% | -0.02% | 94% | 91% |
| Animation | -0.05% | -0.02% | -0.03% | 97% | 94% |  |  |  |  |  |
| Mixed | -0.10% | -0.07% | -0.09% | 99% | 100% | -0.19% | -0.18% | -0.08% | 98% | 99% |
| Camera | -0.02% | -0.06% | 0.03% | 97% | 96% |  |  |  |  |  |
| Overall | -0.28% | -0.22% | -0.22% | 98% | 96% |  |  |  |  |  |

- Performance results of suggested method B3.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  | x | x | x | x | x |  |
| Class A2 |  |  |  |  |  | x | x | x | x | x |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.02% | -0.02% | -0.16% | 99% | 93% | 0.10% | 0.09% | 0.21% | 98% | 89% |  |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 99% | 91% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.10% | 0.12% | 99% | 99% | 0.13% | -0.02% | 1.33% | 101% | 98% |  |
| Class F | -0.03% | 0.05% | 0.05% | 101% | 100% |  |  |  |  |  |  |
| Class SCC | -0.81% | -0.58% | -0.56% | 100% | 98% |  |  |  |  |  |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.94% | -0.69% | -0.75% | 101% | 99% |  |  |  |  |  |
| TGM720 | -0.11% | -0.08% | -0.11% | 100% | 93% | -0.30% | -0.47% | -0.40% | 985 | 91% |
| Animation | -0.05% | -0.02% | -0.03% | 97% | 94% |  |  |  |  |  |
| Mixed | -0.06% | -0.03% | -0.07% | 101% | 100% | -0.18% | 0.02% | 0.00% | 103% | 99% |
| Camera | -0.02% | -0.06% | 0.03% | 98% | 96% |  |  |  |  |  |
| Overall | -0.30% | -0.22% | -0.24% | 100% | 96% |  |  |  |  |  |

- Performance results of TPM off.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 | 0.36% | 0.42% | 0.77% | 92% | 99% | x | x | x | x | x |  |
| Class A2 | 0.47% | 1.00% | 0.91% | 90% | 97% | x | x | x | x | x |  |
| Class B | 0.45% | 0.69% | 0.66% | 92% | 99% | 0.80% | 1.37% | 1.35% | 90% | 101% |  |
| Class C | 0.87% | 1.23% | 1.16% | 95% | 105% | 1.31% | 1.99% | 1.86% | 90% | 96% |  |
| Class E | x | x | x | x | x | 1.34% | 1.74% | 0.82% | 86% | 103% |  |
| Overall | 0.55% | 0.84% | 0.87% | 92% | 100% | 1.10% | 1.67% | 1.39% | 89% | 100% |  |
| Class D | 0.54% | 0.98% | 1.09% | 96% | 119% | 1.19% | 2.09% | 2.66% | 95% | 105% |  |
| Class F | 0.66% | 0.79% | 0.78% | 113% | 122% | 0.58% | 1.43% | 1.62% | 97% | 107% |  |
| Class SCC | 1.15% | 1.02% | 1.00% | 95% | 101% | 0.20% | 0.36% | 0.28% | 90% | 100% |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | 1.25% | 1.22% | 1.30% | 94% | 100% | 0.28% | 0.55% | 0.47% | 89% | 98% |
| TGM720 | 0.33% | 0.25% | 0.31% | 94% | 103% | 0.40% | 0.19% | 0.57% | 88% | 101% |
| Animation | 0.77% | 1.52% | 1.34% | 93% | 101% | 1.07% | 2.39% | 2.02% | 86% | 96% |
| Mixed | 0.23% | 0.23% | 0.32% | 91% | 100% | 0.47% | 0.66% | 0.72% | 87% | 103% |
| Camera | 0.25% | 0.65% | 0.50% | 93% | 99% | 0.78% | 1.03% | 1.13% | 89% | 98% |
| Overall | 0.60% | 0.73% | 0.74% | 93% | 101% | 0.52% | 0.79% | 0.84% | 88% | 100% |

- Performance results of JVET-O0411 simplified motion storage B.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |  |
| Class A1 | 0.00% | -0.15% | 0.07% | 99% | 100% | x | x | x | x | x |  |
| Class A2 | 0.02% | 0.06% | 0.05% | 100% | 99% | x | x | x | x | x |  |
| Class B | 0.02% | -0.06% | 0.00% | 99% | 99% | 0.08% | 0.39% | 0.06% | 100% | 104% |  |
| Class C | -0.03% | -0.02% | -0.16% | 102% | 104% | 0.09% | 0.09% | 0.21% | 100% | 97% |  |
| Class E | x | x | x | x | x | 0.03% | 0.28% | -0.19% | 101% | 105% |  |
| Overall | 0.00% | -0.05% | -0.02% | 100% | 100% | 0.07% | 0.26% | 0.05% | 100% | 102% |  |
| Class D | -0.02% | 0.07% | 0.08% | 102% | 107% | 0.12% | -0.02% | 1.33% | 109% | 113% |  |
| Class F | 0.00% | 0.03% | 0.00% | 113% | 111% | 0.09% | 0.02% | -0.38% | 101% | 98% |  |
| Class SCC | -0.02% | -0.02% | -0.05% | 100% | 101% | -0.06% | -0.01% | -0.05% | 99% | 102% |  |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.07% | -0.02% | -0.06% | 100% | 99% | 0.10% | 0.04% | 0.00% | 101% | 101% |
| TGM720 | 0.02% | 0.02% | -0.02% | 103% | 103% | 0.22% | -0.05% | -0.10% | 101% | 104% |
| Animation | -0.05% | -0.03% | -0.04% | 102% | 103% | 0.02% | -0.08% | -0.07% | 99% | 99% |
| Mixed | 0.09% | 0.11% | 0.12% | 101% | 101% | 0.02% | 0.15% | 0.23% | 102% | 103% |
| Camera | -0.03% | -0.06% | 0.02% | 99% | 95% | 0.12% | 0.11% | 0.15% | 100% | 98% |
| Overall | -0.01% | 0.01% | 0.00% | 101% | 101% | 0.11% | 0.03% | 0.03% | 101% | 102% |

# Text Changes

The implementation of the proposed method in the draft is as follows.

The proposed texts are drafted based on JVET-N1001 (v9). The following section highlights the change in VVC working draft of each respective test. Additions to the text are highlighted and deletion are strikethrough.

## General changes

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag ) { |  |
| sps_tpm_blending_off_flag | u(1) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| … |  |

sps_tpm_blending_off_flag equal to 1 specifies that triangle partition merge mode disables the blending process applied to the boundary between two triangle partitions inside a coding unit and the blending process is replaced by direct copy. sps_tpm_blending_off_flag equal to 0 specifies that the blending process is always enabled. When not present, the value of sps_tpm_blending_off_flag is inferred to be 0.

#### 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ) { |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if( sps_tpm_blending_off_flag ) |  |
| slice_triangle_blending_off_flag | u(1) |
| } |  |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| slice_qp_delta | se(v) |
| … |  |

slice_triangle_blending_off_flag equal to 1 specifies that triangle partition merge mode disables the blending process applied to the boundary between two triangle partitions inside each coding unit of the current slice and the blending process is replaced by direct copy. slice_triangle_blending_off_flag equal to 0 specifies that the blending process is always enabled in the current slice. When not present, slice_triangle_blending_off_flag is inferred to be 0.

## Disabling blending in TPM

### Method 1 (JVET-O0645)

In addition to the syntax change as suggested in Section 4.1, method 1 applies the following changes.

#### 8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-821)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-822)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-823)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-824)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-825)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-826)

–	If triangle_blending_off_flag is equal to 1, the following applies:

wValue = ( wValue != 4 ) ? ( wValue < 4 ? 0 : 8 ) : ( triangleDir == 1 ? 0 : 8 )	(8-827)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-827)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

### Method 2 (JVET-O0563)

In addition to the syntax change as suggested in Section 4.1, method 2 applies the following changes.

8.5.7 Decoding process for triangle inter blocks

8.5.7.1 General

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the chroma motion vectors mvCA and mvCB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

Outputs of this process are:

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCb of chroma prediction samples for the component Cb,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCr of chroma prediction samples for the component Cr.

Let predSamplesLAL and predSamplesLBL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesLACb, predSamplesLBCb, predSamplesLACr and predSamplesLBCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- The reference picture consisting of an ordered two-dimensional array refPicLNL of luma samples and two ordered two-dimensional arrays refPicLNCb and refPicLNCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X set equal to predListFlagN and refIdxX set equal to refIdxN as input.

- The array predSamplesLNL is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the luma coding block width sbWidth set equal to cbWidth, the luma coding block height sbHeight set equal to cbHeight, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvN and the reference array refPicLXL set equal to refPicLNL, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 0 as inputs.

- The array predSamplesLNCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / 2, the coding block height sbHeight set equal to cbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCb set equal to refPicLNCb, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 1 as inputs.

- The array predSamplesLNCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / 2, the coding block height sbHeight set equal to cbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCr set equal to refPicLNCr, the variable bdofFlag set euqal to FALSE, and the variable cIdx is set equal to 2 as inputs.

- The partition direction of merge triangle mode variable triangleDir is set equal to merge_triangle_split_dir[ xCb ][ yCb ].

- The prediction samples inside the current luma coding block, predSamplesL[ xL ][ yL ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth, the coding block height nCbH set equal to cbHeight, the sample arrays predSamplesLAL and predSamplesLBL, the MVa and MVb set equal to mvA and mvB and the variables triangleDir, and cIdx equal to 0 as inputs.

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC ][ yC ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesLACb and predSamplesLBCb, and the MVa and MVb set equal to mvCA and mvCB, and the variables triangleDir, and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC ][ yC ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesLACr and predSamplesLBCr, the MVa and MVb set equal to mvCA and mvCB, and the variables triangleDir, and cIdx equal to 2 as inputs.

- The motion vector storing process for merge triangle mode specified in clause 8.5.7.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the partition direction triangleDir, the luma motion vectors mvA and mvB, the reference indices refIdxA and refIdxB, and the prediction list flags predListFlagA and predListFlagB as inputs.

8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

- a variable MVa specifying the motion vector of the prediction block A

- a variable MVb specifying the motion vector of the prediction block B

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-820)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-821)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-822)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-823)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-824)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

if triangle_blending_off_flag is equal to 1, the followings apply:

The variable wOffset is derived as

wOffset = (Max(Abs(MVa[0]),Abs(MVa[1])) <= Max(Abs(MVb[0]),Abs(MVb[1]))) ? 0.5 : -0.5

wValue = ( cIdx == 0 ) ? Clip3(0, 8, (wIdx+wOffset-4)*16) : Clip3(0, 8, (wIdx + wOffset-2)*16)

Otherwise (triangle_blending_off_flag is equal to 0),

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-825)

–	The prediction sample values are derived as follows

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 ) 	(8-826)

### Method 3 (JVET-O0513)

In addition to the syntax change as suggested in Section 4.1, method 3 applies the following changes.

#### 8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-821)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-822)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-823)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-824)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-825)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

- The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

- if triangle_blending_off_flag is equal to 1,

- if y < (nCbH >>1) :

wValue = ( cIdx = = 0 )  ? (wIdx <= 4 ? 0 : 8) : (wIdx * 2 <= 4 ? 0 : 8)	(8-826)

else :

wValue = ( cIdx = = 0 )  ? (wIdx < 4 ? 0 : 8) : (wIdx * 2 < 4 ? 0 : 8)	(8-826)

- Otherwise (triangle_blending_off_flag is equal to 0),

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-826)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-827)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

## TPM storage

### Method A (adopted in Göteborg)

In addition to the syntax change as suggested in Section 4.1, and the changes from the methods 1 or 2 or 3, method A applies the following changes.

#### Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-812)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-813)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-814)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-815)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-816)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-817)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-818)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-819)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-820)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-821)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-822)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-823)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-824)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(8-825)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(8-826)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-827)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-828)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-829)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-830)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-831)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-832)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-833)

predFlagL1 = 1		(8-834)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-835)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-836)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-837)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-838)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-839)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-840)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-841)

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-842)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-843)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-844)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-845)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-846)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-847)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-848)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-849)

predFlagL1 = 1		(8-850)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-851)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-852)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-853)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-854)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-855)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-856)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-857)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-858)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-859)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-860)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-861)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(8-862)

#### Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

The variable refPicPoc is derived as follows:

refPicPoc = ( X = = 0 )  ?  RefPicList[ 0 ][ refIdxN ]  :  RefPicList[ 1 ][ refIdxN ]	(8-863)

The reference picture list refPicListTemp is derived as follows:

refPicListTemp = ( X = = 0 )  ?  RefPicList[ 1 ]  :  RefPicList[ 0 ]	(8-864)

The variable refIdxTemp is derived as follows:

- The variable mapStop is set equal to FALSE.

- For the variable refIdxm with m = 0..NumRefIdxActive[ 1 ] − 1, the following applies until mapStop is equal to FALSE:

refIdxTemp = ( refPicListTemp[ refIdxm ] = = refPicPoc )  ?  refIdxm  :  −1	(8-865)

mapStop = ( refIdxTemp != −1 )  ?  TRUE  :  FALSE	(8-866)

### Method B (JVET-O0563)

In addition to the syntax change as suggested in Section 4.1, and the changes from the methods 1 or 2 or 3, method B applies the following changes.

#### 8.5.7.3 Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-812)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-813)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-814)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-815)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-816)

- The varibale storeFromA is derived as follows

If slice_tpm_blending_off _flag is equal to 1,

storeFromA = (Max(Abs(MVa[0]),Abs(MVa[1])) <= Max(Abs(MVb[0]),Abs(MVb[1])) ? 1 : 0

otherwise

storeFromA = -1

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, or storeFromA is equal to 1, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-817)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-818)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-819)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-820)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-821)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-822)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-823)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-824)

- Otherwise, if sType is equal to 1, or storeFromA is equal to 0, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(8-825)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(8-826)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-827)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-828)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-829)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-830)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-831)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-832)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-833)

predFlagL1 = 1		(8-834)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-835)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-836)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-837)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-838)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-839)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-840)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-841)

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-842)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-843)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-844)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-845)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-846)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-847)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-848)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-849)

predFlagL1 = 1		(8-850)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-851)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-852)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-853)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-854)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-855)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-856)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-857)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-858)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-859)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-860)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-861)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(8-862)

#### Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

The variable refPicPoc is derived as follows:

refPicPoc = ( X = = 0 )  ?  RefPicList[ 0 ][ refIdxN ]  :  RefPicList[ 1 ][ refIdxN ]	(8-863)

The reference picture list refPicListTemp is derived as follows:

refPicListTemp = ( X = = 0 )  ?  RefPicList[ 1 ]  :  RefPicList[ 0 ]	(8-864)

The variable refIdxTemp is derived as follows:

- The variable mapStop is set equal to FALSE.

- For the variable refIdxm with m = 0..NumRefIdxActive[ 1 ] − 1, the following applies until mapStop is equal to FALSE:

refIdxTemp = ( refPicListTemp[ refIdxm ] = = refPicPoc )  ?  refIdxm  :  −1	(8-865)

mapStop = ( refIdxTemp != −1 )  ?  TRUE  :  FALSE	(8-866)

# References

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1010, Mar. 2019.

- X. Xu, Y.-H. Chao and et. al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, Mar. 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1001, Mar. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.